| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G1026 WG11 Number: m22807 |
| --- | --- |

| Title: | Using Multiple APSs for Quantization Matrix Parameters Signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ming LI Ping WU No. 50 Ruanjiandadao Road, ZTE R&D Center District II, Nanjing, China 210012 Junichi Tanaka Yoshitaka Morigami Teruhiko Suzuki Kazushi Sato 2-10-1 Osaki, Shinagawa-ku Tokyo, 141-8610, Japan | Tel: Email: | +86-25-88014641 li.ming42@zte.com.cn ping.wu@zte.com.cn +81-50-3750-2745 tanaka.junichi@jp.sony.com Yoshitaka.Morigami@jp.sony.com teruhikos@jp.sony.com Kazushi.Sato@jp.sony.com |
| Source: | ZTE Corporation, Sony Corporation |  |  |

_____________________________

# Abstract

In this document, Quantization matrix is proposed to be signaled in adaptation parameter set (APS). In order to extend the existing APS to code the Quantization matrix as well as the SAO and ALF parameters, a multiple adaptation parameter sets (APSs) referring approach is proposed to save bits from coding repeated tool parameters.

# Introduction

Similar to SAO and ALF, Quantization matrix parameters can also be changed picture by picture, so that APS [1] has been considered as the right place to signal them.

Although APS provides an effective approach to achieve slice level common information sharing, the approach that one slice can only refer to a single APS for the parameters of Quantization matrix, SAO and ALF could bring a problem of duplicating of the coded APS information. To tackle this problem, a new slice layer APS referring method is proposed, which is a combined re-design based on contributions JCTVC-G332 [2] and JCTVC-G295 [3]. This method allows a slice to refer to multiple APSs. The decoder parses the referred APSs to get the presented tool parameters (including the ON/OFF flag and matrix or filter parameters) of Quantization matrix, SAO and ALF from the referred APSs, then combines the obtained parameters together and activates them to initialize the tools for decoding the current slice, as illustrated in Figure 1.

Figure 1. Example of slice layer multiple APSs referring

Note for encoder operation: inside all APSs with aps_id[i], {i = 0,…N}, for each of the present flag type, e.g. aps_quantization_matrix_data_present_flag, aps_sample_adaptive_offset_data_present_flag, and aps_adaptive_loop_filter_data_present_flag, can only be equal to “1” once for avoiding confusion on which set of parameter data (referred to through different aps_id[i]) should be activated and applied. If there is such a confusion, the decoder operating rule was not defined.

# Modified Syntax and Semantics

The modifications of syntax and semantics for the proposed design are listed in the table below.

Table 1  Modifications Summary

| SPS | Adding one new flag signaling whether Quantization matrix is enabled. |
| --- | --- |
| APS | Adding three new elements signaling whether Quantization matrix, SAO, ALF parameters are presented in this APS; Moving SAO, ALF ON/OFF control flags to the “if(aps_sao/alf _data_present_flag)” branches. |
| Slice Header | Adding new syntax elements signaling multiple APSs’ IDs. |

The modified SPS syntax is presented in Table 2, where the added syntax elements are highlighted in yellow bars.

Table 2  Modified SPS Syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| reserved_zero_8bits /* equal to 0 */ | u(8) |
| level_idc | u(8) |
| seq_parameter_set_id | ue(v) |
| …… |  |
| chroma_pred_from_luma_enabled_flag | u(1) |
| quantization_matrix_enabled_flag | u(1) |
| loop_filter_across_slice_flag | u(1) |
| sample_adaptive_offset_enabled_flag | u(1) |
| adaptive_loop_filter_enabled_flag | u(1) |
| …… | u(1) |
| } |  |

The modified APS syntax based on the latest WD document [3] is presented in Table 3, where the added syntax elements are highlighted in yellow bars.

Table 3  Modified APS Syntax

| aps_rbsp( ) { | Descriptor |
| --- | --- |
| aps_id | ue(v) |
| aps_quantization_matrix_data_present_flag | u(1) |
| aps_sample_adaptive_offset_data_present_flag | u(1) |
| aps_adaptive_loop_filter_data_present_flag | u(1) |
| if( aps_sample_adaptive_offset_data_present_flag ¦¦ aps_adaptive_loop_filter_data_present_flag ) { |  |
| aps_cabac_use_flag | u(1) |
| if( aps_cabac_use_flag ) { |  |
| aps_cabac_init_idc | ue(v) |
| aps_cabac_init_qp_minus26 | se(v) |
| } |  |
| } /*If CABAC is removed from coding syntax elements in APS, the above marked lines will be deleted. */ |  |
| if(aps_quantization_matrix_data_present_flag) { |  |
| qmatrix_data_byte_count | u(8) |
| byte_align () |  |
| qmatrix_param() |  |
| byte_align () |  |
| } |  |
| if( aps_sample_adaptive_offset_data_present_flag ) { |  |
| aps_sample_adaptive_offset_flag | u(1) |
| if (aps_sample_adaptive_offset_flag) { |  |
| sao_data_byte_count | u(8) |
| byte_align() |  |
| sao_param( ) |  |
| byte_align() |  |
| } |  |
| } |  |
| if( aps_adaptive_loop_filter_data_present_flag ) { |  |
| aps_adaptive_loop_filter_flag | u(1) |
| if ( aps_adaptive_loop_filter_ flag ) { |  |
| alf_data_byte_count | u(8) |
| byte_align () |  |
| alf_param( ) |  |
| } |  |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

The modified slice header is presented in Table 4, where the added syntax elements are highlighted in yellow bars.

Table 4  Modified slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| lightweight_slice_flag | u(1) |
| if( !lightweight_slice_flag ) { |  |
| slice_type | ue(v) |
| pic_parameter_set_id | ue(v) |
| if( quantization_matrix_enabled_flag ¦¦ sample_adaptive_offset_enabled_flag ¦¦ adaptive_loop_filter_enabled_flag ) { |  |
| number_of_valid_aps | u(v) |
| if (number_of_valid_aps != 0 ) { |  |
| for (i = 0; i < number_of_valid_aps; i++) { |  |
| aps_id[i] | ue(v) |
| } |  |
| } |  |
| } |  |
| …… |  |
| } |  |
| …… |  |
| } |  |

The semantics of the new elements in SPS syntax are presented below.

quantization_matrix_enabled_flag equal to 1 specifies that Quantization matrix process is applied in decoding the transform coefficients.

The semantics of the new elements in APS syntax are presented below.

aps_quantization_matrix_data_present_flag equal to 1 specifies that the Quantization matrix parameters exist in this APS, equal to 0 specifies that the Quantization matrix parameters do not exist in this APS.

aps_sample_adaptive_offset_data_present_flag equal to 1 specifies that the SAO parameters exist in this APS, equal to 0 specifies that the SAO parameters do not exist in this APS.

aps_sample_adaptive_offset_flag equal to 1 specifies that the SAO is on for slices referred to this APS; equal to 0 specifies that the SAO is off for slices referred to this APS. If there is no active APS or the value of aps_sample_adaptive_offset_data_present_flag in this APS equals to 0, the value of aps_sample_adaptive_offset_flag in this APS is inferred to be 0.

aps_adaptive_loop_filter_data_present_flag equal to 1 specifies that the ALF parameters exist in this APS, equal to 0 specifies that the ALF parameters do not exist in this APS.

aps_adaptive_loop_filter_flag equal to 1 specifies that the ALF is on for slices referred to this APS; equal to 0 specifies that the ALF is off for slices referred to this APS. If there is no active APS or the value of aps_adaptive_loop_filter_data_present_flag in this APS equals to 0, the value of aps_adaptive_loop_filter_flag in this APS is inferred to be 0.

The semantics of the new elements in slice header syntax are presented as below.

number_of_valid_aps specifies the number of valid APSs in decoding this slice. The value of number_of_valid_aps shall be in the range of [0, MaxNumberValidAps], inclusively. The value of MaxNumberValidAps is TBD. If number_of_valid_aps does not exist, its value is inferred to be 0. If number_of_valid_aps is equal to 0, it is inferred as no active APSs.

aps_id[i] specifies the IDs of the valid APSs in decoding this slice, where i shall be in the range of [0, number_of_valid_aps-1], inclusively.

# Conclusions

In this document, an extension of APS syntax for Quantization matrix is proposed. In order to facilitate flexible signaling of Quantization matrix, SAO and ALF parameters in APS, a slice layer multiple APSs referring approach is also proposed. The recommendation is to adopt both the extension of Quantization matrix to APS and the slice layer multiple APSs referring approach described in this document to HEVC.

# References

- Stephan Wenger, Jill Boyce, Yu-Wen Huang, Chia-Yang Tsai, Ping Wu and Ming Li, “Adaptation Parameter Set (APS)”, JCT-VC Document JCTVC-F747, July 2011, Torino, Italy.

- Ming Li and Ping Wu, “Multiple Adaptation Parameter Sets Referring”, JCT-VC Document JCTVC-G332, November 2011, Geneva, Switzerland.

- Junichi Tanaka, Yoshitaka Morigami, and Teruhiko Suzuki, “Non-CE4 Subtest3 : Extension of Adaptation Parameter Sets syntax for Quantization matrix”, JCT-VC Document JCTVC-G295, November 2011, Geneva, Switzerland.

- Benjamin Bross, Woo-Jin Han, Jens-Rainer Ohm, Gary J. Sullivan and Thomas Wiegand, “WD4: Working Draft 4 of High-Efficiency Video Coding”, JCT-VC Document JCTVC-F803_d6, July 2011, Torino, Italy.

# Patent rights declaration(s)

ZTE Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).