| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 Nov., 2011 | Document: JCTVC-G1027 WG11 Number: m22808 |
| --- | --- |

| Title: | Non-CE9: Additional test result on LG’s simplification of merge/skip TMVP refidx derivation (JCTVC-G163) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Toshiyasu Sugio1 Shigeru Fukushima2 | Tel: Email: | +81 6 6900 9689 sugio.toshiyasu@jp.panasonic.com fukushima.shigeru@jvckenwood.com |
| Source: | Panasonic Corporation1 JVC KENWOOD Corporation2 |  |  |

_____________________________

# Abstract

This contribution is a report of an additional test result on LG’s simplification SP05 of merge/skip TMVP refidx derivation in JCTVC-G163. Experimental results reportedly showed no loss on average relative to the HM4.0 without the macro setting REF_SETTING_FOR_LD.

- Introduction

The simplified derivation process of merge/skip TMVP refidx was proposed in JCTVC-G163 [1]. It includes six simplified methods (SP01-06), and the following SP05 method was recommended because the process can be simplified without any harm on coding efficiency.

SP05. Use left as default. Fix to 0 only if left is not available.

- If refIdxLXA is not equal to -1, the following applies.

  - refIdxLX = refIdxLXA

- Otherwise, the following applies.

  - refIdxLX = 0

To confirm the affect of this simplification SP05 in the different reference structure, it was conducted to test this simplification SP05 without the macro setting REF_SETTING_FOR_LD.

- Experimental Condition

The software to be tested was provided by LG and its performance relative to HM4.0 without the macro REF_SETTING_FOR_LD was checked under the LDB and LDP test condition described in JCTVC-F900 [2]. The computing platform is Linux 64bits and the executables were compiled on 64-bit Linux with gcc 4.4.3.

- Results

R-D performance and execution time of each experimental result is summarized in Table 1. Detailed result is included in the attached excel sheet. It was confirmed that there was no loss on average relative to the HM4.0 without the macro setting REF_SETTING_FOR_LD. This result is perfectly matched with the one provided by cross checker JVC KENWOOD.

Table 1: Experimental result of JCTVC-G163 SP05 compared to HM4.0 w/o REF_SETTING_FOR_LD

- Conclusion

The result of LG’s simplification SP05 of merge/skip TMVP refidx derivation in JCTVC-G163 was reported. It was confirmed that there was no loss on average relative to the HM4.0 without the macro setting REF_SETTING_FOR_LD.

- References

- Yongjoon Jeon et al., “Non-CE9: simplification of merge/skip TMVP ref_idx derivation”, JCTVC-G163, Geneva, November 2011.

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-F900, Torino, July 2011.