| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G1031 |
| --- | --- |

| Title: | Support of varying QP in deblocking |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Geert Van der Auwera Xianglin Wang Marta Karczewicz 5775 Morehouse Dr San Diego, CA 92121 USA Matthias Narroschke Anand Kotra Thomas Wedi Panasonic R&D Center Germany Monzastr. 4c, 63225 Langen, Germany | Email: | geertv@qualcomm.com Matthias.Narroschke@eu.panasonic.com |
| Source: | Qualcomm Inc., Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution provides the working draft changes and the corresponding software to support the varying QP capabilities in the deblocking filter, which were recommended to have during the discussions in Track A. All changes are based on JCTVC-G640 and JVTVC-G384.

# Introduction

The QP values QPP and QPQ associated to the samples of each side of an edge may differ.

The following section shows recommended modifications to the deblocking filter necessary to support this varying QP capability. These recommended modifications were derived from the contributions [2] and [3].

# Modifications to support varying QP

The values β and tC for the current edge to be processed (on/off decision, strong/weak decision, tC based clipping) are derived based on:

QPL = ( ( QPP + QPQ + 1 ) >> 1 )

In case of the luma weak filter decisions on each side of the edge for modifying one or two samples, the QP that is associated to the CU of the respective edge side is used to derive the value β.

ThrP = ( β(QPP) + (β(QPP)>>1) )>>3

IF ( dp < ThrP ) THEN p1’ = p1 + p

ThrQ = ( β(QPQ) + (β(QPQ)>>1) )>>3

IF ( dq < ThrQ ) THEN q1’ = q1 + q

# Conclusion

It is proposed to adopt the described modifications to the deblocking filter to support the varying QP capability.

# References

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “WD4: Working Draft 4 of High-Efficiency Video Coding,” 6th JCT-VC Meeting, Turin, Italy, July 2011, Doc. JCTVC-F803.

- A. Kotra, M. Narroschke, T. Wedi, “Deblocking bug fix for CU-Varying QP’s and IPCM blocks,” 7th JCT-VC Meeting, Geneva, CH, November 2011, Doc. JCTVC-G640.

- G. Van der Auwera, X. Wang, M. Karczewicz, “Varying QP Deblocking,” 7th JCT-VC Meeting, Geneva, CH, November 2011, Doc. JCTVC-G384.

# Working Draft Changes

This section describes the working draft changes for luma. The changes for chroma conform to the luma changes.

8.6.1.4.1 Decision process for luma block edge

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top left luma sample of the current coding unit,

–	a variable verticalEdgeFlag,

–	a variable bS specifying the boundary filtering strength,

Output of this process is:

–	variables dE, dEp1 and dEq1 containing decisions,

–	one-dimensional array of size (8), dS containing decisions.

Let s’ represent the luma sample array recPictureL of the current picture.

Depending on verticalEdgeFlag, the following applies:

–	If verticalEdgeFlag is equal to 1, the following ordered steps apply:

- The sample values pi,k and qi,k with i = 0..3 and k = 2,5 are derived as follows:

qi,k = s’[ xC + xB +i, yC + yB + k ]		(8-1)

pi,k = s’[ xC + xB – i – 1, yC + yB + k ]		(8-2)

- The quantization parameter QPP of the neighboring block is assigned the QPY value of the coding unit containing sample p0,0

- The loop filter quantization parameter QPL is derived as follows:

QPL = ( ( QPY + QPP + 1) >>1 )

- A variable β is specified as Error: Reference source not found with loop filter quantization parameter QPL as input.

- A variable βP is specified as Error: Reference source not found with quantization parameter QPP as input.

- A variable βQ is specified as Error: Reference source not found with luma quantization parameter QPY as input.

- A variable tC is specified as follows:

  - If bS is greater than 2, the variable tC is specified as Error: Reference source not found with quantization parameter Clip3(0, 55, QPL + 2 ) as input,

  - Otherwise (bS is equal or less than 2), the variable tC is specified as Error: Reference source not found with quantization parameter QPL as input.

- The variables dp, dq and d are derived as follows:

dp = | p2,2 – 2*p1,2 + p0,2 | + | p2,5 – 2*p1,5 + p0,5 |		(8-3)

dq = | q2,2 – 2*q1,2 + q0,2 | + | q2,5 – 2*q1,5 + q0,5 |		(8-4)

d = dp + dq		(8-5)

- The variables dE, dEp1 and dEq1 are set equal to 0.

- If bS is not equal to 0 and d is less than β, the following ordered steps apply:

- for each sample location ( xC + xB, yC + yB + k ), k = 0..7, the following ordered steps apply:

  - The decision process for a luma sample specified in subclause Error: Reference source not found is invoked with sample values pi,k, qi,k with i = 0..3, the boundary filtering strength bS and the variables d, β and tC as inputs and a decision dSam as output.

  - The variable dS[k] is set equal to dSam

- The variable dE is set equal to 1.

- If dp is less than ( βP + ( βP >> 1 ) ) >> 3, the variable dEp1 is set equal to 1.

- If dq is less than ( βQ + ( βQ >> 1 ) ) >> 3, the variable dEq1 is set equal to 1.

–	Otherwise (verticalEdgeFlag is equal to 0), the following ordered steps apply:

- The sample values pi,k and qi,k with i = 0..3 and k = 2,5 are derived as follows:

qi,k = s’[ xC + xB +k, yC + yB + i ]		(8-6)

pi,k = s’[ xC + xB +k, yC + yB – i – 1 ]		(8-7)

- The quantization parameter QPP of the neighboring block is assigned the QPY value of the coding unit containing sample p0,0

- The loop filter quantization parameter QPL is derived as follows:

QPL = ( ( QPY + QPP +1 ) >> 1 )

- A variable β is specified as Error: Reference source not found with loop filter quantization parameter QPL as input.

- A variable βP is specified as Error: Reference source not found with quantization parameter QPP as input.

- A variable βQ is specified as Error: Reference source not found with quantization parameter QPY as input.

- A variable tC is specified as follows:

  - If bS is greater than 2, the variable tC is specified as Error: Reference source not found with quantization parameter Clip3(0, 55, QPL + 2 ) as input,

  - Otherwise (bS is equal or less than 2), the variable tC is specified as Error: Reference source not found with loop filter quantization parameter QPL as input.

- The variables dp, dq and d are derived as follows:

dp = | p2,2 – 2*p1,2 + p0,2 | + | p2,5 – 2*p1,5 + p0,5 |		(8-8)

dq = | q2,2 – 2*q1,2 + q0,2 | + | q2,5 – 2*q1,5 + q0,5 |		(8-9)

d = dp + dq		(8-10)

- The variables dE, dEp1 and dEq1 are set equal to 0.

- If bS is not equal to 0 and d is less than β, the following ordered steps apply:

- For each sample location ( xC + xB + k, yC + yB ), k = 0..7, the following ordered steps apply:

  - The decision process for a luma sample specified in subclause Error: Reference source not found is invoked with sample values pi,k, qi,k with i = 0..3, the boundary filtering strength bS and the variables d, β and tC as inputs and a decision dSam as output.

  - The variable dS[k] is set equal to dSam.

- The variable dE is set equal to 1.

- If dp is less than ( βP + ( βP >> 1 ) ) >> 3, the variable dEp1 is set equal to 1.

- If dq is less than ( βQ + ( βQ >> 1 ) ) >> 3, the variable dEq1 is set equal to 1.

# Patent rights declaration

Qualcomm Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).