| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G1032 WG11 Number: m22816 |
| --- | --- |

| Title: | BoG report on intra prediction complexity reduction and filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Rajan Joshi (chair) 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Email: | rajanj@qualcomm.com Tel. +1-858-658-4511 |
| Source: | Break-out group |  |  |

_____________________________

# Abstract

Break-out group meetings for intra prediction complexity and filtering were held on Saturday, Sunday and Monday, Nov. 24–26, 2011. Approximately 25 delegates were present.

# Madates

The mandates of the BoG were:

- Review proposals related to intra prediction complexity reduction and  filtering

- Assess the rate-distortion performance and complexity of the proposals

- Make recommendations for adoption into WD/HM

# Recommendations

The following proposals were discussed by the break out group:

- JCTVC-G447 Reduced number of intra 64x64 prediction mode [K. Sugimoto, A. Minezawa, S. Sekiguchi (Mitsubishi)]

- JCTVC-G449 Reduced number of intra chroma prediction mode [K. Sugimoto, A. Minezawa, S. Sekiguchi (Mitsubishi)]

- Simplified DC prediction [J. Lainema, K. Ugur (Nokia)]

- JCTVC-G139 Non-CE6.d: Intra Prediction With Selective Secondary Boundary [G. Van der Auwera, M. Karczewicz (Qualcomm)]

- JCTVC-G373 Non-CE6: Performance of secondary boundary DC intra prediction [C. Auyeung (Sony)

- JCTVC-G443 Improved directional intra prediction smoothing [A. Minezawa, K. Sugimoto, S. Sekiguchi (Mitsubishi)]

- JCTVC-G119 Modifications to intra frame coding [Y. Lin (Hisilicon), H. Yang(Huawei), L. Liu, J. Zheng(Hisilicon), H. Yu(Huawei)]

G447: Proposes only 1 mode at 64x64 block size, either DC or planar. It was commented that in case of multiple modes, the encoder may choose to check only one mode. The additional bits needed to signal are not a big issue at 64x64 block size. The impact on coding efficiency of the proposed is very small. There was some concern that for different testing conditions and higher resolutions, more coding efficiency loss may be observed. It was agreed that the current design of having only 6 modes for 64x64 intra prediction is not very elegant. In track A, there was another suggestion (from Frank Bossen) that 64x64 should use 35 modes. For JCTVC-G447, when only planar mode is used the BD-rates in AI case are 0.0%, 0.1%, and 0.1%. The group recommends that either 35 modes or a single mode (planar) be adopted for 64x64 block size.

G449: Proposes to remove DC (or planar) mode from the allowed explicit modes for intra chroma modes. If the mode is not a derived mode and there is an overlap between the luma mode and one of the explicit modes, that mode is replaced by the DC (or planar) mode. Proponents claimed encoder complexity reduction and getting rid of diagonal mode as benefits. Proponents do not claim any advantage (simplification) on the decoder side. It was commented that it may be dangerous to remove the DC mode. In certain hardware scenarios, after an MTU slice boundary is enforced, the DC mode is used to re-encode some blocks. It was commented that because of the derived modes, the encoder may have to search through all the chroma modes any way. There was a concern about visual aspects for removing either DC or planar.

G567: Proposes simplification of the DC prediction. The DC prediction value is calculated using every 4th reference pixel. There was no impact on compression efficiency. The maximum BD-rate loss is 0.1% for WVGA RaceHorses. The proponents claimed reduction in complexity. It was commented that DC mode may be the least complex. The proponents noted that DC has additional calculations before prediction. The proponents further noted that there may be an additional benefit in software SIMD environment because only 4th line needs to be accessed for calculating the DC prediction value.

One of the concerns expressed was that this may reduce the number of non-border 4x4 blocks encoded in the DC mode. For encoders not using RDO, this may result in drop in coding efficiency. Proponents provided additional results showing that the number of blocks using the DC mode decreased by about 6% (from 9.2% to 8.7% of all blocks). Comparing the percentages of pixels using the DC mode, it appears that the use of DC mode for lower block sizes is affected more.

One additional suggestion was to always use 8 pixels (4 horizontal and 4 vertical) for calculating DC prediction. The proponents commented that it was a good idea and we should test this in CE6.

The group recommends that G567 and the proposed modification should be tested in a CE.

G139: Proposes introducing a check to avoid secondary boundary filtering in areas with strong edges. This proposal is dependent on one of the unified proposals (test 10 or test 13 from CE 6.d) getting adopted. One of the unified solutions (test 13) for secondary boundary had significant loss for some class F sequences (0.5% for ChinaSpeed and 1.0% for SlideEditing). This proposal attempts to fix this issue.

It was commented that the loss was much smaller (0.2% ChinaSpeed and 0.0% for SlideEditing) for test 10. It was commented that other sequences and classes show consistent gains, so there is some room for improvement of the unified solution (test 10). It was also remarked that class F sequences are not mandatory and that this proposal did not provide any benefit for classes A to E.

It was queried whether the relative drop in performance for certain class F sequences is due to intra prediction or secondary boundary filtering? It was commented that if we specifically want to improve class F, we may have other tools (such as SDIP) available which provide much larger improvement. It was noted that the comment is not directly related to this discussion. Even with adoption of other tools, this problem may still be present.

It was queried whether it would be possible to disable secondary filtering with a flag, possibly at the SPS level?

G373: This proposal reports the gain from secondary boundary prediction for DC mode with respect to the unified solution (test 12 from CE6.d). The reported gain from secondary boundary prediction for DC mode is about 0.04% when tested on top of test 12. It is conjectured that it may have similar gains with other unified solutions in consideration (tests 10 and 13).

Proponents commented that the document was more for informational purposes and suggested that the secondary boundary prediction for DC mode could be removed from a low complexity profile. It was commented that removal of secondary boundary filtering for the DC mode was not a significant reduction in complexity. It was commented that in test 12, secondary boundary prediction is used for other intra modes as well. So removing it just from the DC mode does not make sense. Proponents are not strongly advocating removal from Low complexity profile.

G443: Proposes improved directional intra smoothing. This solution is implemented on top of unified solution 13 from CE6.d. Only the reference sample direction is changed for intra prediction angle +-13 and +-17. Very small luma gain is observed (0.01-0.02)%. In Test 13, a number of diagonal modes were processed in an identical manner. Since with this proposal, the processing is split into 2 groups. Hence some additional logic needed. Crosschecker commented that it was not worth adding the extra logic to achieve the small gain.

G119 (part 3): Part 3 of G119 has two parts. In the first part, for all block sizes, two diagonal modes (hor+8 and vert+8) are combined into a single mode. In the second part, one additional mode is added for 4x4 blocks. The BD-rate gain is 0.1% for all classes. In case of 8x8 and higher, the number of modes is reduced by 1. The proponents commented that this simplifies remaining mode coding by eliminating the need for an escape mode. In case of 4x4, the number of modes is the same.

It was queried as to what would happen if we just dropped hor+8 (or vert+8) for 8x8 and higher blocks? Proponents mentioned that there would be a loss, but there was no data available.

It was queried separate BD-rate numbers were available for only adding combining the diagonal modes (without adding the extra mode in 4x4 case). Proponents provided additional results to show that when in that case, there is no loss with respect to HM4.0.

It was commented that the gain was small for the amount of change. It was agreed that the complexity after combining the modes would be comparable to other modes.

There was concern regarding the visual impact of removing the original hor+8 and vert+8 modes. It was commented that potentially, there would be blockiness on both boundaries instead of only one. For the combined mode, the proposal uses the same transform as used for vert+8. For 4x4 blocks, HM4.0 uses a different transform for hor+8 and vert+8 modes, which may take care of blockiness on one boundary more effectively. It was also commented that there is a possible interaction with secondary boundary filtering solutions under consideration.

Proponents reported that they observed no visual artifacts.

The group recommends that part 3 of G119 should be tested in a CE.