| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G1037 |
| --- | --- |

| Title: | Non-CE6: Crosscheck of JCTVC-G419 with alpha7bit bugfix |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kazushi Sato Sony City Osaki 9F 2-10-1 Osaki, Shinagawa, 141-8610 Tokyo, Japan | Tel: Email: | +81 50 3750 2741 Kazushi.Sato@jp.sony.com |
| Source: | Sony Corp. |  |  |

_____________________________

# Abstract

This document reports cross-check result of JCTVC-G419 with alpha7bit bugfix.

# Introduction

In current HM specification second left pixels are used for alpha&beta calculation with luma-based chroma intra prediction.

JCTVC-G419 proposes not using second left pixels but first left ones [4]. Merit of this modification is that reference sample padding method will be unified.

In JCTVC-G419 it is evaluated how much does this modification impact on coding efficiency. Original HM-4.0 contains bug on bit accuracy for alpha calculation, and the bugfixed software has been provided to CE6 participants. The proposed method should be evaluated based on this software.

This document provides cross-check result of this modification with this bugfixed software.

# Simulation Condition

HM-4.0 with bugfix (alpha-7bits) is used both for anchor. On top of this anchor the proponent implemented the proposed method and distributed to the author of this document. Source code has been studies and confirmed to be implemented correctly.

To enable the proposed method the following macro in “Typedef.h” shall be set:

# define G419_LM 1

IT condition as described in [3] is tested with class A to E sequences.

# Simulation Result

Summary of the results are shown in Table 3.1.

Complete result is shown in JCTVC-G1037.xls.

The result provided by the proponent has been verified.

Table 3.1

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.1% | 2.3% | 2.0% |
| Class B | 0.1% | 0.4% | 0.2% |
| Class C | 0.2% | 0.8% | 1.0% |
| Class D | 0.1% | 0.4% | 0.4% |
| Class E | 0.0% | 0.2% | 0.2% |
| Class F |  |  |  |
| Overall | 0.1% | 0.8% | 0.8% |
|  | 0.1% | 0.8% | 0.8% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 98% |  |  |

# Conclusion

The result of JCTVC-G419 has been verified.

The proposed method enables unification of reference pixel padding process with LM_CHROMA mode and other modes, but average loss by 0.8% is observed for U&V components.

The author encourages to study method to compensate this loss.

# References

[1] “WD4: Working Draft 4 of High-Efficiency Video Coding”, JCTVC-F803, Jul 2011

[2] “CE6.a.4: Chroma intra prediction by reconstructed luma samples”, JCTVC-E266, Mar 2011

[3] “Common test conditions and software reference configurations”, JCTVC-F900, Jul 2011

[4] “Inconsistency of intra LM mode between HM and WD”, JCTVC-G419, Nov 2011

# Patent rights declaration(s)

Sony Corp. does not have any current or pending patent rights relating to the technology described in this contribution.