| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G1038_r1 |
| --- | --- |

| Title: | JCT-VC break-out report: Harmonization of NSQT with residual coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Joel Sole Qualcomm | Tel: Email: | +1-858-845-2429 joels@qualcomm.com |
| Source: | Break out Group |  |  |

_____________________________

# Abstract

A break-out group meeting was held Monday Nov. 28 2011 from 11:00 through 13:00. Approximately 20 delegates were present. A second meeting was held on Tuesday Nov. 29 2011 from 10.00 through 11:30. Approximately 12 delegates were present. A straw man solution for NSQT harmonization is recommended.

# Mandates

The mandates of the BoG are:

- Study the interaction between the contexts for residual coding in JCTVC-G1015 and the sub-block scans in JCTVC-G323 with NSQT

  - Analyze possible issues (including  transpose blocks and remapping)

  - Recommend harmonization if needed

    - 4 proposals on the topic: JCTVC-G123, JCTVC-G517, JCTVC-G724, JCTVC-G750

- Generate a straw man solution for the NSQT harmonization with residual coding

# Discussion

With respect to the current remapping:

Discussion on that direct integration of G1015 with the current design HM4.0 appears to have no additional issues with respect to NSQT (interaction with 8x8 and 16x16/32x32).

Discussion on that direct integration of G323 with the current design HM4.0 appears to have no additional issues with respect to NSQT (no interaction with 16x4 and 4x16, interaction only in the 32x8 and 8x32 that map to the 16x16)

Agreement that the remapping from non-square to square blocks should be removed.

Proposals (G123, G517, G724, G750) removing remapping have interaction with G1015 (interaction with 8x8 and 16x16/32x32) and G323.

There is a CE established on NSQT harmonization with entropy coding.

Request confirmation that bug fix 214 will be in HM5.0.

Straw man solution

Use the coding method of 16x16 and 32x32 and apply it to the non-square blocks.

No new contexts are added.

- Scans

  - Use 4x4 sub-block scan process for all non-square block sizes (16x4, 4x16, 32x8, 8x32)

    - Diagonal scan within sub-blocks

    - Diagonal scan across sub-blocks

  - This is scanning method of JCTVC-G323 for 16x16 and 32x32

- Last significant coefficient coding

  - Signal the X and Y according to the width and height of block

  - Contexts according to width and height shared with square block of corresponding width and height

- Significance map

  - Use the 2-level significance map in JCTVC-G644

    - Rules and context for significant_coeffgroup_flag are the same

    - Contexts are shared

  - Contexts for coefficients are shared with 16x16 and 32x32

    - Context to determine the low frequency depends on a threshold based on X + Y

      - 16x4 and 4x16: threshold is set to 4

      - 32x8 and 8x32: threshold is set to 8

      - Note: the threshold is the max of width and height divided by 4

        - Threshold=Max(width, height)>>2

  - Context derivation of corner cases in JCTVC-G323 (top-left corner coefficient and coefficient above the bottom-right corner in a sub-block)

- Coefficient level

  - No interaction with NSQT

Example 16x4 (width 16, height 4) for last significant coding:

- For X it is signaled as it would be done for X in a 16x16 block

  - Use the width contexts

- For Y it is signaled as it would be done for Y in a 4x4 block

  - Use the height contexts