| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, 21-30 November, 2011 | Document: JCTVC-G1041_r1 WG11 Number: m22837 |
| --- | --- |

| Title: | BoG report on subjective viewing test for deblocking filter proposals |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Andrey Norkin Matthias Narroschke Kenneth Andersson David Flynn Xun Guo Geert van der Auwera | Email: | andrey.norkin@ericsson.com matthias.narroschke@eu.panasonic.com kenneth.r.andersson@ericsson.com davidf@rd.bbc.co.uk xun.guo@mediatek.com geertv@qualcomm.com |
| Source: | BoG on subjective viewing |  |  |

_____________________________

This contribution is a report on informal subjective viewing for the deblocking filtering that was held during the Geneva meeting on November 26, 2011. The subjective viewing was conducted according to the mandate of core experiment 12 on deblocking filtering F912. The goal of informal subjective viewing test was to determine how the deblocking filter proposals affect the subjective quality. In total, ten proposals from CE12 and were evaluated in the subjective test. There we two different tests

# Test procedure

There were thirteen proposals tested and the anchor, in total 9 different proposals. Both the CE and non-CE proposals have been shown in one test. The proposals in each test session were evaluated by a group of 4-5 experts. In total, three viewing sessions have been held. The identities of the proposals and the anchor were hidden (the test subjects were shown a label from P0 to P9).

Each session consisted of two tests, a normal test and a high-QP test.

The following six sequences have been used in the normal test.

Sequence 1: BQMall, low-delay low-complexity, QP 32
Sequence 2: BQMall, random-access high-efficiency, QP 37 
Sequence 3: Kimono, low-delay low-complexity, QP 32
Sequence 4: Kimono, random-access high-efficiency, QP 37 
Sequence 5: Vidyo3, low-delay low-complexity, QP 32
Sequence 6: Vidyo3, low-delay high-efficiency, QP 37

The sequences were encoded according to common test conditions E900 [2].

Proposals from CE12.5 have also participated in the high-QP test. The following sequences have been tested in the high-QP test:

Sequence 1: BasketballDrill, random-access low-complexity, QP 37 
Sequence 2: RaceHorses, low-delay high-efficiency, QP37
Sequence 3: Cactus, low-delay high-efficiency, QP37
Sequence 4: ParkScene, all-intra high-efficiency QP37

The test subjects were given score sheets and asked to grade each proposal with a grade from 1 to 10. The test procedure was as follows. The test subjects were given instructions first. The proposals were shown in a randomized order one after another with a five-second break for test 1 and ten-second break for test 2 voting after each test sequence. The test subjects were asked to grade the sequence on a 1 to 10 scale with the average quality ranged in a level of 5. The order of the sequences was randomized for each sequence and each test.

The length of one session was about 25 minutes. In total, 14 test subjects participated in scoring. The test environment was set-up by David Flynn and Kenneth Andersson. The test sessions were conducted by CE12 coordinators Andrey Norkin and Matthias Narroschke, and processing of score sheets and counting the test results was done by Kenneth Andersson, Geert van der Auwera, Xun Guo and Andrey Norkin.

# Test equipment

The test was conducted with the test program and scripts provided by David Flynn and Kenneth Andersson. A WXQGA (2560x1600@60Hz) monitor was used with a 1920x1200 computer output.  The monitor was configured to display the lower resolution in a pixel mapped manner, surrounding it with black boarders.  Video playback was performed using the BBC's video player software, presenting pixel mapped video with a mid-grey surround when sub 1920x1200 video was displayed.  Non-60Hz material was displayed using an appropriate frame repetition period.

# Proposals

The following CE12 proposals have been tested in the subjective test.

- [JCTVC-G409, Microsoft] CE12, Subset 1: Report of Deblocking for Large Size Blocks

- [JCTVC-G287, MediaTek, Ericsson] CE12 Subtest 1: Simplified Chroma Deblocking Filter in JCTVC-G286

- [JCTVC-G383, Qualcomm] CE12 Subtest 1: Deblocking of New Non-Square Blocks: Edge Shift for AMP

- [JCTVC-G590, Panasonic] CE12 Subtest 1: Results for modified decisions for deblocking

- [JCTVC-G229, MediaTek] CE12.3.3: Reducing motion data line buffers

- [JCTVC-G174, Toshiba, Qualcomm] CE12: Deblocking filter parameter adjustment in slice level

- [JCTVC-G466, Mitsubishi Electric] CE12: results on signaling of boundary filtering strength

- [JCTVC-G574, Panasonic] CE12 Subtest 5: Deblocking filter using adaptive weighting factors.

Proposal G574 has been tested with both CE and non-CE (subjective) encoder control. Proposals G174, G466 and G574 (with two variants of encoder control) have also participated in a high-QP test.

# Problems during test

There was one problem during the test. When testing sequence Vidyo3 LDLC QP32, one of the proposals crashed during the test (it appeared green soon after the start of the sequence). The test subjects were instructed not to grade this sequence. After the test, the problem has been investigated and it has been found that the file with sequence Vidyo3 LDLC QP32 of the proposal G466 has been corrupted. Therefore, the results for proposal G466 are not available for this sequence.

# Test results

The graphs below show the test results. MOS value with overlapping CI values cannot be rated as distinguishable.

## Test 1

HM	     174    229       287       383     409     466    574obj   574subj   590

HM	      174        229        287         383       409      466       574obj   574subj   590

HM	      174        229        287         383       409      466       574obj   574subj   590

HM	      174        229        287         383       409      466       574obj   574subj   590

HM	      174        229        287         383       409      466       574obj   574subj   590

HM	      174        229        287         383       409      466       574obj   574subj   590

## Test 2 (High QPs)

HM	                  174                  466                574obj            574subj

HM	                  174                  466                574obj            574subj

HM	                  174                  466                574obj            574subj

HM	                  174                  466                574obj            574subj

# Conclusions

The conclusion of test results is that the confidence intervals overlap for all sequences in both Tests 1 and Test 2. Therefore it is not possible to claim that one proposal is better than another.

# References

[1]	A. Norkin, X. Guo, B. Jeon, M. Narroschke, “Description of CE12: deblocking filtering,” JCTVC-F912, Torino, July 2011.

[2]	A. Norkin, X. Guo, B. Jeon, M. Narroschke, “CE12: Summary report of Core Experiment on deblocking filtering,” JCTVC-G042, Geneva, November 2011.

[3]	F. Bossen, “Common test conditions and software reference configurations,” JCTVC-G900, Torino, July 2011.