| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G1042_r2 |
| --- | --- |

| Title: | Harmonization of the prediction and partitioning mode binarization of P and B slices |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Simon Oudin Benjamin Bross Y. Piao J. Min J. Park X. Zhang S. Liu S. Lei W.-J. Chien J. Sole M. Karczewicz | Tel: Email: | simon.oudin@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI, Samsung, MediaTek, Qualcomm |  |  |

_____________________________

# Abstract

This document reports a unified solution of two proposed approaches for harmonizing the prediction mode and the partitioning mode binarization of P slices and B slices. The two approaches that have been studied are trying to achieve this harmonization in a different way. One suggests using the P slice scheme for B slices and the other one suggests doing it the other way round. It has been studied which of the two approaches is better in a sense of simplicity for the HM and the Working Draft. Based on that study, a unified solution which uses the P slice binarization for B slices and fixes a bug in HM4.0 in the binarization of partitioning modes for P slices from one of the proposals is presented.

# JCTVC-G785 Proposal

The proposal JCTVC-G785 [1] describes this harmonization using the binarization from P slices for B slices. This harmonization comes along with the change of context initialization values for the prediction mode and the partitioning mode.

Furthermore it was identified that the binarization for B slices in the current WD is wrong.

# JCTVC-G151/G718 Proposals

The proposals JCTVC-G151 [2] and the first part of JCTVC-G718 [3] describe this harmonization using the binarization from B slices B for P slices.

Furthermore, JCTVC-G151 also reports a bug which occurs in the binarization of the partitioning size in Slice P. A fix for this bug is also provided.

# Unified solution

After having seen the code of each approach and the implication of the harmonization in the logic and simplicity of the binarization for these parameters, the preferred solution is using the P slice binarization for B slices. The following advantages have been identified:

- Prediction mode and partition mode can be described as two separate syntax elements.

- The functions for coding prediction mode and partition mode can be implemented clean and separately.

Consequently the bug fix proposed in JCTVC-G151 [2] is applied to the unified solution.

A final, cleaned-up version of this unified solution has been integrated in HM4.0 adding the macro PREDTYPE_CLEANUP. The source code (JCTVC-G1042_source.zip) and the working draft text (JCTVC-G1042_WD.zip) of the unified solution are included in this proposal.

# Simulation Environment and Results

## Software

The binaries were compiled on a 64 bit Ubuntu 10.10 Linux using the included make files and gcc 4.4.5. The coding conditions used for the simulations are specified in [5]. The HM4.0 default settings, both for configuration file and compile-time, are used in this test as anchor.

## Results

The coding efficiency results in this section are given in terms of BD-rate. The values are averaged over all sequences from one class. BD-PSNR, PSNR values and bit rates for every test point can be found in the attached Excel spreadsheet JCTVC-G1042.xls. The encoding and decoding times shown shouldn’t be considered due to tests running on different platforms.

It should be noticed the results for the Table 4-3 and the Low delay P HE case of the Table 4-4 are the same case to the bug fix integrated in the proposed solution. Consequently the results of Table 4-1 and the Random Access HE and Low Delay HE cases of Table 4-4 are slightly different but still very close.

Table 4-1  Harmonization from JCTVC-G785

|  | Random Access HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0,0% | 0,0% | -0,1% |
| Class B | 0,0% | 0,0% | 0,1% |
| Class C | 0,0% | -0,1% | 0,0% |
| Class D | 0,1% | -0,1% | -0,1% |
| Class E |  |  |  |
| Overall | 0,0% | -0,1% | 0,0% |
|  | 0,0% | -0,1% | 0,0% |
| Enc Time[%] | 100,0% |  |  |
| Dec Time[%] | 99,6% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0,0% | -0,1% | 0,1% |
| Class C | 0,0% | -0,1% | 0,0% |
| Class D | 0,1% | 0,2% | 0,3% |
| Class E | 0,0% | -0,3% | 0,5% |
| Overall | 0,0% | -0,1% | 0,2% |
|  | 0,0% | 0,0% | 0,1% |
| Enc Time[%] | 99,9% |  |  |
| Dec Time[%] | 100,1% |  |  |

Table 4-2  Harmonization from JCTVC-G151

|  | Low delay P HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0,0% | 0,2% | 0,3% |
| Class C | 0,0% | 0,1% | -0,1% |
| Class D | 0,0% | -0,1% | -0,1% |
| Class E | 0,0% | 0,0% | 0,1% |
| Overall | 0,0% | 0,0% | 0,0% |
|  | 0,0% | 0,0% | -0,1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

Table 4-3  Bug fix from JCTVC-G151

|  | Low delay P HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0,0% | 0,0% | 0,1% |
| Class C | 0,0% | 0,0% | -0,1% |
| Class D | 0,0% | -0,3% | 0,0% |
| Class E | 0,0% | 0,1% | 0,4% |
| Overall | 0,0% | -0,1% | 0,1% |
|  | 0,0% | -0,1% | 0,1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

Table 4-4  Solution proposed in JCTVC-G1042

|  | Random Access HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0,0% | 0,0% | -0,2% |
| Class B | 0,0% | 0,1% | 0,0% |
| Class C | 0,0% | -0,1% | 0,1% |
| Class D | 0,0% | 0,1% | 0,0% |
| Class E |  |  |  |
| Overall | 0,0% | 0,0% | 0,0% |
|  | 0,0% | 0,0% | 0,0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 90% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0,0% | -0,1% | -0,2% |
| Class C | 0,0% | 0,0% | -0,1% |
| Class D | 0,0% | 0,1% | 0,2% |
| Class E | 0,0% | 0,2% | 0,4% |
| Overall | 0,0% | 0,0% | 0,0% |
|  | 0,0% | 0,0% | 0,0% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 89% |  |  |
|  | Low delay P HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0,0% | 0,0% | 0,1% |
| Class C | 0,0% | 0,0% | -0,1% |
| Class D | 0,0% | -0,3% | 0,0% |
| Class E | 0,0% | 0,1% | 0,4% |
| Overall | 0,0% | -0,1% | 0,1% |
|  | 0,0% | -0,1% | 0,1% |
| Enc Time[%] | 108% |  |  |
| Dec Time[%] | 107% |  |  |

# References

- Y. Piao, J. Min, J. Park, “Unified Pred_type coding in CABAC”, JCTVC-G785, Geneva, November, 2011.

- X. Zhang, S. Liu, S. Lei, “Prediction and partition mode binarization for Low Delay P”, JCTVC-G151, Geneva, November, 2011.

- W.-J. Chien, J. Sole, M. Karczewicz, “Context reduction for CABAC”, JCTVC-G718, Geneva, November, 2011.

- JCT-VC, “HEVC Test Model Software 4.0”, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-4.0

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-F900, Turin, July, 2011.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd., MediaTek Inc. and Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).