| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G1044 |
| --- | --- |

| Title: | BoG report on Quantization |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Madhukar Budagavi Louie Kerofsky | Email: | madhukar@ti.com lkerofsky@sharplabs.com |
| Source: | BoG on Quantization |  |  |

_____________________________

# Abstract

This contribution is a report on complexity analysis of dequantization proposals: JCTVC-G382, JCTVC-G773, JCTVC-G850.

# Introduction

Table below provide a summary of different dequantization techniques studied in comparison with HM4 dequantization.

|  | QP range |
| --- | --- |
| HM4 | 0 … 51 |
| JCTVC-G850 | 0 … 51 |
| JCTVC-G382 | 0 … 51 |
| JCTVC-G773 | 0 .. 103 |

# Flat quantization

## HM4

B       = source bit width (8 or 10 bit in the experiments described below)

DB      = B-8 (internal bit-depth increase with 8-bit input)

N        = transform size

M       = log2(N)

IQ(x)       = g(x),       where g(x)  =   {40,45,51,57,64,72}, x=0,…,5

Dequantizer is specified as follows:

coeffQ   =  ((level*IQ[QP%6] << (QP/6)) + offset)>>(M-1+DB),    offset = 1<<(M-2+DB)

Bitwidth analysis:

- coeffQ   =  ((level*IQ[QP%6] << (QP/6)) + offset

- level is assumed to be 16 b (signed)

- IQ = 7 b

- << QP/6 = 8 b

- Total bitwidth of coeffQ before right shift = 31 b (signed)

## JCTVC-G773

IQ(x)       = g(x),       where g(x)  =   {40, 42, 45, 48, 51, 54, 57, 60, 64, 68, 72, 76}, x=0,…,11

Dequantizer is specified as follows:

coeffQ   =  ((level*IQ[QP%12] << (QP/12)) + offset)>>(M-1+DB),    offset = 1<<(M-2+DB)

Bitwidth analysis:

- coeffQ   =  ((level*IQ[QP%12] << (QP/12)) + offset

- level is assumed to be 16 b (signed)

- IQ = 7 b

- << QP/12 = 8 b

- Total bitwidth of coeffQ before right shift = 31 b (signed)

No additional computations are required when compared to HM4.

## JCTVC-G850

### CE version

A delta value in the range: [-15, 15] is received at slice level in bitstream for each of the Y, U, V components. A table FQPLevel[c][level] for levels in the range for [-4096, 4096] is then calculated using the received delta value: FQPLevel[c][level] = level*(128+delta[c]) ([c] accounts for color component). Each entry of FQPLevel table is 19 bits. This table is populated using recursive addition of (128+delta). The number of operations required for this is: 4096 additions and negations per color component. Note that three different FQPLevel tables are maintained one for each color component.

Dequantizer equations are as follows:

- Clip level to –4096 to 4096

- coeffQ =  ((FQPLevel[c][level]*IQ[QP%6] << (QP/6)) + offset)>>(M+6+DB),  offset = 1<<(M+5+DB)

Bit width analysis:

- level is assumed to be 16 b (signed)

- FQPLevel = 19 b

- IQ = 7 b

- << QP/6 – 8 b

- Total bitwidth of coeffQ before right shift =  19+7+8 = 34 b

Additional TU level complexity when compared to HM4: 2 CMP and 1 LUT per sample.

### New version [Source code released on 11/28/2011]

A delta value in the range: [-15, 15] is received at slice level in bitstream for each of the Y, U, V components. IQ’ tables are calculated at slice level as follows: IQ’[c][QP%6] = ( IQ[QP%6] * (128+delta[c] ) +64) >> 7. Slice level computations for calculating IQ’ tables for each color component are: 7 ADD, 6 MULT. The table size required is: 3*6*7 bits in total.

Dequantizer equations are as follows:

- coeffQ 	=  ((level*IQ’[c][QP%6] << (QP/6)) + offset)>>(M-1+DB),  offset = 1<<(M-2+DB)

[c] account for color component.

Bit width analysis:

- level = 16 bits (signed)

- IQ’ = 7 bits

- << QP/6 = 8 bits

- Total = 31 bits (signed)

No additional computations are required when compared to HM4.

## JCTVC-G382

### CE version

A delta_s value in range of [-28 to 32] and a delta_o value in range of [-28 to 32] is received at slice level in bitstream for each of IntraY, InterY, IntraUV, Inter UV. Two tables IQ_s and IQ_o are calculated for each type as follows:

- IQ_s[c][QP%6] = IQ[QP%6]*(128+delta_s[c])

- IQ_o[c][QP%6] = IQ[QP%6]*delta_o[c]

where [c] ranges from 0 to 3 (IntraY, InterY, IntraUV, Inter UV).

Slice level computations for calculating these tables require 48 MULT and 4 ADD for (128+delta_s).

Dequantizer equations are as follows:

If level[i][j] >= 0

coeffQ[i][j] = ( level[i][j] *( IQ_s[c][QP%6] << (QP/6)) + offset ) >> (M-1+DB+7)

elseif  level[i][j] < 0

coeffQ[i][j] = ( level[i][j] *( IQ_s[c][QP%6] << (QP/6)) – offset ) >> (M-1+DB+7)

where

offset = (1<<(M-2+DB)) +IQ_o[c][QP%6]

Bit width analysis:

- level is assumed to be 16 b (signed)

- IQ_s = 15 b

- << QP/6 = 8 b

- Total bitwidth of coeffQ before right shift =  16+15+8 = 39 b

Computation complexity over HM4 is: 1 CMP per sample and 1 ADD+1 LUT per TU.

### New version [Source code released on 11/28/2011]

A delta_s value in range of [-28 to 32] and a delta_o value in range of [-28 to 32] is received at slice level in bitstream for each of IntraY, InterY, IntraUV, Inter UV. Two tables IQ_s and IQ_o are calculated as follows:

- IQ_s[c][QP%6] = (IQ[QP%6]*(128+delta_s[c])+64)  >> 7

- IQ_o[c][QP%6] = (IQ[QP%6]*delta_o[c]+64) >> 7

where [c] ranges from 0 to 3 (IntraY, InterY, IntraUV, Inter UV).

Slice level computations: IQ_s: 6*4 ADD (factor of 4 is for InterY, InterUV, IntraY, IntraUV)  + 4 ADD ( for 128+delta_s[c]), 6*4 MULT. IQ_o: 6*4 MULT, 6*4 ADD.

Table size: 2*6*4*7bits

Dequantizer equations are as follows:

- coeffQ[i][j] = sgn(level[i][j]) * ( abs(level[i][j]) * IQ_s[c][QP%6] + IQ_o[c][QP%6]) << (QP/6)  >> (M-1+DB)

where sgn() = 1 for positive number, -1 for negative numbers, and 0 for 0.

Bit width analysis:

- abs(level) = 16 bits (unsigned)

- delta_s varies from -28 to 32

- IQ_s = 7 bits

- <<QP/6 = 8 bits

- Total bitwidth = 31 unsigned bits + 1 sign = 32 signed bits

Computation complexity over HM4 is: 1 LUT per TU. Note that explicit calculation of sgn() and abs() not needed as those values come in bitstream.

## Summary

Bit-width summary for flat dequantization:

|  | CE | New ### [Source code released on 11/28/2011] |
| --- | --- | --- |
| HM4 | 31 b | - |
| G850 | 34 b | 31 b |
| G382 | 39 b | 32 b |
| G773 | 31 b | - |

Additional computational complexity at CU level (on top of HM4 complexity):

|  | CE | New [Source code released on 11/28/2011] |
| --- | --- | --- |
| G850 | Per Sample: 2 CMP, 1 LUT | 0 |
| G382 (*) | Per Sample: 1 CMP, Per TU: 1 ADD, 1 LUT | Per TU: 1 LUT |
| G773 | 0 | - |

(*) Additional Intra/Inter parameter needed for G382. G382 also uses sgn() and abs() in dequant eqauation. No consensus reached on complexity impact of these.

Computational complexity at slice level:

|  | CE | New [Source code released on 11/28/2011] |
| --- | --- | --- |
| G850 | 12288 ADD, 12288 NEG | 21 ADD, 18 MULT |
| G382 | 48 MULT, 4 ADD | 48 MULT, 52A |
| G773 | 0 | - |

Table usage at slice level:

|  | CE | New [Source code released on 11/28/2011] |
| --- | --- | --- |
| G850 | 24579 (19 b) | 18 (8 b) |
| G382 | 48 (8 b) | 48 (8 b) |
| G773 | 12 (7 b) | - |

# Quantization matrix

## HMx

For dequantization with quantization matrices, the dequantization process will in general be as follows

coeffQ[i][j]       =  ((level[i][j]*W[i][j]*IQ[QP%6] << (QP/6)) + offset)>>(M-1+DB+4),          offset = 1<<(M-2+DB+4), with i=0…nW-1, j = 0..nH-1      (2)

where W[i][j] denotes quantization matrix and nW is TU width and nH is TU height. This equation assumes that the flat quantization matrix will have a value of 16 in all entries as in AVC. This accounts for the additional shift of 4.

To avoid overflow issues, the following equations are used:

iShift = M-1+DB+4;

If (iShift > QP/6)

coeffQ[i][j]       =  (level[i][j]*W[i][j]*IQ[QP%6] + offset)>>(iShift-QP/6),          offset = 1<<(iShift-QP/6-1), with i=0…nW-1, j = 0..nH-1

else

coeffQ[i][j]       =  (level[i][j]*W[i][j]*IQ[QP%6]) << (QP/6-iShift)

## JCTVC-G773

IQ(x)       = g(x),       where g(x)  =   {40, 42, 45, 48, 51, 54, 57, 60, 64, 68, 72, 76}, x=0,…,11

Same equations as for HMx.

coeffQ[i][j]       =  ((level[i][j]*W[i][j]*IQ[QP%12] << (QP/12)) + offset)>>(M-1+DB+4),          offset = 1<<(M-2+DB+4)

Bit-width required is asserted to be the same as HMx.

## JCTVC-G850

Let iShift= M-1+DB+4

If(iShift > QP/6)

coeffQ[i][j]	=  ((level[i][j]*W[i][j]*IQ’[c][QP%6]+ offset)>>(iShift-QP/6),  offset = 1<<(iShift-1-QP/6), with i=0…nW-1, j = 0..nH-1

else

coeffQ[i][j]	=  ((level[i][j]*W[i][j]*IQ’[c][QP%6])<<(QP/6-iShift),  with i=0…nW-1, j = 0..nH-1	(5)

where IQ’[c][QP%6] = ( IQ[QP%6] * (128+delta[c] ) +64) >> 7

The same as the case of no quantization matrix, IQ’[QP%6] is used instead of IQ[QP%6].  Bit-width required is asserted to be the same as HMx. IQ’ table is calculated at slice level. It is asserted that there are no additional computations when compared to HMx equations.

## JCTVC-G382

If (iShift>QP/6)

coeffQ[i][j] = sgn(level[i][j]) * ( abs(level[i][j])* IQ_s[c][QP%6] + IQ_o[c][QP%6]  ) * W[i][j] >> (iShift-QP/6)

else

coeffQ[i][j] = sgn(level[i][j]) * ( abs(level[i][j])* IQ_s[c][QP%6] + IQ_o[c][QP%6]  ) * W[i][j] <<  (QP/6-iShift)

where sgn() = 1 for positive number, -1 for negative numbers, and 0 for 0.

IQ_s[c][QP%6]  = ((128+delta_s[c]) * IQ[QP%6] + 64) >>7

IQ_o[c][QP%6]  = (delta_o[c] * IQ[QP%6] +64) >> 7

IQ_s and IQ_o tables (6 bytes) are calculated at slice level. Bit-width required is asserted to be the same as HMx. When compared to HMx equations, 1 LUT per TU is additionally required.

## Comparison

Additional computational complexity at CU level (on top of HMx complexity):

|  | Equation analysis |
| --- | --- |
| G850 | 0 |
| G382 (*) | Per TU: 1 LUT |
| G773 | - |

(*) Additional Intra/Inter parameter needed for G382. G382 also uses sgn() and abs() in dequant eqauation.  No consensus reached on complexity impact of these.

# Analysis of HMx quantization matrix dequantization equation

iShift = M-1+DB+4;

If (iShift > QP/6)

coeffQ[i][j]       =  (level[i][j]*W[i][j]*IQ[QP%6] + offset)>>(iShift-QP/6),          offset = 1<<(iShift-QP/6-1), with i=0…nW-1, j = 0..nH-1

else

coeffQ[i][j]       =  (level[i][j]*W[i][j]*IQ[QP%6]) << (QP/6-iShift)

Assuming level is 16 bits, when M=2, iShift = 5, QP = 51 so QP/6-iShift =3 coeffQ is 16+7+8+3=34 b which exceeds desired 32 b. This is a purely mathematical example and it is asserted that this will not occur in practice.

Also, the largest coefficient that can be represented at low weight is limited. Example largest coeffQ when M=5, W=1, QP=0 (IQ=40),

W=1, coeffQ = (2^15)*1*40*>>8 =5120

This needs to be further studied.