| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 19-30 November, 2011 | Document: JCTVC-G105 M21653 |
| --- | --- |

| Title: | Non-CE9: cross-verification of JVC’s proposal JCTVC-G219 on construction of MVP list without using scaling operation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Minhua Zhou Texas Instruments Inc., USA | Tel: Email: : | +1-214-480-3816 zhou@ti.com |
| Source: | Texas Instruments Inc; |  |  |

_____________________________

# Abstract

This contribution reports cross-check results for JVC’s proposal JCTVC-G219 on construction of MVP list without using scaling operation. In this proposal, the spatial MVP and temporal MVP scaling have been moved to after the pruning process of the AMVP list construction.  The temporal distance for each MVP in the list needs to be stored for the MVP scaling afterwards. This technique can potentially reduce the AMVP worst case MVP scaling from 2 to 1 on the decoder, but no saving on the encoder, with penalty of increased latency and data storage. There is no BD-rate loss against the current design. The results match the ones reported by proponents. The source code is also checked to verify its consistency with the proposal description.

.

# Introduction

This document reports cross-check results for JVC’s proposal JCTVC-G219 on construction of MVP list without using scaling operation. The proposal basically advocates to perform the MVP scaling after the MVP pruning process of the AMVP list construction for potential complexity reduction. No changes have been introduced to the merge/skip MVP list construction.

# Test Settings and Conditions

The simulations of this document have used HM4.0 software, the simulation platform is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz 64 bits Linux machines, the common test conditions and reference configurations specified in [1] are followed.

# Experimental results

The experimental results are summarized in Table 1. No loss is observed. Those results match what proponents have reported. Please be advised in Table 1 runtime may not be accurate.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 94% |  |  | 99% |  |  |
| Dec Time[%] | 93% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | 0.2% |
| Class C | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.3% |
| Class D | 0.0% | 0.0% | 0.4% | 0.0% | -0.1% | -0.1% |
| Class E | 0.0% | 0.3% | 0.7% | 0.0% | -0.6% | 0.1% |
| Class F |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.3% | 0.0% | -0.1% | 0.0% |
|  | 0.0% | 0.0% | 0.2% | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 103% |  |  | 96% |  |  |
| Dec Time[%] | 107% |  |  | 94% |  |  |

Table 1.  Experimental results

# Comments

# On the encoder side, this technique won’t save the number of MVP scaling,  in the worst case still two scales are needed. But it will reduce the worst case scale from 2 to 1 on the decoder side. The penalties are increased latency because non-scaled and scaled MVP derivation processes now run sequentially (In the current design the non-scaled MVP derivation can be hidden by the scaled MVP derivation); motion vector reconstruction on decoder side needs two stages, i.e. first to use mvp_idx to compute the final MVP, and then add MVDs to obtain the motion vectors, and additional storage for buffering the temporal distances for all the MVPs.

# References

[1]	F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-F900, Torino, IT, July 2011.

[2]	B. Bross, W.-J. Han,  J.-R. Ohm, G. J. Sullivan, T. Wiegand  “WD4: Working Draft 4 of High-Efficiency Video Coding,” JCT-VC Document, JCTVC-F803, Torino, IT, July 2011

[4] B. Bross, W. Chien, I. Kim, M. Zhou, “Core Experiment 9: MV Coding and Skip/Merge Operations”, JCT-VC Document, JCTVC-F909, Torino, IT, July 2011.