| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G111 |
| --- | --- |

| Title: | Common test conditions to specify 8-bit internal bit depth for 8-bit source material |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tim Hellman, Yong Yu Broadcom Corporation 200 Brickstone Square Andover, MA 01810 USA Wade Wan Broadcom Corporation 5300 California Avenue Irvine, CA 92617 USA | Tel: Email: Tel: Email: | +1-978-719-1348 timhellman@broadcom.com yongyu@broadcom.com +1-949-926-6387 wwan@broadcom.com |
| Source: | Broadcom Corporation |  |  |

_____________________________

# Abstract

This proposal recommends changing the common test conditions to specify an internal bit depth (IBD) of 8 bits for all 8-bit source material.  It presents results that show a coding loss from this change relative to the current 10-bit IBD configuration, but claims that the cost of 10-bit IBD cannot be justified.  A cost increase of 20-23% is reported for a sample hardware codec implementation as well as additional memory and bandwidth costs.  It also recommends keeping 10-bit IBD for 10-bit source material, in preparation for a future 10-bit encoding profile.

# Introduction

The common test conditions document specifies 10-bit internal bit depth (IBD) for the high-efficiency settings, regardless of the source bit depth [2].  The encoder automatically converts 8-bit source data to 10 bit before encoding, by multiplying each pixel value by 4.  The increase in IBD has been shown to increase coding efficiency, but that coding efficiency comes at a significant cost to both hardware and software codec implementations.  This submission summarizes those costs, and presents the coding gain results seen with the HM 4.0 model.

# IBDI System Cost

## Hardware Codec Area Increase

2-bit IBDI for 8-bit source represents a 25% increase in pixel size.  The pixel size increase translates almost one-to-one into an area increase for the pixel-handling portions of both hardware encoders and decoders.  This is because the datapath logic (registers, adders/multipliers, buffer memories) scales directly with pixel size, and the datapath represents almost 100% of the area of those design sections.  There are, of course, portions of a codec which don’t scale with pixel size, such as high-level control, entropy encoding/decoding, and vector/coefficient construction.  These represent a smaller portion of the codec area than they did in AVC, because the HEVC datapath size has increased significantly relative to AVC designs, due to the new filter stages and need for more buffering for LCU-sized processing.

A reasonable estimate for the HEVC pixel-handling area is 80% of a high-performance decoder and 90% of an encoder.  Thus, using 10-bit IBD adds 20-23% more to the cost of a hardware codec compared to 8-bit IBD.

## Software Codec Performance Cost

The IBDI cost to software codecs results from the granularity of multi-media processor instructions.  These instructions, which are critical to software codec performance, parallelize 8- and 16-bit arithmetic operations.  Therefore, all of the 10-bit pixel processing would be forced into using 16-bit representations rather than 8-bit representations.  This represents a 50% decrease in performance for those instructions.  Likewise, storage of processed pixels is likely to be done at 16-bit granularity, which also slows performance (more memory operations needed per pixel).

## Bandwidth & Memory Footprint Cost

Going from 8- to 10-bit pixels increases the memory bandwidth by 25% in the best case.  In practical implementations, the actual cost is often higher, because it’s not possible to optimally pack 10-bit pixels in byte-oriented memory systems which likely results in an additional 5-10% overhead.  This is on top of the bandwidth increases already in HEVC due to the larger motion compensation filter extents relative to AVC.

Various memory compression schemes have been proposed during the HEVC standardization process to mitigate this bandwidth increase.  It is often overlooked but important to realize that memory compression can reduce the average memory bandwidth, but it doesn’t help the worst-case scenario, and in fact, it may make it worse.  This is because the memory compression schemes all have a certain granularity (such as 4x4 pixels), which in the worst-case causes more pixels to be fetched than would otherwise be needed.  This contribution reports that worst-case bandwidth is extremely important to high-performance real-time codecs and often much more significant than average memory bandwidth.

Another cost that should be considered is the cost of the increased memory footprint of 10-bit reference frames.  In AVC systems, the reference frames and display frames are generally one and the same in the display picture buffer (DPB).  It’s highly unlikely that an HEVC-based system would be interested in using 10-bit picture buffers throughout the system, as this would have memory footprint and bandwidth implications everywhere, e.g. video/graphics compositing, video scaling, de-interlacing and display.  Therefore an IBDI-based decoder would have to produce two sets of output:  reference frames (10-bit) and display frames (8-bit).  Reference frame compression suffers from this problem as well.  This effectively doubles the size of the required DPB, adding another 16 Mbytes for HD.

# Experimental Results

Table 1 shows the results comparing the HM 4.0 code with IBDI turned on and turned off.  Only the high-efficiency results are included, as the low-complexity streams always use 8-bit processing depth.

Note that decoding simulations were done on the same type of computers but encoding simulations were performed on different computers so conclusions drawn from comparing encoding times may not be accurate.

Table 1 - Full Results

|  | AI-HE |  |  |  | RA-HE |  |  | LB-HE |  |  | LP-HE |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V |  | Y | U | V | Y | U | V | Y |  | U | V |
| Class A | 2.2 | 13.0 | 16.3 |  | 4.0 | 33.6 | 45.8 |  |  |  |  |  |  |  |
| Class B | 1.7 | 3.5 | 3.3 |  | 3.0 | 10.2 | 9.9 | 3.1 | 11.7 | 13.5 | 2.9 |  | 11.3 | 14.0 |
| Class C | 0.9 | 1.4 | 1.5 |  | 1.7 | 3.8 | 4.5 | 1.7 | 4.8 | 5.4 | 1.5 |  | 4.3 | 4.5 |
| Class D | 0.6 | 1.4 | 1.3 |  | 1.1 | 3.3 | 3.6 | 1.1 | 6.5 | 6.2 | 0.9 |  | 5.4 | 5.3 |
| Class E | 2.3 | 5.6 | 5.4 |  |  |  |  | 7.1 | 20.9 | 15.8 | 6.2 |  | 21.7 | 16.4 |
| Class F | 0.2 | 0.2 | 0.3 |  | 0.8 | 1.9 | 1.6 | 0.2 | 0.4 | 0.3 | 0.2 |  | 0.0 | -0.1 |
| All | 1.3 | 4.1 | 4.6 |  | 2.2 | 10.5 | 12.9 | 2.4 | 8.4 | 8.1 | 2.2 |  | 8.0 | 7.9 |
| Enc Time | 99% |  |  | 98% |  |  |  | 126% |  |  |  | 73% |  |  |
| Dec Time | 94% |  |  | 96% |  |  |  | 97% |  |  |  | 95% |  |  |

Clearly, the largest effect is seen for the chroma components.  It is useful to analyze further to better understand these results.

For Class A, this contribution would like to point out that an unusually large contributor to the relatively high chroma losses (33.6% for U and 45.8% for V) comes from the SteamLocomotive sequence as showin in Table 2.  Note that this 4Kx2K sequence is being coded at a very high quantitative quality even at the lowest QP=37.  An example of this is shown in Table 2 for the RA-HE case.  It is unclear whether gains in this (saturated) operating region are subjectively significant and whether encoders would choose not to spend bits encoding chroma at these high quantitive levels and instead spend more on luma.  Table 2 also shows that the chroma losses are much more in line with the other sequences if SteamLocomotive is removed from the average calculation.

Table 2 – RA-HE Class A results

| Sequence | PSNR at lowest QP=37 for Reference (10-bit IBDI) |  |  | PSNR at lowest QP=37 for Test (8-bit IBDI) |  |  | BD-rate |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Traffic | 34.12 | 37.75 | 39.81 | 34.10 | 37.70 | 39.78 | 3.1% | 6.0% | 6.5% |
| PeopleOnStreet | 31.74 | 40.47 | 41.12 | 31.73 | 40.39 | 41.04 | 1.1% | 6.3% | 7.2% |
| Nebuta | 28.09 | 36.69 | 34.90 | 28.08 | 36.56 | 34.83 | 0.8% | 19.7% | 9.8% |
| SteamLocomotive | 37.22 | 44.99 | 45.09 | 37.13 | 44.60 | 44.63 | 11.0% | 102.4% | 159.8% |
| Average |  |  |  |  |  |  | 4.0% | 33.6% | 45.8% |
| Average without SteamLocomotive |  |  |  |  |  |  | 1.7% | 10.7% | 7.8% |

A similar effect is occurring with the Class E results.  In this case, there is not a single sequence that is an outlier but all the sequences in this class exhibit the same property that even at the lowest QP, the chroma is already being coded at a very high quantitative quality.  An example of this is shown in Table 2 for the LB-HE case.

Table 3– LB-HE Class E results

| Sequence | PSNR at lowest QP=37 for Reference (10-bit IBDI) |  |  | PSNR at lowest QP=37 for Test (8-bit IBDI) |  |  | BD-rate |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Vidyo1 | 36.14 | 43.26 | 43.49 | 36.08 | 42.91 | 43.18 | 7.2% | 23.7% | 17.9% |
| Vidyo3 | 35.70 | 45.12 | 43.01 | 35.63 | 44.77 | 42.88 | 6.7% | 25.2% | 15.0% |
| Vidyo4 | 35.77 | 43.54 | 43.54 | 35.71 | 43.56 | 43.51 | 7.3% | 13.9% | 14.6% |
| Average |  |  |  |  |  |  | 7.1% | 20.9% | 15.8% |

# Conclusions

Although 2-bit IBDI has a small average coding gain (mostly in the chroma components), its associated system cost far outweighs its benefits.  Therefore, this contribution recommends that it remain for use in a profile supporting 10-bit input source material, but that the common test conditions be changed to use 8-bit internal bit depth for all 8-bit input source material.

# References

- T. Wiegand, B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, “WD4: Working Draft 4 of High-Efficiency Video Coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-F803, Oct. 2011.

- F. Bossen, “Common test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-F900, Torino, IT, July 2011.

# Patent rights declaration(s)

Broadcom, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).