| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G112 |
| --- | --- |

| Title: | Changing Luma/Chroma Coefficient Interleaving from CU to TU level |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tim Hellman, Yong Yu Broadcom Corporation 200 Brickstone Square Andover, MA 01810 USA | Tel: Email: | +1-978-719-1348 timhellman@broadcom.com yongyu@broadcom.com |
| Source: | Broadcom Corporation |  |  |

_____________________________

# Abstract

In Working Draft 4, the luma and chroma coefficient data are interleaved at the coding unit (CU) level.  This submission proposes to shift the interleaving to the transform unit (TU) level instead.  It purports to show that this results in a size reduction in hardware decoder implementations, and claims that it has no measurable effect on coding efficiency.  It also recommends limits on two SPS parameters that it claims are needed to enable this size reduction.

# Introduction

In HM-4.0, transform residual luma and chroma data is interleaved at the coding unit (CU) level.  This is true no matter how the transform units (TUs) are split within the CU.  Figure 1 has an example of a TU split in a CU.

Figure 1 - Sample TU Split in a CU

The coefficient data for the CU in Figure 1 would be sent as follows:

Luma 1, Luma 2, Luma 3, Luma 4, Luma 5, Luma 6, Luma 7,

Cb 1, Cb 2, Cb 3, Cb 4, Cb 5, Cb 6, Cb 7,

Cr 1, Cr 2, Cr 3, Cr 4, Cr 5, Cr 6, Cr 7

where “Luma i” represents all the luma coefficients for Block i and “Cb i” and “Cri i” represent all the chroma coefficients for Block i.

The example shown is for square TU partitioning, but the ordering is identical with rectangular splitting.

With this ordering, a given TU doesn’t receive all its pixel data until the whole CU is almost entirely sent.  This is true even though the maximum CU size (64x64) is larger than the maximum transform size (32x32).

Hardware-based decoders are generally designed with a pipelined structure, which allows various portions of the decoder to process data in parallel.  In such a design, the coefficient decoding is typically done in parallel with the inverse transform, as shown in Figure 2.

Figure 2 - HW Inverse Transform Block Diagram

Each of the buffers needs to be double the size of the amount of data needed in the worst-case by the processing stage, which is a prerequisite to parallel processing.  The coefficient buffer only needs to hold 2 times the 1024 coefficients, since that’s the size of the largest permitted transform (32x32), but the residual buffer can’t pass along its data until it has all three components for each pixel.  That means it has to store 2 times the 4096 sets of residual data (64x64 LCU).  At 1.5 bytes/luma pixel (for 8-bit residual data), a 12 Kbyte residual buffer (4096 * 1.5 * 2) is required.  That’s a RAM amount equal to four HD line buffers just for residual data storage between stages!  This size could be reduced by 75% if the three color components were available simultaneously, because then the residual data could be passed once the maximum transform size (32x32) is reached, rather than the maximum LCU size (64x64).

# Proposed Interleaving Change

The proposed solution is to move the luma/chroma interleave to the TU level. This solves the buffering problem, because all components for a given TU will be sent before the following TUs, which allows the residual buffer to be sized for the maximum TU size instead of the maximum CU size.

For the example in figure 1, the interleaving would now be:

Luma 1, Cb 1, Cr 1,

Luma 2, Cb 2, Cr 2,

Luma 3, Cb 3, Cr 3,

…

Luma 7, Cb 7, Cr 7

When TU size equals MinTUSize, the interleaving is slightly different.  Here there is only one Cb and one Cr transform for each 4 luma transforms, so the Cb and Cr are sent after the first luma block of the 4 (the choice of the first block is somewhat arbitrary. The last block would work as well).  Assume in figure 1 that blocks 2, 3, 4 and 5 are at the minimum transform size.  In this case the order of coefficient data would be:

Luma 1, Cb 1, Cr 1,

Luma 2, Cb 2:5, Cr 2:5,

Luma 3,

Luma 4,

Luma 5,

Luma 6, Cb 6, Cr 6,

Luma 7, Cb 7, Cr 7

Where Cb 2:5 represents the coefficients for the single Cb transform associated with Luma Blocks 2 through 5, and Cr 2:5 represents the coefficients for the single Cr transform.

An addition benefit of this interleave is that the transform split tree only has to be traversed once per CU, as opposed to the three times required by the existing interleave.  Also, if the delta QP is allowed to change at the sub-CU level (as has been proposed), then it certainly seems to make sense to group coefficients with the same QP together, as this interleave would do.  The existing interleave would shuffle between high and low QP coefficients within a CU.

The actual change to the coding syntax is listed in Annex A, but the change can be summarized graphically as shown in Error: Reference source not found.  As this figure demonstrates, the substance of the change is simply to swap the two loops that surround the residual coding process.  The change is identical for both CABAC and CAVLC, and the rest of the coding syntax is unaffected.

Figure 3  - Graphical Syntax Change

# Other Syntax Changes

Two other syntax changes are needed to allow the smaller buffer size to be used:

- Limit MinTrafoSize to be less than MaxTrafoSize, unless MaxTrafoSize = 4x4.

This is necessary to prevent the odd chroma transform size behaviors that result when MinTrafoSize = MaxTrafoSize.  For example, currently, if MinTrafoSize is set to 32x32, the resulting chroma transform for the CU is 32x32, which is something that does not occur under any other circumstances.  Limiting this behavior seems generally desirable.

Alternatively, a new ‘MinChromaTrafoSize’ variable could be defined as being one size smaller than MinTrafoSize.  This would prevent the unusual chroma transform sizes while keeping the full flexibility of MinTrafoSize.

- Set a new variable: MaxIPCMCUSize, with a maximum value of 32x32.

This corresponds to the existing ‘MinIPCMCUSize’ variable.  PCM data has the same luma/chroma interleaving problem if it’s allowed to extend to the maximum LCU size.  Putting an upper limit on PCM CU size solves this.  It also reduces the number of times that the ‘pcm_flag’ syntax element has to be inserted in the stream.

# Experimental Results

It is expected that this change would have no substantial effect on bit rate, since only the order of the syntax elements has been changed.  Indeed, simulations have shown no change to (average) coding efficiency.  Encoding and decoding time are likewise unaffected.

Table 1 - Result Summary (using 6Y + U + V averaging)

|  | HE-AI | LC-AI | HE-RA | LC-RA | HE-LD | LC-LD |
| --- | --- | --- | --- | --- | --- | --- |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class E | 0.0% | 0.0% |  |  | -0.1% | -0.2% |
| All | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |

Table 2 - Full Results

|  | HE-AI |  |  | LC-AI |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 101% |  |  | 87% |  |  |
| Dec Time[%] | 102% |  |  | 88% |  |  |
|  | HE-RA |  |  | LC-RA |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | -0.1 | -0.1 | 0.0 | 0.2 | -0.1 |
| Class B | 0.0 | 0.0 | 0.1 | 0.0 | -0.1 | 0.1 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | 0.0 |
| Class D | 0.0 | 0.2 | -0.2 | 0.0 | 0.1 | 0.2 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 93% |  |  | 99% |  |  |
| Dec Time[%] | 114%1 |  |  | 97% |  |  |
|  | HE-LD |  |  | LC-LD |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.1 | -0.2 | 0.0 | 0.0 | 0.1 |
| Class C | 0.0 | 0.1 | -0.1 | 0.0 | 0.2 | 0.0 |
| Class D | 0.0 | 0.6 | 0.0 | 0.0 | 0.2 | 0.1 |
| Class E | 0.0 | -0.1 | -0.5 | -0.1 | -0.1 | -0.6 |
| All | 0.0 | 0.2 | -0.2 | 0.0 | 0.1 | -0.1 |
| Enc Time[%] | 99% |  |  | 102% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |

1 Runtimes are recorded as reported, but should be largely dismissed as the experiments were run on heterogeneous Linux clusters.  There is no reason to believe that a change to the interleave order would have any actual impact on real runtime.

# Conclusions

The proposed change allows hardware decoder implementations to be much more area-efficient.  As the change has no effect on complexity or coding efficiency, and should not affect software implementations, we recommend that it be adopted.

# References

- T. Wiegand, B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, “WD4: Working Draft 4 of High-Efficiency Video Coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-F803, Oct. 2011.

# Patent rights declaration(s)

Broadcom, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex A: Draft Text Changes

This highlight indicates deletes or changes

This highlight indicates additions

7.3.6 Coding unit syntax

| coding_unit( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| … |  |
| if( !pcm_flag ) { |  |
| transform_tree( x0, y0, log2CUSize, log2CUSize, 0, 0 ) |  |
| transform_coeff( x0, y0, log2CUSize, log2CUSize, 0, 0) |  |
| transform_coeff( x0, y0, log2CUSize, log2CUSize, 0, 1 ) |  |
| transform_coeff( x0, y0, log2CUSize, log2CUSize, 0, 2 ) |  |
| } |  |
| } |  |

7.3.7  Prediction unit syntax

| prediction_unit( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| merge_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else if( PredMode = = MODE_INTRA ) { |  |
| if( PartMode == PART_2Nx2N && log2CUSize >= Log2MinIPCMCUSize && log2CUSize <= Log2MaxIPCMCUSize ) |  |
| pcm_flag | u(1) ¦ ae(v) |
| if( pcm_flag ) { |  |
| … |  |

7.3.9 Transform coefficient syntax

| transform_coeff( x0, y0, log2TrafoWidth, log2TrafoHeight, trafoDepth, blkIdx ) { | Descriptor |
| --- | --- |
| if( ( ( cIdx = = 0 && cbf_luma[ x0 ][ y0 ][ trafoDepth ] ) ¦ ¦ ( cIdx = = 1 && cbf_cb[ x0 ][ y0 ][ trafoDepth ] ) ¦ ¦ ( cIdx = = 2 && cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) ) ) { |  |
| if( (cbf_luma[ x0 ][ y0 ][ trafoDepth ] ) ¦ ¦ cbf_cb[ x0 ][ y0 ][ trafoDepth ] ) ¦ ¦ cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) { |  |
| if( cu_qp_delta_enabled_flag && !IsCuQpDeltaCoded ) { |  |
| cu_qp_delta | se(v) ¦ ae(v) |
| IsCuQpDeltaCoded = 1 |  |
| } |  |
| if( split_transform_flag[ x0 ][ y0 ][ trafoDepth ] && ( cIdx = = 0 ¦ ¦ log2TrafoSize > Log2MinTrafoSize + 1 ) ) { |  |
| if( InterTUSplitDirection = = 2 ) { |  |
| … |  |
| } |  |
| transform_coeff( x0, y0, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 0 ) |  |
| transform_coeff( x1, y1, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 1 ) |  |
| transform_coeff( x2, y2, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 2 ) |  |
| transform_coeff( x3, y3, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 3 ) |  |
| } else { |  |
| log2TrafoSize = ( ( log2TrafoWidth + log2TrafoHeight ) >> 1 ) |  |
| log2TrafoSizeC = (log2TrafoSize == log2MinTrafoSize) ? log2TrafoSize : (log2TrafoSize – 1) |  |
| if ( PredMode = = MODE_INTRA ) { |  |
| if (cIdx = = 0) |  |
| scanIdx = ScanType[ log2TrafoSize – 2 ][ IntraPredMode ] |  |
| Else |  |
| scanIdxC = ScanTypeC[ log2TrafoSizeC – 2 ][ IntraPredModeC ] |  |
| } else { |  |
| scanIdx = 0 |  |
| scanIdxC = 0 |  |
| } |  |

| if ( entropy_coding_mode_flag ) { |  |
| --- | --- |
| if ( scanIdx = = 0 ) |  |
| scanIdx = 3 |  |
| if ( scanIdxC = = 0 ) |  |
| scanIdxC = 3 |  |
| if (cbf_luma[x0][y0][trafoDepth]) |  |
| residual_coding_cabac( x0, y0, log2TrafoSize, trafoDepth, scanIdx, 0 ) |  |
| if (log2TrafoSize > log2MinTrafoSize ¦¦ blkIdx == 0) { |  |
| if (cbf_cb[x0][y0][trafoDepth]) |  |
| residual_coding_cabac( x0, y0, log2TrafoSizeC, trafoDepth, scanIdxC, 1 ) |  |
| if (cbf_cr[x0][y0][trafoDepth]) |  |
| residual_coding_cabac( x0, y0, log2TrafoSizeC, trafoDepth, scanIdxC, 2 ) |  |
| } |  |
| } |  |
| else if ( !entropy_coding_mode_flag ) { |  |
| if (cbf_luma[x0][y0][trafoDepth]) |  |
| residual_coding_cavlc( x0, y0, log2TrafoSize, trafoDepth, 0 ) |  |
| if (log2TrafoSize > log2MinTrafoSize ¦¦ blkIdx == 0) { |  |
| if (cbf_cb[x0][y0][trafoDepth]) |  |
| residual_coding_cavlc( x0, y0, log2TrafoSizeC, trafoDepth, 1 ) |  |
| if (cbf_cr[x0][y0][trafoDepth]) |  |
| residual_coding_cavlc( x0, y0, log2TrafoSizeC, trafoDepth, 2 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |