| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G113 |
| --- | --- |

| Title: | CE4 Subtest 1.1.a: Cross-verification of Motorola’s QP adaptation at sub_CU level (JCTVC-F577) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Keiichi Chono Hirofumi Aoki 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN | Tel: Email: | +81 50 3757 5439 chono@ct.jp.nec.com hao@bu.jp.nec.com |
| Source: | [Information and Media Processing Labs, NEC Corporation] |  |  |

_____________________________

# Abstract

This contribution presents cross-verification results of Motorola’s QP adaptation at sub_CU level (JCTVC-F577). The cross-verification task was successfully completed and it is confirmed that proponent's results are accurate. If the JCTVC- F577 is considered for HM5.0 adoption, the following actions are recommended:

- JCTVC-F577 software is revised so as to locate all cu_delta_qp syntaxes in transfrom_coeff() as specified by the current WD text: having cu_delta_qp syntaxes in different locations complicates the definition of the semantics.

- The revised software is inspected by JCTVC experts prior to the final decision.

- Software analysis

Through the software analysis, the following are confirmed:

- cu_delta_qp syntaxes are signaled for TUs larger than or equal to the minimum CU dQP size specified by log2MinCUDQPSize;

- cu_delta_qp syntaxes except the first cu_delta_qp syntax of a CU are located after transfrom_coeff(), that is, the syntax locations of the first and remaining cu_delta_qp are different;

- the derivation process of the left available QP value is modified so as to use the QP of the immediate left TU in current CU instead of that of immediate left CU.

- Simulation results

Simulation was conducted on Windows-based computing platform (Windows 7 64-bit Professional, Xeon 3.57GHz and Memory 32GB). Table 1 shows the simulation results (for dQP depth=3). It is confirmed that the results match with proponent’s. (Detailed results are found in the attached Excel sheet.)

Table1: Simulation results of CE4 Subset 1.1 a

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | 1.1% | 2.8% | 2.8% | 20.0% | 1.5% | 1.4% | 0.8% | 22.4% |
| Class B | 1.0% | 2.7% | 3.5% | 15.3% | 1.4% | 1.0% | 1.1% | 17.8% |
| Class C | 0.4% | 2.6% | 3.1% | 7.2% | 0.7% | 0.7% | 0.8% | 7.6% |
| Class D | 0.3% | 2.1% | 2.3% | 6.0% | 0.5% | 0.5% | 0.5% | 5.8% |
| Class E | 1.0% | 4.0% | 3.2% | 10.1% | 1.4% | 1.4% | 1.2% | 13.2% |
| Overall | 0.8% | 2.8% | 3.0% | 12.0% | 1.1% | 1.0% | 0.8% | 13.6% |
|  | 0.8% | 2.8% | 2.9% |  | 1.1% | 1.0% | 0.9% |  |
| Enc Time[%] | 102% |  |  |  | 102% |  |  |  |
| Dec Time[%] | 102% |  |  |  | 102% |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | 1.6% | 0.9% | 0.8% | 20.3% | 1.6% | 0.3% | -0.1% | 20.7% |
| Class B | 1.3% | 2.6% | 3.8% | 15.5% | 1.4% | 0.9% | 1.0% | 15.7% |
| Class C | 0.7% | 1.8% | 2.0% | 9.5% | 0.8% | 0.6% | 0.8% | 9.4% |
| Class D | 0.5% | 2.0% | 2.1% | 9.2% | 0.6% | 0.5% | 0.4% | 8.3% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | 1.0% | 1.8% | 2.3% | 13.7% | 1.1% | 0.6% | 0.5% | 13.6% |
|  | 1.0% | 1.8% | 2.2% |  | 1.1% | 0.6% | 0.6% |  |
| Enc Time[%] | 103% |  |  |  | 103% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 101% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | 1.9% | 4.4% | 5.5% | 16.0% | 1.9% | 2.3% | 2.1% | 13.8% |
| Class C | 1.1% | 2.4% | 3.0% | 10.5% | 1.2% | 1.9% | 1.8% | 10.1% |
| Class D | 1.0% | 3.1% | 3.5% | 11.8% | 1.1% | 1.5% | 0.8% | 10.1% |
| Class E | 2.1% | 6.2% | 4.3% | 11.7% | 3.0% | 5.1% | 4.1% | 17.6% |
| Overall | 1.5% | 3.9% | 4.2% | 12.7% | 1.7% | 2.5% | 2.1% | 12.7% |
|  | 1.5% | 3.9% | 4.2% |  | 1.7% | 2.6% | 2.0% |  |
| Enc Time[%] | 102% |  |  |  | 103% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 99% |  |  |  |

- Conclusions and recommendation

This contribution has presented the cross-verification results of QP adaptation at sub_CU level (JCTVC-F577). The cross-verification task was successfully completed and it is confirmed that proponent's results are accurate. If the JCTVC- F577 is considered for HM5.0 adoption, the following actions are recommended:

- JCTVC-F577 software is revised so as to locate all cu_delta_qp syntaxes in transfrom_coeff() as specified by the current WD text: having cu_delta_qp syntaxes in different locations complicates the definition of the semantics.

- The revised software is inspected by JCTVC experts prior to the final decision.

# Reference

- K. Panusopone et al., "QP adaptation at sub_CU level," JCTVC-F577, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011.