| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G115 |
| --- | --- |

| Title: | AHG22: Cross-verification of TI’s Sample-based angular prediction (SAP) for HEVC lossless coding (JCTVC-G093) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Keiichi Chono Hirofumi Aoki 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN | Tel: Email: | +81 50 3757 5439 chono@ct.jp.nec.com hao@bu.jp.nec.com |
| Source: | [Information and Media Processing Labs, NEC Corporation] |  |  |

_____________________________

# Abstract

This contribution presents cross-verification results of TI’s HEVC lossless coding proposal. The cross-verification task was successfully completed and it is confirmed that proponent's algorithm description and simulation results are correct. If picture-level lossless coding schemes are further studied in the context of CE, it is recommended that an HM5.0 that bypasses transform and quantization is used for its Anchor.

- Simulation results

Through the software inspection, it is confirmed that HM4.0 is modified according to the proponent’s algorithm descriptions (bypass transform and quantization, and conduct sample-by-sample angular intra prediction). The software was successfully compiled and run on Window-based computing platform. Table1 shows the simulation results. Note that Reference in table is a modified HM4.0 (bypass transform and quantization) and that Test is the modified HM4.0 using sample-based angular intra prediction. Table2 shows additional simulation results that enables PCM mode with 64x64 according to the AHG22 common test conditions. Note that as informed by the proponent in advance, decoder crashes a observed for some RA-LC and LB-LC sequences due to the HM CAVLC bugs (announced on JCTVC reflector with a title "[JCTVC] potential bugs in HM4.0 code" on Nov. 1st). However, it was confirmed that other sequences are correctly decoded.

Table1: Simulation results (ALF, SAO, and RDQO are disabled, internal-bit depth is set to the original input-bit depth, and PCM with 64x64 is enabled.)

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Comp. ratio |  | bit-saving % | Comp. ratio |  | bit-saving % |
|  | Reference | Tested |  | Reference | Tested |  |
| Class A | 1.72 | 1.87 | -7.43% | 1.61 | 1.79 | -9.81% |
| Class B | 2.07 | 2.16 | -4.05% | 1.90 | 2.00 | -5.09% |
| Class C | 1.96 | 2.05 | -4.30% | 1.82 | 1.92 | -5.51% |
| Class D | 1.82 | 1.97 | -7.23% | 1.70 | 1.87 | -8.94% |
| Class E | 2.88 | 3.19 | -9.70% | 2.66 | 2.95 | -9.97% |
| Class F | 4.49 | 5.06 | -8.95% | 4.23 | 4.76 | -8.88% |
| Overall | 2.46 | 2.67 | -6.71% | 2.29 | 2.51 | -7.83% |
| Enc Time[%] | 120% |  |  | #NUM! |  |  |
| Dec Time[%] | 99% |  |  | 103% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Comp. ratio |  | bit-saving % | Comp. ratio |  | bit-saving % |
|  | Reference | Tested |  | Reference | Tested |  |
| Class A | 2.02 | 2.07 | -2.49% | 1.85 | 1.92 | -3.37% |
| Class B | 2.37 | 2.39 | -0.75% | 2.15 | 2.17 | -1.05% |
| Class C | 2.54 | 2.57 | -1.26% | 2.27 | 2.31 | -1.47% |
| Class D | 2.70 | 2.69 | #VALUE! | 2.38 | 2.43 | -2.08% |
| Class E |  |  |  |  |  |  |
| Class F | 28.03 | 29.77 | -5.64% | 26.11 | 27.60 | -5.61% |
| Overall | 7.52 | 7.64 | #VALUE! | 6.72 | 7.04 | -2.64% |
| Enc Time[%] | #NUM! |  |  | 107% |  |  |
| Dec Time[%] | 91% |  |  | NA |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Comp. ratio |  | bit-saving % | Comp. ratio |  | bit-saving % |
|  | Reference | Tested |  | Reference | Tested |  |
| Class A |  |  |  |  |  |  |
| Class B | 2.37 | 2.39 | -0.56% | 2.15 | 2.17 | -0.85% |
| Class C | 2.54 | 2.57 | -1.10% | 2.27 | 2.30 | -1.27% |
| Class D | 2.64 | 2.68 | -1.29% | 2.37 | 2.41 | -1.66% |
| Class E | 4.13 | 4.14 | -0.27% | 3.41 | 3.43 | -0.86% |
| Class F | 46.53 | 48.01 | -4.58% | 42.01 | 43.23 | -4.61% |
| Overall | 11.56 | 11.87 | -1.57% | 10.38 | 10.65 | -1.85% |
| Enc Time[%] | 105% |  |  | 106% |  |  |
| Dec Time[%] | 103% |  |  | NA |  |  |

Table2: Simulation results (ALF, SAO, and RDQO are disabled, internal-bit depth is set to the original input-bit depth, 64x64 PCM is enabled.)

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Comp. ratio |  | bit-saving % | Comp. ratio |  | bit-saving % |
|  | Reference | Tested |  | Reference | Tested |  |
| Class A | 1.72 | 1.87 | -7.46% | 1.61 | 1.79 | -9.81% |
| Class B | 2.07 | 2.16 | -4.05% | 1.90 | 2.00 | -5.09% |
| Class C | 1.96 | 2.05 | -4.30% | 1.82 | 1.92 | -5.51% |
| Class D | 1.82 | 1.97 | -7.23% | 1.70 | 1.87 | -8.94% |
| Class E | 2.88 | 3.19 | -9.70% | 2.66 | 2.95 | -9.97% |
| Class F | 4.49 | 5.06 | -8.95% | 4.23 | 4.76 | -8.88% |
| Overall | 2.46 | 2.67 | -6.71% | 2.29 | 2.51 | -7.83% |
| Enc Time[%] | 120% |  |  | 105% |  |  |
| Dec Time[%] | 100% |  |  | 104% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Comp. ratio |  | bit-saving % | Comp. ratio |  | bit-saving % |
|  | Reference | Tested |  | Reference | Tested |  |
| Class A | 2.02 | 2.07 | -2.50% | 1.85 | 1.92 | -3.37% |
| Class B | 2.37 | 2.39 | -0.75% | 2.15 | 2.17 | -1.05% |
| Class C | 2.54 | 2.57 | -1.26% | 2.27 | 2.31 | -1.47% |
| Class D | 2.64 | 2.69 | -1.66% | 2.38 | 2.43 | -2.08% |
| Class E |  |  |  |  |  |  |
| Class F | 28.03 | 29.77 | -5.64% | 26.11 | 27.60 | -5.61% |
| Overall | 7.28 | 7.64 | -2.29% | 6.73 | 7.04 | -2.64% |
| Enc Time[%] | 105% |  |  | 107% |  |  |
| Dec Time[%] | 102% |  |  | NA |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Comp. ratio |  | bit-saving % | Comp. ratio |  | bit-saving % |
|  | Reference | Tested |  | Reference | Tested |  |
| Class A |  |  |  |  |  |  |
| Class B | 2.37 | 2.39 | -0.56% | 2.15 | 2.17 | -0.85% |
| Class C | 2.54 | 2.57 | -1.10% | 2.27 | 2.30 | -1.27% |
| Class D | 2.64 | 2.68 | -1.29% | 2.37 | 2.41 | -1.66% |
| Class E | 4.13 | 4.14 | -0.27% | 3.41 | 3.43 | -0.86% |
| Class F | 46.53 | 48.01 | -4.58% | 42.01 | 43.23 | -4.61% |
| Overall | 11.56 | 11.87 | -1.57% | 10.38 | 10.65 | -1.85% |
| Enc Time[%] | 105% |  |  | 106% |  |  |
| Dec Time[%] | 103% |  |  | NA |  |  |

- Conclusions and recommendation

This contribution presented cross-verification results of TI’s HEVC lossless coding proposal. The cross-verification task was successfully completed and it is confirmed that proponent's algorithm description and simulation results are correct. If (picture-level) lossless coding schemes are further studied in the context of CE, it is recommended that an HM5.0 that bypasses transform and quantization is used for its Anchor.

# Reference

- M. Zhou, " AHG14: Sample-based angular prediction for HEVC lossless coding," JCTVC-GXXX, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011.