| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G116-r1 |
| --- | --- |

| Title: | On PCM memory usage reduction in HM software |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Keiichi Chono 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN | Tel: Email: | +81 50 3757 5439 chono@ct.jp.nec.com |
| Source: | [Information and Media Processing Labs, NEC Corporation] |  |  |

_____________________________

# Abstract

This contribution provides information of an HM software patch that removes PCM buffer stored in DPB. The HM software patch modifies PCM related codes so as to use the original and reconstructed picture buffers instead of PCM buffer. It is reportedly shown that removing PCM buffer stored in DPB does not change HM encoder and decoder running time significantly (2-3% reduction in the common test conditions).

- Problem statement

Ticket 57 on the JCTVC HM site states the excessive memory allocation issue of HM software. The excessive memory allocation comes from its encoder design that creates CU-data (e.g., luma and chroma transform coefficients) storage for every CU in every frame in DPB. The encoder design allows experts to easily integrate new tools. However, it sometimes leads to crashes when naive users run the software on a computer with insufficient memory capacity. And also, it is asserted that it may slow down encoder and decoder running time due to the potential cache misses associated with the excessive memory allocation.

- Removal of PCM buffer stored in DPB

CU-data also includes PCM data (luma and chroma samples). Therefore PCM buffer is also created for every CU in every frame in DPB although such an excessive allocation is unnecessary. Existing functions, xCompressCU(), xEncodeCU(), xDecodeCU(), and xDecompressCU, are modified, and a new function, prePCMLFDisableProcess () is added into TComAdaptiveLoopFilter.c, respectively, as follows:

- xCompressCU(): PCM estimation process is modified so as to directly write the original pixel values to the reconstructed picture buffer instead of PCM buffer;

- xEncodeCU(): PCM entropy coding process is modified so as to directly encode the PCM samples of the original picture buffer with a specified PCM bit-depth precision;

- xDecodeCU(): PCM entropy decoding process is modified so as to parse PCM samples and store them to the reconstructed picture buffer directly;

- xDecompressCU: PCM reconstruction process is modified to scale the bit-depth the PCM samples of the reconstructed picture buffer to the internal bit-depth in-place;

- prePCMLFDisableProcess (): It locates before SAO and ALF functions and stores the reconstructed picture buffer to a new single frame buffer that are used for PCM sample substitution when PCM LF is disabled. (Note that the new single frame buffer, m_pcIPCMSampleYuv in TComAdaptiveLoopFilter.h, is allocated when it is necessary.)

Regarding the xEncodeCU() modification, note that since SAO and ALF are called before calling encodeSlice, they break the PCM sample values of the reconstructed picture buffer when pcm_sample_loop_filter_disable_flag is set to 0. Therefore we use the original picture buffer instead of one.

- Simulation results

Simulation was conducted to see how the PCM buffer reduction changes HM-4.0 encoder and decoder running time. The simulation conditions are the same as the common test conditions in JCTVC-F900. Table 1 shows the simulation results ran on Window7 with iCore7 2.8GHz and 12GB memory. (The detailed results are found in attached Excel sheet.) The results show that removing PCM buffer stored in DPB does not change the HM encoder and decoder running time significantly.

Table 1: Simulation results (Anchor: HM-4.0, Tested: HM-4.0 with pcm-buffer-reduction patch.)

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class E | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Enc Time[%] | 97% |  |  | 96% |  |  |
| Dec Time[%] | 97% |  |  | 98% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class E |  |  |  |  |  |  |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Enc Time[%] | 98% |  |  | 98% |  |  |
| Dec Time[%] | 97% |  |  | 97% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class E | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Enc Time[%] | 98% |  |  | 97% |  |  |
| Dec Time[%] | 97% |  |  | 96% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class E | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Enc Time[%] | 98% |  |  | 98% |  |  |
| Dec Time[%] | 97% |  |  | 97% |  |  |

# Conclusions

This contribution has described the HM software patch that removes PCM buffer stored in DPB. It has been reportedly shown that removing PCM buffer stored in DPB does not change HM-4.0 encoder and decoder running time significantly.

# Acknowledgement

The author thanks to Panasonic who kindly inspected the HM-4.0 patch for PCM buffer reduction.

# Patent rights declaration(s)

NEC Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).