| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G118 |
| --- | --- |

| Title: | Potential enhancement of signaling of I_PCM blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Keiichi Chono Hirofumi Aoki 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN | Tel: Email: | +81 50 3757 5439 chono@ct.jp.nec.com |
| Source: | [Information and Media Processing Labs, NEC Corporation] |  |  |

_____________________________

# Abstract

This contribution presents the concept-level idea of a method for transmitting successive I_PCM blocks. The method counts the number of successive I_PCM blocks in the Z-scan order in the same layer of the CTB and signals that number minus 1 at the PU header of the first I_PCM block. PCM sample data of the subsequent I_PCM blocks immediately follow that of the first I_PCM block, that is, signaling of the PU headers of the subsequence I_PCM blocks is skipped. Thereby the method enhances the throughput of transmitting PCM sample data of successive I_PCM block while reducing some bits of side-information. It is recommended that JCTVC group reviews and discusses the presented method.

- Background

The PCM mode in the H.264/MPEG-4 AVC standard transmits PCM samples of its associated MB without prediction, transform coding, and entropy coding, and thus it allows encoders to adjust the number of bits of MB to a predetermined value or less without complicated computation. The current HM adopts the PCM mode coding in JCTVC-E057 which signals flags, pcm_flags, in the Intra CUs larger than the minimum I_PCM block size specified by a SPS parameter, log2_min_pcm_coding_block_size_minus3. Therefore the current HM can also adjust the number of bits of CU to a predetermined value or less without complicated computation.

- Signaling of successive I_PCM blocks

Since an I_PCM block consists of variable-length data of PU header (pcm_flag, intra_part_mode, pred_type, pcm_alignment_zero_bit) and fixed-length data of PCM sample (pcm_sample_luma and pcm_sample_chroma), the variable-length data and fixed-length data of successive I_PCM blocks are interleaved when a certain layer of CTB contains successive I_PCM blocks. Figure 1(a) shows an example that the certain layer of CTB contains one non-I_PCM block and three I_PCM blocks. Figure 1(b) illustrates the bistream of Figure 1(a). As it may be deduced from Figure 1 (b), throughput of transmitting PCM sample data of three I_PCM blocks is affected due to the presence of the variable-length data in the second and third PU headers. Specifically, the current I_PCM signaling need to terminate PCM sample data transmission at each of I_PCM blocks. Therefore it affects the throughput of transmitting PCM sample data of successive I_PCM blocks and increases some bits of side-information.

Figure 1: (a) Example of CTB that contains three successive I_PCM blocks. (b) HM bistream of (a). (c) Proposed bitstream of (a). Note that PU header (pcm_flag, intra_part_mode, pred_type, pcm_alignment_zero_bit) is variable length data and PCM sample is fixed length data.

- Burst transmission of successive I_PCM blocks

In order to enhance the throughput of transmitting PCM sample data of successive I_PCM blocks, we propose to signal, in the PU header of the first I_PCM block, the number of subsequent I-PCM blocks that successively follow the first I-PCM block in the same layer of a CTB. The number of subsequent I-PCM blocks is specified as subsequent_pcm_num syntax, and Figure 1 (c) illustrates the bitstream of Figure 1 (a). Notice that PCM sample data of the two subsequent I_PCM blocks (block 2 and 3) immediately follow that of the first I_PCM block, that is, signaling of the PU headers of the two subsequent I_PCM blocks is skipped. The method thus enhances the throughput of transmitting PCM sample data of successive I_PCM block while reducing some bits of side-information. (Note that subsequent_pcm_num can be interpreted as pcm_flags of the subsequent I_PCM blocks and therefore the first pcm_flag and subsequent_pcm_num can be jointly coded.)

A potential disadvantage of introducing the subsequent_pcm_num syntax is side-information increase of I_PCM block in the case that I_PCM block is sparsely distributed. However, the side-information increase is expected to be small because of the presence of pcm_alignment_zero_bit. For example, if we assume the uniform distribution of the remainder of total bits divided by 8, the expectation of the side-information increase is roughly 1bit. (0.125*8 + 0.125*0 + 0.125*0  … + 0.125*0).

# Conclusions

This contribution has described a burst transmission scheme of successive I_PCM blocks of CTB. It is recommended that JCTVC group reviews and discusses the burst transmission scheme. If the JCTVC group agrees to its necessity, it is recommended that the burst transmission scheme is studied and evaluated in the context of CE.

# Patent rights declaration(s)

NEC Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- K. Chono et al., "Pulse code modulation mode for HEVC," JCTVC-E057, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March 2011

Appendix: Potential changes in WD text

Modified portions in 7.3.4-7.2.7 are highlighted with red font and yellow color background.

### 7.3.4. Slice data syntax

| slice_data( ) { | Descriptor |
| --- | --- |
| CurrTbAddr = LCUAddress |  |
| moreDataFlag = 1 |  |
| if( adaptive_loop_filter_flag && alf_cu_control_flag ) |  |
| AlfCuFlagIdx = -1 |  |
| do { |  |
| xCU = HorLumaLocation( CurrTbAddr ) |  |
| yCU = VerLumaLocation( CurrTbAddr ) |  |
| NumPCMBlock = 0 |  |
| moreDataFlag = coding_tree( xCU, yCU, Log2TbSize ) |  |
| CurrTbAddr = NextTbAddress( CurrTbAddr ) |  |
| } while( moreDataFlag ) |  |
| } |  |

7.3.5 Coding Tree syntax

| coding_tree( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| if( x0 + ( 1 << log2CUSize ) <= PicWidthInSamplesL && y0 + ( 1 << log2CUSize ) <= PicHeightInSamplesL && cuAddress( x0, y0 ) >= SliceAddress NumPCMBlock == 0 ) { |  |
| if( !entropy_coding_mode_flag && slice_type != I ) |  |
| cu_split_pred_part_mode[ x0 ][ y0 ] | ce(v) |
| else if( log2CUSize > Log2MinCUSize ) |  |
| split_coding_unit_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| } |  |
| if( adaptive_loop_filter_flag && alf_cu_control_flag ) { |  |
| cuDepth = Log2MaxCUSize – log2CUSize |  |
| if( cuDepth <= alf_cu_control_max_depth ) |  |
| if( cuDepth == alf_cu_control_max_depth ¦¦ |  |
| split_coding_unit_flag[ x0 ][ y0 ] == 0 ) |  |
| AlfCuFlagIdx++ |  |
| } |  |
| if( split_coding_unit_flag[ x0 ][ y0 ] ) { |  |
| if( cu_qp_depta_enabled_flag && log2CUSize == log2MinCUDQPSize ) |  |
| IsCuQpDeltaCoded = 0 |  |
| x1 = x0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| if( cuAddress( x1, y0 ) > SliceAddress ) |  |
| moreDataFlag = coding_tree( x0, y0, log2CUSize – 1 ) |  |
| if( cuAddress( x0, y1 ) > SliceAddress && moreDataFlag && x1 < PicWidthInSamplesL ) |  |
| moreDataFlag = coding_tree( x1, y0, log2CUSize − 1 ) |  |
| if( cuAddress( x1, y1 ) > SliceAddress && moreDataFlag && y1 < PicHeightInSamplesL ) |  |
| moreDataFlag = coding_tree( x0, y1, log2CUSize − 1 ) |  |
| if( moreDataFlag && x1 < PicWidthInSamplesL && y1 < PicHeightInSamplesL ) |  |
| moreDataFlag = coding_tree( x1, y1, log2CUSize − 1 ) |  |
| } else { |  |
| if(adaptive_loop_filter_flag && alf_cu_control_flag ) |  |
| AlfCuFlag[ x0 ][ y0 ] = alf_cu_flag[ AlfCuFlagIdx ] |  |
| if(NumPCMBlock == 0) |  |
| coding_unit( x0, y0, log2CUSize ) |  |
| else |  |
| NumPCMBlock-- |  |
| if(NumPCMBlock == 0) { |  |
| if( !entropy_coding_mode_flag ) |  |
| moreDataFlag = more_rbsp_data( ) |  |
| else { |  |
| if( granularity_block_boundary( x0, y0, log2CUSize ) ) { |  |
| end_of_slice_flag | ae(v) |
| moreDataFlag = !end_of_slice_flag |  |
| } else |  |
| moreDataFlag = 1 |  |
| } |  |
| } |  |
| else |  |
| moreDataFlag = 1 |  |
| } |  |
| return moreDataFlag |  |
| } |  |

7.3.6	Coding unit syntax

| coding_unit( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| if( entropy_coding_mode_flag && slice_type != I ) |  |
| skip_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0 ) |  |
| else { |  |
| if( !entropy_coding_mode_flag ) { |  |
| if( slice_type == I && log2CUSize == Log2MinCUSize ) |  |
| intra_part_mode | u(1) |
| } else if( slice_type != I ¦ ¦ log2CUSize = = Log2MinCUSize ) |  |
| pred_type | u(v) ¦ ae(v) |
| x1 = x0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| x2 = x1 − ( ( 1 << log2CUSize ) >> 2 ) |  |
| y2 = y1 − ( ( 1 << log2CUSize ) >> 2 ) |  |
| x3 = x1 + ( ( 1 << log2CUSize ) >> 2 ) |  |
| y3 = y1 + ( ( 1 << log2CUSize ) >> 2 ) |  |
| if( PartMode == PART_2Nx2N ) { |  |
| prediction_unit( x0, y0, log2CUSize ) |  |
| } else if( PartMode == PART_2NxN ) { |  |
| prediction_unit( x0, y0, log2CUSize ) |  |
| prediction_unit( x0, y1, log2CUSize ) |  |
| } else if( PartMode == PART_Nx2N ) { |  |
| prediction_unit( x0, y0, log2CUSize ) |  |
| prediction_unit( x1, y0, log2CUSize ) |  |
| } else if( PartMode == PART_2NxnU ) { |  |
| prediction_unit( x0, y0, log2CUSize ) |  |
| prediction_unit( x0, y2, log2CUSize ) |  |
| } else if( PartMode == PART_2NxnD ) { |  |
| prediction_unit( x0, y0, log2CUSize ) |  |
| prediction_unit( x0, y3, log2CUSize ) |  |
| } else if( PartMode == PART_nLx2N ) { |  |
| prediction_unit( x0, y0, log2CUSize ) |  |
| prediction_unit( x2, y0, log2CUSize ) |  |
| } else if( PartMode == PART_nRx2N ) { |  |
| prediction_unit( x0, y0, log2CUSize ) |  |
| prediction_unit( x3, y0, log2CUSize ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0, log2CUSize ) |  |
| prediction_unit( x1, y0, log2CUSize ) |  |
| prediction_unit( x0, y1, log2CUSize ) |  |
| prediction_unit( x1, y1, log2CUSize ) |  |
| } |  |
| } |  |
| if( !pcm_flag ) { |  |
| transform_tree( x0, y0, log2CUSize, log2CUSize, 0, 0 ) |  |
| transform_coeff( x0, y0, log2CUSize, log2CUSize, 0, 0 ) |  |
| transform_coeff( x0, y0, log2CUSize, log2CUSize, 0, 1 ) |  |
| transform_coeff( x0, y0, log2CUSize, log2CUSize, 0, 2 ) |  |
| } |  |
| } |  |

7.3.7	Prediction unit syntax

| prediction_unit( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| merge_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else if( PredMode = = MODE_INTRA ) { |  |
| if( PartMode == PART_2Nx2N && log2CUSize >= Log2MinIPCMCUSize ) |  |
| pcm_flag | u(1) ¦ ae(v) |
| if( pcm_flag ) { |  |
| subsequent_pcm_num | u(v) ¦ ae(v) |
| NumPCMBlock=1 + subsequent_pcm_num |  |
| while ( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | u(v) |
| for(j = 0; j < NumPCMBlock; j++ ){ |  |
| for( i = 0; i < 1 << ( log2CUSize << 1 ); i++ ) |  |
| pcm_sample_luma[ i ] | u(v) |
| for( i = 0; i < ( 1 << ( log2CUSize << 1 ) ) >> 1; i++ ) |  |
| pcm_sample_chroma[ i ] | u(v) |
| } |  |
| NumPCMBlock-- |  |
| } else { |  |
| prev_intra_luma_pred_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( prev_intra_luma_pred_flag[ x0 ][ y0 ] ) |  |
| if( NumMPMCand > 1 ) |  |
| mpm_idx[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 ][ y0 ] | ce(v) ¦ ae(v) |
| if( IntraPredMode[ x0 ][ y0 ] == 2 ) |  |
| planar_flag_luma[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| SignaledAsChromaDC = ( chroma_pred_from_luma_enabled_flag ? intra_chroma_pred_mode[ x0 ][ y0 ] == 3 : intra_chroma_pred_mode[ x0 ][ y0 ] == 2 ) |  |
| if( IntraPredMode[ x0 ][ y0 ] != 2 && IntraPredMode[ x0 ][ y0 ]!=34 && SignaledAsChromaDC ) |  |
| planar_flag_chroma[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| } |  |
| } else { /* MODE_INTER */ |  |
| if( entropy_coding_mode_flag ¦¦ PartMode != PART_2Nx2N ) |  |
| merge_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( merge_flag[ x0 ][ y0 ] ) { |  |
| merge_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else { |  |
| if( slice_type = = B ) { |  |
| if( !entropy_coding_mode_flag ) { |  |
| combined_inter_pred_ref_idx | ue(v) |
| if( combined_inter_pred_ref_idx == MaxPredRef ) |  |
| inter_pred_flag[ x0 ][ y0 ] | ue(v) |
| } else |  |
| inter_pred_flag[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| if( inter_pred_flag[ x0 ][ y0 ] = = Pred_LC ) { |  |
| if( num_ref_idx_lc_active_minus1 > 0 ) { |  |
| if( !entropy_coding_mode_flag ) { |  |
| if( combined_inter_pred_ref_idx == MaxPredRef ) |  |
| ref_idx_lc_minus4[ x0 ][ y0 ] | ue(v) |
| } else |  |
| ref_idx_lc[ x0 ][ y0 ] | ae(v) |
| } |  |
| mvd_lc[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_lc[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| mvp_idx_lc[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| else { /* Pred_L0 or Pred_BI */ |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) { |  |
| if( !entropy_coding_mode_flag ) { |  |
| if( combined_inter_pred_ref_idx == MaxPredRef ) |  |
| ref_idx_l0_minusX[ x0 ][ y0 ] | ue(v) |
| } else |  |
| ref_idx_l0_minusX[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| mvd_l0[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_l0[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| mvp_idx_l0[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| if( inter_pred_flag[ x0 ][ y0 ] = = Pred_BI ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) { |  |
| if( !entropy_coding_mode_flag ) { |  |
| if( combined_inter_pred_ref_idx == MaxPredRef ) |  |
| ref_idx_l1_minusX[ x0 ][ y0 ] | ue(v) |
| } else |  |
| ref_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| mvd_l1[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_l1[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| mvp_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

### subsequent_pcm_num specifies the number of subsequent I-PCM coding units associated with the current log2CUSize that successively follow the current I-PCM coding unit. The value subsequent_pcm_num shall be in the range of 0 to 3, inclusive. When the subsequent_pcm_num is not present, it shall be inferred to be 0.

The binary representations and variable length codes of given a subsequence_pcm_num value are specified in Table A and Table B, respectively.

Table A: Binary representation of subsequence_pcm_num syntax. (Note that one context will be assigned for 1st bin and another context will be assigned for remaining bins.)

| subsequent_pcm_num | binary representation |  |  |
| --- | --- | --- | --- |
|  | 1st | 2nd | 3rd |
| 0 | 0 |  |  |
| 1 | 1 | 0 |  |
| 2 | 1 | 1 | 0 |
| 3 | 1 | 1 | 1 |

Table B: Variable length code  of subsequence_pcm_num syntax

| subsequent_pcm_num | variable length code |
| --- | --- |
| 0 | 0 |
| 1 | 10 |
| 2 | 110 |
| 3 | 111 |