| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G119 |
| --- | --- |

| Title: | Modifications to intra frame coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongbing Lin Haitao Yang Lingzhi Liu Jianhua Zheng Haoping Yu | Tel: Email: | +8610-60612142 yblin@huawei.com |
| Source: | Huawei Technologies |  |  |

_____________________________

# Abstract

This contribution presents three modifications to intra frame coding: (1) down-sampling filter for left reference luma samples in LM mode is changed from 2-tap to 4-tap, (2) removal of the remapping tables used in MPM prediction, (3) simplification of Intra mode coding by using FLC instead of VLC. As a result, it is reported that BD-rate results for the first modification is: AI-HE: Y: 0.0, U: -0.2, V: -0.2, and AI-LC: Y: 0.0, U: -0.2, V: -0.2. The second modification removes three MPM remapping tables and takes planar mode as the remapped mode. BD-Rate results for this modification are reported to be: AI-HE: Y: 0.0, U: 0.1, V: 0.0, and AI-LC: Y: 0.0, U: 0.0, V: 0.1. The third modification enables total number of the remaining Intra modes to be expressed as powers of 2. Meanwhile Intra mode coding method in Intra LC configuration is changed from VLC to FLC, which unifies binarization for Intra mode coding in both HE and LC Intra configurations. BD-Rate results for this harmonization are reported to be: AI-HE: Y: -0.1, U: 0.0, V: 0.0, and AI-LC: Y: 0.1, U: 0.1, V: 0.1.

# Introduction

This contribution presents three modifications to Intra frame coding. One is related to down-sampling filter selection in LM mode for performance improvement, the other two modifications are related to Intra mode coding for complexity reduction.

# Proposed modifications

## Down-sampling filter in LM mode

Luma-based chroma Intra prediction (LM) has been adopted into HEVC Test Model [1] [2]. Chroma values in a block are predicted from down sampled reconstructed luma values of same block. In the down-sampling process, HM4.0 uses a 3-tap horizontal filter [1, 2, 1] to down-sample above neighbor reconstructed luma samples and a 2-tap vertical filter [1, 1]’ for left neighbor reconstructed luma samples.

A 4-tap vertical filter [1, 3, 3, 1]’, instead of 2-tap filter, is proposed to be used to down-sample the left neighbor reconstructed luma samples. Table below shows simulation results when the proposed 4-tap vertical filter is used. BD-rate saving of 0.2% for chroma components can be observed.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.5% | -0.6% | 0.0% | -0.5% | -0.6% |
| Class B | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| Class C | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| Class D | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% |
| Class E | 0.0% | -0.1% | -0.2% | 0.0% | -0.1% | -0.2% |
| Overall | 0.0% | -0.2% | -0.2% | 0.0% | -0.2% | -0.2% |
|  | 0.0% | -0.2% | -0.2% | 0.0% | -0.2% | -0.2% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

## Removal of MPM remapping tables

In Intra mode coding, MPM of current block is predicted based on neighbor blocks. When Intra mode number of neighbor block is larger than the maximum Intra mode number of current block, a remapping table is used to obtain the MPM of current block. There are three tables used in HM4.0 since different block size uses different total number of Intra modes.

For example, in HM4.0 these remapping tables are used as follows:

if ( iLeftIntraDir >= g_aucIntraModeNumAng[iIdx] )

{

if ( g_aucIntraModeNumAng[iIdx] == 6 )

{

iLeftIntraDir = g_aucAngModeMapping[0][g_aucAngIntraModeOrder[iLeftIntraDir]];

}

else if ( g_aucIntraModeNumAng[iIdx] == 4 )

{

iLeftIntraDir = g_aucAngModeMapping[3][g_aucAngIntraModeOrder[iLeftIntraDir]];

}

else

{

iLeftIntraDir = g_aucAngModeMapping[1][g_aucAngIntraModeOrder[iLeftIntraDir]];

}

}

It is proposed to remove all these remapping tables and take planar mode as default MPM mode.

For example, the proposed text is:

if ( iLeftIntraDir >= g_aucIntraModeNumAng[iIdx] )

{

iLeftIntraDir = Intra_Planar;

}

In addition, when neighbor block is not available or is not an Intra coded block, the default MPM mode is also set to planar mode.

Table below shows BD-Rate results for the modifications.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class C | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% | 0.1% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| Class E | 0.1% | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% |
| Overall | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

## Harmonization of Intra mode coding

Total number of Intra modes is reduced by introduction of one bi-prediction mode which merges two existing diagonal Intra modes (VOR+8 and VER+8). The bi-prediction mode averages two prediction values of VOR+8 and VER+8 modes. As a result, the total number of remaining Intra modes can be expressed as powers of 2 for all block sizes, as shown in table below. It brings the following advantages:

| Block size | Total number of Intra modes ( HM4.0 / proposal ) | Total number of remaining Intra modes ( HM4.0 / proposal ) |
| --- | --- | --- |
| 2x2 | 4 / 4 | 2 / 2 |
| 4x4 | 18 / 18 | 16 / 16 |
| 8x8 | 35 / 34 | 33 / 32 |
| 16x16 | 35 / 34 | 33 / 32 |
| 32x32 | 35 / 34 | 33 / 32 |
| 64x64 | 4 / 4 | 2 / 2 |

- For 8x8/16x16/32x32 block sizes, total number of remaining Intra modes is reduced from 33 to 32. For Intra HE configuration, the escape bit in HM4.0 used to coding total 33 remaining Intra modes can be removed. No condition checking of the escape bit helps to increase parsing throughput.

- For block size of 4x4, the total number of Intra modes remains 18, which actually includes an additional prediction mode (HOR+6). As shown in following figure, better symmetric distribution of prediction modes can be observed [3].

Fig. 1 Intra prediction modes for 4x4 block

Secondly, it is proposed to use FL code to replace VLC in Intra mode coding for Intra LC configuration. As a result, binarization for Intra mode coding in both Intra HE and LC configurations can be unified. Furthermore, the complexity is reduced in the following aspects: a) storage saving by removing the code table, b) computational complexity saving due to no code table adaptation, c) increased parsing throughput.

The table below shows the test results of above two modifications. It is shown the proposed harmonization can achieve complexity reduction with almost no performance loss.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | 0.0% | 0.1% | 0.1% | 0.2% | 0.2% |
| Class B | -0.1% | 0.0% | 0.1% | 0.2% | 0.1% | 0.2% |
| Class C | -0.1% | -0.1% | -0.1% | 0.1% | 0.0% | 0.1% |
| Class D | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| Class E | -0.1% | 0.0% | 0.1% | 0.2% | 0.3% | 0.4% |
| Overall | -0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% |
|  | -0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

As for implementation of the proposed bi-prediction Intra mode, it is designed to average the main and side reference samples before performing angular Intra prediction. Only one change to WD text needs to be made as follows:

8.3.3.1.6 Specification of Intra_Angular prediction mode

…

The reference pixel array refMain[ x ], with x=-nS..2*nS is specified as follows.

–	If intraPredOrder is equal to 17,

refMain[ x ] = ( p[ -1+x, -1 ] + p[ -1, -1+x ] +1 )>>1,  with x=0..2*nS	(8-1)

–	Otherwise, if intraPredOrder is less than 18,

refMain[ x ] = p[ -1+x, -1 ], with x=0..nS		(8-2)

- If intraPredAngle is less than 0,

refMain[ x ] = p[ -1, -1+( ( x*invAngle+128 )>>8 ) ], with x=( nS*intraPredAngle ) >>5..-1	(8-3)

- Otherwise,

refMain[ x ] = p[ -1+x, -1 ], with x=nS+1..2*nS		(8-4)

Otherwise,

…

# References

- J. Chen et. al., “CE6.a.4: Chroma intra prediction by reconstructed luma samples”, JCTVC-E266, JCTVC meeting, Geneva, Switzerland, 16-23 March, 2011.

- J. Chen et. al., “BoG: Chroma intra prediction by reconstructed luma samples”, JCTVC-F907, JCTVC meeting, Torino, IT, 14-22 July, 2011.

- Toru Kumakura, Shigeru Fukushima, Motoharu Ueda, “Fixing the number of mpm candidates”, JCTVC-F340, JCTVC meeting, Torino, IT, 14-22 July, 2011.

- Frank Bossen, “Common test conditions and software reference configurations”, Doc. JCTVC-F900, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Torino, IT, July 2011.

# Acknowledge

We would like to thank NEC for the crosscheck.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).