The proposed change to WD4 text

# Decoding process

## Decoding process for coding units coded in intra prediction mode

### Derivation process for luma intra prediction mode

Inputs to this process are:

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture,

–	a variable log2PUSize specifying the size of the current prediction unit,

–	variable arrays IntraPredMode (If available) that are previously (in decoding order) derived for adjacent coding units.

Output of this process is the variable IntraPredMode[ xB ][ yB ].

Table 8-1 specifies the value for the intra prediction mode and the associated names.

Table 8-1 – Specification of intra prediction mode and associated names

| Intra prediction mode | Associated names |
| --- | --- |
| 0 | Intra_Planar |
| 1 | Intra_Vertical |
| 2 | Intra_Horizontal |
| 3 | Intra_DC |
| Otherwise (4..33) | Intra_Angular |
| 34 | Intra_FromLuma (used only for chroma) |

Table 8-2 specifies the number of luma intra prediction modes intraPredModeNum depending on log2PUSize.

Table 8-2 – Specification of intraPredModeNum

| log2PUSize | intraPredModeNum |
| --- | --- |
| 2 | 18 |
| 3 | 34 |
| 4 | 34 |
| 5 | 34 |
| 6 | 4 |

-

IntraPredMode[ xB ][ yB ] labelled 0, 1, 2, .., 34 represents directions of predictions as illustrated in Figure 8-1.

4

18

10

19

5

20

11

21

1

22

12

23

6

24

25

13

7

0:

Intra_Planar

3:

Intra_DC

34:

Intra_FromLuma

26

14

27

8

28

15

29

2

30

16

31

9

32

17

33

7

Figure 8-1 – Intra prediction mode directions (informative)

IntraPredMode[ xB ][ yB ] is derived as the following ordered steps. [Ed. (WJ): proponent suggests to move this part to the syntax since the other syntax elements utilize IntraPredMode. But it seems too complex to move all the following process to the syntax table. Maybe it’s better to move this part to the semantics section or simply avoid the use of IntraPredMode to parse the syntax item]

- The derivation process for neighbouring treeblocks specified in subclause XXX with ( xB,  yB ) given as input and the output is assigned to tbAddrA and tbAddrB specifying the treeblock addresses of treeblocks covering ( xBA,  yBA ) and ( xBB, yBB ) respectively where ( xBA,  yBA ) is set equal to ( xB-1,  yB ) and ( xBB,  yBB ) is set equal to ( xB,  yB-1 ).

- For N being either replaced A or B, the variables intraPredModeN are derived as follows.

- If the treeblock with address tbAddrN is not available, intraPredModeN is set equal to Intra_Planar

- Otherwise, if the coding unit covering ( xBN,  yBN ) is not coded as intra mode, intraPredModeN is set equal to Intra_Planar,

- Otherwise, If intraPredModeN is greater than or equal to intraPredModeNum, intraPredModeN is set equal to Intra_Planar,

- Otherwise, intraPredModeN is set equal to IntraPredMode[ xBN ][ yBN ], where IntraPredMode is the variable array assigned to the coding unit covering the luma location ( xBN, yBN ).

- if both candIntraPredModeN (N=A or B) are the same, the candIntraPredModeA is derived as follows:

- If candIntraPredModeA is not equal to 0, the value 0 is assigned to candIntraPredModeA

- Otherwise, the value 3 is assigned to candIntraPredModeA

- 4.  The candModeList[x] is  derived as follows:

- both candIntraPredModeN are assigned to the candidate modes list with the smaller of the two candidates at candModeList[ 0 ] and the larger at candModeList[ 1 ].

- IntraPredMode[ xB ][ yB ] is derived by applying the following procedure:

- If prev_intra_pred_flag[ xB ][ yB ] is true, the IntraPredMode[ xB ][ yB ] is set equal to  candModeList[mpm_idx][ xB ][ yB ]]

- Otherwise IntraPredMode[ xB ][ yB ] is derived by applying the following ordered steps:

  - IntraPredMode[ xB ][ yB ] = rem_intra_pred_mode

  - If candModeList[ 0 ] is greater than candModeList[ 1 ], then two values are swapped. [Ed. (WJ): may be better expression needed]

  - When IntraPredMode[ xB ][ yB ] is equal or greater than candModeList[ 0 ], the value of IntraPredMode[ xB ][ yB ] is increased by one

  - When IntraPredMode[ xB ][ yB ] is equal or greater than candModeList[ 1 ], the value of IntraPredMode[ xB ][ yB ] is increased by one

### Derivation process for chroma intra prediction mode

#### Intra sample prediction

##### Specification of Intra_Angular prediction mode

Inputs to this process are:

–	neighbouring samples p[ x, y ], with x, y = -1..2*nS-1,

–	a variable nS specifying the prediction size.

Output of this process is:

–	predicted samples predSamples[ x, y ], with x, y =0..nS-1.

This intra prediction mode is invoked when intraPredMode is in the range of 3..33.

Table 8-4 specifies the mapping table between intraPredMode and the rearranged intra prediction order intraPredOrder.

Table 8-4 – Specification of intraPredOrder

| intraPredMode | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| intraPredOrder | - | - | - | - | 1 | 5 | 13 | 17 | 21 | 29 | 33 | 3 | 7 | 11 | 15 | 19 | 23 | 27 |
| intraPredMode | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |  |
| intraPredOrder | 31 | 2 | 4 | 6 | 8 | 10 | 12 | 14 | 16 | 18 | 20 | 22 | 24 | 26 | 28 | 30 | 32 |  |

Figure 8-2 illustrates the total 34 intra angles and Table 8-5 specifies the mapping table between intraPredOrder and the angle parameter intraPredAngle.

Figure 8-2 – Intra prediction angle definition (informative)

Table 8-5 – Specification of intraPredAngle

| intraPredOrder | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| intraPredAngle | - | -32 | -26 | -21 | -17 | -13 | -9 | -5 | -2 | - | 2 | 5 | 9 | 13 | 17 | 21 | 26 |
| intraPredOrder | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |
| intraPredAngle | 32 | -26 | -21 | -17 | -13 | -9 | -5 | -2 | - | 2 | 5 | 9 | 13 | 17 | 21 | 26 | 32 |

Table 8-6 further specifies the mapping table between intraPredOrder and the inverse angle parameter invAngle.

Table 8-6 – Specification of invAngle

| intraPredOrder | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| invAngle | -256 | -315 | -390 | -482 | -630 | -910 | -1638 | -4096 |
| intraPredOrder | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 |
| invAngle | -315 | -390 | -482 | -630 | -910 | -1638 | -4096 | - |

The reference pixel array refMain[ x ], with x=-nS..2*nS is specified as follows.

–	If intraPredOrder is equal to 17,

refMain[ x ] = ( p[ -1+x, -1 ] + p[ -1, -1+x ] +1 )>>1,  with x=0..2*nS	(8-1)

–	Otherwise,  if intraPredOrder is less than 18,

refMain[ x ] = p[ -1+x, -1 ], with x=0..nS		(8-2)

- If intraPredAngle is less than 0,

refMain[ x ] = p[ -1, -1+( ( x*invAngle+128 )>>8 ) ], with x=( nS*intraPredAngle ) >>5..-1	(8-3)

- Otherwise,

refMain[ x ] = p[ -1+x, -1 ], with x=nS+1..2*nS		(8-4)

Otherwise,

refMain[ x ] = p[ -1, -1+x ], with x=0..nS		(8-5)

- If intraPredAngle is less than 0,

refMain[ x ] = p[ -1+( ( x*invAngle+128 )>>8 ), -1 ], with x=( nS*intraPredAngle ) >>5..-1	(8-6)

- Otherwise,

refMain[ x ] = p[ -1, -1+x ], with x=nS+1..2*nS		(8-7)

The values of the prediction samples predSamples[ x, y ], with x, y = 0..nS-1 are derived by the following procedures.

–	The index variable iIdx and the multiplication factor iFact are derived by

iIdx = ( ( y + 1 )*intraPredAngle ) >> 5		(8-8)

iFact = ( ( y + 1 )*intraPredAngle ) && 31		(8-9)

–	Depending on the value of iFact, the following applies.

- If iFact is not equal to 0, the value of the prediction samples predSamples[ x, y ] is derived by

predSamples[ x, y ] = ( ( 32 – iFact )*refMain[ x+iIdx+1 ] + iFact*refMain[ x+iIdx+2] + 16 ) >> 5	(8-10)

- Otherwise, the value of the prediction samples predSamples[ x, y ] is derived by

predSamples[ x, y ] = refMain[ x+iIdx+1 ]		(8-11)

##### Specification of Intra_FromLuma prediction mode

Inputs to this process are:

–	a sample location ( xB, yB ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

–	neighbouring samples p[ x, y ], with x, y = -1..2*nS-1,

–	a variable nS specifying the prediction size.

Output of this process is:

–	predicted samples predSamples[ x, y ], with x, y =0..nS-1.

This intra prediction mode is invoked when intraPredMode is equal to 35.

The values of the prediction samples predSamples[ x, y ], with x, y = 0..nS-1, are derived as the following ordered steps:

- Variable k3 and the sample array pY’ are derived as:

k3 = Max( 0, BitDepthC + log2( nS ) – 14 )		(8-12)

pY’[ x, y ] = ( recSamplesL[ 2x-1, 2y+1 ] + 
2*recSamplesL[ 2x, 2y+1 ] + recSamplesL[ 2x+1, 2y+1 ] + 2 ) >> 2, with x=0..nS-1, y = -1	(8-13)

pY’[ x, y ] = ( recSamplesL[ 2x, 2y ] + recSamplesL[ 2x, 2y+1 ] ) >> 1, with x=0..nS-1, y = 0..nS-1	(8-14)

pY’[ x, y ] = ( recSamplesL[ 2x, 2y-1 ] + 3*recSamplesL[ 2x, 2y ] + 3*recSamplesL[ 2x, 2y+1 ] + recSamplesL[ 2x, 2y+2 ] +4) >> 3,      with x=-1, y = 0..nS-1	(8-15)

- Variables L, C, LL, LC and k2 are derived as follows:

L = y=0nS−1pY'[−1, y]+x=0nS−1pY'[x, −1]>>k3		(8-16)

C = y=0nS−1p[−1, y]+x=0nS−1p[x, −1]>>k3		(8-17)

LL = y=0nS−1pY'[−1, y]2+x=0nS−1pY'[x, −1]2>>k3	(8-18)

LC = y=0nS−1pY'[−1, y]∗p[−1, y]+y=0nS−1pY'[x, −1]∗p[x,−1]>>k3	(8-19)

k2 = log2( (2*nS) >> k3 )		(8-20)

- Variables a, b and k are derived as:

a1 	= ( LC << k2 ) – L*C		(8-21)
a2 	= ( LL << k2 ) – L*L		(8-22)
k1 	= Max( 0, log2( abs( a2 ) ) – 5 ) – Max( 0, log2( abs( a1 ) ) – 14 ) + 2	(8-23)
a1s	= a1 >> Max(0, log2( abs( a1 ) ) – 14 )		(8-24)
a2s	= abs( a2 >> Max(0, log2( abs( a2 ) ) – 5 ) )		(8-25)
a3	= a2s < 1 ? 0 : Clip3( -215, 215-1, a1s*lmDiv + ( 1 << ( k1 – 1 ) ) >> k1 )	(8-26)

a	= a3 >> Max( 0, log2( abs( a3 ) ) – 6 )		(8-27)
k	= 13 – Max( 0, log2( abs( a ) ) – 6 )		(8-28)

b	= ( L – ( ( a*C ) >> k1 ) + ( 1 << ( k2 – 1 ) ) ) >> k2	(8-29)

where lmDiv is specified in Table 8-7 with the input a2s.

- The values of the prediction samples predSamples[ x, y ] are derived as:

predSamples[ x, y ] = Clip1C( ( ( pY’[ x, y ] * a ) >> k ) + b ), with x, y = 0..nS-1	(8-30)

Table 8-7 – Specification of lmDiv

| a2s | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| lmDiv | 32768 | 16384 | 10923 | 8192 | 6554 | 5461 | 4681 | 4096 | 3641 | 3277 | 2979 | 2731 | 2521 |
| a2s | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 |
| lmDiv | 2341 | 2185 | 2048 | 1928 | 1820 | 1725 | 1638 | 1560 | 1489 | 1425 | 1365 | 1311 | 1260 |
| a2s | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 |
| lmDiv | 1214 | 1170 | 1130 | 1092 | 1057 | 1024 | 993 | 964 | 936 | 910 | 886 | 862 | 840 |
| a2s | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 |
| lmDiv | 819 | 799 | 780 | 762 | 745 | 728 | 712 | 697 | 683 | 669 | 655 | 643 | 630 |
| a2s | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 | 64 |  |
| lmDiv | 618 | 607 | 596 | 585 | 575 | 565 | 555 | 546 | 537 | 529 | 520 | 512 |  |

# Parsing process

## CAVLC parsing process for slice data

### Initialisation process

#### - Specification of intraModeTable[k][codeNum]

### Parsing process for transform coefficient syntax elements

#### Parsing process for rem_intra_luma_pred_mode

This process is invoked when entropy_coding_mode_flag is equal to 0 for parsing syntax element rem_intra_luma_pred_mode in subclause Error: Reference source not found.

Inputs to this process are bits from slice data, a variable log2CUSize specifying the size of the current coding unit

Outputs of this process are the syntax element rem_intra_luma_pred_mode

The variable log2PUSize is derived as follows.

–	If PartMode is equal to PART_NxN, log2PUSize is set equal to log2CUSize – 1,

–	Otherwise, log2PUSize is set equal to log2CUSize.

The value of rem_intra_luma_pred_mode is derived as follows.

–	Based on log2PUSize, the value of a variable intraPredModeNum indicating the number of intra prediction modes for the given size of prediction unit is obtained according to Table 8-2.

–	If intraPredModeNum is equal to 4, read one bit and assign its value to rem_intra_luma_pred_mode. No further step is carried out. Othewise,

–	If intraPredModeNum is equal to 18, read 4 bits and assign its value to rem_intra_luma_pred_mode. No further step is carried out. Othewise,

–	If intraPredModeNum is equal to 34, read 5 bits and assign its value to rem_intra_luma_pred_mode. No further step is carried out.

  - CABAC parsing process for slice data

### Binarization process

#### Binarization process for rem_intra_luma_pred_mode

Input to this process is a request for the syntax element rem_intra_luma_pred_mode and cNumBins.

Output of this process is the binarization of the syntax element.

The binarization for rem_intra_luma_pred_mode is given by Table 9-9.

Table 9-9 – Binarization for rem_intra_luma_pred_mode

| Value of rem_intra_luma_pred_mode | Bin string |
| --- | --- |
| less than 32 | FL, cMax = cNumBins |