| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G1204 |
| --- | --- |

| Title: | Description of Core Experiment 4: Quantization |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | CE Description |  |  |
| Author(s) or Contact(s): | K. Sato (Sony) M. Budagavi (TI) M. Coban (Qualcomm) H. Aoki (NEC) X. Li (MediaTek) | Email: | Kazushi.Sato@jp.sony.com madhukar@ti.com mcoban@qualcomm.com hao@bu.jp.nec.com x.li@mediatek.com |
| Source: | CE Coordinators |  |  |

_____________________________

# Introduction

This document provides description on CE4: Quantization. This core experiment contains following subtests:

Subtest 1: QP Prediction

Subtest 2: Quantization Matrices

Subtest 3: QP Signaling Unit Granularity

Subtest 4: Fine Granularity of Quantization Step

# Participants

| Participant | Person, Email address | P | C |
| --- | --- | --- | --- |
| Canon | M.Shima (shima.masato@canon.co.jp) | X | X |
| HiSilicon | J. Zheng (zhengjianhua@huawei.com) X.Zheng (zhengxiaozhen@huawei.com) | X | X |
| JVCKenwood | H. Nakamura (nakamura.hiroya@jvckenwood.com) S. Fukushima (fukushima.shigeru@jvckenwood.com) M. Nishitani (nishitani.masayoshi@jvckenwood.com) | X | X |
| MediaTek | X.Li (x.li@mediatek.com) X. Guo (xun.guo@mediatek.com) S.Liu (Shan.Liu@mediatek.com) X.Zhang (ximin.zhang@mediatek.com) | X | X |
| Motorola Mobility | K. Panusopone (krit@motorola.com) | X | X |
| NEC | H.Aoki (hao@bu.jp.nec.com) K.Chono (chono@ct.jp.nec.com) | X | X |
| Qualcomm | M. Coban (mcoban@qualcomm.com) R. Joshi (rajanj@qualcomm.com) X. Wang (xianglin@qualcomm.com) | X |  |
| Samsung | C. Kim (dionism@samsung.com) T. Lee (tammy.lee@samsung.com) | X |  |
| Sony | M. Haque (munsi.haque@am.sony.com) E. Maani (ehsan.maani@am.sony.com) K.Sato (Kazushi.Sato@jp.sony.com) T.Suzuki (teruhikos@jp.sony.com) A. Tabatabai (ali.tabatabai@am.sony.com) J.Xu (Jun.Xu@am.sony.com) | X | X |
| TI | M.Zhou (zhou@ti.com) | X |  |

# Tools under Test

## Subtest 1: QP Prediction

Subtest 1.1: AVC-style QP prediction

QP of the preceding quantization group of coding units, which is hereafter referred to as QGCU in this document, in scan order is always employed as the predicted QP.

Subtest 1.2: HM 4 QP prediction with reset to preceding CU QP in scan order at LCU boundary

- If the left neighbouring QGCU is located within the current LCU, QP of the left QGCU is employed as the predicted QP.

- Otherwise, QP of the preceding QGCU in scan order is employed as the predicted QP.

Subtest 1.3: JCTVC-G1028 Rate control friendly spatial QP prediction

CU, PU and QGCUs employed in this method are illustrated in Figure 1.3.1. Brief algorithm of the proposed method is described as follows. In the following description, each QGCU which is outside of the LCU is regarded as unavailable, as shown in Figure 1.3.2.

- If the top-left-most CU in the current QGCU is intra-coded, the predicted QP is derived along intra prediction direction of the top-left-most PU with the G357 method, as shown in Figure 1.3.3.

- Otherwise if at least either of the above QGCU or the left QGCU is available within the LCU, predicted QP is calculated by averaging QPs of those QGCUs available.

- Otherwise, QP of the preceding QGCU in scan order is employed as the predicted QP.

Figure 1.3.1: CU, PU and neighbouring QGCUs employed for QP prediction of the subtest 1.3 method.

Figure 1.3.2: QP reference relationship between QGCUs.

Figure 1.3.3: QP prediction along intra prediction direction.

Subtest 1.4: JCTVC-G353 Non-CE4: QP prediction using spatial QP

Brief algorithm of Subtest 1.4 is described as follows.

In the following condition, each neighbouring QGCU which is not in the current LCU is regarded as not available in the same way as in subtest 1.3.

- If both the left QGCU and the above QGCU are not in the current LCU (If the current QGCU is the most top-left in the current LCU), the predictive QP is set equal to the QP of the preceding QGCU in scan order.

- Otherwise if a neighbouring QGCU is not available, the predictive QP is set equal to the QP of the neighboring QGCU which is available.

- Otherwise, if the QPL* is equal to the QPAL*, the predictive QP is derived as follows:	predictiveQP=QPL+3∗QPA+2>>2

- Otherwise, if the QPA* is equal to the QPAL, the predictive QP is derived as follows:	predictiveQP=3∗QPL+QPA+2>>2

- Otherwise, the predictive QP is derived as follows:
	predictiveQP=QPL+QPA+1>>1

Remarks:

- QPL: QP of the left neighbouring QGCU

- QPA: QP of the above neighbouring QGCU

- QPAL: QP of the above-left neighbouring QGCU

- predictiveQP=QPL+3∗QPA+2>>2

- predictiveQP=3∗QPL+QPA+2>>2

- predictiveQP=QPL+QPA+1>>1

Assignment

| Test | Proponent (Contact Person) | Cross-Checker (Contact Person) |
| --- | --- | --- |
| 1.1 | Sony: K.Sato (Kazushi.Sato@jp.sony.com) | NEC: H.Aoki (hao@bu.jp.nec.com) |
| 1.2 | NEC: H.Aoki (hao@bu.jp.nec.com) | Sony: K.Sato (Kazushi.Sato@jp.sony.com) |
| 1.3 | NEC: H.Aoki (hao@bu.jp.nec.com) Canon: M.Shima (shima.masato@canon.co.jp) Qualcomm: M. Coban (mcoban@qualcomm.com) Sony: K. Sato (Kazushi.Sato@jp.sony.com) J.Xu (Jun.Xu@am.sony.com) | JVCKenwood: H. Nakamura (nakamura.hiroya@jvckenwood.com) |
| 1.4 | JVCKenwood: H. Nakamura (nakamura.hiroya@jvckenwood.com) | HiSilicon: J. Zheng (zhengjianhua@huawei.com) |

Evaluation

TM-5 step 3, which has already been implemented on HM-5.0 as an encoding method, shall be used for QP adaptation method with modulation by ±12. This will be enabled by command option “-aq 1 –aqr 12”. MaxCUDepthDQP =3 [8x8] is mandatory and 2 [16x16], 1 [32x32] and 0 [64x64] are options. Other seting is same as the one specified in [1].

Each proposal will be evaluated based on coding efficiency (BD-rate) and complexity (enc/dec time). Additionally dQP bit increase should be reported. Patch to add functionality of dQP incr output shall be provided by NEC/Sony with the timeline specified in section 4 of this description. Subtest 1.1 is the anchor for this subtest. Software will also be provided with the timeline in section 4.

## Subtest 2: Quantization Matrices

Subtest 2.1: JCTVC-G083, JCTVC-G094 and JCTVC-G434: Enhancements of AVC quantization matrix in HEVC

The following combinations of the tools proposed in JCTVC-G083, JCTVC-G094 and G434 are tested.

Option 1: Lossless coding

Matrix copy + matrix prediction of same size + DPCM + 135 degree processing + signed/unsigned exp-golomb coding

Option 2a: Lossy coding

Matrix up/dowm-smapling + matrix copy + matrix prediction of arbitrary size + DPCM + 135 degree processing + signed/unsigned exp-golomb coding + up-sampled quantization for 16x16 and 32x32 block size

Option 2b:

Matrix up/dowm-smapling + matrix copy + matrix prediction of arbitrary size + DPCM + 135 degree processing + signed/unsigned exp-golomb coding

Subtest 2.2: JCTVC-G152: Method and syntax for quantization matrices representation

Proposal JCTVC-G152 proposed to implicitly generate 16x4, 4x16, 16x16, 32x8, 8x32 and 32x32 quantization matrices for both default and user defined quantization matrices. The corresponding syntax is also proposed.

In one method in JCTVC-G152, each coefficient of square 16x16 and 32x32 matrices is calculated by the following two steps.

1.    Mapped the coefficient of the smaller available quantization matrix to the coefficient of the bigger quantization matrix if the two coefficients share similar frequency.

2.   Generate the non-mapped coefficient with interpolation from the mapped coefficients.

The following tests are proposed.

Test1: Test implicitly generating 16x16 and 32x32 matrices from 4x4 and 8x8 matrices in “Symmetry1” and “Asymmetry1”.

Test2: Test implicitly generating default 16x16 and 32x32 matrices from down-sampled HVS model based default 16x16 and 32x32 matrices.

Subtest 2.3: JCTVC-G352 Parameterization of Default Quantization Matrices

JCTVC-G352 shows how a 4-parameter model can efficiently represent a set of symmetric, AVC-type, quantization matrices of sizes 4x4 and 8x8. The error in such quantization matrices due to parameterization is negligible and may represent such matrices. This type of parameterization can also save significant amount of memory (more than 8 KB of ROM) at both the encoder and the decoder when matrices are derived during the run time. Larger size matrices (16x16, 32x32) are also derived from parameterization of AVC matrices and scaling. Furthermore, updating the matrices through their parameters is relatively easy if later a better visual quantization matrix is found. Visual quality and model complexity of these AVC-like matrices will be evaluated in the context of HEVC.

Subtest 2.4: JCTVC-G530 Layered quantization matrices compression

This proposal provides a layered representation and coding structure for quantization matrix compression. Matrix elements are divided into multiple layers and coded from lower layer to higher layer. In the coded layers, element is predicted from coded elements in the previous layer and current layer, and the residue of the predicted value and the original element is coded. Matrix element can be reconstructed using the predicted value and residue in lossless way. In predicted layers, matrix elements are not coded and reconstructed using predicted values, hence the generated matrix bits can be reduced. The number of coded layers can be configured in encoder and need to be notified to the decoder. By changing the number of coded layers, it easily shifts from quantization matrix lossless coding to lossy coding. The prediction value of an element in a layer is interpolated by the reconstructed value of reference elements. Vertical interpolation and horizontal interpolation are used in prediction. Other aspects of quantization matrix prediction in JCTVC-G530 are as follows,

–	Copy mode and down-sampling

–	Symmetry mode

–	Residue Coding

–	Differential coding

Fig.X  Layered coding of quantization matrix

Subtest 2.5: JCTVC-G578 Non-CE4: Quantization matrix compression and signaling

The AVC/H.264 method was adopted for the coding of quantization matrices in the 7th JCTVC meeting in Geneva, Switzerland. G578 proposes an alternate method of coding the quantization matrices. It uses raster scanning and prediction based on top and left neighbors. The resulting prediction residual is biased towards positive values. A biased mapping of signed prediction residuals to unsigned values, followed by Golomb coding is proposed to code the prediction residuals. This method is reported to work well with both symmetric and asymmetric matrices. Combinations with other techniques such as sampling rate changes, symmetry flags will also be investigated.

Assignment

| Test | Proponent (Contact Person) | Cross-Checker (Contact Person) |
| --- | --- | --- |
| 2.1 | TI: M.Zhou (zhou@ti.com) Sony: T.Suzuki (teruhikos@jp.sony.com) | Canon: M.Shima (shima.masato@canon.co.jp) HiSilicon: J. Zheng (zhengjianhua@huawei.com) |
| 2.2 | MediaTek: S.Liu (Shan.Liu@mediatek.com) X.Zhang (ximin.zhang@mediatek.com) | Canon: M.Shima (shima.masato@canon.co.jp) HiSilicon: J. Zheng (zhengjianhua@huawei.com) |
| 2.3 | Sony: M. Haque (munsi.haque@am.sony.com) E. Maani (ehsan.maani@am.sony.com) | Canon: M.Shima (shima.masato@canon.co.jp) |
| 2.4 | HiSilicon: J. Zheng (zhengjianhua@huawei.com) | Canon: M.Shima (shima.masato@canon.co.jp) Sony: T.Suzuki (teruhikos@jp.sony.com) |
| 2.5 | Qualcomm: R. Joshi (rajanj@qualcomm.com) | Canon: M.Shima (shima.masato@canon.co.jp) HiSilicon: J. Zheng (zhengjianhua@huawei.com) |

Evaluation

Matrices to be Tested:

The matrices to test are included in the attached file “ScalingList_TestSet.zip” and “scaling_list_proposed_QC.zip”. The matrices “Symmetry1” and “Asymmetry1” are used to evaluate the coding performance of the proposed methods in the subtest 3.

Criteria

Both coding performance and complexity are tested. Coding efficiency of matrix itself will be evaluated. .The coding efficiency of matrix only is investigated. The proposed method is compared with HM5 method. In case of lossy coding, coding error of matrix coefficients is also evaluated. For complexity evaluation, encoding time and decoding time compared to HM5 method is evaluated. More detail complexity analysis should be presented.

## Subtest 3: QP Signaling Granularity

Subtest 3.1: JCTVC-G721 and JCTVC-G893: QP adaptation at sub_CU level

This proposal studies trade-off between level of granularity in QP adaptation and overhead bits. The proposal also investigates QP coding schemes when QP adaptation at sub_CU level is allowed. In this experiment, 2 coding methods will be tested.

1.1.a) The first method follows the current QP adaptation method in HM-5.0, except to allow QP adaptation at sub_CU level, to serve as anchor. Specifically, if depth of a leaf CU is smaller than the max_cu_qp_delta_depth, a TU can signal its own dQP if the full depth (CU depth + transform depth) of that TU is less than the given threshold. Otherwise, leaf TUs will share a single TU dQP when their full depths are over the max_cu_qp_delta_depth threshold

1.1.b) The second method allows QP adaptation at sub_CU level, with additional constraints to limit the minimum area size for QP adaption. Specifically, the new constraint adds another limit for QP adaptation to allow only leaf TUs that are immediate child nodes of CU to have its own TU dQP. Otherwise, leaf TUs will share a single TU dQP. Another constraint will also be tested to force all TUs within the same PU to share a single TU dQP.

Subtest 3.2: JCTVC-G550: TU size based QP offset

This proposal studies implicit signaling QP offset relative to CU QP which is based on fact that larger TU has more possibility to be a reference area for Motion Estimation. The test will be performed.

Assignment

| Test | Proponent (Contact Person) | Cross-Checker (Contact Person) |
| --- | --- | --- |
| 3.1 | Motorola Mobility K. Panusopone (krit@motorola.com) | NEC: K.Chono (chono@ct.jp.nec.com) |
| 3.2 | Samsung: C. Kim (dionism@samsung.com) | Motorola Mobility K. Panusopone (krit@motorola.com) |

Evaluation

[Subtest 3.1]

QP modulation algorithm is needed for evaluation of this subtest.

HM-5.0 with TM-5 step 3 is used both for anchor and to be tested. QP within a picture is modulated ±12. MaxCUDepthDQP =3 [8x8] is mandatory and 2 [16x16], 1 [32x32] and 0 [64x64] are options. Other setting is same as the one specified in [1]. Subtest 1.1 and subtest 3.1 (test 1.1.a) are the anchors for this subtest. Software will also be provided with the timeline in section 4.

The following evaluation criteria will be used:

- BD performance including dQP bit increase/saving

- Dec time as complexity measure

- Variance of encoded TU bits in coding order as rate fluctuation measurement

- Subjective quality aspect including reporting noticeable degradation or improvement in visual quality compared to anchor

[Subtest 3.2]

The test will be performed based on Common Condition based on HM5.0.

## Subtest4: Fine Granularity of Quantization Step

Subtest 4.1: JCTVC-G773: CE4 Subtest 1.2.c: Higher granularity of quantization parameter scaling

In current HEVC, the quantization step size increases by approximately 12.25% with each increment of QP which lead to average 18.7% and up to 44.8% bit increase. This contribution proposes to increase the granularity of the QP scaling in HEVC test model, i.e. the quantization step size increases by approximately 5.95% with each increment of QP which lead to average 9.0% and up to 20.7% bit increase. In addition to higher granularity, this contribution also proposes to specify dQP_scale value at the slice level to support the tradeoff between dQP bits overhead and fine rate/quality control.

Subtest 4.2: JCTVC-G850: Fine granularity QP offset

According to this proposal, offset values are sent at slice level to modify the quantizer step size. The offset is signaled as a percentage of the original quantizer step-size, resulting in much higher granularity compared to what could be achieved through a QP change.

Assignment

| Test | Proponent (Contact Person) | Cross-Checker (Contact Person) |
| --- | --- | --- |
| 4.1 | Samsung: T. Lee (tammy.lee@samsung.com) | MediaTek: X. Li (x.li@mediatek.com) X. Guo (xun.guo@mediatek.com) Qualcomm: R. Joshi (rajanj@qualcomm.com) X. Wang (xianglin@qualcomm.com) |
| 4.2 | MediaTek: X.Li (x.li@mediatek.com) Qualcomm: R. Joshi (rajanj@qualcomm.com) X. Wang (xianglin@qualcomm.com) | Samsung: T. Lee (tammy.lee@samsung.com) |

Evaluation

To justify the functionality of finer quantization step, following tests shall be conducted.

JCTVC-G773:

- Under common test conditions, code sequences with QP which matches QP in HM5.

- Under common test conditions, code sequences with QP+1 where QP matches that in HM5.

JCTVC-G850:

- Under common test conditions, code sequences with QP and offset 0 (for all color components) where QP matches that in HM5.

- Under common test conditions, code sequences with QP and offset 1 (for all color components) where QP matches that in HM5.

Coding results including BD-rate, encoding and decoding time shall be provided for the tests defined above. Additional results to further justify the functionality are encouraged.

# Time-line

[Subtest 1]

T1:2011-Dec-14:	Final CE description and test condition definition

T2:2011-Dec-22:	HM-5.0 available

T2.1:2012-Jan-06: 	Anchor software and patch for dQP incr output functionality provided

T3: 2012-Jan-10: 	Completion of tool integration, test configuration announcement

Start of cross-check

T4: 2012-Jan-17: 	Report cross-checkers’ results to coordinators

T5: 2012-Jan-20: 	Input document upload

T6: 2012-Jan-25: 	Summary report uploaded

[Subtest 2]

T1:2011-Dec-14:	Final CE description and test condition definition

T2:2011-Dec-22:	HM-5.0 available

T2.1:2012-Jan-6: 	Release of the branch of HM-5.0 to support quantization matrix

T3: 2012-Jan-10: 	Completion of tool integration, test configuration announcement

Start of cross-check

T4: 2012-Jan-17: 	Report cross-checkers’ results to coordinators

T5: 2012-Jan-20: 	Input document upload

T6: 2012-Jan-25: 	Summary report uploaded

[Subtest 3]

T1:2011-Dec-14:	Final CE description and test condition definition

T2:2011-Dec-22:	HM-5.0 available

T3:2011-Dec-26:	Subtest 3 anchor software and patch for variance of encoded TU bits output functionality provided

T4: 2011-Jan-10: 	Completion of tool integration, test configuration announcement

Start of cross-check

T5: 2012-Jan-17: 	Report cross-checkers・results to coordinators

T5: 2012-Jan-20:	Input document upload

T6: 2012-Jan-25: 	Summary report uploaded

[Subtest 4]

T1:2011-Dec-14:	Final CE description and test condition definition

T2:2011-Dec-22:	HM-5.0 available

T3: 2011-Dec-26: 	Completion of tool integration, test configuration announcement

Start of cross-check

T4: 2012-Jan-13: 	Report cross-checkers’ results to coordinators

T5: 2012-Jan-20: 	Input document upload

T6: 2012-Jan-25: 	Summary report uploaded

# References

[1] “Common HM test conditions and software reference configurations”, JCTVC-G1200, Nov 2011