| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G1205_r1 |
| --- | --- |

| Title: | Description of Core Experiment 5: Transform Skipping |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Core Experiment Description |  |  |
| Author(s) or Contact(s): | Marta Mrak (BBC) Joel Sole (Qualcomm) Jizheng Xu (Microsoft) Ankur Saxena (Samsung) | Email: | marta.mrak@bbc.co.uk joels@qualcomm.com jzxu@microsoft.com asaxena@sta.samsung.com |
| Source: | CE5 coordinators |  |  |

_____________________________

# Abstract

This document defines Core Experiment 5 (CE5) on Transform Skipping to be performed for the upcoming 8th JCT-VC meeting. Following work from AHG19 on Transform Skipping, during the 7th JCT-VC meeting, a core experiment on transform skipping and supporting tools was recommended for a more thorough assessment. This assessment will be used to further evaluate each part of the modifications to HM to support transform skipping and evaluate possible transform skipping settings. In addition to the tools related to transform skipping (as proposed in JCTVC-1003 for motion compensated blocks and JCTVC-E145 for intra blocks) an additional proposal on horizontal and vertical scans for larger blocks (JCTVC-G586) was also recommended for study in this core experiment, because of its relationship to skipping the transform in one direction only.

# Introduction

For the November 2011 JCT-VC meeting in Geneva, AHG19 on Transform Skipping [1] investigated and evaluated transform skipping and related tools. Transform skipping is proposed for application on motion compensated blocks. Instead of performing transforms on both rows and columns, the skipping concept enables that transform of rows and/or transform of columns can be skipped. When the transform is skipped in only one direction, a transpose is applied, to keep the coefficients in the order required by other coding stages. Preserving the 2-D transforms (on columns and rows) is still supported and the choice of keeping the transform or skipping is defined by one of the four Transform Skip Modes (TSM), [2].

JCT-VC made the following recommendations:

- Evaluate transform skipping with minimum changes and with additional changes with respect to HM

- Determine the amount of additional changes justified by related gains

- Study interaction of skipping with quantization matrices

# Participants

| Name | Affiliation | Email |
| --- | --- | --- |
| Marta Mrak (coordinator) | BBC | marta.mrak@bbc.co.uk |
| Joel Sole (coordinator) | Qualcomm | joels@qualcomm.com |
| Jizheng Xu (coordinator) | Microsoft | jzxu@microsoft.com |
| Ankur Saxena (coordinator) | Samsung | asaxena@sta.samsung.com |
| Glenn Van Wallendael | Ghent University | glenn.vanwallendael@ugent.be |
| Rajan Joshi | Qualcomm | rajanj@qualcomm.com |
| Matteo Naccari | BBC | matteo.naccari@bbc.co.uk |
| Andrea Gabriellini | BBC | andrea.gabriellini@bbc.co.uk |
| Jewon Kang | Nokia | jewonkan@usc.edu |
| Jani Lainema | Nokia | jani.lainema@nokia.com |
| Zhengxiao Zhen | Huawei | zhengxiaozhen@huawei.com |
| Thomas Davies | CISCO | thdavies@cisco.com |
| Da-ke He | Research In Motion Limited | dhe@rim.com |
| TK Tan | NTT DOCOMO | ttk@pacific.net.sg |
| Patrice Onno | Canon | patrice.onno@crf.canon.fr |
| Jie Dong | InterDigital | jie.dong@interdigital.com |
| Simon Oudin | Fraunhofer HHI | simon.oudin@hhi.fraunhofer.de |
| Shih-Ta Hsiang | Mediatek | shih-ta.hsiang@mediatek.com |
| Youji Shibahara | Panasonic | shibahara.youji@jp.panasonic.com |
| Elena Alshina | Samsung | elena.alshina@gmail.com |
| Xing Wen | HKUST | wxxab@ust.hk |

# Description of Experiments

The experiments related to transform skipping on motion compensated blocks will be carried out using a basic TSM implementation. This software will consist of the support for TSM in HM-5.0. The support for TSM will be derived from JCTVC-G1003 by enabling TSM choice for each TU. This includes support for TSM in the rate-distortion optimisation search for motion compensated blocks, actual transform skip, related quantisation and TSM choice signalling. Transform skipping for intra coded blocks will be evaluated using HM-5.0.

The following experiments will be carried out and evaluated for CE5. Additional details can be found in the related contributions associated with considered tests. Bold text indicates the primary contact for the corresponding entry.

## CE5.a: Quantization for transform skipping

Related contribution: JCTVC-G575

Proponents: M. Naccari, M. Mrak (BBC)

Cross-checkers: S.-T. Hsiang (Mediatek)

Description: The purpose of this experiment is to evaluate the efficiency of TSM quantisation, which is dependent on transform skipping modes and block sizes, as described in JCTVC-G575. Proposed quantisation adjusts signal levels when the transform is skipped in any direction. Additionally, the test will be performed where weighting matrices from HM-5.0 are used together with TSM.

## CE5.b: Transform skipping choices based on block parameters

Related contribution: JCTVC-G577

Proponents: G. Van Wallendael (Ghent University)

Cross-checker: A. Saxena (Samsung), X. Zheng (Huawei)

Description: In contribution JCTVC-G577 a mechanism for selection of TSM depending on current block parameters was presented. The solution demonstrated an improved BD-rate performance compared to HM and a lower encoder's execution time compared to TSM that is applied on all block sizes for all QPs. To support such more customized selection of TSM, SPS signalling for TSM on larger blocks was proposed. Within this experiment, TSM with restricted block sizes will be evaluated.

## CE5.c: Mode dependant significance scans for transform skip

Related contribution: JCTVC-G575

Proponents: M. Mrak, A. Gabriellini (BBC)

Cross-checker:  R. Joshi (Qualcomm), J. Kang (Nokia)

Description: In JCTVC-G575 the entropy coding was changed to support cases when the transform is skipped in one direction only. When the transform is skipped on rows only or columns only, vertical or horizontal scans were used, respectively. In this way, TSM-dependent scans were introduced, addressing ordering of transform coefficients imposed by TSM. The purpose of this experiment is to evaluate the efficiency of such scans, in comparison to the default HM-5.0 entropy coding settings used with TSM.

## CE5.d: Coefficients coding for transform skipping

Related contribution: JCTVC-G663

Proponents: R. Joshi, J. Sole (Qualcomm)

Cross-checker: J. Dong (InterDigital)

Description: This experiment will evaluate tools related to transform skipping that were proposed in JCTVC-G663. This proposal introduced enhancements to the coding of the last non-zero transform coefficient and coding of significance flags. When a transform is skipped in a particular direction, the last position in that direction is coded using fixed length code (dependent on the block size in that direction) in the bypass mode. The coding of last position in the direction in which transform is applied is unchanged from the HM. Modification for coding of significance flags are applied for coding of 4x4 and 8x8 blocks.

## CE5.e: Parallelizable context for significance coding of large transform blocks

Related contribution: JCTVC-G586

Proponents: J. Kang, J. Lainema, A. Hallapuro, K. Ugur (Nokia)

Cross-checker: J. Sole (Qualcomm), A. Gabriellini (BBC)

Description: This experiment will evaluate effect of the parallelizable contexts proposed in JCTVC-G586. The proposed contexts for significant coefficient flags are applied for TUs utilizing horizontal and vertical scans with neighbourhood-based modelling. The intention in the proposed scheme is to allow same level of parallelism for the horizontal or vertical scanning in TSM as what is currently available for the diagonal scans in the large TU sizes.

## CE5.f: Residual Scalar Quantization (RSQ)

Related contributions: JCTVC-E145

Proponents: J. Xu (Microsoft)

Cross-checker: X. Wen (HKUST)

In the RSQ mode, directional prediction is also used for each block to generate the prediction residue. After the intra prediction, different from the current intra coding modes, no transform is applied on the residue. Instead, the quantization is directly applied on the residual signals, similar to APEC used in KTA inter block coding. Since no transform is performed, the residual quantization and sample reconstruction can be done pixel by pixel within a block in this scheme. The reconstructed pixels from the former row or column can be used for the prediction of the pixels in the current row or column. The use of intra block reconstructed pixels for prediction is based on the assumption that the shorter the prediction distance is, the stronger the correlation will be.

## CE5.g: Integration of tools CE5.a to CE5.e

Related contributions: JCTVC-G1003 and JCTVC-G586

Proponents: M. Mrak and all CE5.a to CE5.e proponents

Cross-checker: D. He (RIM), T. Davies (CISCO)

Description: The goal of this test is to evaluate overall performance of transform skipping with supporting tools tested in this CE (CE5.a to CE5.e). This test will be carried out without quantisation weighting matrices which are part of CE5.a experiment. Transform skipping will be tested for different NSQT/RQT levels. As a result, the amount of additional changes justified by related gains will be determined.

# Core Experiment Tasks

The objectives for this Core Experiment are to:

- Enable testing of transform skipping with minimum changes as well as with additional improvements;

- Specify the software platform, common software configurations and common test conditions for these experiments;

- Evaluate compression efficiency and complexity of the tools studied in this CE;

- Investigate any unique interactions among the tools investigated in this CE;

- Investigate the interaction of transform skipping with other transform related tools in HM (NSQT, RQT);

- Report the results and conclusions of these experiments.

# Test Conditions

The following test conditions shall be used for this CE:

- Software

The base for tests in this CE is transform skipping implementation provided by BBC on top of HM version 5.0.

- Test configurations

This CE shall use the same encoder and decoder configurations as described in JCTVC-G1200. Experiments shall be run for the Random Access and Low Delay B coding configurations, both for high efficiency and low complexity configurations. Low Delay P configurations are not mandatory.

- Test sequences

The test sequences as described in JCTVC- G1200 shall be used for these tests. The results shall be reported for classes A to E averaged and for class F separately.

# Evaluation Criteria

This section contains details about evaluation of each experiment. The objective rate-distortion measurements using 4-point BD-rate for all tools will be used, as provided in the spreadsheet associated with the common conditions JCTVC- G1200. For the complexity measurement, the computational time will be measured of the anchor and the proposed method implemented.

Two sets of results should be presented:

- results with the HM-5.0 as the reference,

- results with the starting transform skipping software as the reference, on top of which the experiment is carried out.

Proponents are welcome to include additional results.

If sequences coded using transform skipping exhibit subjective improvements or degradations, proponents and cross-checkers should describe these subjective differences. For tests CE5.a and CE5.g coded sequences and anchors will be available for viewing during the 8th JCT-VC meeting in February 2012.

# Timeline

- 19 December 2011; Final CE5 description uploaded

- 19 December 2011; HM-5.0 available

- 30 December 2012 or earlier; Software based on HM-5.0 that supports transform skipping distributed to all CE5 participants

- 11 January 2012; Software of each CE5 experiment distributed to all CE5 participants so cross-checking and integration can begin

- 15 January 2012; Integrated software distributed to all CE5 participants so cross-checking can begin

- 20 January 2012; Proponents report results to CE5 coordinators

- 20 January 2012; Input documents uploaded

# References

- M. Mrak, J. Sole, I.-K. Kim, J. Xu, H. Yu, “JCT-VC AHG Report: Transform Skipping (AHG19)”, JCTVC-G019, 7th JCT-VC Meeting, Geneva, CH, November 2011.

- M. Mrak, A. Gabriellini, D. Flynn, “Transform skip mode”, JCTVC-G575, 7th JCT-VC Meeting, Geneva, CH, November 2011

- J. Kang, J. Lainema, A. Hallapuro, K. Ugur, “Parallelizable context for significance coding of large transform blocks”, JCTVC-G586, 7th JCT-VC Meeting, Geneva, CH, November 2011.

- C. Lan, X. Peng, J. Xu, F. Wu, "Intra and inter coding tools for screen contents", JCTVC-E145, 5th JCT-VC Meeting, Geneva, CH, March 2011.