| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, IT, 21-30 November, 2011 | Document: JCTVC-G1206 |
| --- | --- |

| Title: | CE6: Intra Coding Improvements |  |  |
| --- | --- | --- | --- |
| Status: | Output Document |  |  |
| Purpose: | CE Description |  |  |
| Author(s) or Contact(s): | Ali Tabatabai (Primary) Edouard Francois Keiichi Chono | Tel: Email: Tel: Email: Tel: Email: | +1-408-352-4715 ali.tabatabai@am.sony.com edouard.francois@canon.crf.fr chono@ct.jp.nec.com |

_____________________________

# Introduction

This output document provides the description of Core Experiments 6 on “Intra prediction and improvements”. Proposals on intra coding and intra prediction are divided in several subsets of this CE:

- Subset CE6a:	Intra Chroma Prediction

- Subset CE6b:	Improved Intra Prediction

- Subset CE6c:	Intra mode coding cleanup and simplification

# Experimental Conditions

## Software

Tools under test in this CE will be implemented on top of HM 5.0 software that will be released by the software Ad-Hoc group after integrating changes adopted at the 7th JCT-VC meeting in Geneva.

## Common Test Conditions

The common test conditions for all-Intra configuration as defined in the document JCTVC-G1000 released by the software Ad-Hoc group will be used as anchor configuration for tests in all subsets.

Performance evaluations will be mandatory reported for All Intra configuration and for classes A to E.

Optional conditions are the inclusion of class F and the use of Random Access, Low Delay B and Low Delay P configuration.

.

## Evaluation of CE Results

Results of this CE will be evaluated on the basis of BDR metric. In addition, subjective evaluation to support the advantage of the proposed tools is highly desirable.

## Evaluation of Complexity

For the complexity measurement, the anchor and the reference software with the tools implemented will be executed on the same machine and the computational time will be measured for each configuration. Then, a time factor is calculated which compares the reference software including the sub-CE test tool and the reference software without the tool.

# Timelines

| T1 | December 19, 2011 | HM 5.0 software available |
| --- | --- | --- |
| T2 | T1 + 2 Wks, 2011 | Codes to be released by proponents |
| T3 | January 13, 2012 | Cross-verification completed CE. |
| T4 | January 20, 2012 | Report the verification results uploading. |

# CE6a: Intra Chroma Prediction

The following tools are considered in this CE:

- Cross-channel intra chroma residual prediction (JCTVC-G173)

- Luma-based chroma prediction - Model correction (JCTVC-G244)

- Chroma intra prediction based on residual luma samples (JCTVC-G346)

- New modes for chroma intra prediction (JCTVC-G358)

These tools will be evaluated independently. Combinations will also be explored to identify potential optimal combination in terms of trade-off coding efficiency/complexity.

## Participants list

| Name | Affiliation | Email | Proponent/Cross-checker |
| --- | --- | --- | --- |
| Oscar Au | HKUST | eeau@ust.hk | P/C |
| Xingyu Zhang | HKUST | eexyzhang@ust.hk | P/C |
| Xing Wen | HKUST | wxxab@ust.hk | P/C |
| Edouard François | Canon | edouard.francois@crf.canon.fr | P/C |
| Christophe Gisquet | Canon | christophe.gisquet@crf.canon.fr | P/C |
| Yi-Jen Chiu | Intel | yi-jen.chiu@intel.com | P/C |
| Lidong Xu | Intel | lidong.xu@intel.com | P/C |
| Wenhao Zhang | Intel | wenhao.zhang@intel.com | P/C |
| Yu Han | Intel | yu.han@intel.com | P/C |
| Kei Kawamura | KDDI | ki-kawamura@kddi.com | P/C |
| Tomonobu Yoshino | KDDI | to-yoshino@kddi.com | P/C |
| Shohei Matsuo | NTT | matsuo.shohei@lab.ntt.co.jp | C |
| Haitao Yang | Huawei | haitao.yang@huawei.com | C |

## Proposals description

### JCTVC-G173 Cross-channel intra chroma residual prediction [Intel]

G173 presents a cross-channel prediction refinement technique among the chroma channels. The prediction refinement technique applies on the process of intra chroma prediction and no additional control flag is required. This test is focused on a cross-channel residual prediction technique to improve the intra chroma prediction of HM 5.0. It produces a refinement of prediction samples based on the residual of another channel. The test includes the following

- The test of Cb prediction refined based upon the residual of Cr channel, and the test of Cr prediction refined based upon the residual of Cb channel

To test the coding efficiency performance based upon a linear model with a fixed parameter value

- The test of Cb prediction and Cr prediction refined based upon the residual of Y channel.

To test the coding efficiency performance based upon a linear model with a fixed parameter value

- A combination of #1 and #2.

### JCTVC-G244 Luma-based chroma prediction – Model correction [Canon]

General Concept

In luma-based chroma intra prediction mode (LM), 2 linear mode parameters (alpha, beta) are estimated using OLS from reconstructed neighboring luma and chroma samples. alpha value is computed in OLS as the ratio of two intermediate parameters A1 and A2. OLS can lead to wrong or even undetermined estimation in specific cases when these two intermediate parameters are of low range. The aim of the proposed method is to detect such ill-cases and to replace the alpha value by a pre-determined value, depending on the ill-case type. In the current design, 4 types of ill-cases are considered. For each ill-case, one pre-determined alpha value, picked from a table of fixed values (the index is signaled in the slide header or APS), is used.

The ill-cases classification consists at most of 5 comparisons involving parameters A1, A2 and alpha. The thresholds (2 values per chroma component) involved in these comparisons are currently signaled in the APS. If none of the 4 possible ill-cases is detected, the normal alpha value is used.

Bitstream organization

12 syntax elements (38 bits) are inserted in slice header or APS.

The algorithm settings (number of types, pre-defined alpha values) could be adjusted to reach the best performance. In particular, other settings will be tested when combining G244 with other tools.

### JCTVC-G346 Chroma intra prediction based on residual luma samples [KDDI]

JCTVC-G346 presents additional chroma intra mode based on inter-channel correlation of residual samples. Predicted Cb/Cr values are sum of regular prediction (same as DM) and linear equation using luma residual values with parameter alpha. For further harmonization with HM5.0, each process in the tool, including mode coding, is tuned and simplified. In order to combine JCTVC-G346 with other tools in CE, the test includes the following:

- The parameter alpha is derived and coded on the encoder side.

- The parameter alpha is fixed with simplified preparation of luma residual values.

- A combination of JCTVC-G346 and other tools.

### JCTVC-G358 New modes for chroma intra prediction (LML and LMA) [HKUST]

General Concept

JCTVC-G358 contains two new modes, LML and LMA, for chroma intra prediction.  Basically, these two modes take the same steps as LM mode to predict Chroma component, except that the neighboring samples used for deriving alpha and beta are from different locations. For an NxN chroma block, 2N left (and down left) neighboring samples are used in LML mode to calculate alpha and beta, while 2N above (and above right) neighboring samples are used in LMA mode. The down-sampling process of luma component is the same as in LM mode.

There are 3 schemes: Scheme 1(S1) simply adds LMA and LML to the existing 6 modes. Scheme 3 (S3) has 4 modes: DM, LM, LML, LMA. Scheme 2 (S2) has 4+2 modes – it modifies Scheme 3 by adding 2 of 4 removed modes back. Things to be tested include different number and combination of modes, choice of samples to be used, and other variations, especially when combined with other tools.

Bitstream organization

No need to modify the bitstream syntax.

## Timeline for software integration

| T1 | T | HM 5.0 software available |
| --- | --- | --- |
| T2 | T+2 days | Intel finishes integrating G173 into HM5.0 |
| T3 | T+5 days | HKUST finishes integrating G358 |
| T4 | T+9 days | Canon finishes integrating G244 |
| T5 | T+13 days | KDDI finishes integrating G346 |
| T6 | T+14 days | Crosscheck of source code, experiments begin |

## Test plan

The following tests will be performed using the tests conditions defined in section 2.

Proponents in bold font are in charge of providing the simulation results to the cross-checkers.

| Combination | Contribution | Tools | Proponent | Cross-check |
| --- | --- | --- | --- | --- |
| Individual Proposal | JCTVC-G173 | Cross-channel intra chroma residual prediction | Intel | HKUST Canon |
|  | JCTVC-G244 | Luma-based chroma prediction – Model correction | Canon | Intel KDDI |
|  | JCTVC-G346 | Chroma intra prediction based on residual luma samples | KDDI | HKUST Canon |
|  | JCTVC-G358* | New modes for chroma intra prediction (LML and LMA) | HKUST | Intel KDDI |
| Combination of two proposals | Combination of G173, G244 | Cross-channel intra chroma residual prediction Luma-based chroma prediction – Model correction | Intel, Canon | HKUST KDDI |
|  | Combination of G173, G346 | Cross-channel intra chroma residual prediction Chroma intra prediction based on residual luma samples | Intel, KDDI | Canon HKUST |
|  | Combination of G173, G358* | Cross-channel intra chroma residual prediction New modes for chroma intra prediction (LML and LMA) | Intel, HKUST | Canon KDDI |
|  | Combination of G244, G346 | Luma-based chroma prediction – Model correction Chroma intra prediction based on residual luma samples | Canon, KDDI | HKUST Intel |
|  | Combination of G244, G358* | Luma-based chroma prediction – Model correction; New modes for chroma intra prediction (LML and LMA) | Canon, HKUST | KDDI Intel |
|  | Combination of G346, G358* | Chroma intra prediction based on residual luma samples New modes for chroma intra prediction (LML and LMA) | KDDI, HKUST | Canon Intel |
| Combination of three proposals | Combination of G173, G244, G346 | Cross-channel intra chroma residual prediction; Luma-based chroma prediction – Model correction Chroma intra prediction based on residual luma samples | Intel, Canon, KDDI | HKUST NTT |
|  | Combination of G173, G244, G358* | Cross-channel intra chroma residual prediction; Luma-based chroma prediction – Model correction New modes for chroma intra prediction (LML and LMA) | Intel, Canon, HKUST | KDDI NTT |
|  | Combination of G244, G346, G358* | Luma-based chroma prediction – Model correction; Chroma intra prediction based on residual luma samples New modes for chroma intra prediction (LML and LMA) | Canon, KDDI, HKUST | Intel Huawei |
|  | Combination of G173, G346, G358* | Cross-channel intra chroma residual prediction; Chroma intra prediction based on residual luma samples New modes for chroma intra prediction (LML and LMA) | Intel, KDDI, HKUST | Canon Huawei |
| Combination of four proposals | Combination of G173, G244, G346, G358* | Cross-channel intra chroma residual prediction; Luma-based chroma prediction – Model correction Chroma intra prediction based on residual luma samples New modes for chroma intra prediction (LML and LMA) | Intel, Canon, KDDI, HKUST | NTT Huawei |

*  G358 has 3 schemes. All of the 3 schemes should be tested.

# CE6b: Improved Intra Prediction

The goal of this experiment is to enhance Intra prediction of the HEVC Test Model.

The following tools are considered in this experiment:

## Proposals description

## Proposals description

### Rectangular (2NxN and Nx2N) Intra Prediction (JCTVC-G135)

JCTVC-G135 proposes to add two prediction unit (PU) types (i.e. 2NxN and Nx2N) to Intra coding units (CU) in addition to the existing PU types (i.e. 2Nx2N and NxN) in current HM. Consequently, one Intra prediction mode is applied to each 2NxN and Nx2N PU. In one version of implementation, the transforms and prediction block sizes that have been used in SDIP (JCTVC-E278) are utilized. Experimental results report further coding gain on top of SDIP compared with the anchor. The harmonization between the proposed PU structure for Intra CU and previously adopted NSQT is being investigated.

The purpose of this experiment is to test the coding efficiency and run-time of the proposed method in harmonization with (a) SDIP, and (b) NSQT, based on HM5.0 software. The harmonization of JCTVC-G135 and some other Intra prediction tools (adopted or in CE) can also be evaluated based on HM5.0 software.

Coordinators: This description is too general needs to be revised and become more specific.

### Tradeoff of coding efficiency and encoding/decoding running time of SDIP

## Test plan

Following tests will be performed using the tests conditions defined in section 2.

| CE6c: Short Distance Intra Prediction |  |  |  |
| --- | --- | --- | --- |
| Contribution | Tools | Proponent | Cross-check |
| JCTVC-G135 | Rectangular (2NxN and Nx2N) Intra Prediction | MediaTek Shan.Liu@mediatek.com | Intel yi-jen.chiu@intel.com |

Coordinators: Please list individual test items!

# CE6c: Intra mode coding cleanup and simplification

The goal of this experiment is to cleanup and simplify intra mode coding of the HEVC Test Model.

The following tools are considered in this experiment:

- On intra prediction mode coding (JCTVC-G153)

- Modifications to Intra-frame coding (JCTVC-G119)

## Proposals description

### Modifications to Intra-frame coding (JCTVC-G119)

?

Part3 of JCTVC-G119 proposes to merge the two diagonal intra prediction modes (HOR+8 and VER+8) into one bi-prediction mode. As a result, the total number of remaining Intra modes can be expressed as powers of 2 for all block sizes. The intra mode coding can be harmonized. Specifically, escape code for intra mode coding is removed for 8x8/16x16/32x32/64x64 block sizes. Besides, intra prediction mode (HOR+6) is allowed for 4x4 block size.The purpose of this experiment is to evaluate the impact on performance and complexity based on HM5.0.

## Test plan

The following tests will be performed using the tests conditions defined in section 2.

| CE6d: Intra mode coding cleanup and simplification |  |  |  |
| --- | --- | --- | --- |
| Contribution | Tools | Proponent | Cross-check |
| JCTVC-G153 | On intra prediction mode coding | I2R |  |
| JCTVC-G119 | Modifications to Intra-frame coding | Huawei |  |