| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G1207_r3 WG11 Number: m22846 |
| --- | --- |

| Title: | Description of Core Experiment 7: Additional Transforms |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Core Experiment Description |  |  |
| Author(s) or Contact(s): | Robert Cohen Mitsubishi Electric Research Laboratories Chuohao Yeo Institute for Infocomm Research Rajan Joshi Qualcomm Felix Fernandes Samsung Electronics Co., Ltd. | Email: | cohen@merl.com chyeo@i2r.a-star.edu.sg rajanj@qualcomm.com felix.f@sta.samsung.com |
| Source: | CE7 Coordinators |  |  |

_____________________________

# Abstract

This document defines Core Experiment 7 (CE7) on Additional Transforms, to be performed for the upcoming 8th JCT-VC meeting February 2012. During the 7h JCT-VC meeting in November 2011, three additional transform proposals JCTVC-G108, JCTVC-G281, and JCTVC-G632 were recommended for further study in a CE. This assessment will be used to determine whether the improvement in compression efficiency is worthwhile given the associated increase in complexity.

# Introduction

For the November 2011 JCT-VC meeting in Geneva, Core Experiment 7 (CE7) [1] investigated the performance of several proposed non-core transforms. JCT-VC made the following recommendations:

- There were concerns over the gain and complexity specific to the use of the mode-dependent secondary transform of JCTVC-G108 [2], which was applied only for intra. Further study in a CE including combinations with inter was recommended.

- JCTVC-G281 [3] applied a boundary dependent transform on inter. It was recommended for further study along with JCTVC-G632, which combines it with a secondary transform.

# Description of tools

The following tools shall be evaluated for CE 7. Full details for the tools can be found in their associated documents. A “Primary” transform is applied in place of the HM core transform. A “Secondary” transform is applied in addition to the HM core transform. Bold text indicates the primary contact for the corresponding entry.

| Tool # | Participants | Doc. | Transform Type | Tool Description and cross-checkers |
| --- | --- | --- | --- | --- |
| 1 | A. Saxena, Y. Shibahara, F. Fernandes, T. Nishi | JCTVC-G108 | Secondary | Mode-dependent secondary transform for intra luma: Depending on the intra prediction mode, a secondary transform may operate on the low-frequency coefficients of the 8x8 and larger HM core transform, for luma. When used, this transform is applied in addition to the HM core transform. Secondary transform sizes of 4x4 and 8x8 will be tested. |
|  | Cross-checks: |  |  | 1. Marta Mrak 2. 3. … |
| 2 | J. An, X. Guo, X. Zhao, S. Lei | JCTVC-G281 | Primary | Boundary-dependent transform for inter: For 4x4, 8x8, and 16x16 inter TUs, different transforms (DST-VII/DCT-IV or DCT-II, using the existing HM transform cores) are selected for luma depending on whether the TU boundary is a PU boundary. When used, this transform is applied in place of the HM core transform. |
|  | Cross-checks: |  |  | 1. Robert Cohen 2. Ankur Saxena, Felix Fernandes 3. … |
| 3 | A. Saxena, F. Fernandes | JCTVC-G632 | Secondary (Some tests are primary on 4x4) | Unified boundary-dependent secondary transform for inter luma: For 8x8 and larger TUs, the HM core transform is followed by a secondary transform. For 4x4 TUs, tests will include a secondary transform and a boundary-dependent transform in the framework of JCTVC-G281. Unifying this tool with JCTVC-G108 for Intra blocks will also be investigated. Secondary transform sizes of 4x4 and 8x8 will be tested. |
|  | Cross-checks: |  |  | 1. Marta Mrak 2. 3. … |

# Core Experiment tasks

The objectives for this Core Experiment are to:

- Specify the software platform, common software configurations and common test conditions for these experiments;

- Evaluate compression efficiency and complexity of the additional transforms studied in this CE;

- Provide informal comments regarding subjective differences, if any.

- Investigate any unique interactions among the additional transforms and coefficient scanning schemes;

- Report the results and conclusions of these experiments.

- Provide changes, if any, to the Working Draft syntax and semantics text.

# Test conditions

The following test conditions shall be used for this CE:

## Software

HM version 5.0 shall be used for these experiments, as specified in JCTVC-G1200 [4].

## Test configurations

This CE shall use the common test conditions as described in JCTVC-G1200. For proposed transforms that operate only on intra TUs, all low delay test conditions are optional.

## Test sequences

The test sequences as described in JCTVC- G1200 shall be used for these tests.

## Evaluation Criteria

This section contains details about how the transforms shall be evaluated. If a proposed transform includes scan orders not already included in HM, simulation results at a minimum shall be reported for the additional transform without these additional scan orders. Results with the additional scan orders may be reported as well. This is done to help JCT-VC determine the benefits of these additional scans. Proponents are welcome to include additional results at their discretion.

## Compression efficiency

The spreadsheet associated with the common conditions JCTVC- G1200 shall be used to evaluate coding efficiency. This spreadsheet includes a summary of BD metrics and encoder/decoder run times, along with detailed rate and PSNR tables and plots, and possibly other data. Data associated with the CE7 configurations described above in Section 4.2 shall be reported. The references for these computations shall be generated from the unmodified HM 5.0 software. Participants are free to provide additional data at their discretion.

## Complexity evaluation

### Tool description

In the JCT-VC text document associated with each proposed tool, the abstract shall include a brief text description of the proposed additional transform, along with a brief summary of the compression performance and complexity evaluation.

### Additional transform description

The additional transforms for each transform size shall be described in such a way that makes it clear what operations are being performed. Pseudo-code or data-flow diagrams are preferred. The number of additional transform cores to be added shall also be reported.

### Arithmetic operations

For each size of each inverse transform, the worst-case number of arithmetic operations per square transform unit (multiplies, adds, and shifts) shall be reported. These counts shall be based upon the pseudo-code or data-flow diagrams. The operations of the final scaling performed during the inverse transform process shall also be included in the operations count. Data for the JCTVC-E243 partial butterfly implementation are provided in Table 1 for reference. Note that the number of additions reported includes the offset before the shift. The correct number of shifts is also shown.

The following table format shall be used. The contents of the shaded areas are fixed, and the non-shaded areas shall contain data for each of the proposed transforms. Additional columns may be added as needed. If the forward and inverse transforms do not have similar operation counts, forward transform data shall also be reported in a separate column. For secondary transforms applied in addition to the core HM transform, the operation counts of the secondary transform shall be preceded by a “+” sign. Example entries for additional transforms are shown in the last two columns of Table 1.

In addition to worst-case operation counts, averaged operation counts shall be provided to give an indication of the additional complexity incurred under typical operating conditions. For the mode-dependent toolsError: Reference source not found, the average complexity shall be computed as a weighted average based upon the number of modes for which that configuration is used. For example,a if a tool uses +0 multiplies for 1 of 5  lumaiIntra prediction mod I, +32 multiplies for 9 of the modes, +32 for 9 other modes, and +64 for the remaining 16 modes, then the average number of multiplies for the tool would be (0 × 1/35) + (32 × 9/35) + (32 × 9/35) + (64 × 16/35) = 46 (rounded). So, the worst-case number of multiplies for this secondary transform would be reported as +64, and the average case would be +ses.

Table 1. Operation count examples

| Transform size |  | JCTVC-E243 partial butterfly implementation of 2-D inverse DCT | Example transform that replaces the core HM 2-D transform | Example secondary transform applied after 8x8 and larger HM 2-D transforms. For 4x4, a replacement transform is used |
| --- | --- | --- | --- | --- |
| 4x4 | Mults | 48 | 72 | 128 |
|  | Adds | 96 | 160 | 144 |
|  | Shifts | 32 | 80 | 32 |
| 8x8 | Mults | 352 | 1024 | +128 |
|  | Adds | 576 | 1024 | +128 |
|  | Shifts | 128 | 352 | +16 |
| 16x16 | Mults | 2752 |  | +128 |
|  | Adds | 3712 |  | +128 |
|  | Shifts | 512 |  | +16 |
| 32x32 | Mults | 21888 |  | +128 |
|  | Adds | 25856 |  | +128 |
|  | Shifts | 2048 |  | +16 |

### Bit precision

Report the maximum bit precision needed for each transform size.

In the decoder, because the output of the secondary inverse transform is input to the core inverse transform, proponents shall provide information that confirms that the data output from the secondary inverse transform does not exceed 16 bits.

### Memory requirements

For each transform size, report any additional memory requirements in bytes.

### Quantization

Report the presence and number of additional quantization matrices, for each transform size.

Any additional bit-depth required by use of these additional quantization matrices shall also be reported.

### R-D optimization

If any additional rate-distortion loops or iterations are being performed in the encoder or decoder, they shall be described.

### Encoder/Decoder run times

Encoder and decoder run times shall be reported using the common spreadsheet as described in Section 5.1.

### Other statistics

If present, memory transfers, reordering, and other data operations not included above shall be reported.

## Subjective effects

If sequences coded using a proposed additional transform exhibit subjective improvements or degradations, proponents and/or cross-verifiers shall describe these subjective differences. These descriptions are to be considered as informal comments, as formal subjective testing is not required by CE7 at this time.

## Syntax and Semantics

For each proposed tool, text for any changes to the syntax and semantics of the Working Draft shall be provided.

# Timeline

December 14, 2011: Final CE7 description uploaded

December 22, 2011: HM 5.0 available

January 5, 2011: Software distributed to all CE7 participants so cross-checking can begin

January 19, 2011 or earlier: Proponents report results to CE7 coordinators

January 20, 2011: Input documents uploaded

- Participant list

| Name | Affiliation | Email |
| --- | --- | --- |
| Robert Cohen (coordinator) | Mitsubishi Electric Research Labs | cohen@merl.com |
| Chuohao Yeo (coordinator) | Institute for Infocomm Research | chyeo@i2r.a-star.edu.sg |
| Rajan Joshi (coordinator) | Qualcomm | rajanj@qualcomm.com |
| Felix Fernandes (coordinator) | Samsung | felix.f@sta.samsung.com |
| Ankur Saxena | Samsung | asaxena@sta.samsung.com |
| Youji Shibahara | Panasonic | shibahara.youji@jp.panasonic.com |
| Takahiro Nishi | Panasonic | nishi.takahiro@jp.panasonic.com |
| Jicheng An | MediaTek | jicheng.an@mediatek.com |
| Xun Guo | MediaTek | xun.guo@mediatek.com |
| Xin Zhao | MediaTek | x.zhao@mediatek.com |
| Shawmin Lei | MediaTek | shawmin.lei@mediatek.com |
| Marta Mrak | BBC | Marta.Mrak@bbc.co.uk |
| Observing participants: |  |  |
| Il-Koo Kim | Samsung | ilkoo.kim@samsung.com |
| Elena Alshina | Samsung | elena_a.alshina@samsung.com |
| Akiyuki Tanizawa | Toshiba | akiyuki.tanizawa@toshiba.co.jp |
| Jun Yamaguchi | Toshiba | jun2.yamaguchi@toshiba.co.jp |
| Atsuro Ichigaya | NHK | ichigaya.a-go@nhk.or.jp |
| Shinichi Sakaida | NHK | sakaida.s-gq@nhk.or.jp |
| Tung Nguyen | HHI | tung.nguyen@hhi.fraunhofer.de |
| Madhukar Budagavi | TI | madhukar@ti.com |
| TK Tan | NTT DOCOMO | ttk@pacific.net.sg |
| Kiran Misra | Sharp Labs of America | misrak@sharplabs.com |
| Jewon Kang | Nokia | jewonkan@usc.edu |
| Pankaj Topiwala | FastVDO | pankaj@fastvdo.com |
| Amir Naghdinezhad | eBrisk Video | amir@ebriskvideo.com |

# References

- R. Cohen, C. Yeo, R. Joshi and F. Fernandes, “CE7: Summary Report of Core Experiment on Additional Transforms,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G037, 7th JCT-VC Meeting, Torino, IT, Nov. 2011.

- A. Saxena, Y. Shibihara, F. Fernandes and T. Nishi, “CE 7 : On secondary transforms for intra prediction residual,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G108, 7th JCT-VC Meeting, Geneva, CH, Nov. 2011.

- J. An, X. Zhao, X. Guo and S. Lei, “Non-CE7: Boundary Dependent Transform for Inter Predicted Residue,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G281, 7th JCT-VC Meeting, Geneva, CH, Nov. 2011.

- F. Bossen., “Common conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC- G1200, 7th JCT-VC Meeting, Geneva, CH, Nov. 2011.