| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-G1208 WG11 Number: m22849 |
| --- | --- |

| Title: | Description of Core Experiment 8 (CE8): Non-deblocking loop filtering |  |  |
| --- | --- | --- | --- |
| Status: | Output Document of JCT-VC |  |  |
| Purpose: | CE description |  |  |
| Author(s) or Contact(s): | Tomoo Yamakage (Primary) In Suk Chong Matthias Narroschke Yu-Wen Huang |  | tomoo.yamakage@toshiba.co.jp ichong@qualcomm.com Matthias.Narroschke@eu.panasonic.com yuwen.huang@mediatek.com |
| Source: | CE Coordinators |  |  |

_____________________________

# Introduction

The goal of this Core Experiment (CE) is to enhance non-deblocking loop filtering techniques in the HEVC Test Model.  This is not limited to improving coding efficiency, but also reducing complexity and harmonization of the various schemes that are technically feasible.

Techniques are evaluated according to BD-rate gain, complexity and/or subjective picture quality. The following items are categorized as subtests:

- Implementation friendly improvement of non-deblocking in-loop filtering.  4 proposals.

JCTVC-G218 (MediaTek/Qualcomm/Toshiba)

JCTVC-G499 (Cisco)

JCTVC-G831 (MediaTek/Qualcomm)

JCTVC-G499, G218 and G831 harmonization (MediaTek/Qualcomm/Toshiba)

- ALF simplification.  5 proposals.

JCTVC-G351 (Sony)

JCTVC-G813 (TI)

- Necessity of sign bits for SAO offsets.  2 proposals.

JCTVC-G680 (TI)

JCTVC-Gxxx or HM5.0 plus signed offset encoder control (MediaTek)	Note: This would show the necessity of sign bits for offset.

- SAO simplification and line buffer reduction.  2 proposals.

JCTVC-G682 (TI)

Panasonic (JCTVC-G212-Tool2)

- SAO improvement (lower priority than the other subtests when discussing at the San Jose meeting, if the improvement is minor).  5 proposals.

JCTVC-G222 (MediaTek/Qualcomm)

JCTVC-G246 (Canon)

JCTVC-G309 (Sharp)

JCTVC-G490 (Ericsson)

JCTVC-G915 (Motorola Mobility)

JCTVC-G490 and JCTVC-G246 harmonization (Ericsson/Canon)

- Subjective picture quality assessment of ALF. (Volunteered by TI.)

JCTVC-G684 (TI)

The software basis for this CE is HM 5.0, and the experiments are conducted based on the test conditions in JCTVC-G1xxx in general.  However, there is an exception that require multiple-slice coding configuration.  Details are described in section 3.

# Participants

| Nr. | Name | Company | Email | Remarks |
| --- | --- | --- | --- | --- |
| 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 21 22 23 24 25 26 27 28 29 30 31 32 33 34 35 36 37 38 39 40 41 42 43 44 45 46 47 48 49 50 51 52 53 54 55 56 57 58 59 60 61 62 | Chia-Yang Tsai Chih-Ming Fu Ching-Yeh Chen Shawmin Lei Xun Guo Yu-Wen Huang (coordinator) Matthias Narroschke (coordinator) Thomas Wedi Semih Esenlik Marta Karczewicz In Suk Chong (coordinator) Wei-Jung Chien Wang (PoLin) Lai Felix C. Fernandes Elena Alshina Il-Koo Kim Tomohiro Ikai Tomoyuki Yamamoto Yukinobu Yasugi Andrew Segall Takeshi Chujoh Tomoo Yamakage (Primary coordinator) Andrey Norkin Kenneth Andersson Faouzi Kossentini Michael Horowitz Hsan Guermazi Madhukar Budagavi Minhua Zhou Woo-Shik Kim Shun-ichi Sekiguchi Kazuo Sugimoto Ali Tabatabai Ehsan Maani Masaru Ikeda Y.-J. Chiu L. Xu W. Zhang Han Yu B. Bross T. K. Tan Chuohao Yeo Yih Han Tan Kemal Ugur Oguz Bici Arild Fuldseth Thomas Davies Siwei Ma Masaaki Matsumura Jinho Lee Hui Yong Kim Sung-Chang Lim Ju Ock Lee Joo-Hee Moon Jeongyoen Lim Edouard Francois Patrice Onno Seungwook Park Yong-Joon Jeon Joonyoung Park David Baylon Koohyar Minoo | MediaTek MediaTek MediaTek MediaTek MediaTek MediaTek Panasonic Panasonic Panasonic Qualcomm Qualcomm Qualcomm Samsung Samsung Samsung Samsung Sharp Sharp Sharp Sharp Toshiba Toshiba Ericsson Ericsson eBrisk Video eBrisk Video eBrisk Video TI TI TI Mitsubishi Mitsubishi Sony Sony Sony Intel Intel Intel Intel HHI DOCOMO I2R I2R Nokia Nokia Cisco Cisco Peking Univ. NTT ETRI ETRI ETRI Sejong Univ. SK Telecom SK Telecom Canon Canon LGE LGE LGE Motorola M. Motorola M. | chiayang.tsai@mediatek.com chihming.fu@mediatek.com chingyeh.chen@mediatek.com shawmin.lei@mediatek.com xun.guo@mediatek.com yuwen.huang@mediatek.com Matthias.Narroschke@panasonic.de Thomas.Wedi@eu.panasonic.com Semih.Esenlik@eu.panasonic.com martak@qualcomm.com ichong@qualcomm.com wchien@qualcomm.com wlai@sta.samsung.com ffernandes@sta.samsung.com elena_a.alshina@samsung.com ilkoo.kim@samsung.com ikai.tomohiro@sharp.co.jp yamamoto.tomoyuki@sharp.co.jp yasugi.yukinobu@sharp.co.jp asegall@sharplabs.com takeshi.chujoh@toshiba.co.jp tomoo.yamakage@toshiba.co.jp andrey.norkin@ericsson.com kenneth.r.andersson@ericsson.com faouzi@ebriskvideo.com michael@ebriskvideo.com hsan@ebriskvideo.com madhukar@ti.com zhou@ti.com wskim@ti.com Sekiguchi.Shunichi@eb.MitsubishiElectric.co.jp Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp Ali.Tabatabai@am.sony.com Ehsan.Maani@am.sony.com Masaru.Ikeda@jp.sony.com Yi-jen.chiu@intel.com Lidong.xu@intel.com Wenhao.zhang@intel.com yu.han@intel.com benjamin.bross@hhi.fraunhofer.de ttk@pacific.net.sg chyeo@i2r.a-star.edu.sg yhtan@i2r.a-star.edu.sg Kemal.Ugur@nokia.com ext-oguz.bici@nokia.com arilfuld@cisco.com thdavies@cisco.com swma@jdl.ac.cn matsumura.masaaki@lab.ntt.co.jp jinosoul@etri.re.kr hykim5@etri.re.kr sclim@etri.re.kr jolee@sju.ac.kr jhmoon@sejong.ac.kr jeongyeon@sk.com edouard.francois@crf.canon.fr Patrice.Onno@crf.canon.fr Seungwook.park@lge.com yongjoon.jeon@lge.com jy.park@lge.com david.baylon@motorola.com kminoo@motorola.com | Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent |

# Experimental Conditions

## Software

HM 5.0 shall be used, which will be released on Dec. xx, 2011.

## Test Sequences, Bit Rates and Coding Conditions

Unless it is described as optional in JCTVC-G1xxx, test all test sequences under the configurations described in JCTVC-G1xxx.

If a proposal is related to SAO, both Low Complexity and High Efficiency cases (AI, RA, LDB and LDP) shall be tested.

If a proposal is related to only ALF, all High Efficiency cases (AI, RA, LDB and LDP) shall be tested.  However, if the proposal intends to adopt ALF in Low Complexity cases, it is mandatory for the proposal to provide experimental results in Low Complexity cases (AI, RA, LDB and LDP).

Since some proposals relate to multi-slice coding, an alternative test conditions for multi-slice coding will be specified during the editing period.  (Ed. We need to identify the proposals and define the alternative test conditions.)

## Evaluation of CE Results

Results of the CE will be evaluated on the basis of BD-measures, computational and implementation complexity and/or functionality.

BD-rate gain/loss and runtime numbers shall be provided according to JCTVC-G1xxx.

For the complexity measurement, it is recommended that the HM 5.0 software with the anchor configuration and the proposal implemented on the HM 5.0 software will be executed on the same (or identically configured) machine(s), and the computational time will be measured for each test sequence and test case. Relative computational time is calculated against the anchor.

In addition, subjective quality evaluation will be performed if necessary.

# Subtest a: Implementation friendly improvement of non-deblocking in-loop filtering

## Functionality Addressed

- Provide low latency processing capability on non-deblocking in-loop filtering

## Expected Gains

- Provide low latecy friendly design

## Evaluation Criteria

- Coding efficiency: BD-rate

- Encoding and decoding time

- Number of coefficients

- Maximum number of vertical taps

- Number of filter types

- Prediction method for filter coefficients

Note: Results will be required for low latency scenario in addition to frame latency scenario.

## Tool Description

### CE8.a.1: One-stage non-deblocking loop filtering (JCTVC-G218, MediaTek/Qualcomm/Toshiba)

In G218, SAO and ALF are combined into one stage. Three different modes, ALF+SAO, SAO, and OFF, can be used and adapted one block by one block. The ALF+SAO mode is a replacement of sequential process of SAO and ALF in current WD/HM design to enable 1-stage process of SAO and ALF. In HE case, ALF+SAO, SAO, or OFF can be used for one block, and in LC case, only SAO or OFF can be selected. In this core experiment, the size of block and harmonization of other ALF and SAO coding tools are investigated. Moreover, low-latency encoding algorithm will be designed for low latency applications.

### CE8.a.2: Improved ALF with low latency and reduced complexity (JCTVC-G499, Cisco)

One single set of ALF filter coefficients are transmitted for each LCU using single-pass encoder-side estimation. For each LCU, one can choose between a 5x5 snowflake pixel mask or a 9x3 cross pixel filter shape. Also, for each LCU, one can choose between transmitting a new set of filter coefficients and using one of up to 16 previously transmitted sets of filter coefficients (adaptive codebook) for each mask.

As suggested during the 7th JCT-VC meeting the method will be extended to support:

- ALF applied to chroma samples

- Methods for line buffer removal and slice boundary processing

- No look-ahead to LCUs below and to the right during the coefficient estimation/quantization process

- Reset of adaptive codebook at:

  - Slice boundaries

  - Each LCU line if wavefront processing is used

- Larger pixel masks (and other techniques) to improve class A & B in particular.

- Filter coding improvements to address some of the loss in Classes A & B.

### CE8.a.3: Sample Adaptive Offset with LCU-based Syntax (JCTVC-G831, MediaTek/Qualcomm)

Allow SAO parameters to be adaptively changed at any largest coding unit (LCU) so that LCU-based syntax structure is provided.  SAO parameters can be signalled LCU by LCU or can be copied from left or above LCU or above LCU line.  Prediction of offset is also applied.

### CE8.a.4: Harmonization of JCTVC-G499, G218 and G831 (JCTVC-G499, JCTVC-G218, JCTVC-G831,MediaTek/Qualcomm/Toshiba)

Harmonization of JCTVC-G218 and JCTVC-499 is studied so that SAO and ALF can provide slice-level latency coding and transmittion, which has following aspects:

Introduce 1-stage SAO and ALF from JCTVC-G218.

Harmonize LCU syntax from JCTVC-G499, JCTVC-G218 and JCTVC-G831.

Exploit LCU-based encoding scheme in JCTVC-G499.

## Participants

| Participant | Contact |
| --- | --- |
| MediaTek | Yu-Wen Huang yuwen.huang@mediatek.com |
| Qualcomm |  |
| Toshiba | Tomoo Yamakage tomoo.yamakage@toshiba.co.jp |
| Cisco | Arild Fuldseth arild.fuldseth@cisco.com |

Proponent(s)						Cross-checker(s)

- MediaTek/Qualcomm/Toshiba			Sharp

- Cisco						Toshiba

- MediaTek/Qualcomm

- MediaTek/Qualcomm/Toshiba			Cisco

# Subtest b: ALF simplification

## Functionality Addressed

- Reduce complexity of ALF

## Expected Gains

- Complexity reduction in terms of number of operations, reduction of pipeline processing cycles and silmpler design

## Evaluation Criteria

- Coding efficiency: BD-rate

- Encoding and decoding time

## Tool Description

### CE8.b.1 Modification of ALF classification (JCTVC-G351, Sony)

In current block-based ALF (BA), both classification and calculation are performed independently. Therefore, the current BA classification requires extra buffers and iterative processing, i.e.,the classification is performed first for the 4x4 block and then the filtering process starts.

Based on this proposal, the classification for only top-left pixel in 4x4 block is performed with using the same pixel range (5x5) as the filtering for the top-left pixel. Thus, the classification for top-left is performed and then the filtering of that pixel can start. This results in a reduced buffer size which is needed for the classification and therefore allows single-pass processing of the 4x4 block (since pixel buffer for both processes can be shared).

### CE8.b.2 ALF with single filter type (JCTVC-G813, TI)

Test different filter shapes for single filter type.

## Participants

| Participant | Contact |
| --- | --- |
| Sony | Ehsan Maani Ehsan.Maani@am.sony.com Masaru Ikeda Masaru.Ikeda@jp.sony.com |
| TI | Madhukar Budagavi madhukar@ti.com |

Proponent(s)						Cross-checker(s)

(1) Sony (G351)

(2) TI (G813)

# Subtest c: Necessity of sign bits for SAO offsets

## Functionality Addressed

- Potential undesired artifact by SAO

- Evaluation of necessity of sign bits for SAO edge offsets

- Bit saving to signal SAO edge offsets

- SAO simplification by eliminating edge offset sign coding

## Expected Gains

- Assessment of subjective picture quality improvement

- Complexity reduction by eliminating edge offset sign coding

## Evaluation Criteria

- Coding efficiency: BD-rate

- Encoding and decoding time

- Subjective picture quality

- Computational and implementation complexity

## Tool Description

### CE8.d.1 Method of visual coding artifact removal for SAO (TI, JCTVC-G680)

The sign value of edge offset is restricted according to edge category, so that contrast between the current and neighboring pixels is not increased after adding the offset.  This is asserted to remove SAO visual artifacts.  This also reduces complexity by eliminating offset sign coding. Comparison will be with respect to encoder only solution.

### CE8.d.2 xxxxx (MediaTek, JCTVC-Gxxx???)

Which document we should refer to?

## Participants

| Participant | Contact |
| --- | --- |
| TI | Madhukar Budagavi madhukar@ti.com |
| MediaTek | Yu-Wen Huang yuwen.huang@mediatek.com |

Proponent(s)						Cross-checker(s)

(1) TI (G680)

(2) MediaTek (Gxxx or HM5.0 plus signed offset encoder control)

# Subtest d: SAO simplification

## Functionality Addressed

- Reduction of buffers to store SAO offsets

## Expected Gains

- Complexity reduction

## Evaluation Criteria

- Coding efficiency: BD-rate

- Required memory size

## Tool Description

### CE8.e.1 Reduced number of band offsets in SAO (TI, JCTVC-G682)

The number of band offsets is reduced from 16 to 8, and number of bands is increased from 2 to 4.  The purpose is to reduce the memory to store offsets in decoder.  Proposal 1: 4 band, 8 offsets.  Proposal 2: The coverage of offsets of the first and the last sub-band are extended to the pixels outside the band.

### CE8.e.2 Line memory reduction for SAO (Panasonic, JCTVC-G212-Tool2)

For LCU-based filtering with SAO, a line buffer of 2 bits per pixel is required to be implemented in order to store partial SAO pixel classification results. Since the line memory requirement is directly proportional to the picture width, the SAO line buffer constitutes a big implementation burden.

The proposal explores a method of line memory reduction in order to reduce the line memory requirement for SAO.

## Participants

| Participant | Contact |
| --- | --- |
| TI | Madhukar Budagavi madhukar@ti.com |
| Panasonic | Semih Esenlik Semih.Esenlik@eu.panasonic.com |

Proponent(s)						Cross-checker(s)

(1) TI (G682)

(2) Panasonic (G212-Tool2)				MediaTek

# Subtest e: SAO improvement

## Functionality Addressed

- Coding efficienty improvement of SAO

## Expected Gains

- Coding efficienty improvement of SAO

## Evaluation Criteria

- Coding efficiency: BD-rate

- Encoding and decoding time

## Tool Description

### CE8.f.1 Offset coding in SAO (MediaTek/Qualcomm, JCTVC-G222)

New codewords to better fit the offset distribution. A simple prediction technique to reduce offset information.

### CE8.f.2 On additional SAO Band Offset classifications (Canon, JCTVC-G246)

Add 8 Band Offset classifications to the 2 current HM4.0 BO groups.  In particular, 3 additional intensity subdivision ranges are added.  Worst case comlexity does not increase since it comes from EO.  Decrease of decoding time by selecting BO more frequently.   New RDO selection for all SAO classification.  Another test is to add 6 Band Offset classification.

The goal of the experiment is:

- test G246 with different settings regarding the number of offset ranging from lower number for simplifications to higher number for coding efficiency improvement

- test G246 in combination with various loop filter settings with EO on/off.

### CE8.f.3 Modified SAO edge offsets (Ericsson, JCTVC-G490)

By adding the possibility to use more specific edge offsets when needed both the subjective and the objective quality can be improved.  Switch between current 4 constrained edge offset or new 10 unconstrained edge offset.  Prediction of edge offsets is also applied. Investigate if the number of edge offsets can be reduced.

### CE8.f.4 Coding and selection of SAO parameters (Motorola Mobility, JCTVC-G915)

This contribution proposes to increase the number of EO classes from four to eight (spec. change), change the coding of the SAO type, and optimize the selection and coding of SAO offsets.  The newly adopted encoder side optimization in JCTVC-G915, demands a revisit of the optimal number of SAO types and number of classes per type.

### CE8.f.5 Combination of JCTVC-G490 (Ericsson) and JCTVC-G246 (Canon)

The purpose is to investigate if improvements of EO in JCTVC-G490 (CE8.f.4) is additive to improvements of BO in JCTVC-G246 (CE8.f.2).

## Participants

| Participant | Contact |
| --- | --- |
| MediaTek | Yu-Wen Huang yuwen.huang@mediatek.com |
| Qualcomm |  |
| Canon | Guillaume Laroche guillaume@crf.canon.fr Patrice Onno patrice.onno@crf.canon.fr |
| Ericsson | Kenneth Andersson kenneth.r.andersson@ericsson.com |
| Motrola Mobility | Koohyar Minoo kminoo@motorola.com |

Proponent(s)						Cross-checker(s)

(1) MediaTek/Qualcomm (G222)

(2) Canon (G246)

(3) Ericsson (G490)

(4) Motrola Mobility (G915)

(5) Canon/Motorola Mobility (G246, G490)

# Subtest f: Subjective picture quality assessment of ALF

## Functionality Addressed

- Assessment of subjective picture quality

## Expected Gains

## Evaluation Criteria

- Subjective picture quality

## Tool Description

### CE8.g.1 Subjective Tests on ALF and SAO Using HM-4.0 (TI, JCTVC-G684)

This test evaluates visual quality of ALF filter on top of SAO.

Bitstreams will be generated with following options:

|  | SAO | ALF | Postfilter |
| --- | --- | --- | --- |
| Anchor | ON | ON | OFF |
| Test1 | ON | OFF | OFF |
| Test2 | ON | OFF | ON |

Following configurations will be tested:

- Test1 v/s Anchor

- Test2 v/s Anchor

Following material will be used for visual testing.

| Sequence | Class | QP value | Duration |
| --- | --- | --- | --- |
| BasketballDrill | C | 32, 37 | 10 s |
| RaceHorses | C | 32, 37 | 10 s |
| Vidyo1 | E | 32, 37 | 10 s |
| Vidyo3 | E | 32, 37 | 10 s |
| Vidyo4 | E | 32, 37 | 10 s |
| BasketballDrive | B | 32, 37 | 10 s (*) |
| Cactus | B | 32, 37 | 10 s (*) |

(*) – Length of sequence will be finalized  based on test equipment availability.

Volunteers is allowed some other sequences and/or test configurations to demonstrate the picture quality improvement.

Test method:

To be defined after consultation with Vittorio.

Logistics:

TBD.

## Participants

| Participant | Contact |
| --- | --- |
| TI | Madhukar Budagavi madhukar@ti.com |
| MediaTek | Yu-Wen Huang yuwen.huang@mediatek.com |

Proponent(s)						Cross-checker(s)

(1) TI (G684)

# Time-line and Responsibilities (T.B.D.)

T1:		2011-Dec-14:	Finalize CE description and upload

T2:		2011-Dec-xx:	HM 5.0 available

T3:		2012-Jan–xx:	Software and draft contribution text available to all CE participants and start cross-checking for regular tests

T4:		2012-Jan-xx:		Report proponents’ results to Coordinators

T5:		2012-Jan-xx:		Contribution documents upload, and report the document number and title to coordinators

T6: 	2012-Jan-xx		Summary report upload

Note: Time-line changed to reflect the release date of HM 5.0.

# References

[1]	“Common HM test conditions and software reference configurations”, JCTVC-G1xxx, Geneva, CH, November 2011.

[2]	“High Efficiency Video Coding (HEVC) Test Model 5 (HM 5) Encoder Description”, JCTVC-G1xxx, Geneva, CH, November 2011.

[3]	“High Efficiency Video Coding (HEVC) text specification Working Draft 5”, JCTVC-G1xxx, Geneva, CH, November 2011.