| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-G1208_r1 WG11 Number: m22849 |
| --- | --- |

| Title: | Description of Core Experiment 8 (CE8): Non-deblocking loop filtering |  |  |
| --- | --- | --- | --- |
| Status: | Output Document of JCT-VC |  |  |
| Purpose: | CE description |  |  |
| Author(s) or Contact(s): | Tomoo Yamakage (Primary) In Suk Chong Matthias Narroschke Yu-Wen Huang |  | tomoo.yamakage@toshiba.co.jp ichong@qualcomm.com Matthias.Narroschke@eu.panasonic.com yuwen.huang@mediatek.com |
| Source: | CE Coordinators |  |  |

_____________________________

# Introduction

The goal of this Core Experiment (CE) is to enhance non-deblocking loop filtering techniques in the HEVC Test Model.  This is not limited to improving coding efficiency, but also reducing complexity and harmonization of the various schemes that are technically feasible.

Techniques are evaluated according to BD-rate gain, complexity and/or subjective picture quality. The following items are categorized as subtests:

- Implementation friendly improvement of non-deblocking in-loop filtering.  4 proposals.

JCTVC-G218 (MediaTek/Qualcomm/Toshiba)

JCTVC-G499 (Cisco)

JCTVC-G831 (MediaTek/Qualcomm)

JCTVC-G499, G218 and G831 harmonization (MediaTek/Qualcomm/Toshiba)

- ALF simplification.  2 proposals.

JCTVC-G351 (Sony/Samsung)

JCTVC-G813 (TI)

- Necessity of sign bits for SAO offsets.  1 proposal.

JCTVC-G680 (TI)

- SAO simplification and line buffer reduction.  2 proposals.

JCTVC-G682 (TI)

Panasonic (JCTVC-G212-Tool2)

- SAO improvement (lower priority than the other subtests when discussing at the San Jose meeting, if the improvement is minor).  5 proposals.

JCTVC-G222 (MediaTek/Qualcomm)

JCTVC-G246 (Canon)

JCTVC-G490 (Ericsson)

JCTVC-G915 (Motorola Mobility)

JCTVC-G490 and JCTVC-G246 harmonization (Ericsson/Canon)

- Subjective picture quality assessment of ALF. (Volunteered by TI.)

JCTVC-G684 (TI)

The subjective viewing of all subtests will be conducted.  On the first day of the San Jose meeting, the viewing of subtests a to e will be conducted.  Prior to the San Jose meeting, some formal subjective viewing of subtest f will be conducted.  Test sequences and logististic will be determined after the release of HM 5.0.  (It is expected after the deadline of finalization of CE description.)

The software basis for this CE is HM 5.0, and the experiments are conducted based on the test conditions in JCTVC-G1200 in general.  However, there is an exception that requires multiple-slice coding configuration additionally.  Details are described in section 3.

# Participants

| Nr. | Name | Company | Email | Remarks |
| --- | --- | --- | --- | --- |
| 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 21 22 23 24 25 26 27 28 29 30 31 32 33 34 35 36 37 38 39 40 41 42 43 44 45 46 47 48 49 50 51 52 53 54 55 56 57 58 59 60 61 62 63 64 65 66 67 68 69 70 | Chia-Yang Tsai Chih-Ming Fu Ching-Yeh Chen Shawmin Lei Xun Guo Yu-Wen Huang (coordinator) Matthias Narroschke (coordinator) Thomas Wedi Semih Esenlik Toru Matsunobu Marta Karczewicz In Suk Chong (coordinator) Wei-Jung Chien Wang (PoLin) Lai Felix C. Fernandes Elena Alshina Il-Koo Kim Ankur Saxena Tomohiro Ikai Tomoyuki Yamamoto Yukinobu Yasugi Andrew Segall Jane Zhao Takeshi Chujoh Tomoo Yamakage (Primary coordinator) Andrey Norkin Kenneth Andersson Faouzi Kossentini Michael Horowitz Hsan Guermazi Madhukar Budagavi Minhua Zhou Woo-Shik Kim Do-Kyoung Kwon Shun-ichi Sekiguchi Kazuo Sugimoto Ali Tabatabai Ehsan Maani Masaru Ikeda Y.-J. Chiu L. Xu W. Zhang Han Yu B. Bross T. K. Tan Frank Bossen Chuohao Yeo Yih Han Tan Kemal Ugur Oguz Bici Arild Fuldseth Thomas Davies Siwei Ma Masaaki Matsumura Jinho Lee Hui Yong Kim Sung-Chang Lim Ju Ock Lee Joo-Hee Moon Jeongyoen Lim Edouard Francois Patrice Onno Guillaume Laroche Seungwook Park Yong-Joon Jeon Joonyoung Park David Baylon Koohyar Minoo Wade Wan Yongbing Lin | MediaTek MediaTek MediaTek MediaTek MediaTek MediaTek Panasonic Panasonic Panasonic Panasonic Qualcomm Qualcomm Qualcomm Samsung Samsung Samsung Samsung Samsung Sharp Sharp Sharp Sharp Sharp Toshiba Toshiba Ericsson Ericsson eBrisk Video eBrisk Video eBrisk Video TI TI TI TI Mitsubishi Mitsubishi Sony Sony Sony Intel Intel Intel Intel HHI DOCOMO NTT Docomo I2R I2R Nokia Nokia Cisco Cisco Peking Univ. NTT ETRI ETRI ETRI Sejong Univ. SK Telecom SK Telecom Canon Canon Canon LGE LGE LGE Motorola M. Motorola M. Broadcom Huawei | chiayang.tsai@mediatek.com chihming.fu@mediatek.com chingyeh.chen@mediatek.com shawmin.lei@mediatek.com xun.guo@mediatek.com yuwen.huang@mediatek.com Matthias.Narroschke@panasonic.de Thomas.Wedi@eu.panasonic.com Semih.Esenlik@eu.panasonic.com matsunobu.toru@jp.panasonic.com martak@qualcomm.com ichong@qualcomm.com wchien@qualcomm.com wlai@sta.samsung.com ffernandes@sta.samsung.com elena_a.alshina@samsung.com ilkoo.kim@samsung.com asaxena@sta.samsung.com ikai.tomohiro@sharp.co.jp yamamoto.tomoyuki@sharp.co.jp yasugi.yukinobu@sharp.co.jp asegall@sharplabs.com jzhao@sharplabs.com takeshi.chujoh@toshiba.co.jp tomoo.yamakage@toshiba.co.jp andrey.norkin@ericsson.com kenneth.r.andersson@ericsson.com faouzi@ebriskvideo.com michael@ebriskvideo.com hsan@ebriskvideo.com madhukar@ti.com zhou@ti.com wskim@ti.com d-kwon@ti.com Sekiguchi.Shunichi@eb.MitsubishiElectric.co.jp Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp Ali.Tabatabai@am.sony.com Ehsan.Maani@am.sony.com Masaru.Ikeda@jp.sony.com Yi-jen.chiu@intel.com Lidong.xu@intel.com Wenhao.zhang@intel.com yu.han@intel.com benjamin.bross@hhi.fraunhofer.de ttk@pacific.net.sg bossen@docomoinnovations.com chyeo@i2r.a-star.edu.sg yhtan@i2r.a-star.edu.sg Kemal.Ugur@nokia.com ext-oguz.bici@nokia.com arilfuld@cisco.com thdavies@cisco.com swma@jdl.ac.cn matsumura.masaaki@lab.ntt.co.jp jinosoul@etri.re.kr hykim5@etri.re.kr sclim@etri.re.kr jolee@sju.ac.kr jhmoon@sejong.ac.kr jeongyeon@sk.com edouard.francois@crf.canon.fr patrice.onno@crf.canon.fr guillaume.laroche@crf.canon.fr Seungwook.park@lge.com yongjoon.jeon@lge.com jy.park@lge.com david.baylon@motorola.com kminoo@motorola.com wwan@broadcom.com linyongbing@huawei.com | Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent |

# Experimental Conditions

## Software

HM 5.0 shall be used, which will be released on Dec. xx, 2011.

## Test Sequences, Bit Rates and Coding Conditions

All test sequences under the configurations described in JCTVC-G1200 shall be tested.

If a proposal is related to SAO, High Efficiency cases (AI, RA, LDB and LDP) and High Efficiency Random Access 10 bit cases (RA10) shall be tested.

If a proposal is related to only ALF, all High Efficiency cases (AI, RA, LDB and LDP) and High Efficiency Random Access 10 bit cases (RA10) shall be tested.  However, if the proposal intends to adopt ALF in Low Complexity cases, it is mandatory for the proposal to provide experimental results in Low Complexity cases (AI, RA, LDB and LDP).

Since some proposals relate to multi-slice coding, alternative test conditions for multi-slice coding will be specified in the related subtests.

## Evaluation of CE Results

Results of the CE will be evaluated on the basis of BD-measures, computational and implementation complexity and/or functionality.

BD-rate gain/loss and runtime numbers shall be provided according to JCTVC-G1200.

For the complexity measurement, it is recommended that the HM 5.0 software with the anchor configuration and the proposal implemented on the HM 5.0 software will be executed on the same (or identically configured) machine(s), and the computational time will be measured for each test sequence and test case. Relative computational time is calculated against the anchor. Furthermore, if a proposal modifies complexity assessment criteria from WD5 such as number of ALF filter coefficients, maximum number of vertical taps for ALF, number of candidate filters, filter shapes, input source signals to SAO and/or ALF, number of pixels to be used for SAO/ALF filter classification and area for the classification, those numbers shall be reported in comparison with WD5. If a proposal will claim the benefit (and assumption) that can not be assessed by the above metrics, it shall be explained in the contribution document so that all committee members can study in advance to the discussion during the San Jose meeting.

In addition, subjective quality evaluation will be performed.

# Subtest a: Implementation friendly improvement of non-deblocking in-loop filtering

## Functionality Addressed

- Provide low latency processing capability on non-deblocking in-loop filtering

## Expected Gains

- Provide low latency friendly design

## Evaluation Criteria

General evaluation criteria are listed in section 3.3.  The following criteria may be highlighted:

- Coding efficiency: BD-rate

- Encoding and decoding time

- Number of coefficients

- Maximum number of vertical taps

- Number of filter types

- Prediction method for filter coefficients

- Subjective picture quality

Note: Results for LDB and LDP will be required for low latency scenario with multiple slices per frame, in addition to frame latency scenario.  Multiple slice configurations are 64, 104, 28, 60, and 96 LCUs per slice for 1920x1080, 832x480, 416x240, 1280x720, and 1024x768 resolutions, respectively.  Since HM 5.0 does not have slice-based ALF to support low latency scnerio, the anchor for low latency scenario (i.e., multiple slice encoding) shall not use ALF, which means that the anchor is just intended to be used as the common base of the comparison between proposals.

## Tool Description

### CE8.a.1: One-stage non-deblocking loop filtering (JCTVC-G218, MediaTek/Qualcomm/Toshiba)

In G218, SAO and ALF are combined into one stage. Three different modes, ALF+SAO, SAO, and OFF, can be used and adapted one block by one block. The ALF+SAO mode is a replacement of sequential process of SAO and ALF in the current WD/HM design to enable 1-stage process of SAO and ALF. For SAO and ALF+SAO, LCU based syntax is used. As an encoding algorithm, frame based algorithm is used.

In this core experiment, following aspects will be additionally studied on top of JCTVC-G218:

- Low latency encoding algorithm of SAO and ALF (i.e., LCU based encoding algorithm or using previous frame's coefficients).

- The performance of subset of this approach  (i.e., SAO or ALF+SAO only case).

  - As a piece of information, ALF only case can also be considered.

- Multiple slices use cases, the possibility of moving coefficients from APS to slice header (or additionally signalling coefficients in slice header) can be considered.

### CE8.a.2: Improved ALF with low latency and reduced complexity (JCTVC-G499, Cisco)

One single set of ALF filter coefficients are transmitted for each LCU using single-pass encoder-side estimation. For each LCU, one can choose between a 5x5 snowflake pixel mask or a 9x3 cross pixel filter shape. Also, for each LCU, one can choose between transmitting a new set of filter coefficients and using one of up to 16 previously transmitted sets of filter coefficients (adaptive codebook) for each mask.

As suggested during the 7th JCT-VC meeting the method will be extended to support:

- ALF applied to chroma samples

- Methods for line buffer removal and slice boundary processing

- No look-ahead to LCUs below and to the right during the coefficient estimation/quantization process

- Reset of adaptive codebook at:

  - Slice boundaries

  - Each LCU line if wavefront processing is used

- Larger pixel masks (and other techniques) to improve class A & B in particular.

- Filter coding improvements to address some of the loss in Classes A & B.

### CE8.a.3: Sample Adaptive Offset with LCU-based Syntax (JCTVC-G831, MediaTek/Qualcomm)

Allow SAO parameters to be adaptively changed at any largest coding unit (LCU) so that LCU-based syntax structure is provided.  SAO parameters can be signalled LCU by LCU or can be copied from left or above LCU or above LCU line.  Prediction of offset is also applied.

### CE8.a.4: Harmonization of JCTVC-G499, G218 and G831 (JCTVC-G499, JCTVC-G218, JCTVC-G831,MediaTek/Qualcomm/Toshiba)

Harmonization of JCTVC-G218, JCTVC-G831 and JCTVC-G499 is studied so that SAO and ALF can provide slice-level latency coding and transmission, which has following aspects:

- Introduce 1-stage SAO and ALF from JCTVC-G218.

- Harmonize LCU syntax from JCTVC-G499, JCTVC-G218 and JCTVC-G831.

- Exploit LCU-based encoding scheme in JCTVC-G499.

Tests related to CE8.a are as follows:

| Test | Tools | Tester |
| --- | --- | --- |
| HM5.0 - One slice per frame (anchor) | Frame-based ALF (HM5.0) | - |
| Test1a - One slice per frame | LCU-based 1-stage ALF (G218, CE8.a.1) | MQT, Sharp |
|  | LCU-based ALF (G499, CE8.a.2) | Cisco |
|  | LCU-based ALF (G831, CE8.a.3) | MQ, Sharp |
|  | LCU-based harmonized ALF (CE8.a.4) | MQT |
| HM5.0 - Multile slices per frame (additional anchor) | Slice-based encoding with ALF off (HM 5.0 with ALF off) | - |
| Test1b – Multiple slices per frame | LCU-based 1-stage ALF (G218, CE8.a.1) | MQT |
|  | LCU-based ALF (G499, CE8.a.2) | Cisco |
|  | LCU-based ALF (G831, CE8.a.3) | MQ, Sharp |
|  | LCU-based harmonized ALF (CE8.a.4) | MQT |

Note: MQ means MediaTek and Qualcomm, MQT means MediaTek, Qualcomm and Toshiba.

## Participants

| Participant | Contact |
| --- | --- |
| MediaTek | Yu-Wen Huang yuwen.huang@mediatek.com |
| Qualcomm | In Suk Chong ichong@qualcomm.com |
| Toshiba | Tomoo Yamakage tomoo.yamakage@toshiba.co.jp |
| Cisco | Arild Fuldseth arild.fuldseth@cisco.com |
| Sharp | Tomohiro Ikai ikai.tomohiro@sharp.co.jp |

Proponent(s)						Cross-checker(s)

- MediaTek/Qualcomm/Toshiba	 (G218)		Sharp

- Cisco	 (G499)					Toshiba

- MediaTek/Qualcomm

- (G831)			Sharp

- MediaTek/Qualcomm/Toshiba		 (G218, G499, G831)	Cisco

# Subtest b: ALF simplification

## Functionality Addressed

- Reduce complexity of ALF

## Expected Gains

- Complexity reduction in terms of number of operations, reduction of pipeline processing cycles and silmpler design

## Evaluation Criteria

General evaluation criteria are listed in section 3.3.  The following criteria may be highlighted:

- Coding efficiency: BD-rate

- Encoding and decoding time

- Subjective picture quality

Note: Since the purpose of this subtest is to simplify SAO, negligible impact on subjective quality needs to be confirmed before discussing the adoption of the proposals.

## Tool Description

### CE8.b.1 Modification of ALF classification (JCTVC-G351, Sony/Samsung)

In current block-based ALF (BA), both classification and calculation are performed independently. Therefore, the current BA classification requires extra buffers and iterative processing, i.e.,the classification is performed first for the 4x4 block and then the filtering process starts.

Based on this proposal, the classification for only top-left subset of pixels in a 4x4 block is performed with using the same pixel range (5x5) as the filtering for the first top-left pixel in the 4x4 block. Thus, the classification for top-left is performed and then the filtering of that pixel can start. This results in a reduced buffer size which is needed for the classification and therefore allows single-pass processing of the 4x4 block (since pixel buffer for both processes can be shared). This proposal evaluates coding efficiency improvements to JCTVC-G351 while still using the same pixel range to compute directional features for classification.

### CE8.b.2 ALF with single filter type (JCTVC-G813, TI)

Test different filter shapes and sizes for single filter type.  Example filters that will be tested include cross filter with rectangular center and variants.

Tests related to CE8.b are as follows:

| Test | Tools | Tester1 | Tester2 |
| --- | --- | --- | --- |
| HM5.0 (anchor) | Adaptive star and cross shape ALF filters | - | - |
| Test1 | HM5.0 star shape ALF filter only | TI | SONY |
| Test2 | HM5.0 cross shape ALF filter only | TI | SONY |
| Test3 | Unifed single ALF filter | TI | SONY |

## Participants

| Participant | Contact |
| --- | --- |
| Sony | Ehsan Maani Ehsan.Maani@am.sony.com Masaru Ikeda Masaru.Ikeda@jp.sony.com |
| TI | Madhukar Budagavi madhukar@ti.com |

Proponent(s)						Cross-checker(s)

(1) Sony/Samsung (G351)

Qualcomm, TI

(2) TI (G813)						Samsung

# Subtest c: Necessity of sign bits for SAO offsets

## Functionality Addressed

General evaluation criteria are listed in section 3.3.  The following criteria may be highlighted:

- Potential undesired artifact by SAO

- Evaluation of necessity of sign bits for SAO edge offsets

- Bit saving to signal SAO edge offsets

- SAO simplification by eliminating edge offset sign coding

## Expected Gains

- Assessment of subjective picture quality improvement

- Complexity reduction by eliminating edge offset sign coding

## Evaluation Criteria

- Coding efficiency: BD-rate

- Encoding and decoding time

- Subjective picture quality

- Computational and implementation complexity

## Tool Description

### CE8.c.1 Method of visual coding artifact removal for SAO (TI, JCTVC-G680)

The sign value of edge offset is restricted according to edge category, so that contrast between the current and neighboring pixels is not increased after adding the offset.  This is asserted to remove SAO visual artifacts.  This also reduces complexity by eliminating offset sign coding. Comparison will be with respect to encoder only solution.

### Tests related to CE8.c are as follows:

| Test | Tools | Tester1 | Tester2 |
| --- | --- | --- | --- |
| HM5.0 | Both negative and positive EO offsets are allowed for all SAO categories | - | - |
| Test1 | Encoder only method, set SAO EO offset to zero if it is negative for categoty 1 and 2, and positive for category 3 and 4. | TI | MediaTek |
| Test2 | Removal of EO offset sign bit | TI | MediaTek |

## Participants

| Participant | Contact |
| --- | --- |
| TI | Madhukar Budagavi madhukar@ti.com |
| MediaTek | Yu-Wen Huang yuwen.huang@mediatek.com |
| Samsung | Elena Alshina elena_a.alshina@samsung.com |

Proponent(s)						Cross-checker(s)

(1) TI (G680)

MediaTek	, Samsung

# Subtest d: SAO simplification

## Functionality Addressed

- Reduction of buffers to store SAO offsets

## Expected Gains

- Complexity reduction

## Evaluation Criteria

General evaluation criteria are listed in section 3.3.  The following criteria may be highlighted:

- Coding efficiency: BD-rate

- Required memory size

- Subjective picture quality

Note: Since the purpose of this subtest is to simplify SAO, negligible impact on subjective quality needs to be confirmed before discussing the adoption of the proposals.

## Tool Description

### CE8.d.1 Reduced number of band offsets in SAO (TI, JCTVC-G682)

The number of band offsets is reduced from 16 to 8, and number of bands is increased from 2 to 4.  The purpose is to reduce the memory to store offsets in decoder.  Proposal 1: 4 bands, 8 offsets.  Proposal 2: The coverage of offsets of the first and the last sub-band are extended to the pixels outside the band.

Tests related to CE8.d.1 are as follows:

| Test | Tools | Tester1 | Tester2 |
| --- | --- | --- | --- |
| HM5.0 (anchor) | Number of SAO band offsets 16, number of bands 2 | - | - |
| Test1 | Tool1: Number of SAO band offsets 8, number of bands 4 | TI | MediaTek |
| Test2 | Tool1 plus coverage of offsets of the first and the last sub-band are extended to the pixels outside the band | TI | MediaTek |

### CE8.d.2 Line memory reduction for SAO (Panasonic, JCTVC-G212-Tool2)

For LCU-based filtering with SAO, a line buffer of 2 bits per pixel is required to be implemented in order to store partial SAO pixel classification results. Since the line memory requirement is directly proportional to the picture width, the SAO line buffer constitutes a big implementation burden.

The proposal explores a method of line memory reduction in order to reduce the line memory requirement for SAO.

## Participants

| Participant | Contact |
| --- | --- |
| TI | Madhukar Budagavi madhukar@ti.com |
| Panasonic | Semih Esenlik Semih.Esenlik@eu.panasonic.com |

Proponent(s)						Cross-checker(s)

(1) TI (G682)						MediaTek

(2) Panasonic (G212-Tool2)				MediaTek

# Subtest e: SAO improvement

## Functionality Addressed

- Coding efficienty improvement of SAO

## Expected Gains

- Coding efficienty improvement of SAO

## Evaluation Criteria

General evaluation criteria are listed in section 3.3.  The following criteria may be highlighted:

- Coding efficiency: BD-rate

- Encoding and decoding time

- Subjective picture quality

Note: Since the purpose of this subtest is to improve SAO, subjective picture quality improvement and/or objective improvement needs to be confirmed before discussing the adoption of the proposals.  Proponents and cross-checkers are requested to conduct such test prior to the meeting to determine whether the proposal should be evaluated during the meeting.

## Tool Description

### CE8.e.1 Offset coding in SAO (MediaTek/Qualcomm, JCTVC-G222)

New codewords to better fit the offset distribution. A simple prediction technique to reduce offset information.

### CE8.e.2 On additional SAO Band Offset classifications (Canon, JCTVC-G246)

Add 8 Band Offset classifications to the 2 current HM4.0 BO groups.  In particular, 3 additional intensity subdivision ranges are added.  Worst case comlexity does not increase since it comes from EO.  Decrease of decoding time by selecting BO more frequently.   New RDO selection for all SAO classification.  Another test is to add 6 Band Offset classification.

The goal of the experiment is:

- test G246 with different settings regarding the number of offset ranging from lower number for simplifications to higher number for coding efficiency improvement

- test G246 in combination with various loop filter settings with EO on/off.

One particular configuration is tested in this experiment. It will consist in limiting the number of offsets for SAO to 8. The current SAO band offset B0 and B1 are removed and changes by 3 different band offset as proposed in JCTVC-G246. These classifications are BO_NB_8_SUB_1, BO_NB_4_SUB_2 and BO_NB_2_SUB_4 as described in JCTVC-G246. For all this classes always 8 offsets are used.

A second experiment (corresponding to a simplification) is proposed in order to limit to only one SAO band offset classification with 8 offsets. In this proposed experiment, the SAO band offset classification is removed and changed by one SAO band with 8 offsets (corresponding to BO_NB_8_SUB_1 classification in G246).

### CE8.e.3 Modified SAO edge offsets (Ericsson, JCTVC-G490)

By adding the possibility to use more specific edge offsets when needed both the subjective and the objective quality can be improved.  Switch between current 4 constrained edge offset or new 10 unconstrained edge offset.  Prediction of edge offsets is also applied. Investigate if the number of edge offsets can be reduced.

### CE8.e.4 Coding and selection of SAO parameters (Motorola Mobility, JCTVC-G915)

This contribution proposes to increase the number of EO classes from four to eight (spec. change), change the coding of the SAO type, and optimize the selection and coding of SAO offsets.  The newly adopted encoder side optimization in JCTVC-G915, demands a revisit of the optimal number of SAO types and number of classes per type.

### CE8.e.5 Combination of JCTVC-G490 (Ericsson) and JCTVC-G246 (Canon)

The purpose is to investigate if improvements of EO in JCTVC-G490 (CE8.e.3) is additive to improvements of BO in JCTVC-G246 (CE8.e.2).  It will be tested how the number of offsets effect the objective and subjective performance.  It will also be investigated if line buffer requirement for EO for LCU based processing can be reduced without affecting objective and subjective quality.

## Participants

| Participant | Contact |
| --- | --- |
| MediaTek | Yu-Wen Huang yuwen.huang@mediatek.com |
| Qualcomm |  |
| Canon | Guillaume Laroche guillaume@crf.canon.fr Patrice Onno patrice.onno@crf.canon.fr |
| Ericsson | Kenneth Andersson kenneth.r.andersson@ericsson.com |
| Motrola Mobility | Koohyar Minoo kminoo@motorola.com |

Proponent(s)						Cross-checker(s)

(1) MediaTek/Qualcomm (G222)			Canon

(2) Canon (G246)					Ericsson

(3) Ericsson (G490)					Qualcomm

(4) Motrola Mobility (G915)				MediaTek

(5) Canon/Ericsson (G246, G490)			MediaTek

# Subtest f: Subjective picture quality assessment of ALF

## Functionality Addressed

- Assessment of subjective picture quality

## Expected Gains

- Understanding of ALF subjective gains

## Evaluation Criteria

The following criteria will be highlighted:

- Subjective picture quality

General evaluation criteria listed in section 3.3 (e.g. criteria to assess the expected complexity) may additionally be used.

## Tool Description

### CE8.f.1 Subjective Tests on ALF and SAO Using HM-5.0 (TI, JCTVC-G684)

This test evaluates visual quality of ALF filter on top of SAO.

Bitstreams will be generated with following options:

|  | SAO | ALF | Postfilter(*) |  | Testers (**) |
| --- | --- | --- | --- | --- | --- |
| HM5.0 (Anchor) | ON | ON | OFF |  | - |
| Test1 | ON | OFF | OFF |  | TI, MMI, Qualcomm, Toshiba |
| Test2 | ON | OFF | ON |  | TI, SONY, Toshiba |

(*) Postfilter scheme adopts off-loop ALF. (i.e., Just apply ALF as a post filter.)

(**) Testers may share the task or cross-check the results.

Following configurations will be tested:

- Test1 v/s Anchor (LDP, tentative, needs to be finalized after the release of HM 5.0).

- Test2 v/s Anchor (LDP, tentative, needs to be finalized after the release of HM 5.0).

Following material will be used for visual testing (tentative, needs to be finalized after the release of HM 5.0).

| Sequence | Class | QP value | Duration |
| --- | --- | --- | --- |
| BasketballDrill | C | 32, 37 | 10 s |
| RaceHorses | C | 32, 37 | 10 s |
| Vidyo1 | E | 32, 37 | 10 s |
| Vidyo3 | E | 32, 37 | 10 s |
| Vidyo4 | E | 32, 37 | 10 s |
| BasketballDrive | B | 32, 37 | 10 s (*) |
| Cactus | B | 32, 37 | 10 s (*) |
| Kimono | B | 32, 37 | 10 s (*) |

(*) – Length of sequence will be finalized based on test equipment availability.

The percentage difference between bitrates of test and anchor bitstreams will be reported. The bitrate will be attempted to be controlled within 2% range (no rate control will be used, fixed QP value will be increased or decreased by 1 or more for bitrate matching purposes).

Volunteers is allowed some other sequences including but not limited to Class A seuqnences and/or test configurations to demonstrate the picture quality improvement.

Test method:

To be defined after consultation with Vittorio.

Logistics:

TBD.

## Participants

| Participant | Contact |
| --- | --- |
| TI | Madhukar Budagavi madhukar@ti.com |
| MediaTek | Yu-Wen Huang yuwen.huang@mediatek.com |
| Sony | Ali Tabatabai Ali.Tabatabai@am.sony.com |
| Motorola Mobility | Koohyar Minoo kminoo@motorola.com |
| Broadcom | Wade Wan wwan@broadcom.com |
| NTT Docomo | Frank Bossen bossen@docomoinnovations.com |
| Toshiba | Tomoo Yamakage tomoo.yamakage@toshiba.co.jp |
| Qualcomm | In Suk Chong ichong@qualcomm.com |
| Sharp | Tomohiro Ikai ikai.tomohiro@sharp.co.jp |

Cross-checkers will be assigned after the test sequences and configurations are identified.

# Time-line and Responsibilities

T1:		2011-Dec-xx:	HM 5.0 available (expected 2.5 weeks after the meeting)

T2:		2011-Dec-21:	Finalize CE description and upload

T3:		2012-Jan–xx:	Software and draft contribution text available to all CE participants and start cross-checking for regular tests (two weeks after the release of HM 5.0 software)

T4:		2012-Jan-xx:		Report proponents’ results to Coordinators (three days after T3)

T5:		2012-Jan-20:		Contribution documents upload, and report the document number and title to coordinators

T6: 	2012-Jan-26		Summary report upload

Note: Time-line may be changed to reflect the release date of HM 5.0.

# References

[1]	“Common HM test conditions and software reference configurations”, JCTVC-G1200, Geneva, CH, November 2011.

[2]	“High Efficiency Video Coding (HEVC) Test Model 5 (HM 5) Encoder Description”, JCTVC-G1102, Geneva, CH, November 2011.

[3]	“High Efficiency Video Coding (HEVC) text specification Working Draft 5”, JCTVC-G1103, Geneva, CH, November 2011.