| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G1209 |
| --- | --- |

| Title: | Description of Core Experiment 9 (CE9): MV Coding and Skip/Merge Operations |  |  |
| --- | --- | --- | --- |
| Status: | Output Document |  |  |
| Purpose: | Core Experiments Description |  |  |
| Author(s) or Contact(s): | Il-Koo Kim Joel Jung Simon Oudin | Email: | ilkoo.kim@samsung.com joelb.jung@orange-ftgroup.com simon.oudin@hhi.fraunhofer.de |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

At the 7th JCT-VC meeting in Geneva, several contributions were proposed to improve motion vector prediction and coding. In this CE, two methods among them will be evaluated: 1) modification on bi-prediction (JCTVC-G305) 2) Simplification of the first merge pruning process (Part 1 of JCTVC-G593).

# Participants list

| Company/Institute | Name | E-mail address |
| --- | --- | --- |
| Ateme | Thiesse Jean-Marc | jm.thiesse@ateme.com |
| HiSilicon | Xiaozhen Zheng | zhengxiaozhen@huawei.com |
| Huawei | Yongbing Lin | linyongbing@huawei.com |
| Fraunhofer HHI | Simon Oudin Benjamin Bross Philipp Helle | simon.oudin@hhi.fraunhofer.de benjamin.bross@hhi.fraunhofer.de philipp.helle@hhi.fraunhofer.de |
| I2R | Y. H. Tan | yhtan@i2r.a-star.edu.sg |
| JVC KENWOOD | S. Fukushima H. Nakamura H. Takehara M. Ueda | fukushima.shigeru@jvckenwood.com nakamura.hiroya@jvckenwood.com takehara.hideki@jvckenwood.com ueda.motoharu@jvckenwood.com |
| LG | Yongjoon Jeon | yongjoon.jeon@lge.com |
| MediaTek | Yu-Wen Huang Jian-Liang Lin Yi-Wen Chen | yuwen.huang@mediatek.com jl.lin@mediatek.com yiwen.chen@mediatek.com |
| NTT DOCOMO | T. K. Tan Yoshinori Suzuki | ttk@pacific.net.sg suzukiyos@rd.nttdocomo.co.jp |
| Nokia | Oguz Bici | ext-oguz.bici@nokia.com |
| Orange Labs | Joel Jung | joelb.jung@orange.com |
| Panasonic | Toshiyasu Sugio | Sugio.toshiyasu@jp.panasonic.com |
| Qualcomm | Vadim Seregin | vseregin@qualcomm.com |
| Samsung | Il-Koo Kim Ankur Saxena | ilkoo.kim@samsung.com asaxena@sta.samsung.com |
| Santa Clara University | Guichun Li Chia-Chi Li | g1li@scu.edu cli1@scu.edu |
| Sony | Kazushi Sato | kazushi.sato@jp.sony.com |

# Contribution and Experiment Description

## Modification of bi-prediction

- JCTVC-G305 Non-CE9: Bi-prediction for low delay coding [Y. Suzuki, A. Fujibayashi (NTT DOCOMO)]

JCTVC-G305 proposed the additional option of bi-prediction for low delay conditions and key pictures of random access conditions. The motion vector difference for List 1 is not signaled and it sets to (0, 0) for GPB pictures (reference picture list 1 is identical to reference picture list 0). In this test, the impact on visual quality by the modification of ME part is investigated. And also, the effect of syntax change in JCTVC-G305 is investigated by comparing two encoder only change scenarios.

- JCTVC-G305 (BP01, BP02 and BP03)

There are four points to be modified.

- In the HM encoder, the motion estimation of the list 0 motion vector for bi-prediction is conducted on the condition that the list 1 motion vector detected at uni-prediction ME process is fixed. In the test method, instead of list 1 motion vector detected at uni-prediction ME process, the list 1 motion vector predictor is selected as the list 1 motion vector for bi-prediction.

- To find the best list 1 motion vector predictor which minimizes a prediction error, the best combination of the list 1 reference pictures and motion vector predictor candidates is considered in the uni-prediction ME process.

- When the new option is enabled, the difference between a list 1 motion vector components and its prediction is not signaled, and it sets to (0, 0) at decoder. In other words, list 1 motion vector components set to its prediction.

- A flag to identify if this option is utilized is signaled at the slice header. In this test, the additional option on bi-prediction is used when reference picture list 1 is identical to reference picture list 0 (GPB pictures). Note that other implementation or criteria by the encoder is also possible.

The modifications described above are tested as BP01. The method in which list 1 is swapped for list 0 is tested as BP02 since there was the question why the list 1 motion vector difference is not signaled, but the list 0 motion vector difference is signaled during the 7th JCT-VC meeting. To know if these modifications are only useful for GPB pictures, additionally, the method in which the flag indicated in point 4 above is enabled for all inter pictures in random access conditions is tested as BP03.

- Encoder only scenario 1 (BP04)

Regarding BP01, only the points 1 and 2 above are conducted for GPB pictures. In the test, the impact of signaling zero mvd for list 1 is studied.

- Encoder only scenario 2 (BP05 and BP06)

HM5.0 with the 2nd motion estimation of list 0/1 (depending on picture) for bi-prediction is tested as BP05. For comparison, BP01 with the 2nd motion estimation of list 1 for non-GPB pictures in random access conditions is tested as BP06. In this test, the trade-off between encoder RT and BD rate saving by the 2nd motion estimation for bi-prediction is studied to investigate the impact of modifications by JCTVC-G305. Note that the motion estimation for bi-prediction is conducted only for one reference picture list (List 0 or List 1) when the fast encoder setting (FEN) is enabled in the HM encoder.

- HM ticket #175 (BP07, BP08)

The HM ticket #175 may have an influence on BP series since it is related to bi-predictive ME. HM5.0 and BP01 with modification reported by ticket #175 is tested as BP07 and BP08, respectively. The clipping in the function TComYuv::removeHighFreq is omitted as follows:

HM5.0 (for luma component)

pDst[x ] = Clip( (pDst[x ]<<1) - pSrc[x ] );

Ticket #175

pDst[x ] =  (pDst[x ]<<1) - pSrc[x ] ;

HM5.0 (for chroma components)

pDstU[x ] = Clip( (pDstU[x ]<<1) - pSrcU[x ] );

pDstV[x ] = Clip( (pDstV[x ]<<1) - pSrcV[x ] );

Ticket #175

pDstU[x ] =  (pDstU[x ]<<1) - pSrcU[x ] ;

pDstV[x ] =  (pDstV[x ]<<1) - pSrcV[x ] ;

Table 1. Description of experiments – BP series

| Test | Description | Related Doc. | Tester1 | Tester2 |
| --- | --- | --- | --- | --- |
| BP01 | L1 MVD is set to (0,0) and not signaled for GPB pictures | JCTVC-G305 | NTT DOCOMO | HHI |
| BP02 | L0 MVD is set to (0,0) and not signaled for GPB pictures | JCTVC-G305 | NTT DOCOMO | Orange Labs |
| BP03 | L1 MVD is set to (0, 0) and not signaled for all inter pictures (RA conditions only) | JCTVC-G305 | NTT DOCOMO | JVC Kenwood |
| BP04 | L1 MVD is set to (0, 0) at encoder for GPB pictures | JCTVC-G305 | NTT DOCOMO | HHI |
| BP05 | HM5.0 with the 2nd motion estimation for bi-prediction | JCTVC-G305 | NTT DOCOMO | Panasonic |
| BP06 | BP01 with the 2nd motion estimation for bi-prediction in non-GPB pictures (RA conditions only) | JCTVC-G305 | NTT DOCOMO | Orange Labs |
| BP07 | HM5.0 with HM ticket #175 | HM ticket #175 | NTT DOCOMO | Samsung |
| BP08 | BP01 with HM ticket #175 | HM ticket #175 JCTVC-G305 | NTT DOCOMO | Samsung |

## Simplification of the first merge pruning process

- JCTVC-G593 Non-CE13: Simplification of merge mode [O. Bici, J. Lainema, K. Ugur (Nokia)]

Only the part 1 (Section 2) of JCTVC-G593 is further studied in the CE. In the current design, up to 4 spatial candidates and one temporal candidate are inserted into the merge list which is composed of their motion information (one or two motion vectors and an index specifying the corresponding reference picture). The redundant candidates are removed by comparing motion information of each candidate, which is also known as the first pruning process. In this contribution, it is proposed to reduce the number of motion information comparison during the first pruning process for the merge/skip mode by comparing each spatial candidate with a limited number of spatial candidates and exempting the temporal motion vector prediction candidate from the pruning process.

The description of the experiment is as follows: the first pruning process is removed and instead, it is proposed to perform a smaller number of parallelizable motion information comparisons only between the spatial candidates. The motion information of a spatial candidate is inserted into the merge list depending of a specific condition on this spatial candidate. :

The conditions for each spatial candidate to be excluded from the merge list are as follows:

•	A1: No change

•	B1: B1 has the same motion information as A1

•	B0: B0 has the same motion information as B1

•	A0: A0 has the same motion information as A1

•	B2: B2 has the same motion information as A1 or has the same motion information as B1

Figure 1. Spatial neighbors which are the potential candidates for the merge list. Arrows indicate which spatial candidate(s) are to be compared.

Table 2 Description of experiments – SP

| Test | Description | Related Doc. | Tester1 | Tester2 |
| --- | --- | --- | --- | --- |
| SP | The first merge pruning process using parallelizable motion information comparisons on the spatial candidates | JCTVC-G593 | Nokia | Sony |

# Test Sequences / Test Points / SW Configuration

Simulation results will be generated for sequences and coding conditions specified in the document JCTVC-G1200. Latest version of the HM (HM 5.0) will be used for evaluation.

# Definition of Performance Measurement Criteria

## Coding Performance Measurements

Measure impact on bitrate/PSNR using provided data (4 points, BD-Rate) as described in JCTVC-G1200.

## Complexity Considerations

To provide an indicative assessment of complexity, the following numbers should be provided:

- Encoding time (compared to the default configuration)

- Decoding time (compared to the default configuration)

# Timeline

| T1 | 2 weeks after the meeting (2011 December 14) | Final CE description |
| --- | --- | --- |
| T2 | HM5.0 is available (2011 December 19) | Start of tool integration |
| T3 | 2 weeks after HM5.0 is available (2012 January 2) | Completion of tool integration Start of testing |
| T4 | 2012 January 11 | Testing complete Reporting of all results to CE9 participants complete |
| T5 | 2012 January 20 | Input documents uploaded |
| T6 | 2012 January 27 | CE9 summary report uploaded |