| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, 21-30 November, 2011 | Document: JCTVC-G1210_r1 WG11 Number: m22850 |
| --- | --- |

| Title: | Description of Core Experiment 10: Deblocking Filtering |  |  |
| --- | --- | --- | --- |
| Status: | Output Document of JCT-VC |  |  |
| Purpose: | CE description |  |  |
| Author(s) or Contact(s): | Andrey Norkin (primary) Xun Guo Byeungwoo Jeon Matthias Narroschke | Email: | andrey.norkin@ericsson.com xun.guo@mediatek.com bjeon@skku.edu matthias.narroschke@eu.panasonic.com |
| Source: | CE coordinators |  |  |

_____________________________

# Introduction

The goal of this Core Experiment (CE) is to enhance deblocking filtering techniques in the HEVC Test Model. This is not limited to improving coding efficiency and subjective quality, but also reducing complexity, parallelization and harmonization of various schemes that are technically feasible. Techniques covered by this CE are deblocking filter improvements, line memory reduction, complexity reduction and high-level syntax for signaling the deblocking filter parameters.

The contributions in this CE are organized into the following subtests:

Subtest 1: Subjective and objective quality improvements.

- Chroma deblocking filter (JCTVC-G286, MediaTek/Ericsson)

- Simplified chroma deblocking (JCTVC-G287, MediaTek/Ericsson)

- Deblocking of new non-square blocks (JCTVC-G383, Qualcomm)

- Unification of luma and chroma filtering (JCTVC-G087, TI)

- Optimization of deblocking filter (JCTVC-G639 Part2, Panasonic)

Subtest 2: Line memory reduction.

- Deblocking filter line buffer reduction (JCTVC-G230, MediaTek/Sony)

- Tiles for line buffer reduction (JCTVC-G618, Ericsson)

Subtest 3: Deblocking filtering simplification.

- Simplification of deblocking filter (JCTVC-G639 Part1, Panasonic)

- Simplification of Bs decision process for deblocking (JCTVC-G176, LG)

- Simplification of filtering decisions and Bs decision process in deblocking (G089, TI / JCTVC-G616, Ericsson)

- Simplification of deblocking weak filter (JCTVC-G290 Part 1, Qualcomm)

- Strong filtering decision simplification (JCTVC-G290 Part 2, Qualcomm)

- Simplification of first delta calculation in deblocking (JCTVC- F258, Part 1, SKT/SKKU)

- Simplification of second delta calculation in deblocking (JCTVC- F258, Part 2, SKT/SKKU)

Subtest 4: Controlling strength of deblocking filter at slice level.

- Deblocking filter using adaptive weighting factors (JCTVC-G574, Panasonic)

- Transform dependent deblocking filter parameter adjustment in slice level (JCTVC-G291, Qualcomm)

- Deblocking parameters signaling in slice header (JCTVC-G619, Ericsson)

For the proposals in this CE, the impact of modifications on subjective and objective quality is evaluated. The complexity of the proposed schemes relative to the current HEVC deblocking is also evaluated. In addition, subjective viewing should be conducted according to the suggestions of viewing experts. 

The proponents are encouraged to test harmonization of various proposals in this CE in order to find the best solution and provide a basis for convergence of the proposals towards a recommendation.

The software basis for this CE is HM 5.0, and the experiments are conducted based on the test conditions in JCTVC-G1200 [1].

# Participants

| Nr. | Name | Company | Email | Remarks |
| --- | --- | --- | --- | --- |
| 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 21 22 23 24 25 26 27 28 30 31 32 33 34 35 36 37 38 39 40 41 42 43 | Andrey Norkin (primary coordinator) Shawmin Lei Xun Guo (coordinator) Yu-Wen Huang Jicheng An Qian Huang Jizheng Xu Xiaoyan Sun Matthias Narroschke (coordinator) Jeongyeon Lim Jungyoup Yang Byeungwoo Jeon (coordinator) Kenneth Andersson Teruhiko Suzuki Masaru Ikeda Tomoo Yamakage In Suk Chong Geert Van der Auwera Minhua Zhou TK Tan Jani Lainema Oguz Bici Il-Koo Kim Vittorio Baroncini Simon Oudin Han Yu Chih-Wei Hsu Sugimoto.Kazuo Shun-ichi Sekiguchi Chih-Wei Hsu Chia-Yang Tsai Zhongbo Shi Kwanghyun Won Jinho Lee Hui Yong Kim Sung-Chang Lim Tzu-Der Chuang Ching-Yeh Chen Seungwook Park Jie Zhao Vadim Seregin Lidong Xu | Ericsson MediaTek MediaTek MediaTek MediaTek MediaTek Microsoft Microsoft Panasonic SK Telecom SKKU SKKU Ericsson Sony Sony Toshiba Qualcomm Qualcomm Texas Instruments NTT Docomo Nokia Nokia Samsung FUB HHI Intel MediaTek Mitsubishi Electric Mitsubishi Electric MediaTek MediaTek USTC SKKU ETRI ETRI ETRI MediaTek MediaTek LG Sharp Qualcomm Intel | andrey.norkin@ericsson.com shawmin.lei@mediatek.com xun.guo@mediatek.com yuwen.huang@mediatek.com jicheng.an@mediatek.com qian.huang@mediatek.com jzxu@microsoft.com xysun@microsoft.com Matthias.Narroschke@panasonic.de jeongyeon@sktelecom.com binbak1@ece.skku.ac.kr bjeon@skku.edu kenneth.r.andersson@ericsson.com teruhikos@jp.sony.com Masaru.Ikeda@jp.sony.com tomoo.yamakage@toshiba.co.jp ichong@qualcomm.com geertv@qualcomm.com zhou@ti.com ttk@pacific.net.sg jani.lainema@nokia.com ext-oguz.bici@nokia.com ilkoo.kim@samsung.com vittorio@fub.it simon.oudin@hhi.fraunhofer.de yu.han@intel.com cw.hsu@mediatek.com Sugimoto.Kazuo@ ak.MitsubishiElectric.co.jp Sekiguchi.Shunichi@eb.MitsubishiElectric.co.jp cw.hsu@mediatek.com chiayang.tsai@mediatek.com v-zhshi@microsoft.com wkh12345@gmail.com jinosoul@etri.re.kr hykim5@etri.re.kr sclim@etri.re.kr peter.chuang@mediatek.com chingyeh.chen@mediatek.com seungwook.park@lge.com jzhao@sharplabs.com vseregin@qualcomm.com lidong.xu@intel.com | Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent |

# Experimental Conditions

  - Software

HM 5.0 shall be used (will be provided on December 19, 2011).

Proposals from Subtest 4 should use the version of HM 5.x with implemented deblocking filter parameters signaling from G1035. The same version should be used as the anchor in objective evaluation in Subtest 4.

  - Test Sequences, Bit Rates and Coding Conditions

The recommended sequences, configurations and test cases described in JCTVC-G1200 will be used. Providing results for additional sequences is encouraged but not mandated. Results for all mandatory test cases and for HE LD-P and LC LD-P configurations shall be provided. Therefore, results for HE LD-P and LC LD-P are mandatory for this CE.

  - Evaluation of CE Results

Results of the CE will be evaluated on the basis of BD-measures and complexity as well as subjective quality. Complexity assessment is performed on the base of decoding complexity measurement. For complexity assessment, HM 5.0 software with the anchor configuration and the proposal implemented on the HM 5.0 software shall be used. For decoder complexity measurements, the decoding shall be executed on the same machine without output of decoded YUV data. The computational time must be measured for each test sequence and test case for both anchor and proposal. Relative computational time calculated against the anchor must be presented.

An informal subjective viewing by experts during the San Jose meeting should be performed and results provided during the meeting in order to ensure that simplification proposals do not cause degradation of subjective quality and assess improvements in subjective quality by the proposals claiming subjective quality improvements. Test sequences should be determined before the beginning of cross-check.

The following test procedure will be used. The same test sequence is shown for a number of proposals with the anchor being one of the proposals. The order of proposals is randomized for every test and sequence and identities of the proposals are hidden. Test subjects are asked to rate each proposal on a scale from 1 to 10. The MOS results are later calculated as well as 95-procent confidence intervals for every proposal and test sequence.

# Subtest 1. Subjective and objective quality improvements

## Functionality addressed

- Reduction of blocking artifacts

- Improvement of coding efficiency

- Improvements in subjective quality

## Expected gains

- Subjective quality improvement

- BD-rate reduction

## Evaluation criteria

- Subjective video quality

- BD-rate relative to the anchor(s)

- Complexity (decoding time measurement, memory storage, etc.)

Note 1: HM 5.0 shall be used as the anchor. 

Note 2: Cross-checking includes informal viewing of proposals. All cross-checkers shall report the informal viewing results (without scoring) and their equipments (e.g., monitor, etc). If a room, equipment, and time-slot for informal viewing are available at the San Jose meeting, informal viewing will be conducted. The test sequences, QP values, and coding conditions for the potential informal viewing at the San Jose meeting are chosen from the sequences, configurations and test cases described in JCTVC-G1200. The proponents should be ready to provide the requested sequences. The selection should be conducted by CE10 participants including non-proponents.

Note 3: The proponents are encouraged to provide additional results for separate tools (groups of tools) in their proposals if the proposal consists of several independent modifications of the HM software. The possibility to turn on and off these groups of tools should be provided in the software. This specifically applies to adjustments of the thresholds and tables as well as to modifications to the encoder control and implementations, which could also be applied to improve the anchor. The cross-checkers should have a possibility to assess, which part of the coding gain comes from the thresholds/tables adjustment or from modifications to the encoder control or implementations. The cross-checkers are encouraged to perform the subjective viewing of the separate tools/combinations of tools in order to ensure that removing or adding certain tools from the proposal does not degrade the subjective quality.

  - Chroma deblocking filter (JCTVC-G286, MediaTek/Ericsson)

### General Concept

In this contribution, a modified deblocking filter for chroma is proposed.  There are two major modifications. First, a filtering decision is added for chroma boundaries to guarantee the accuracy of the filter, which is performed independently for U and V components. For 4:2:0 sampling format, the filtering decision is done on the 4-pixel block boundary. Two pixel lines are used for the filtering decision. Second, boundaries across inter blocks are also filtered to reduce the possible blocking artifacts in inter-predicted frames.

### Bitstream organization

No need to modify the bitstream syntax (i.e. only requires the modification of decoding process).

  - Simplified chroma deblocking (JCTVC-G287, MediaTek/Ericsson)

### General Concept

This contribution unifies chroma filtering decision with luma component. For inter frames, such as P and B frames, the filtering decision of chroma is exactly the same as that of luma. For intra frames, chroma first reuses the gradient computed for luma to compare with a new threshold. Then the filtering decision is made based on the comparison.

### Bitstream organization

No need to modify the bitstream syntax (i.e. only requires the modification of decoding process).

  - Deblocking non-square blocks (JCTVC-G383, Qualcomm)

### General concept

The adaptation of the deblocking filter in case of AMP partitions of size 16x4 or 4x16 is studied. Instead of deblocking the central edge on the 8x8 deblocking grid inside the 16x16 CU of the AMP type, the relevant internal AMP PU edge is deblocked, which keeps the number of filtering operations unchanged. The deblocking filter width is adapted to avoid filtering dependencies between nearby edges.

### Bitstream organization

No need to modify the bitstream syntax (i.e. only requires the modification of decoding process).

  - Unification of luma and chroma filtering (JCTVC-G087, TI)

### General concept

This proposal advocates the unification of luma weak and chroma filter for complexity reduction and coding efficiency improvements. In the proposed algorithm the delta calculation for luma weak filter and chroma filter is made the same. Both coding efficiency and subjective quality will be evaluated.

### Bitstream organization

No modification of the bitstream syntax, only the decoding process is modified.

## Optimization of deblocking filter (JCTVC-G639 Part2, Panasonic)

### General concept

The proposal optimizes the rounding operations applied in the deblocking filter. This includes making the rounding operation symmetric and combining it with clipping operations.

### Bitstream organization

No modification of the bitstream syntax, only the decoding process is modified.

### Participants

| Participant | Contact |
| --- | --- |
| MediaTek | Xun Guo xun.guo@mediatek.com |
| Ericsson | Andrey Norkin andrey.norkin@ericsson.com |
| Panasonic | Matthias Narroschke Matthias.Narroschke@eu.panasonic.com |
| Qualcomm | In Suk Chong ichong@qualcomm.com Geert Van Der Auwera geertv@qualcomm.com |
| TI | Minhua Zhou zhou@ti.com |
| SKT/SKKU | Byeungwoo Jeon bjeon@skku.edu Jungyoup Yang binbak1@ece.skku.ac.kr |
| Sony | Teruhiko Suzuki teruhikos@jp.sony.com Masaru Ikeda Masaru.Ikeda@jp.sony.com |
| Intel | Lidong Xu lidong.xu@intel.com |

Cross-check assignment

| Proponent(s) | Cross/checker(s) |
| --- | --- |
| G286, MediaTek/Ericsson | Intel |
| G287, MediaTek/Ericsson | Intel |
| G383, Qualcomm | Mediatek |
| G087, TI | Sony |
| G639 Part2, Panasonic | Sony |

# Subtest 2: Line memory reduction

## Functionality Addressed

- Reduction of required line memory and complexity

## Expected Gains

- Balance of both flexibility of parallel processing and complexity without subjective and objective quality degradation

## Evaluation Criteria

- BD-rate relative to the anchor(s)

- Complexity (decoding time measure, control, etc.)

- Impact on subjective quality

- Degree of achieved line memory reduction

Note 1: HM 5.0 shall be used as the anchor.

Note 2: Cross-checking includes informal viewing of proposals. All cross-checkers shall report the informal viewing results (without scoring) and their equipments (e.g., monitor, etc). If a room, equipment, and a time-slot for informal viewing are available at the San Jose meeting, informal viewing will be conducted. The selection of test sequences, QP values, and coding conditions for the potential informal viewing in the San Jose meeting are decided before the cross-check start date. The selection should be conducted by CE10 participants including non-proponents.

## Deblocking filter line buffer reduction (JCTVC-G230, MediaTek, Sony)

This proposal's motivation is reduction of line buffers required by deblocking filter with keeping visual quality. This proposal consists of the following points based on G230, G228 and G257.

- The target of line buffers reduction is R3W2 or R2W1

- The decision and calculation are modified by few pixels above the LCU boundary

- Modified vertical filtering is applied to horizontal edges in LCU boundary or all horizontal edges

## Tiles for line buffer reduction (JCTVC-G618, Ericsson)

The contribution proposes a tiles-based solution to reduce a line-buffer problem identified in in-loop filtering processes in the current HEVC design. The solution is to split the picture in vertical tiles when the picture width exceeds a certain size. In order to reduce the required line buffer in decoder, level constraints on the maximum tile width are proposed.

## Participants

| Participant | Contact |
| --- | --- |
| Sony | Teruhiko Suzuki teruhikos@jp.sony.com Masaru Ikeda Masaru.Ikeda@jp.sony.com |
| MediaTek | Yu-Wen Huang yuwen.huang@mediatek.com Chih-Wei Hsu cw.hsu@mediatek.com Chia-Yang Tsai chiayang.tsai@mediatek.com Tzu-Der Chuang peter.chuang@mediatek.com Ching-Yeh Chen chingyeh.chen@mediatek.com |
| Ericsson | Andrey Norkin andrey.norkin@ericsson.com |
| Panasonic | Matthias Narroschke Matthias.Narroschke@eu.panasonic.com |

Cross-check assignment

| Proponent(s) | Cross/checker(s) |
| --- | --- |
| G230, MediaTek/Sony | Panasonic, Ericsson |
| G618, Ericsson | Sony |

# Subtest 3. Reduction of computational complexity of deblocking filter

## Functionality addressed

- Reducing computational complexity of deblocking filtering

## Expected gains

- Lower computational complexity

## Evaluation criteria

- BD-rate relative to the anchor(s)

- Complexity (decoding time measurement)

- Check that no degradation of subjective video quality occurs

## Simplification of deblocking filter (JCTVC-G639 part1, Panasonic)

### General concept

This proposal simplifies the deblocking filter considering the reduction of dependencies as well as simplified deblocking operations.

### Bitstream organization

No modification of the bitstream syntax, only the decoding process is modified.

## Simplification of Bs decision process for deblocking (JCTVC-G176, LG)

### General concept

A simplification of BS decision process for deblocking filter is evaluated. Currently in HM4.0, Bs decision process for an 8-pel edge consists of finding Bs decision for every 4-pel edge and then a maximum of two 4-pel Bs is found, because the minimum unit of BS and filtering is different. This proposal suggests using the Bs of the first 4-pixel part as the Bs for the whole 8-pel edge to reduce Bs decision operation and remove BS merging process.

### Bitstream organization

No need to modify the bitstream syntax (i.e. only requires the modification of decoding process).

## Simplification of Bs decision process for deblocking and filtering decisions (JCTVC-G616, Ericsson, JCTVC-089 TI )

### General concept

A simplification of Bs decision process for deblocking filter is evaluated. Currently in HM4.0, Bs decision process for an 8-pel edge consists of finding BS decision for every 4-pel edge and then a maximum of two 4-pel Bs is found. It is proposed to remove the maximum operation in Bs calculation. This would also align the Bs derivation with the transform size. The proposals studies the effect of weak and strong filtering block-based decisions on objective and subjective quality.

### Bitstream organization

No need to modify the bitstream syntax (i.e. only requires the modification of decoding process).

## Strong filtering decision simplification (JCTVC-G290 Part 2, Qualcomm)

### General concept

The proposal is based on decisions G590 adopted at Geneva meeting. The proposals studies the effect of simplifying block-based filtering decisions on objective and subjective quality.

### Bitstream organization

No need to modify the bitstream syntax (i.e. only requires the modification of decoding process).

## Simplification of deblocking weak filter (JCTVC-G290 Part 1, Qualcomm)

### General concept

Study of computational simplification of deblocking weak filter. The delta calculation for the first pixel from the block boundary is modified from (9*(q0-p0)-3*(q1-p1))/16) to (3*(q0-p0)-(q1-p1))/8 therefore reducing number of operations. Deblocking filter characteristics are compared.

### Bitstream organization

No need to modify the bitstream syntax (i.e. only requires the modification of decoding process).

## Simplification of first delta calculation in deblocking (JCTVC- F258, Part1, SKT/SKKU)

### General concept

The delta calculation for a weak filter is changed from d = ( 9*(q0-p0) - 3*(q1-p1) + 8 )/16 to d = (4*(q0-p0) - (q1-p1)+4)/8.

### Bitstream organization

No need to modify the bitstream syntax (i.e. only requires the modification of decoding process).

## Simplification of second delta calculation in deblocking (JCTVC- F258 Part2, SKT/SKKU)

### General concept

The delta values for the second pixels from the filtered boundary are computed by half of the delta value computed for the first-positioned pixels at the filtering boundary.

### - Bitstream organization

No need to modify the bitstream syntax (i.e. only requires the modification of decoding process).

### Participants

| Participant | Contact |
| --- | --- |
| Panasonic | Matthias Narroschke matthias.narroschke@eu.panasonic.com |
| LG | Seungwook Park seungwook.park@lge.com |
| Ericsson | Andrey Norkin andrey.norkin@ericsson.com |
| Qualcomm | Geert Van der Auwera geertv@qualcomm.com |
| SKT/SKKU | Byeungwoo Jeon bjeon@skku.edu Jungyoup Yang binbak1@ece.skku.ac.kr |
| Sony | Teruhiko Suzuki teruhikos@jp.sony.com |

Cross-check assignment

| Proponent(s) | Cross/checker(s) |
| --- | --- |
| G639 part 1, Panasonic | Sony |
| G176, LG | Ericsson |
| G616, Ericsson | LG |
| G290 part 1, Qualcomm | SKKU |
| G290 part 2, Qualcomm | Ericsson |
| F258 part1, SKT/SKKU | Qualcomm |
| F258 part2, SKT/SKKU | ETRI |

# Subtest 4. Controlling deblocking filter strength at slice level

## Functionality addressed

- Encoder optimization capability for subjective picture quality

- High-level syntax for sending deblocking filter parameters at slice level

- Improvements in subjective quality

- Improvements in coding efficiency

## Expected gains

- Give options to encoder to control deblocking filter strength in order to improve subjective picture quality

- Improve coding efficiency

## Evaluation criteria

- Flexibility and clarity of parameter signaling

- Subjective video quality improvements

- Encoding and decoding time

- BD-rate relative to the anchor(s)

Note 1: HM 5.x shall be used as the anchor (the version of HM5 with deblocking parameters signaling from G1035). 

Note 2: The proponents are encouraged to test harmonization of the proposals in this subtest.

Note 3: Cross-checking includes informal viewing of proposals. All cross-checkers shall report the informal viewing results (without scoring) and their equipments (e.g., monitor, etc). If a room, equipment, and time-slot for informal viewing are available at the San Jose meeting, informal viewing will be conducted. The test sequences, QP values, and coding conditions for the potential informal viewing at the San Jose meeting are chosen from the sequences, configurations and test cases described in JCTVC-G1200. The proponents should be ready to provide the requested sequences. The selection should be conducted by CE12 participants including non-proponents.

## Deblocking filter using adaptive weighting factors (JCTVC-G574, Panasonic)

### General concept

This proposal introduces adaptive weighting factors into the deblocking filter. Each weighting factor weights the value, by which the HM deblocking filter would modify a sample value at the corresponding filtering position. The weighting factors are estimated at the encoder side, coded and transmitted to the receiver side. An encoder control is considered including appropriate subjective and objective criteria for the estimation of the weighting factors.

### Bitstream organization

Weighting factors are coded within the slice header

## Transform Dependent Deblocking Filter Parameter Adjustment in Slice Level (JCTVC-G291, Qualcomm)

### General concept

It is proposed to study controlling deblocking parameters depending on the transform size and on intra or inter type of the blocks. Different Tc offsets for differet transform sizes are sent in the slice header.

### Bitstream organization

Deblocking control parameters offsets are coded within the APS/slice header

## Deblocking parameters signaling in slice header (JCTVC-G619, Ericsson)

### General concept

It is proposed to signal deblocking filter parameters in the slice header. It is proposed to signal three deblocking parameters that control which pixels are being filtered and one clipping parameter that determines the largest possible modification of the pixels in the weak filtering mode.

### Bitstream organization

Parameters are coded within the slice header

### Participants

| Participant | Contact |
| --- | --- |
| Panasonic | Matthias Narroschke matthias.narroschke@eu.panasonic.com |
| Qualcomm | Geert Van der Auwera geertv@qualcomm.com |
| Ericsson | Andrey Norkin andrey.norkin@ericsson.com |
| Sony | Teruhiko Suzuki teruhikos@jp.sony.com |
| Toshiba | Tomoo Yamakage tomoo.yamakage@toshiba.co.jp |

Cross-check assignment

| Proponent(s) | Cross/checker(s) |
| --- | --- |
| G574, Panasonic | Sony |
| G291, Qualcomm | Toshiba |
| G619, Ericsson | Qualcomm |

# Time-line and Responsibilities

2011-Dec-21:	CE10 description finalized and uploaded.

2011-Dec-22:	HM5.0 software is available.

2011-Dec–xx:	HM5.x is available with signaling of deblocking parameters from G1035 (for Subtest 4).

2012-Jan–5:	Cross-verification begins for all subtests except Subtest 4, proponents provide software, draft of contribution text and results to cross-checker(s) and CE10 coordinators (two weeks after availability of HM5.0).

2012-Jan–xx:	Cross-verification begins for Subtest 4, proponents provide software, draft of contribution text and results to cross-checker(s) and CE10 coordinators (two weeks after availability of the anchor, HM5.x).

2012-Jan-16:	Cross-verifiers report results to coordinators.

2012-Jan-20:	Contribution documents uploaded, the document number and title are reported to coordinators.

2012-Jan-26:	Summary report uploaded.

# Previous Contributions

JCTVC-F258	[J. Yang, K. Won, B. Jeon (SKKU), J. Lim (SK Telecom)] CE12: SK Telecom/SKKU Deblocking Filter

JCTVC-G087	[M. Zhou, O. Sezer, V. Sze (TI)] CE12 subset 4.10: Test results on unification of luma and chroma filtering

JCTVC-G176	[S. Park, B. Jeon (LG)] Non-CE12: Simplified BS calculation process

JCTVC-G230	[C.-W. Hsu, Y.-W. Huang, S. Lei (MediaTek), M. Ikeda, T. Suzuki (Sony), S. Park, B. Jeon (LG)] Non-CE12.3: Reducing pixel line buffers by modifying DF to R3W2 for horizontal LCU boundaries

JCTVC-G290	[G. Van der Auwera, X. Wang, M. Karczewicz (Qualcomm)] Deblocking filter simplifications

JCTVC-G291	[G. Van der Auwera, R. Joshi, X. Wang, M. Karczewicz (Qualcomm)] Transform Dependent Deblocking Filter Parameter Adjustment in Slice Level

JCTVC-G286 	[Q. Huang, J. An, X. Guo, Y.-W. Huang, S. Lei (MediaTek), A. Norkin, K. Andersson, R. Sjöberg (Ericsson)] CE12 Subtest 1: Chroma Deblocking Filter.

JCTVC-G287 	[Q. Huang, J. An, X. Guo, Y.-W. Huang, S. Lei (MediaTek), A. Norkin, K. Andersson, R. Sjöberg (Ericsson)] CE12 Subtest 1: Simplified Chroma Deblocking Filter in JCTVC-G286

JCTVC-G383	[G. Van der Auwera, X. Wang, M. Karczewicz (Qualcomm)] CE12 Subtest1: Deblocking of New Non-Square Blocks: Edge Shift for AMP

JCTVC-G574	[M. Narroschke, A.-K. Seifert (Panasonic)] CE12 Subtest 5: Deblocking filter using adaptive weighting factors

JCTVC-G616 	[A. Norkin (Ericsson)] On parallel deblocking

JCTVC-G618 	[A. Norkin, R. Sjöberg (Ericsson)] Level restrictions on tiles for line buffer reduction

JCTVC-G619 	[A. Norkin, R. Sjöberg, K. Andersson, J. Enhorn (Ericsson)] Non-CE12: deblocking parameters signaling in slice header.

JCTVC-G639	[A. Kotra, M. Narroschke, T. Wedi (Panasonic)] Deblocking simplification and rounding optimization

# References

- F. Bossen, “Common test conditions and software reference configurations,” JCTVC-G1200, Geneva, November 2011.

- “High Efficiency Video Coding (HEVC) Test Model 5 (HM 5) Encoder Description”, JCTVC-G1102, Geneva, November 2011.

- “High Efficiency Video Coding (HEVC) text specification Working Draft 5”, JCTVC-G1103, Geneva, November 2011.