| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 Nov 2011 | Document: JCTVC-G1211 |
| --- | --- |

| Title: | Description of Core Experiment (CE11): Coefficient scanning and coding |  |  |
| --- | --- | --- | --- |
| Status: | Output document |  |  |
| Purpose: | Core Experiment Description |  |  |
| Author(s) or Contact(s): | Vivienne Sze (primary) Tung Nguyen Yinji Piao Joel Sole Krit Panusopone | Tel: Email: Tel: Email: Tel: Email: Tel: Email: Tel: Email: | +1-214-567-4005 sze@ti.com +49-30-31002-230 tung.nguyen@hhi.fraunhofer.de +82-10-6296-4261. yj1003.piao@samsung.com +1-858-845-2429 joels@qualcomm.com +1-858-404-3618 krit@motorola.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document defines Core Experiment 11 (CE11) on coefficient scanning and coding, to be performed for the 8th JCT-VC meeting. The CE is divided into 4 subtests. The 1st subtest consists of proposals related to changes in significance map coding. The 2nd subtest consists of proposals related to changes in coefficient level coding.  The 3rd subtest consists of proposals related to changes to sign coding.  The 4th subtest consists of proposals related to removing remapping from NSQT to SQT.

# Introduction

Various methods have been proposed for coding the transform coefficients of the residual signal to reduce complexity and/or improve the coding efficiency. This core experiment evaluates the coding efficiency and complexity impact of:

- context modeling/selection for syntax elements related to transform coefficients

- signaling method (e.g. data hiding)

The objectives of this CE include:

- Investigation of the impact of context selection and signaling schemes for transform coefficient coding on the coding efficiency and complexity

- Coding efficiency and complexity measurement of transform coefficient scanning order methods

Complexity measures are described in Section 6.2.

# Participants List

| Company | Participants | P | C |
| --- | --- | --- | --- |
| TI | Vivienne Sze, sze@ti.com | X | X |
| Qualcomm | Joel Sole, joels@qualcomm.com Wei-Jung Chien, wchien@qualcomm.com | X | X |
| Bejing Univ. Tech | Wenpeng Ding, wpding@gmail.com | X |  |
| Microsoft | Jizheng Xu, jzxu@microsoft.com | X |  |
| Motorola Mobility | Yue Yu, yyu@motorola.com | X | X |
| Huawei | Jin Song, andy.songjin@huawei.com Xiaozhen Zheng, zhengxiaozhen@huawei.com Haoping Yu, haoping.yu@huawei.com | X | X |
| Canon | Chris Rosewarne, Chris.Rosewarne@cisra.canon.com.au |  |  |
| MediaTek | Xun Guo, xun.guo@mediatek.com Shih-Ta Hsiang, shih-ta.hsiang@mediatek.com Chih-Wei Hsu, cw.hsu@mediatek.com | X | X |
| Samsung | Ankur Saxena, asaxena@sta.samsung.com Yinji Piao, yj1003.piao@samsung.com Zhan Ma, zhan.ma@sta.samsung.com | X | X |
| Mitsubishi | Kazuo Sugimoto, Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp Akira Minezawa, Minezawa.Akira@ds.MitsubishiElectric.co.jp |  |  |
| I2R | Yeo Chuo Hao, chyeo@i2r.a-star.edu.sg |  |  |
| Nokia | Jewon Kang, ext-jewon.kang@nokia.com |  |  |
| Orange | Felix Henry, felix.henry@orange.com Joel Jung, joelb.jung@orange.com Gordon Clare, gordon.clare@orange.com | X |  |
| Panasonic | Hisao Sasai, sasai.hisao@jp.panasonic.com Terada Kengo, terada.kengo@jp.panasonic.com | X | X |
| Sony | Cheung Ayeung, Cheung.Auyeung@am.sony.com Jun Xu, Jun.Xu@am.sony.com |  | X |
| LGE | Jungsun Kim, jungsun.kim@lge.com Seungwook Park, Seungwook.park@lge.com | X | X |
| RIM | Gergely Korodi, gkorodi@rim.com Dake He, dhe@rim.com Gaëlle Martin-Cocher, gmartincocher@rim.com Tianying Ji, tiji@rim.com Nguyen Nguyen, nnguyen@rim.com Jing Wang, wangj@rim.com |  | X |
| Cisco | Thomas Davies, thdavies@cisco.com |  |  |
| Sharp | Tomohiro Ikai, ikai.tomohiro@sharp.co.jp Tsukuba Takeshi, tsukuba.takeshi@sharp.co.jp Yamamoto Tomoyuki, yamamoto.tomoyuki@sharp.co.jp | X | X |
| NEC | Keiichi Chono, chono@ct.jp.nec.com |  |  |
| BBC | Marta Mrak, Marta.Mrak@bbc.co.uk |  | X |

# Tools under Test

The following tools will be considered:

| Topic | Proponent | Doc # | Tool description | Cross-checker |
| --- | --- | --- | --- | --- |
| Significance map coding | Sharp | G308 | Complexity reduction of significant map coding | TI |
| Significance map coding | MediaTek | G448 | Context reduction for coding transform coefficients | Sharp, BBC |
| Significance map coding | Motorola Mobility | G725 | Simplification of significant map coding for CABAC | MediaTek |
| Significance map coding | TI | G917 | On significance map coding in CABAC | Motorola |
| Coefficient level coding | Bejing Univ. Tech, Microsoft | G301 | Context simplification for coefficients entropy coding | Samsung, Sony |
| Coefficient level coding | Panasonic | G522 | Context reduction for coefficient level | TI |
| Coefficient level coding | Qualcomm | G718 | Coefficient level context reduction | Panasonic |
| Coefficient level coding | Samsung | G783 | Context number reduction for level coding in CABAC | Qualcomm |
| Sign coding | Orange | G271 | Sign Data Hiding | RIM, Samsung |
| NSQT | TI | G123 | Simplified Coefficient Scans for NSQT | Qualcomm, LGE |
| NSQT | MediaTek | G202 | Modified NSQT coefficient scan | TI |
| NSQT | HiSilicon, Tsinghua | G517 | Harmonization of unified scan and NSQT | MediaTek |
| NSQT | Motorola Mobility | G724 | Entropy coding for non square blocks | HiSilicon, Tsinghua |
| NSQT | Qualcomm, LGE | G750 | Harmonization of HE residual coding and NSQT | Motorola, Sony |

## SubTest 1: Significance map coding

### JCTVC-G308: “Improvement of significant_coeff_flag context”

This contribution presents an improvement of significant_coeff_flag context. In the proposed method, the number of references used to derive context in significant_coeff_flag for high frequency area of 16x16 and 32x32 block is reduced 0 to 3 from 5. The number of contexts may be changed. In this CE, the harmonization with adopted method (G1015 and G323/G644) will also be studied.

### JCTVC-G448: “Context reduction for coding transform coefficients”

JCTVC-G448 proposes a modified context modeling method to reduce the number of the contexts used for coding significant_coeff_flag in 4x4 and 8x8 transform blocks. The contexts are reduced by subsampling the context configuration map and having the coefficients from the highest frequency region share the same context.

### JCTVC-G725: “Simplification of significant map coding for CABAC”

JCTVC-G725 proposes reduction in number of context used in significant map coding process by removing position based context. In this core experiment, position based context will be used for (0,0) position and different separation between medium frequency and high frequency zones will be investigated and reduced context derivation method based on 4 or 5 neighbor context will be used.

### JCTVC-G917: “On significance map coding in CABAC”

JCTVC-G917 proposes removing neighboring dependencies for context selection of significant_coeff_flag in order to enable parallel bin processing.  Specifically, dependency on recently processed bins is removed. Harmonization with the newly adopted tools in HM-5.0 is investigated.

## SubTest 2: Coefficient level coding

### JCTVC-G301: “Context simplification for coefficients entropy coding”

JCTVC-G301 proposes to reduce the contexts used for coeff_abs_level_greater1_flag and coeff_abs_level_greater2_flag. The contexts with similar probability will be grouped together to reduce the number of contexts needed.

### JCTVC-G522: "Context reduction for coefficient level"

JCTVC-G522 proposes to reduce the contexts used for coeff_abs_level_greater1_flag and coeff_abs_level_greater2_flag. If context set index is equal to 1, 2, 4 or 5, the contexts for coeff_abs_level_greater1_flag and coeff_abs_level_greater2_flag are each reduced by changing the threshold of c1 and c2. c1 is changed from 4 to 2. c2 is changed from 4 to 1.

### JCTVC-G718: "Coefficient level context reduction"

JCTVC-G718 part 2.6 proposes to reduce the number of contexts for level coding in CABAC. Contexts sets are modified so the number of contexts are not the same in each of them in order to better match the bin usage.

### JCTVC-G783: "Context number reduction for level coding in CABAC"

JCTVC-G783 proposes two new context sets for level coding in CABAC. The context sets are differentiated in luma and chroma by removing rarely used contexts. In this core experiment, only the second context set will be tested.

## SubTest 3: Sign Coding

### JCTVC-G271: “Sign Data Hiding”

This contribution presents a method to embed sign information into the residue. The sign of the first non-nul coefficient is not always explicitly written in the bitstream. At decoding time, the sign is inferred from the parity of the quantized coefficients. At encoding time, if the sign of the first non-nul coefficient and the parity do not match, one of the quantized coefficients is changed in order to obtain the desired parity. We plan to test this proposal when integrated into HM5.0, in conjunction with the different changes related to transform coefficient coding adopted during the 7th JCTVC meeting.

## SubTest 4: Removing remapping from NSQT to SQT

### JCTVC-G123: “Simplified Coefficient Scans for NSQT”

JCTVC-G123 proposes simplified coefficient scanning for NSQT.  Specifically, a scan is proposed that maps 4x16/16x4 and maps 8x32/32x8 to square sub-blocks.  Since context selection of significant_coeff_flag for 4x4 and 8x8 TU does not depend on neighbors, the intermediate mapping to 2D square array is not needed and no remapping is required for NSQT.  Harmonization with the newly adopted tools in HM-5.0 is investigated.

### JCTVC-G202: “Modified NSQT coefficient scan”

JCTVC-G202 proposes to code non-square TUs of NSQT directly without remapping. Simple scanning will be tested. Last significant position coding and context selection will be modified accordingly. Moreover, harmonization of the proposed method with G323 will also be tested.

### JCTVC-G517: "Harmonization of unified scan and NSQT”

This contribution proposes the harmonization solution of diagonal scan and non-square block coefficient coding. At the proposed design, 2Nx0.5N / 0.5Nx2N block shares the contexts of NxN block. Since 16x4 / 4x16 block contains 64 coefficients which is same as 8x8 block, the scanning method used at 16x4 / 4x16 block has the same concept as 8x8 block, which means 16x4 / 4x16 diagonal scan is used. For the case of 32x8 / 8x32 block, 4x4 subblock scan will be used.

### JCTVC-G724: “Entropy coding for non square blocks”

JCTVC-G724 proposes removing non-square to square remapping for non-square TU by changing scanning order, context determination for last significant coefficient coding, and context determination for significant map coding. In this core experiment, harmonization of the proposed method with the newly adopted tools in HM-5.0 will be investigated. Specifically, 4x4 subblock scan in G323, and reduced context model for significant map in G1015 will be integrated and tested.

### JCTVC-G750: “Harmonization of HE residual coding and NSQT”

JCTVC-G750 proposes removing non-square to square remapping for non-square TU. To that end, a scanning order for NSQT blocks is defined, last significant coefficient coding is modified, and context determination for significant map coding is determined. In this core experiment, harmonization of the proposed method with the newly adopted tools in HM-5.0 is investigated.

# Software and Configuration

## Software

The CE will use the HM-5.0 software that is recommended by the Software Ad Hoc group (JCTVC-G1200). Proponents are requested to provide software that is can be compiled under Windows and Linux platforms. Also, the proponents are requested to provide software that fulfils the HEVC software guidelines (JCTVC-F688).

## Test Sequences, Bit Rates and Coding Conditions

The CE will use the test sequences, configuration and conditions that are recommended by the Software Ad Hoc group.

# Timeline and Responsibilities

HM-5.0 software available 2.5 weeks after the meeting (Dec. 22, 2011)

| T1 | Dec. 21 2011 (three weeks after meeting) | Final CE-description |
| --- | --- | --- |
| T2 | Two weeks after HM-5.0 software release (Jan. 5 , 2012) | Completion of tool integration; release software & results for cross-check |
| T3 | Two weeks before start of meeting (Jan. 20, 2012) | Complete cross-check; input document upload |

# Definition of Performance Measurement Criteria

The performance measurements are evaluated by switching on and off individual tools to identify their relative performance. The following measurements are considered to be used in this core experiment.

## Coding Performance Measurements

The objective rate-distortion measurements using 4-point BD-PSNR and BD-Rate for all tools in section 3 are used under common conditions. RDOQ off tested in LC configuration.

Proposals in subtests 1 and 2 should also be tested with low QP (12, 17, 22, 27) based on recommendation of BoG on context reduction at 6th JCT-VC Meeting in Turin (F746).

Proposals in subset 3 should also be tested for sensitivity to TU depth (i.e. test with different TU depths), and visual test for artifacts as suggested by the 7th JCT-VC Meeting notes. Test with low QP (12, 17, 22, 27) is also requested.

## Complexity Measurements

For the complexity measurement, the anchor, the reference software and the reference software with the proposed method implemented will be executed on the same machine and the computational time will be measured. A time factor will then be calculated between the reference software and the reference software with the proposed method implemented.

However, the complexity analysis does not solely rely on the encoding and decoding time measurements of a particular implementation (reference software) of the working draft (e.g. should describe need for LUT, additional logic, memory access, memory storage, etc.)

Core experiment participants who investigate parallelization aspects are requested to provide a description showing how the parallelization is improved and which architecture(s) would see the benefit of the parallelization. Average and worst case throughput gain from parallelized methods should be considered.

Core experiment participants who investigate context reduction and bin reduction aspects are requested to report number of context removed as well as change to context selection that enables reduction.

In addition, any tools proposing a different method of scanning are requested to report

- Scheme of selecting the scan pattern during encoding needs to be described.

- Scheme of updating the scan pattern during parsing needs to be described.