| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G122 |
| --- | --- |

| Title: | VLC for high level syntax (ALF and SAO parameters) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vivienne Sze Texas Instruments Inc. | Email: | sze@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This contribution proposes always using VLC for high level syntax elements; specifically, ALF and SAO parameters are coded with VLC even when CABAC is selected as the entropy coder.  These parameters include alf_cu_control_flag, alf_cu_control_max_depth, alf_length_cu_control_info, alf_cu_flag, sao_split_flag, sao_flag_cb, sao_flag_cr, sao_type_idx, sao_offset.  This change has negligible impact on coding efficiency in HM-4.0 under common conditions (0.0 % AI-HE, 0.1% RA-HE, 0.1% LD-HE).

# Introduction

High level syntax is typically handled by firmware, which traditionally don’t have CABAC acceleration [1].  Thus, for reduced system implementation complexity it is desirable for all syntax elements found in slice header, APS, PPS, and SPS be coded with simple VLC.  Currently, in HM-4.0, SAO and ALF parameters, which are sent in the slice header and PPS, are coded with CABAC. In this proposal, the parameters are encoded with VLC even when CABAC is selected as the entropy coder. Note that in HM-4.0, if CABAC entropy coder is selection, byte alignment is used immediately before transmitting SAO parameters in the slice header. If all syntax elements in the slice header are VLC coded, this byte alignment is not necessary; thus byte alignment is moved to after the slice header.

# Experiment Results

The HM-4.0 software is used for the evaluation, the simulation platforms is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz  64 bits Linux machines, the common test conditions and reference configurations specified by the Software AhG are followed.

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.3% | 0.4% |
| Class B | 0.0% | 0.1% | 0.1% |
| Class C | 0.0% | 0.1% | 0.1% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.2% | 0.0% |
| Class F |  |  |  |
| Overall | 0.0% | 0.1% | 0.1% |
|  | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 98% |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | 0.1% | 0.2% | 0.0% |
| Class B | 0.1% | 0.2% | 0.2% |
| Class C | 0.1% | 0.1% | 0.2% |
| Class D | 0.1% | 0.3% | 0.3% |
| Class E |  |  |  |
| Class F |  |  |  |
| Overall | 0.1% | 0.2% | 0.2% |
|  | 0.1% | 0.2% | 0.2% |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | 108% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.1% | 0.0% | -0.1% |
| Class C | 0.1% | 0.1% | -0.1% |
| Class D | 0.0% | 0.5% | 0.5% |
| Class E | 0.1% | 0.1% | 0.7% |
| Class F |  |  |  |
| Overall | 0.1% | 0.2% | 0.2% |
|  | 0.1% | 0.2% | 0.2% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |

Table 1 Average loss from using VLC for all ALF and SAO parameters under common conditions. Positive numbers in the table means coding loss.

# Text for Working Draft

#### Sample adaptive offset parameter syntax

| sao_param( ) { | Descriptor |
| --- | --- |
| sample_adaptive_offset_flag | u(1) |
| if ( sample_adaptive_offset_flag ) { |  |
| sao_split_param( 0, 0, 0, 0 ) |  |
| sao_offset_param( 0, 0, 0, 0 ) |  |
| sao_flag_cb | u(1) ¦ ae(v) |
| if( sao_flag_cb ) { |  |
| sao_split_param( 0, 0, 0, 1 ) |  |
| sao_split_param( 0, 0, 0, 1 ) |  |
| } |  |
| sao_flag_cr | u(1) ¦ ae(v) |
| if( sao_flag_cr ) { |  |
| sao_split_param( 0, 0, 0, 2 ) |  |
| sao_split_param( 0, 0, 0, 2 ) |  |
| } |  |
| } |  |
| } |  |

| sao_split_param( rx, ry, saoDepth , cIdx ) { | Descriptor |
| --- | --- |
| if( saoDepth < SaoMaxDepth ) |  |
| sao_split_flag[ cIdx ][ saoDepth ][ rx ][ ry ] | u(1) ¦ ae(v) |
| else |  |
| sao_split_flag[ cIdx ][ saoDepth ][ rx ][ ry ] = 0 |  |
| if( sao_split_flag[ cIdx ][ saoDepth ][ rx ][ ry ] ) { |  |
| sao_split_param( 2*rx + 0, 2*ry + 0, saoDepth + 1 , cIdx ) |  |
| sao_split_param( 2*rx + 1, 2*ry + 0, saoDepth + 1 , cIdx ) |  |
| sao_split_param( 2*rx + 0, 2*ry + 1, saoDepth + 1 , cIdx ) |  |
| sao_split_param( 2*rx + 1, 2*ry + 1, saoDepth + 1 , cIdx ) |  |
| } |  |
| } |  |

| sao_offset_param( rx, ry, saoDepth , cIdx ) { | Descriptor |
| --- | --- |
| if( sao_split_flag[ cIdx ][ saoDepth ][ rx ][ ry ] ) { |  |
| sao_offset_param( 2*rx + 0, 2*ry + 0, saoDepth + 1 , cIdx ) |  |
| sao_offset_param( 2*rx + 1, 2*ry + 0, saoDepth + 1 , cIdx ) |  |
| sao_offset_param( 2*rx + 0, 2*ry + 1, saoDepth + 1 , cIdx ) |  |
| sao_offset_param( 2*rx + 1, 2*ry + 1, saoDepth + 1 , cIdx ) |  |
| } else { |  |
| sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] | ue(v) ¦ ae(v) |
| if( sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] != 0 ) |  |
| for( i = 0; i < NumSaoClass[ sao_type_idx ]; i++ ) |  |
| sao_offset[ cIdx ][ saoDepth ][ x0 ][ y0 ][ i ] | se(v) ¦ ae(v) |
| } |  |
| } |  |

#### Adaptive loop filter coding unit control parameter syntax

| alf_cu_control_param() { | Descriptor |
| --- | --- |
| if( adaptive_loop_filter_flag ) { |  |
| alf_cu_control_flag | u(1) |
| if( alf_cu_control_flag ) { |  |
| alf_cu_control_max_depth | ue(v) |
| alf_length_cu_control_info | se(v) |
| for( i = 0; i < NumAlfCuFlag; i++ ) |  |
| alf_cu_flag[ i ] | u(1) ¦ ae(v) |
| } |  |
| } |  |
| } |  |

# Summary

This contribution proposes only using VLC to encode ALF and SAO parameters even when CABAC is selected as the entropy coder to reduce system implementation complexity.

# Summary

[1] T. Suzuki, “JCTVC-F377: Arithmetic coding in high level syntax,” Turin, July 2011.

# Patent rights declaration(s)

Texas Instruments Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).