| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G123 |
| --- | --- |

| Title: | Non-CE11: Simplified Coefficient Scans for NSQT |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vivienne Sze Texas Instruments Inc. | Email: | sze@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

In HM-4.0, NSQT coefficients undergo multiple scans.  First, the coefficients of a 4x16/16x4 and 8x32/32x8 transform are mapped to 8x8 and 16x16 square 2D arrays respectively using zig-zag scans. Second, a diagonal scan is used on the coefficients in the 2D square array for CABAC entropy coding.  This contribution proposes simplified coefficient scanning for NSQT.  Specifically, a scan is proposed that maps 4x16/16x4 to four 4x4, and maps 8x32/32x8 to four 8x8.  Since context selection of significant_coeff_flag for 4x4 and 8x8 TU does not depend on neighbors, the intermediate mapping to 2D square array is not needed and only one scan is required for NSQT.  The coding efficiency impact was measure in HM-4.0 under common conditions.  The change to NSQT scan had negligible impact on coding efficiency (0.0% RA-HE, 0.0% LD-HE), while the changes to the context selection of 8x32/32x8 showed minor loss (0.1% RA-HE, 0.2% LD-HE).

# Introduction

Non-Square Transforms (NSQT) of sizes 4x16/16x4 and 8x32/32x8 were introduced into HM-4.0 [1].  To leverage the existing entropy coding in HM, which was designed for square transforms (e.g. scans and context selection are all based on the size of the square TU), 2-D non square array of NSQT coefficients are mapped to another 2-D coefficient square array before undergoing entropy coding as shown in Fig. 1A and Fig. 1B.  For instance, non-square arrays of 4x16 and 16x4 NSQTs are mapped to 8x8 square arrays and undergo equivalent entropy coding of an 8x8 SQT; and 8x32 and 32x8 NSQTs are mapped to 16x16 square arrays and undergo equivalent entropy coding of a 16x16 SQT.

Fig. 1 – Steps for coding transform coefficients encoding of non-square transforms in HM-4.0

Fig. 1 – Steps for coding transform coefficients decoding of non-square transforms in HM-4.0

In HM-4.0, mapping from 2D non-square array to 2D square array involves two steps.  First, the coefficients of the NSQT are scanned from a non-square 2-D array to a 1-D array using a zig-zag scan.  Second, the 1-D array is then scanned to a square 2-D array using zig-zag scan.  To encode using the CABAC entropy coder, the square transforms use a diagonal scan.  The context selection for the significance map of the coefficients depends on the TU size.  For 4x4 and 8x8 TUs, the context is selection based on the position within the block.  For 16x16 and 32x32 TUs, the context selection is based on the neighboring coefficients; thus for 16x16 and 32x32 TU, the coefficients are stored in an intermediate square 2-D array in order to determine the neighbors before the final stage of arithmetic coding. In summary, the current scan used for NSQT coefficients introduces additional scanning steps during the entropy coding process.

This contribution proposes scanning the coefficients of NSQT into multiple smaller square arrays as shown in Fig. 2.  4x16 and 16x4 are mapped to four 4x4 arrays and 8x32 and 32x8 are mapped to four 8x8 arrays using diagonal scan.  The four 4x4 arrays are coded as an 8x8 array.  Since 8x8 does not use neighbors for context selection of significance map, the intermediate 2-D square array, used in HM-4.0, can be bypassed.   In other words, no additional scanning is needed for entropy coding.  Thus NSQT has the same number of scanning steps as SQT.  Fig. 3A and Fig. 3B shows the resulting data flow for this proposal.

While 4x16/16x4  TU use the 8x8 TU SQT context selection,  8x32/32x8 TU used a modified version of the 8x8 TU SQT context selection.  The low frequency 8x8 significant_coeff_flags of the 8x32/32x8 (i.e. top 8x8 TU block) share the same contexts and context selection procedure as the 8x8 TU SQT.  The high frequency 8x8 significant_coeff_flags  have a very simple context selection. All the high frequency 8x8 significant_coeff_flags of the 8x32/32x8 (i.e. the other three 8x8 TU block) use the same high frequency context (ctxIdx=30) of the 8x8 TU SQT.

Fig. 2– Scan of 4x16 into four 4x4.  Similar scan extended to 32x8/8x32 to scan into four 8x8.

Fig. 3A – Steps for coding transform coefficients encoding of non-square transforms with proposed scan approach.  Notice that only one scan is needed in the NSQT path which gives it the same latency as the square transforms.

Fig. 3B – Steps for coding transform coefficients decoding of non-square transforms with proposed scan approach.

# Experiment Results

The HM-4.0 software is used for the evaluation, the simulation platforms is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz  64 bits Linux machines, the common test conditions and reference configurations specified by the Software AhG are followed.

|  | Random Access HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | -0.2% | -0.3% |
| Class B | 0.0% | -0.2% | 0.0% |
| Class C | 0.0% | -0.1% | -0.1% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |
| Class F |  |  |  |
| Overall | 0.0% | -0.1% | -0.1% |
|  | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | 96% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | -0.1% | -0.3% |
| Class C | 0.0% | -0.2% | 0.0% |
| Class D | 0.0% | 0.0% | 0.2% |
| Class E | 0.0% | 0.2% | 0.4% |
| Class F |  |  |  |
| Overall | 0.0% | -0.1% | 0.0% |
|  | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |

Table 1 Coding efficiency of proposed NSQT scan under common conditions. Positive numbers in the table means coding loss.

|  | Random Access HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.1% | -0.1% | 0.0% |
| Class B | 0.1% | 0.2% | 0.1% |
| Class C | 0.1% | 0.1% | 0.1% |
| Class D | 0.1% | 0.0% | 0.1% |
| Class E |  |  |  |
| Class F |  |  |  |
| Overall | 0.1% | 0.0% | 0.1% |
|  | 0.1% | 0.0% | 0.1% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 96% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.2% | 0.5% | 0.3% |
| Class C | 0.1% | 0.1% | 0.1% |
| Class D | 0.1% | -0.1% | 0.4% |
| Class E | 0.1% | 0.5% | 0.8% |
| Class F |  |  |  |
| Overall | 0.1% | 0.2% | 0.4% |
|  | 0.2% | 0.2% | 0.3% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 102% |  |  |

Table 2 Coding efficiency of proposed NSQT scan and modified context selection under common conditions. Positive numbers in the table means coding loss.

# WD Text

### Up-right diagonal scanning array initialisation process

Input to this process is a block size width blkWidth and a block height blkHeight.

Output of this process is the array DiagScan[ pos ][ comp ]. The array index pos specify the scan position ranging from 0 to ( blkWidthSize * blkHeightSize ) − 1. The array index comp equal to 0 specifies the horizontal component and the array index comp equal to 1 specifies the vertical component. Depending on the values of blkWidth and blkHeight, the array DiagScan is derived as follows.

DiagScan[ 0 ][ 0 ]  =  0
DiagScan [ 0 ][ 1 ]  =  0

- If blkHeight=blkWidth then,

i  =  1
x  =  0
y  =  1
stopLoop  =  ( blkWidth  = = 1 ) ? true : false
while( !stopLoop ) {
	while( y  >=  0 ) {
		if (x  >=  0  &&  x  <  blkWidth  &&  y  >=  0  &&  y  <  blkHeight ) {
			DiagScan [ i ][ 0 ]  =  x
			DiagScan [ i ][ 1 ]  =  y
			i++
		}
		y− −
		x++
	}
	y  =  x
	x  =  0
	if ( i  >=  blkWidth  *  blkHeight ) 
		stopLoop  =  true
}

- Otherwise,

MinDim = ( blkWidth < blkHeight ) ? blkWidth : blkHeight

blk=0
x  =  0
y  =  1

while (blk < 4) {

i  =  1

stopLoop  =  (MinDim = = 1 ) ? true : false
while( !stopLoop ) {
	while( y  >=  0 ) {
		if (x  >=  0  &&  x  <  MinDim &&  y  >=  0  &&  y  <  MinDim) {
			DiagScan [ i ][ 0 ]  =  x + MinDim * blk * (blkWidth > blkHeight )
			DiagScan [ i ][ 1 ]  =  y + MinDim * blk * (blkHeight > blkWidth ) 
			i++
		}
		y− −
		x++
	}
	y  =  x
	x  =  0
	if ( i  >=  MinDim *  MinDim) 
		stopLoop  =  true
}

blk++

}

###### Derivation process of ctxIdxInc for the syntax element significant_coeff_flag

Inputs to this process are the color component index cIdx, the current coefficient scan position ( xC , yC ) and the transform block size log2TrafoSize.

–	a variable nW specifying the width of the current transform unit,

–	a variable nH specifying the height of the current transform unit.

Output of this process is ctxIdxInc.

The variable sigCtx depends on the current position ( xC, yC ), the transform block size and previsously decoded bins of the syntax element significant_coeff_flag. For the derivation of sigCtx, the following applies.

When nW is not equal to nH and log2TrafoSize = 4, log2TrafoSize = log2TrafoSize -1

- If (( xC > ( log2TrafoSize << 1 ) ) || ( yC > ( log2TrafoSize << 1) ))

sigCtx  = 30;

- If log2TrafoSize is less than or equal to 3, sigCtx is derived as follows.

shift  	=  log2TrafoSize  = =  3 ? 1 : 0
sigCtx  =  ( shift  *  15 )  +  ( ( yC  >>  shift )  <<  2 )  +  ( xC  >>  shift )	(9-1)

- Otherwise if xC + yC is less than 2, sigCtx is derived as follows.

sigCtx  =  31  +  ( yC  <<  1 )  +  xC		(9-2)

- Otherwise if xC + yC is less than 5, sigCtx is derived as follows.

temp  =  significant_coeff_flag[ xC + 1 ][ yC ]  +  significant_coeff_flag[ xC + 2 ][ yC ]  +  
		significant_coeff_flag[ xC ][ yC + 1 ]  +  significant_coeff_flag[ xC + 1 ][ yC + 1 ]  +  
		significant_coeff_flag[ xC ][ yC + 2 ]	(9-3)
sigCtx  =  34  +  Min( 4, temp )

- Otherwise (xC  +  yC is greater than 4), sigCtx is derived using previously decoded bins of the syntax element significant_coeff_flag as follows.

- The variable sigCtx is initialized as follows.

sigCtx  =  39		(9-4)

- When xC is less than ( 1 << log2TrafoSize ) − 1, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 1 ][ yC ]	(9-5)

- When xC and yC are less than ( 1 << log2TrafoSize ) − 1, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 1 ][ yC + 1 ]	(9-6)

- When xC is less than ( 1 << log2TrafoSize ) − 2, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 2 ][ yC ]	(9-7)

- When yC is less than ( 1 << log2TrafoSize ) − 1, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 1 ]	(9-8)

- When yC is less than ( 1 << log2TrafoSize ) − 2 and sigCtx is less than 43, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 2 ]	(9-9)

The context index increment ctxIdxInc is derived using the color component index cIdx and sigCtx as follows.

- If cIdx is equal to 0, ctxIdxInc is derived as follows.

ctxIdxInc  =  sigCtx		(9-10)

- Otherwise (cIdx is greater than 0), ctxIdxInc is derived as follows.

ctxIdxInc  =  44  +  sigCtx		(9-11)

[Ed. (BB): The context derivation assumes maximum transform sizes less than or equal to 32x32 for luma and 16x16 for chroma and minimum transform sizes greater than or equal to 4x4.]

# Summary

This contribution proposes changing the coefficient scanning of NSQT such that the number of steps and scans for NSQT coefficient coding is reduced.  This change has negligible impact on coding efficiency.

# Reference

[1] Y. Yuan et al., “CE2: Non-Square Quadtree Transform for symmetric motion partitions,” Turin, July 2011.

# Patent rights declaration(s)

Texas Instruments Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).