| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G127 |
| --- | --- |

| Title: | Cross-check of Sony’s proposal on context reduction for significance map (G366) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Vivienne Sze | Email: | sze@ti.com |
| Source: | Texas Instruments Inc |  |  |

_____________________________

# Abstract

This contribution reports cross-check results from Texas Instruments for G366.  The software was analyzed and no mismatch was found compared to the BD-rate results provided by Sony.  Results are reported with and without changes to context selection for coefficient level proposed in G121/F132. Revision of the code was provided where context initialization was changed.  R1 of this document reflects the results with the changed code.

# Summary of Proposal Based on Analysis of Source Code

The number of context for significant_coeff_flag is reduced from 15 to 11 for 4x4 TU, 16 to 9 for 8x8 TU and 15 to 11 for 16x16/32x32 TU.  The same reduction is achieved for both luma and chroma.

For 4x4TU, the high frequency components share the same context (15 positions mapped to 11 contexts)

For 8x8 TU, the high frequency components share the same context (16 positions mapped to 9 contexts)

For 16x16/32x32 TU, the context for positions with X+Y<5, involve adding 5 neighbors; previously required min operation is eliminated; index of 6 is mapped to 3 contexts.  Similarly, for positions Y< (Width-1) previously required conditional adding of neighbor is removed; index of 6 is mapped to 3 contexts.

# Experiment Results

The HM-4.0 software is used for the evaluation, the simulation platforms is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz  64 bits Linux machines, the common test conditions and reference configurations specified by the Software AhG are followed.

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% |
| Class F |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | 0.0% | -0.1% | 0.1% |
| Class B | 0.0% | 0.2% | 0.2% |
| Class C | 0.0% | -0.1% | 0.0% |
| Class D | 0.0% | -0.1% | 0.0% |
| Class E |  |  |  |
| Class F |  |  |  |
| Overall | 0.0% | 0.0% | 0.1% |
|  | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 98% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | -0.1% | -0.2% |
| Class C | 0.0% | 0.0% | -0.2% |
| Class D | 0.0% | 0.1% | -0.4% |
| Class E | 0.1% | -1.0% | 1.0% |
| Class F |  |  |  |
| Overall | 0.0% | -0.2% | 0.0% |
|  | 0.0% | -0.2% | -0.1% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 98% |  |  |

Table 1 Average coding efficiency for High Efficiency configuration of proposal G366 relative to HM-4.0 anchor. Positive numbers in the table means coding loss. Due to variation in the simulation environment, the runtime may be heavily inaccurate and should not be used to infer the complexity of the proposed method.

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.1% | 0.0% | 0.1% |
| Class B | 0.1% | 0.1% | 0.1% |
| Class C | 0.1% | 0.1% | 0.1% |
| Class D | 0.1% | 0.1% | 0.1% |
| Class E | 0.1% | 0.0% | 0.1% |
| Class F |  |  |  |
| Overall | 0.1% | 0.1% | 0.1% |
|  | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | 0.1% | -0.1% | 0.1% |
| Class B | 0.0% | 0.1% | -0.2% |
| Class C | 0.0% | -0.1% | 0.0% |
| Class D | 0.0% | -0.1% | -0.4% |
| Class E |  |  |  |
| Class F |  |  |  |
| Overall | 0.0% | 0.0% | -0.2% |
|  | 0.0% | 0.0% | -0.2% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 104% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | -0.1% | -0.4% |
| Class C | 0.0% | 0.0% | -0.3% |
| Class D | 0.1% | 0.3% | 0.3% |
| Class E | 0.0% | -0.5% | 1.3% |
| Class F |  |  |  |
| Overall | 0.0% | 0.0% | 0.1% |
|  | 0.0% | -0.1% | 0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 97% |  |  |

Table 2 Average coding efficiency for High Efficiency configuration of proposal G366 with G121 relative to HM-4.0 anchor. Positive numbers in the table means coding loss. Due to variation in the simulation environment, the runtime may be heavily inaccurate and should not be used to infer the complexity of the proposed method.