| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G129 |
| --- | --- |

| Title: | CE6.a: Sub-sampling portion of neighboring pixels in calculation of LM parameters |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Madhukar Budagavi Texas Instruments Inc., USA Kazushi Sato Sony City Osaki 9F 2-10-1 Osaki, Shinagawa, 141-8610 Tokyo, Japan | Tel: Email: Tel: Email: | +1-214-480-1861 madhukar@ti.com +81 50 3750 2741 Kazushi.Sato@jp.sony.com |
| Source: | Texas Instruments Inc., Sony Corp. |  |  |

_____________________________

# Abstract

This contribution presents the following simplification to luma-based chroma intra prediction tool: reduction in number of operations for calculation of   and parameters by sub-sampling reference luma and chroma samples. Results for different sub-sampling factors are reported. Sub-sampling by 1:2 for 16x16 block only is reported to have least BD-Rate loss among various sub-sampling factors tested. The BD-Rate for 1:2 sub-sampling for 16x16 block is: AI-HE: Y: 0.0%, U: 0.2%, V: 0.1%, and AI-LC: Y:0.0%, U:0.1%, V:0.0%. It is reported that the number of multiplication for 16x16 block goes down from 64 to 32 for 1:2 sub-sampling. The number of multiplications for 4x4 and 8x8 block remain the same. Larger block requires more computation for calculation of   and parameters so this sub-sampling will contribute to complexity reduction both for HW and SW implementation. Other subsampling factors provide more computational savings at cost of increased BD-Rate loss.

# Introduction

In Luma-based chroma intra prediction (LM), chroma values in a block are predicted from down sampled reconstructed luma values of same block as follows:

PredC(x, y) =  RecLumaD(x, y) +

where PredC is the predicted chroma samples and RecLumaD is the down sampled reconstructed luma samples.

Parameters  and are derived from reconstructed samples around the current block. The equation used for calculating  and is the ordinary least square (OLS) solution:

whereand indicate reconstructed chroma samples and filtered luma samples around the target block, indicates total samples number of neighboring data. In HM-4.0 I can be 4, 8 or 16 and larger blocks require more computation for calculation of   and parameters . For a target N×N chroma block, assuming intra padding, total involved samples number is .

# Proposed modifications

Computational complexity can be reduced by subsamping and  before passing it on to OLS as shown in Figure 1.

Table below shows comparison of number of multiplications and corresponding BD-Rate results for sub-sampling of reference samples before use in calculation of  and . HM-4.0 reference software with bug-fix for alpha renormalization was used in simulations.

|  | Number of multiplications per block for calculating alpha and beta |  |  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Subsampling factor | 4x4 | 8x8 | 16x16 | Y | U | V | Y | U | V |
| HM-4.0 | 16 | 32 | 64 |  |  |  |  |  |  |
| 1:2 for 16x16 | 16 | 32 | 32 | 0.0% | 0.2% | 0.1% | 0.0% | 0.1% | 0.0% |
| 1:2 for 8x8, 16x16 | 16 | 16 | 32 | 0.1% | 0.5% | 0.4% | 0.1% | 0.5% | 0.3% |
| 1:2 for 8x8, 1:4 for 16x16 | 16 | 16 | 16 | 0.1% | 0.8% | 0.5% | 0.1% | 0.7% | 0.3% |

# Conclusions

It is recommended that 1:2 subsampling for 16x16 be adopted into HM-5.0.

# Patent rights declaration(s)

Texas Instruments Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sony Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).