| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G132 |
| --- | --- |

| Title: | Hardware analysis of transform and quantization |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Madhukar Budagavi Vivienne Sze Texas Instruments Inc. Mangesh Sadafale | Tel: Email: | madhukar@ti.com, sze@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This contribution claims that forward transform, inverse transform, quantization, and de-quantization are typically implemented in a single block in hardware to reduce the number of pipeline stages and also to reuse commonality between them to reduce overall area. It proposes that transform and quantization complexity be evaluated by considering a complete solution that takes into consideration forward and inverse transform and quantization and de-quantization. It presents two architectures that exploit commonalities to reduce overall area of transform and quantization block. The first architecture makes use of a useful symmetry property between forward and inverse HM4 transforms to reduce overall area of hardware block that needs to implement both forward and inverse transform. A joint 32-point forward+inverse transform architecture that exploits the symmetry property was implemented in RTL for HM4. Based on the frequency/latency combinations tested, it is claimed that HM4 joint forward+inverse transform can be implemented at the cost of around 5-10% more area when compared to HM4 forward transform or equivalently at a cost of around 17-20% more when compared to HM4 inverse transform. The second architecture is a special-purpose SIMD architecture that can implement both transform and quantization. This contribution also provides hardware area estimates for forward and inverse transform implementations that do one 32-point 1D transform per cycle for HM4 and F251. HM4 transform was implemented using partial butterfly architecture where as F251 transform was implemented using full factorization.

# Architecture 1: Exploiting symmetry between forward and inverse HM4 transforms

## 4-point transform

Inverse 4-point HM4 transform is defined by following equation where X is input and Y is output:

(Eq. 1)

Partial butterfly decomposition results in:

Even part:

Z0Z1=C32  C32                         (Eq. 2)

Odd part:

Z2Z3=-C48  C16                        (Eq. 3)

Add/sub network:

Y0Y1Y2Y3=Z0−Z3Z1−Z2Z1+Z2Z0+Z3                                 (Eq. 4)

Forward 4-point HM4 transform is defined by following equation where M is input and P is output:

P0P1P2P3=C32  C32  C32  C32C16  C48 -C48 -C16C32 -C32 -C32  C32             (Eq. 5)

Partial butterfly decomposition results in:

Add/sub network:

K0K1K2K3=M0+M3M1+M2−M1+M2−M0+M3                              (Eq. 6)

Even part:

P0P2=C32  C32                         (Eq. 7)

Odd part:

P1P3=-C48 -C16                        (Eq. 8)

Or equivalently:

P3P1=-C48  C16                        (Eq. 9)

Comparing (Eq. 2) and (Eq. 6), it can be observed that the even matrix of forward and inverse transform is identical.

Comparing (Eq. 3) and (Eq. 9), it can be observed that the odd matrix of forward and inverse transform is identical.

So a unified 4-point hardware that implements both forward and inverse transform can be constructed by using common even and odd matrices.

## 8-point transform

Inverse 8-point HM4 transform is defined by following equation where X is input and Y is output:

Y0Y1Y2Y3Y4Y5Y6Y7=C32   C8  C16  C24  C32  C40  C48  C56C32  C24  C48 -C56 -C32  -C8 -C16 -C40C32  C40 -C48  -C8 -C32  C56  C16  C24C32  C56 -C16 -C40  C32  C24 -C48  -C8C32 -C56 -C16  C40  C32 -C24 -C48   C8C32 -C40 -C48   C8 -C32 -C56  C16 -C24C32 -C24  C48  C56 -C32   C8 -C16  C40        (Eq. 10)

Partial butterfly decomposition results in:

Even part:

Z0Z1Z2Z3=C32  C16  C32  C48C32  C48 -C32 -C16C32 -C48 -C32  C16          (Eq. 11)

Odd part:

Z4Z5Z6Z7=-C56  C40 -C24   C8-C40   C8 -C56 -C24-C24  C56   C8  C40         (Eq. 12)

Add/sub network:

Y0Y1Y2Y3Y4Y5Y6Y7=Z0−Z7Z1−Z6Z2−Z5Z3−Z4Z3+Z4Z2+Z5Z1+Z6Z0+Z7                                (Eq. 13)

Forward 8-point HM4 transform is defined by following equation where M is input and P is output:

P0P1P2P3P4P5P6P7=C32  C32  C32  C32  C32  C32  C32  C32 C8  C24  C40  C56 -C56 -C40 -C24  -C8C16  C48 -C48 -C16 -C16 -C48  C48  C16C24 -C56  -C8 -C40  C40   C8  C56 -C24C32 -C32 -C32  C32  C32 -C32 -C32  C32C40  -C8  C56  C24 -C24 -C56   C8 -C40C48 -C16  C16 -C48 -C48  C16 -C16  C48      (Eq. 14)

Partial butterfly decomposition results in:

Add/sub network:

K0K1K2K3K4K5K6K7=M0+M7M1+M6M2+M5M3+M4−M3+M4−M2+M5−M1+M6−M0+N7                              (Eq. 15)

Even part:

P0P2P4P6=C32  C32  C32  C32C16  C48 -C48 -C16C32 -C32 -C32  C32              (Eq. 16)

Odd part:

P1P3P5P7=-C56 -C40 -C24  -C8 C40   C8  C56 -C24-C24 -C56   C8 -C40             (Eq. 17)

Comparing (Eq. 11) and (Eq. 16) to (Eq. 1) and (Eq. 5), it can be observed that the even matrix of forward and inverse transform is identical to 4-point forward and inverse transform matrix. Hence the unified architecture described in Section 1.2 can be used to implement the even part of both 8-point forward and inverse transform.

Comparing (Eq. 12) and (Eq. 17), it can be seen that the odd matrix of forward and inverse transform have identical multiplicands which differ in sign for half the elements. This symmetry can be exploited to design a unified circuit that implements odd matrix multiplication for both forward and inverse transform.

## 16-point and 32-point transform

The symmetry property of 8-point transform explained in Section 1.3 extends to 16-point and 32-point transforms as well.

## Area estimates for HM4 and F251 transforms

HM4 was implemented using partial butterfly and F251 transform was implemented using full factorization. The implementations are for 1D 32-point transform in 1 cycle. Table 1 below list the area estimates for separate inverse and forward transform for HM4 and F251. It also lists area estimate for joint inverse+forward for HM4.

Table 1: Area estimates in (KGates)

|  | 32-pt F251 inverse only |  |  |  | 32-pt F251 forward only |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| frequency (MHz) / L | 2 | 4 | 6 | 8 | 2 | 4 | 6 | 8 |
| 150 | 163 | 71 | 78 | 84 | 161 | 80 | 85 | 91 |
| 200 | 243 | 79 | 80 | 86 |  | 92 | 88 | 94 |
| 250 |  | 100 | 84 | 90 |  | 119 | 94 | 98 |
| 300 |  |  | 91 | 93 |  |  | 104 | 103 |
| 350 |  |  | 108 | 96 |  |  | 122 | 108 |
| 400 |  |  | 129 | 105 |  |  | 150 | 117 |
| 450 |  |  |  | 119 |  |  |  | 132 |
| 500 |  |  |  | 133 |  |  |  | 153 |

|  | 32-pt HM4 inverse only |  |  |  | 32-pt HM4 forward only |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| frequency (MHz) / L | 2 | 4 | 6 | 8 | 2 | 4 | 6 | 8 |
| 150 | 123 | 116 | 122 | 129 | 126 | 129 | 135 | 141 |
| 200 | 152 | 122 | 127 | 133 | 167 | 139 | 141 | 147 |
| 250 | 234 | 130 | 133 | 139 |  | 156 | 148 | 154 |
| 300 |  | 140 | 141 | 147 |  | 190 | 157 | 163 |
| 350 |  | 165 | 150 | 155 |  |  | 170 | 172 |
| 400 |  |  | 167 | 164 |  |  | 192 | 182 |
| 450 |  |  | 195 | 175 |  |  |  | 195 |
| 500 |  |  |  | 189 |  |  |  | 216 |

|  | 32-pt HM4 inverse + forward |  |  |  |
| --- | --- | --- | --- | --- |
| frequency (MHz) / L | 2 | 4 | 6 | 8 |
| 150 | 211 | 139 | 145 | 151 |
| 200 |  | 146 | 150 | 156 |
| 250 |  | 166 | 156 | 162 |
| 300 |  | 209 | 169 | 170 |
| 350 |  |  | 181 | 182 |
| 400 |  |  | 202 | 192 |
| 450 |  |  |  | 206 |
| 500 |  |  |  | 226 |

# Architecture 2: Special purpose SIMD architecture

This architecture is a special-purpose 32-way SIMD architecture that supports 16x8 multiply/accumulate (MAC) operations. This architecture implements 1D 32-point transform in 14 cycles. It carries out de-quantization of 32-points in one cycle. This is sufficient to support transform and quantization throughput of 4Kx2K, 30 fps encode at 450 MHz. Depending on the throughput requirement this architecture can be scaled up or down by increasing or reducing the number of MACS. Area numbers for this architecture were presented in JCTVC-F446.

# Conclusions

This contribution proposes that transform and quantization complexity be evaluated by considering a complete solution that takes into consideration forward and inverse transform and quantization and de-quantization. Transform designs that allow for sharing of hardware between forward and inverse transform and quantization and de-quantization are desirable. The contribution presented a joint forward+inverse transform design to demonstrate hardware sharing that can be efficiently achieved by HM4 transform. This contribution also described a special purpose SIMD architecture for implementing HM4 transform and quantization. The HM4 transform design is a very flexible design and can be tailored to fit several different architectures. This flexibility is a very desirable property. Also, the HM4 design can be scaled easily to meet throughput requirements as in the SIMD architecture design. One more desirable property of HM4 design is that it has low latency since it does not use cascaded multipliers.

# Patent rights declaration(s)

Texas Instruments Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).