| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G133 |
| --- | --- |

| Title: | CE8: Crosscheck of JCTVC-G208 - Snowflake5x5 and cross9x9 for luma and chroma ALF shapes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Madhukar Budagavi | Tel: Email: | +1 214 480 1861 madhukar@ti.com |
| Source: | Texas Instruments, Inc. |  |  |

_____________________________

# Abstract

This contribution presents results of cross-verification of JCTVC-G208 which proposes the use of snowflake5x5 and cross9x9 ALF filter types. The PSNR/bit-rate numbers obtained were compared to those provided by proponents and a match was observed. The worst case number of operations for the proposed filter sets and HM-4.0 filter sets are observed to be the same. However, cross9x9 has larger vertical filter size (9 v/s 5) which could result in significant line buffer/memory bandwidth increase. This aspect should be considered while discussing this proposal.

# Simulation results

Encoder/decoder simulations were run on Linux cluster. Table below presents a summary of BD-Rate and encoder/decoder run-times in comparison with HM 4.0 anchors. The differences in encoder and decoder times could be attributed to the heterogeneous nature of compute cluster used for simulations.

|  | TI cross-check | Proponent | TI cross-check | Proponent | TI cross-check | Proponent |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-Rate | BD-Rate | Enc Time | Enc Time | Dec Time | Dec Time |
| AI HE | -0.1% | -0.1% | 97.02% | 100.15% | 99.27% | 101.01% |
| RA HE | -0.4% | -0.4% | 97.54% | 100.16% | 98.33% | 102.32% |
| LB HE | -0.6% | -0.6% | 99.44% | 100.50% | 98.78% | 101.62% |

# Discussions

- cross9x9 has larger vertical filter size which could result in significant line buffer/memory bandwidth increase. This aspect should be considered while discussing this proposal. The authors claim that line buffer removal techniques could be used in conjunction with this proposal. This aspect is not further discussed in this contribution and but is presented in a different contribution at this meeting.

- Both luma and chroma ALF filters are modified. WD text is provided for luma modifications but not for chroma modifications.