| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G134 |
| --- | --- |

| Title: | Motion Vector Predictor Candidate Clipping Removal |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Muhammed Coban Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-658-3973 mcoban@qualcomm.com martak@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution presents a modification to the motion vector predictor (MVP) candidate derivation process that removes the final normative motion vector clipping operation. In the current HM design, motion vector predictor values are clipped using the non-normative motion vector clipping operation in the motion compensation process. Removal of the motion vector clipping operation on the MVP candidates reduces MVP derivation complexity with negligible impact on coding efficiency. Simulation results show 0.0%, 0.0%, and 0.0% BD rate change for RA_HE, LB_HE and LP_HE cases, respectively.

# Introduction

In the current HM, motion vector predictor candidates values are clipped using the non-normative motion vector clipping operation, clipMV( ) described below, that guarantees motion compensation from regions that lie within the padded frame bounds (padding of maxCUSize) of reference frames. This can be illustrated by Figure 1. Currently this is a normative operation that relies on what should be a non-normative padding size. It also adds complexity to the motion vector predictor candidate derivation in the merge and AMVP processes since it is applied to every added candidate to the predictor list.

Void TComDataCU::clipMv    (TComMv&  rcMv)

{

Int  iMvShift = 2;

Int iHorMax = (m_pcSlice->getSPS()->getWidth() - m_uiCUPelX - 1 )<<iMvShift;

Int iHorMin = (      -(Int)g_uiMaxCUWidth - (Int)m_uiCUPelX + 1 )<<iMvShift;

Int iVerMax = (m_pcSlice->getSPS()->getHeight() - m_uiCUPelY - 1 )<<iMvShift;

Int iVerMin = (      -(Int)g_uiMaxCUHeight - (Int)m_uiCUPelY + 1 )<<iMvShift;

rcMv.setHor( min (iHorMax, max (iHorMin, rcMv.getHor())) );

rcMv.setVer( min (iVerMax, max (iVerMin, rcMv.getVer())) );

}

Figure 1 Motion vector clipping operation.

# Algorithm Description

All normative motion vector clipping operations are removed. Motion vector predictor candidates uses motion vector values that are not clipped. Motion vector differences are derived using unclipped motion vector predictors. Motion vectors pointing outside of frame boundaries as a result of merge or AMVP modes are clipped for the motion compensation process only.

# Results

The proposed motion vector predictor clipping removal algorithm is implemented in HM4.0. The simulations are conducted based on the JCT-VC common test condition defined in [1]. Proposed changes only affect RA_HE, LB_HE and LP_HE as well as RA_LC, LB_LC and LP_LC configurations. Error: Reference source not found shows summary of simulation results. As can be from the results on average there is 0.0% BD rate loss.

Table 1 Summary of simulation results (BD-rate), HM40 anchor

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.01% | -0.08% | -0.26% | 0.00% | 0.02% | 0.04% |
| Class B | 0.01% | 0.06% | 0.01% | 0.00% | -0.02% | 0.08% |
| Class C | 0.02% | 0.01% | -0.01% | -0.01% | 0.02% | 0.05% |
| Class D | 0.02% | 0.03% | -0.02% | -0.02% | 0.03% | 0.02% |
| Class E |  |  |  |  |  |  |
| Overall | 0.01% | 0.01% | -0.06% | -0.01% | 0.01% | 0.05% |
|  | 0.01% | 0.01% | -0.06% | -0.01% | 0.01% | 0.04% |
| Enc Time[%] | 97% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.02% | -0.05% | -0.17% | 0.02% | 0.00% | -0.01% |
| Class C | 0.01% | 0.00% | -0.10% | 0.02% | 0.06% | -0.20% |
| Class D | 0.03% | 0.10% | -0.15% | 0.00% | -0.13% | -0.11% |
| Class E | -0.01% | -0.32% | 0.39% | -0.03% | -0.19% | -0.19% |
| Overall | 0.01% | -0.05% | -0.04% | 0.01% | -0.05% | -0.11% |
|  | 0.01% | -0.05% | -0.04% | 0.01% | -0.10% | -0.16% |
| Enc Time[%] | 98% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.03% | 0.00% | -0.05% | 0.01% | 0.02% | -0.03% |
| Class C | 0.00% | 0.00% | -0.10% | 0.02% | -0.07% | 0.09% |
| Class D | 0.04% | -0.08% | -0.10% | 0.00% | 0.03% | -0.02% |
| Class E | -0.03% | -0.20% | -0.09% | 0.00% | -0.07% | 0.15% |
| Overall | 0.01% | -0.06% | -0.08% | 0.01% | -0.02% | 0.04% |
|  | 0.01% | -0.08% | -0.07% | 0.01% | -0.01% | 0.01% |
| Enc Time[%] | 97% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

# Conclusions

This contribution presented a modification to the motion vector predictor (MVP) candidate derivation process that removes the final normative motion vector clipping operation. Removal of the motion vector clipping operation on the MVP candidates reduces MVP derivation complexity with negligible impact on coding efficiency. Simulation results show 0.0%, 0.0%, and 0.0% BD rate change for RA_HE, LB_HE and LP_HE cases, respectively.

# References

- F. Bossen, “Common test conditions and software reference configurations,” JCT-VC document JCTVC-F900, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 6th meeting, Torino, July 2011.

- B. Bross, et al., “WD4: Working Draft 4 of High-Efficiency Video Coding,” JCT-VC document JCTVC-F803, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/ WG11 6th meeting, Torino, July 2011.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix A: Proposed modification of HEVC Working Draft 4[2]

##### Derivation process for motion vector predictor candidates

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1).

Outputs of this process are (with N being replaced by A, or B)

- the motion vectors mvLXN of the neighbouring prediction units,

- the availability flags availableFlagLXN of the neighbouring prediction units.

Figure 8-2 – Spatial motion vector neighbours

The function RefPicOrderCnt( pic, refidx, LX ) is specified by the value of PicOrderCnt of the picture that is the reference picture RefPicListX[ refidx ] of pic with X being 0 or 1. PicOrderCnt of the reference picture shall be maintained until the picture is marked as “non-existing”

[Ed. (BB): PicOrderCnt(.) needs to be defined. Currently the Decoding process for picture order count is not described]

The variable isScaledFlagLX with X being 0 or 1 is set equal to 0.

The motion vector mvLXA and the availability flag availableFlagLXA are derived in the following ordered steps:

- Let a set of two sample locations be (xAk, yAk), with k = 0, 1, specifies sample locations with xAk = xP – 1, yA0 = yP + nPSH and yA1 = yA0 - MinPuSize. The set of sample locations ( xAk, yAk ) represent the sample locations immediately to the left side of the left partition boundary and it’s extended line. [Ed.: (WJ) MinPuSize should be defined somewhere]

- Let the availability flag availableFlagLXA be initially set equal to 0 and the both components of mvLXA are set equal to 0.

- For ( xAk, yAk ) from ( xA0, yA0 ) to ( xA1, yA1 ) where yA1 = yA0 - MinPuSize, the following applies repeatedly until availableFlagLXA is equal to 1:

- If the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xAk ][ yAk ] is equal to 1 and the reference index refIdxLX[ xAk ][ yAk ] is equal to the reference index of the current prediction unit refIdxLX, availableFlagLXA is set equal to 1 and the motion vector mvLXA is set equal to the motion vector mvLX[ xAk ][ yAk ], refIdxA is set equal to refIdxLX[ xAk ][ yAk ] and ListA is set equal to LX.

- Otherwise if the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xAk ][ yAk ] (with Y = !X) is equal to 1 and RefPicOrderCnt( currPic, refIdxLY[ xAk ][ yAk ], LY) is equal to RefPicOrderCnt( currPic, refIdxLX, LX), availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLY[ xAk ][ yAk ], refIdxA is set equal to refIdxLY[ xAk ][ yAk ] ,  ListA is set equal to LY and mvLXA is set equal to mvLXA.

- When availableFlagLXA is equal to 0, for ( xAk, yAk ) from ( xA0, yA0 ) to ( xA1, yA1 ) where yA1 = yA0 - MinPuSize, the following applies repeatedly until availableFlagLXA is equal to 1:

- If the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xAk ][ yAk ] is equal to 1, availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLX[ xAk ][ yAk ], refIdxA is set equal to refIdxLX[ xAk ][ yAk ], ListA is set equal to LX.

- Otherwise if the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xAk ][ yAk ] (with Y = !X) is equal to 1, availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLY[ xAk ][ yAk ], refIdxA is set equal to refIdxLY[ xAk ][ yAk ], ListA is set equal to LY.

- When availableFlagLXA is equal to 1, isScaledFlagLX is set equal to 1 and mvLXA is derived as specified below.

tx = ( 16384 + Abs( td / 2 ) ) / td	(8-1)

DistScaleFactor = Clip3( -1024, 1023, ( tb * tx + 32 ) >> 6 )	(8-2)

mvLXA = ClipMv( Sign( DistScaleFactor * mvLXA ) * 			
			( (Abs( DistScaleFactor * mvLXA ) + 127 ) >> 8 ) )	(8-3)

[Ed. (WJ): software has a clip function for scaled motion vector. Do we need this? AVC does not have it]

where td and tb are derived as

td = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic , refIdxA, ListA ) )	(8-4)

tb = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic, refIdxLX, LX ) )	(8-5)

The motion vector mvLXB and the availability flag availableFlagLXB are derived in the following ordered steps:

- Let a set of three sample location (xBk, yBk), with k = 0,1,2, specifies sample locations with xB0 = xP + nPSW, xB1 = xB0 MinPuSize , xB2 = xP - MinPuSize and yBk = yP – 1. The set of sample locations ( xBk, yBk ) represent the sample locations immediately to the upper side of the above partition boundary and it’s extended line. [Ed.: (WJ) MinPuSize should be defined somewhere]

- Let the availability flag availableFlagLXB be initially set equal to 0 and the both components of mvLXB are set equal to 0.

- For ( xBk, yBk ) from ( xB0, yB0 ) to ( xB2, yB2 ) where xB0 = xP +nPSW, xB1 = xB0 - MinPuSize , and xB2 =  xP - MinPuSize, the following applies repeatedly until availableFlagLXB is equal to 1:

- If the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xBk ][ yBk ] is equal to 1, and the reference index refIdxLX[ xBk ][ yBk ] is equal to the reference index of the current prediction unit refIdxLX, availableFlagLXB is set equal to 1 and the motion vector mvLXB is set equal to the motion vector mvLX[ xBk ][ yBk ], refIdxB is set equal to refIdxLX[ xBk ][ yBk ] and ListB is set equal to LX.

- Otherwise if the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xBk ][ yBk ] (with Y = !X) is equal to 1 and RefPicOrderCnt( currPic , refIdxLY[ xBk ][ yBk ], LY) is equal to RefPicOrderCnt( currPic, refIdxLX, LX ),  availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLY[ xBk ][ yBk ],  refIdxB is set equal to refIdxLY[ xBk ][ yBk ],  and ListB is set equal to LY.

- When availableFlagLXA or availableFlagLXB is equal to 0 and isScaledFlagLX is equal to 0, availableFlagLXB is set equal to 0 and for ( xBk, yBk ) from ( xB0, yB0 ) to ( xB2, yB2 ) where xB0 = xP +nPSW, xB1 = xB0 - MinPuSize , and xB2 =  xP - MinPuSize, the following applies repeatedly until availableFlagLXB is equal to 1:

- If the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xBk ][ yBk ] is equal to 1, availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLX[ xBk ][ yBk ], refIdxB is set equal to refIdxLX[ xBk ][ yBk ], ListB is set equal to LX.

- Otherwise if the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xBk ][ yBk ] (with Y = !X) is equal to 1, availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLY[ xBk ][ yBk ], refIdxB is set equal to refIdxLY[ xBk ][ yBk ], ListB is set equal to LY.

- When availableFlagLXB is equal to 1 and RefPicOrderCnt( currPic , refIdxB, ListB ) is not equal to RefPicOrderCnt( currPic, refIdxLX, LX ),, mvLXB is derived as specified below.

tx = ( 16384 + Abs( td / 2 ) ) / td	(8-6)

DistScaleFactor = Clip3( -1024, 1023, ( tb * tx + 32 ) >> 6 )	(8-7)

mvLXB = ClipMv( Sign( DistScaleFactor * mvLXA ) * 			
			( (Abs( DistScaleFactor * mvLXA ) + 127 ) >> 8 ) ) 	(8-8)

[Ed. (WJ): software has a clip function for scaled motion vector. Do we need this? AVC does not have it]

where td and tb are derived as

td = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic , refIdxB, ListB ) )	(8-9)

tb = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic, refIdxLX, LX ) )	(8-10)

##### Derivation process for temporal luma motion vector prediction

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1).

Outputs of this process are

- the motion vector prediction mvLXCol,

- the availability flag availableFlagLXCol.

The function RefPicOrderCnt( pic, refidx, LX ) is specified by the value of PicOrderCnt of the picture that is the reference picture RefPicListX[ refidx ] of pic with X being 0 or 1. PicOrderCnt of the reference picture shall be maintained until the picture is marked as “non-existing”.

[Ed. (BB): PicOrderCnt(.) needs to be defined. Currently the Decoding process for picture order count is not described]

Depending on the values of slice_type and collocated_from_l0_flag, the variable colPic, specifying the picture that contains the co-located partition, is derived as follows.

- If slice_type is equal to B and collocated_from_l0_flag is equal to 0, the variable colPic specifies the picture that contains the co-located partition as specified by RefPicList1[ 0 ].

- Otherwise (slice_type is equal to B and collocated_from_l0_flag is equal to 1 or slice_type is equal to P) , the variable colPic specifies the picture that contains the co-located partition as specified by RefPicList0[ 0 ].

Variable colPu and its position ( xPCol, yPCol ) are derived in the following ordered steps:

- Right-bottom luma position of the current prediction unit is defined by

xPRb = xP + nPSW	(8-11)

yPRb = yP + nPSH	(8-12)

- The variable colPu is set as the prediction unit covering the modified position given by ( ( xPRb >> 4 ) << 4, ( yPRb >> 4 ) << 4 ) inside the colPic.

- If colPu is coded in an intra prediction mode or colPu is unavailable, the following applies.

  - Central luma position of the current prediction unit is defined by

xPCtr = ( xP + ( nPSW >> 1 ) – 1	(8-13)

yPCtr = ( yP + ( nPSH >> 1 ) – 1	(8-14)

  - The variable colPu is set as the prediction unit covering the modified position given by ( ( xPCtr >> 4 ) << 4, ( yPCtr >> 4 ) << 4 ) inside the colPic.

- ( xPCol, yPCol ) is set equal to the top-left luma sample of the colPu relative to the top-left luma sample of the colPic.

The variables mvLXCol and availableFlagLXCol are derived as follows.

- If colPu is coded in an intra prediction mode or colPu is unavailable, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

- Otherwise (colPu is not coded in an intra prediction mode and colPu is available), the variables mvCol and refIdxCol are derived as follows,

  - If PredFlagL0[ xPCol ][ yPCol ] is equal to 0, the motion vector mvCol and the reference index refIdxCol are set equal to MvL1[ xPCol ][ yPCol ] and RefIdxL1[ xPCol ][ yPCol ], respectively.

  - Otherwise (PredFlagL0[ xPCol ][ yPCol ] is equal to 1), the following applies.

  - If PredFlagL1[ xPCol ][ yPCol ] is equal to 0, the motion vector mvCol and the reference index refIdxCol are set equal to MvL0[ xPCol ][ yPCol ] and RefIdxL0[ xPCol ][ yPCol ], respectively.

  - Otherwise (PredFlagL1[ xPCol ][ yPCol ] is equal to 1), the following assignments are made.

      - If all PicOrderCnts of pictures in all reference picture lists are smaller than PicOrderCnt( currPic ), the motion vector mvCol, the reference index refIdxCol and ListCol are set equal to MvLN[ xPCol ][ yPCol ], RefIdxColLN and LN, respectively, with N being value of RefIdxLX.

      - Otherwise (one of PicOrderCnt of pictures in all reference picture lists are larger than PicOrderCnt( currPic ) ), the motion vector mvCol, the reference index refIdxCol and ListCol are set equal to MvLN[ xPCol ][ yPCol ], RefIdxColLN and LN, respectively, with N being value of collocated_from_l0_flag.

and the variable availableFlagLXCol is set equal to 1 and the following applies.

  - If PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxCol, ListCol ) is equal to PicOrderCnt( currPic ) - RefPicOrderCnt( currPic, refIdxLX, LX ),

mvLXCol = mvCol	(8-15)

  - Otherwise, mvLXCol is derived as scaled version of the motion vector mvCol as specified below

tx = ( 16384 + Abs( td / 2 ) ) / td	(8-16)

DistScaleFactor = Clip3( -1024, 1023, ( tb * tx + 32 ) >> 6 )	(8-17)

mvLXCol = ClipMv( Sign( DistScaleFactor * mvLXA ) * 			
				( (Abs( DistScaleFactor * mvLXA ) + 127 ) >> 8 ) )	(8-18)

[Ed. (WJ): software has a clip function for scaled motion vector. Do we need this? AVC does not have it]

where td and tb are derived as

td = Clip3( -128, 127, PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxCol, ListCol ) )	(8-19)

tb = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic, refIdxLX, LX ) )	(8-20)