| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, November 21-30, 2011 | Document: JCTVC-G140 |
| --- | --- |

| Title: | CE4 Subtest 2.1.c Cross Check Report for MediaTek's AQO CaseC by HKUST |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Feng Zou Oscar C. Au | Email: | fengzou@ust.hk eeau@ust.hk |
| Source: | The Hong Kong University of Science and Technology (HKUST) |  |  |

_____________________________

# Abstract

This contribution provides a summary of the cross-check performed by HKUST as described in JCTVC-F119. This contribution documents the findings based on the study and simulation conducted using the software provided by MediaTek. The anchor is obtained using Qualcomm’s QPRounding software released in CE4.

# Verification

MediaTek provided the HM-4.0 based source code of their proposed codec as described in JCTVC-F119.

The encoder command --AQO =1 --AQPS=1 was set to enable the MediaTek’s AQO for test Case C defined in JCTVC-F904.

The anchor was obtained using Qualcomm’s QPRounding software by setting macros #define USE_QRL 1 and #define QPRounding 1.

Four configurations were run for the test Case C with the provided code (LD_HE, LD_LoCo, RA_HE, RA_LoCo)

The encoding/decoding times were estimated using a PC with the following configuration:

- CPU: Intel Core i7 CPU 860 @2.80 GHz.

- RAM: 16 GB

- OS:  Windows 7.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | -4.7% | -4.6% | 0.0% | -3.1% | -3.8% |
| Class B | 0.0% | -4.7% | -5.2% | -0.1% | -3.9% | -4.4% |
| Class C | -0.8% | -3.7% | -3.7% | -1.0% | -3.5% | -3.1% |
| Class D | -0.5% | -3.6% | -3.3% | -0.5% | -3.1% | -2.7% |
| Class E |  |  |  |  |  |  |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -0.3% | -4.2% | -4.3% | -0.4% | -3.5% | -3.6% |
|  | -0.2% | -4.1% | -4.2% | -0.3% | -3.4% | -3.5% |
| Enc Time[%] | 116% |  |  | 110% |  |  |
| Dec Time[%] | 115% |  |  | 110% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.4% | -2.7% | -2.7% | -0.1% | -1.8% | -2.5% |
| Class C | -0.7% | -2.1% | -2.0% | -0.4% | -1.3% | -1.0% |
| Class D | 0.0% | -1.8% | -1.4% | -0.1% | -1.2% | -0.8% |
| Class E | -0.5% | -0.8% | -2.2% | -0.2% | -2.0% | -2.6% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -0.4% | -2.3% | -2.1% | -0.2% | -1.5% | -1.7% |
|  | -0.4% | -2.4% | -2.3% | -0.2% | -1.5% | -1.7% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 103% |  |  | 101% |  |  |

# Observations

The obtained BD-Rate results perfectly match the results provided by MediaTek and the run timing is reliable for LD_HE, and LD_LoCo. A crash is observed when we compile the anchor software using 32bit for Class A test sequences in RA cases. Therefore, we compile the anchor software using 64bit for Class A. The proposed AQO is compiled as 32bit and no crash is observed. So the timing for Class A is not reliable. Summary of the results are shown in the table above and more detailed results can be found in the Excel sheet in the compressed file.

# Comments on source code

Source code was roughly reviewed and confirmed that it was consistent with the description in JCTVC- F271.

# Conclusion

The results presented by MediaTek are confirmed.