| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, November 21-30, 2011 | Document: JCTVC-G141 |
| --- | --- |

| Title: | CE4 Subtest 2.2.d Cross Check Report for RIM's ARL Case D by HKUST |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Feng Zou Oscar C. Au | Email: | fengzou@ust.hk eeau@ust.hk |
| Source: | The Hong Kong University of Science and Technology (HKUST) |  |  |

_____________________________

# Abstract

This contribution provides a summary of the cross-check performed by HKUST as described in JCTVC-F276. This contribution documents the findings based on the study and simulation conducted using the software provided by RIM. The anchor is obtained using Miscrosoft’s EEM(Adaptive Rounding) software released in CE4, which is already included into RIM’s software.

# Verification

RIM provided the HM-4.0 based source code of their proposed codec as described in JCTVC-F276.

The following encoder macros were set to enable the RIM’s ARL in Case D defined in F904.

#define QUANTIZATION_ARL         1

#define QP_ROUNDING               0

#define QUANTIZATION_EEM         1

The anchor was obtained using Microsoft’s EEM software by setting

#define QUANTIZATION_ARL         0

#define QP_ROUNDING               0

#define QUANTIZATION_EEM         1

Four configurations were run for Case D with the provided code (LD_HE, LD_LoCo, RA_HE, RA_LoCo) with RDOQ off.

The encoding/decoding times were estimated using a PC with the following configuration:

- CPU: Intel Core i7 CPU 860 @2.80 GHz.

- RAM: 16 GB

- OS:  Windows 7.

# Observations

The obtained BDBR results perfectly match the results provided by RIM and the run timing is reliable. A crash is observed when we compile the anchor software using 32bit for Class A test sequences in RA cases. Therefore, we compile the software using 64bit for Class A. Summary of the results are shown in the table below and more detailed results can be found in the Excel sheet in the compressed file.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -1.8% | 3.6% | 2.0% | -1.3% | 4.8% | 3.3% |
| Class B | -1.4% | 0.5% | 0.2% | -1.5% | 0.5% | 0.2% |
| Class C | -1.3% | 1.7% | 1.8% | -1.5% | 1.2% | 1.5% |
| Class D | -0.9% | 1.8% | 1.8% | -1.1% | 1.3% | 1.5% |
| Class E |  |  |  |  |  |  |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -1.3% | 1.8% | 1.4% | -1.4% | 1.9% | 1.5% |
|  | -1.1% | 1.9% | 1.4% | -1.1% | 1.9% | 1.5% |
| Enc Time[%] | 97% |  |  | 97% |  |  |
| Dec Time[%] | 97% |  |  | 97% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -1.6% | -1.6% | -2.3% | -2.1% | -1.9% | -2.4% |
| Class C | -1.8% | -0.7% | -0.2% | -2.1% | -0.7% | -0.1% |
| Class D | -1.1% | -0.5% | -0.9% | -1.4% | -1.1% | -1.2% |
| Class E | -0.4% | -1.3% | -1.1% | -0.8% | -2.4% | -1.9% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -1.5% | -1.0% | -1.2% | -1.7% | -1.5% | -1.4% |
|  | -1.5% | -1.1% | -1.1% | -1.7% | -1.5% | -1.4% |
| Enc Time[%] | 96% |  |  | 95% |  |  |
| Dec Time[%] | 97% |  |  | 95% |  |  |

# Comments on source code

Source code was reviewed roughly and confirmed that it was consistent with the description in JCTVC- F276.

# Conclusion

The results presented by RIM are confirmed.