| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G142 |
| --- | --- |

| Title: | CE6.c: LM mode harmonization on SDIP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jaehyun Lim Byeongmoon Jeon 221, Yangjae-dong, Seoucho-gu, Seoul 137-130, Korea | Tel: Email: | jaehyun.lim@lge.com bm.jeon@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This document presents a LM mode harmonization method for SDIP. In the previous meeting, the first position of LM mode in the chroma mode coding in SDIP was agreed [1]. In this test, the performance of LM mode position change is investigated under the new SDIP anchor based on HM4 [2]. The simulation result reportedly shows that the proposed method achieves 0.4%/0.3% and 0.6%/0.4% BD rate reduction in U/V component for all intra HE and LC configurations respectively without any encoder and decoder complexity changes. And also, the simulation result shows that the proposed method achieves 0.1%/0.4%/0.3% and 0.0%/0.5%/0.3% BD rate reduction in Y/U/V component including class F sequences.

# Introduction

In the previous meeting, the first position of LM mode in the chroma mode coding in SDIP was agreed. However, several simplification and modification with regard to LM mode are adopted at the same time [3]. In this test, the performance of LM mode position change will be investigated under the new SDIP anchor based on HM4.

# Proposed Methods

In the SDIP anchor, chroma mode candidate list which is including LM mode is created as shown in Figure 1. Maximum four luma modes are mapped to low indices and LM mode is mapped to the next. Mode 0~3 and 7 (Ver+8) are mapped to high ones and only the first six modes are searched in mode decision.

Figure.1 Chroma mode candidate list in the SDIP anchor

In the proposed method, positions are switched between luma modes and LM mode as shown in Figure 2 for harmonizing luma modes and LM mode. LM mode has the lowest index and luma modes and mode 0~3 and 7 (Ver+8) are followed. Only the first six modes are searched in mode decision same as the SDIP anchor.

Figure.2 Chroma mode candidate list in the proposed method

# Experimental Results

The proposed method was implemented on the HM4-SDIP software and the performance was measured under both of intra only configuration (High Efficiency and Low Complexity). Common test condition is used for measuring the performance of the proposed method besides the SDIP option.

Table 1 shows the experimental results in terms of BD rate reduction. The average gains for all intra HE and LC configurations are 0.4%/0.3% and 0.6%/0.4% respectively in the U/V components. There are no noticeable encoder and decoder complexity changes.

Table.1 BD rate (%) reduction of the proposed method

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.5% | -0.2% | 0.0% | -1.0% | -0.4% |
| Class B | 0.0% | -0.2% | -0.2% | 0.0% | -0.4% | -0.3% |
| Class C | -0.1% | -0.4% | -0.3% | -0.1% | -0.4% | -0.4% |
| Class D | -0.1% | -0.3% | -0.2% | 0.0% | -0.4% | -0.3% |
| Class E | 0.0% | -1.0% | -0.7% | 0.0% | -0.7% | -0.6% |
| Overall | 0.0% | -0.4% | -0.3% | 0.0% | -0.6% | -0.4% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 2 also shows the experimental results of LM mode harmonization including class F sequences. The average gains for all intra HE and LC configurations are 0.1%/0.4%/0.3% and 0.0%/0.5%/0.3% respectively in the Y/U/V components.

Table.2 BD rate (%) reduction of the proposed method including class F sequences

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.5% | -0.2% | 0.0% | -1.0% | -0.4% |
| Class B | 0.0% | -0.2% | -0.2% | 0.0% | -0.4% | -0.3% |
| Class C | -0.1% | -0.4% | -0.3% | -0.1% | -0.4% | -0.4% |
| Class D | -0.1% | -0.3% | -0.2% | 0.0% | -0.4% | -0.3% |
| Class E | 0.0% | -1.0% | -0.7% | 0.0% | -0.7% | -0.6% |
| Class F | -0.2% | -0.3% | -0.2% | -0.1% | -0.3% | -0.3% |
| Overall | -0.1% | -0.4% | -0.3% | 0.0% | -0.5% | -0.3% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Conclusion

In this document, LM mode harmonization method for SDIP was proposed. The proposed scheme shows bit-rate reduction without causing any encoder or decoder complexity changes under the new SDIP anchor based on HM4. Accordingly, the proposed scheme shows more harmonized results for the chroma mode decision on SDIP.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

- JCT-VC, “CE6: Summary report of Core Experiment on intra prediction improvements”, JCTVC-F026, JCT-VC Meeting, Torino, IT, July, 2011.

- TBD, JCT-VC, “CE6: XXX”, JCTVC-GXXX, JCT-VC Meeting, Geneva, CH, November, 2011.

- JCT-VC, “BoG report on simplification of intra_chromaFromLuma mode prediction”, JCTVC-F760, JCT-VC Meeting, Torino, IT, July, 2011.