| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G143 |
| --- | --- |

| Title: | CE6.c: VLC improvement for intra partitioning on SDIP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jaehyun Lim Byeongmoon Jeon 221, Yangjae-dong, Seoucho-gu, Seoul 137-130, Korea | Tel: Email: | jaehyun.lim@lge.com bm.jeon@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This document presents a joint coding on split flag and partitioning type for intra prediction mode under the SDIP. When SDIP is enabled, split mode and four kinds of partitioning type exist for intra prediction. Similar joint coding scheme and counters as used for inter mode coding was also applied to intra mode coding. The simulation result shows that the proposed method achieves 0.1% BD rate reduction for all intra LC configuration without any encoder and decoder complexity changes.

# Introduction

When SDIP is enabled, intra prediction modes currently include split flag, 2Nx2N, NxN, 2NxhN and hNx2N. According to the current SDIP anchor software, intra mode information is sent with two types by the size of CU as shown is Figure 1.

If the current CU has larger size than the minimum CU size, firstly the split flag is sent. If split flag is 1, no more mode information is sent; otherwise, a flag which indicates whether SDIP partition is used is sent; if the SDIP flag is 0, the partition type is determined to 2Nx2N and no more information is sent; otherwise a flag which indicates which partition type is used between 2NxhN and hNx2N.

If the current CU has the minimum CU size, firstly partition type flag is sent, If this flag is 0, the partition type is determined to 2Nx2N and no more information is sent; otherwise the SDIP flag is sent; if the SDIP flag is 0, the partition type is determined to Nx2N and no more information is sent; otherwise a flag which indicates which partition type is used between 2NxhN and hNx2N.

Therefore, regardless of the CU size, 3 flags are transmitted for signaling intra mode coding in SDIP. In this contribution, similar joint coding scheme and counters as used for inter mode coding was also applied to intra mode coding.

Figure.1 intra mode signaling in SDIP

# Proposed Methods

According to the proposal, split flag and SDIP flag, SDIP direction flag are grouped and coded together with a symbol intra_cu_split_pred_part_mode. Unary codes are used and mapping between symbol and codeword index is adaptive. The adaptation method is very similar as used in inter mode coding. The adaptation is applied separately at each CU depth. SDIP is enabled for 32x32, 16x16 and 8x8 CU sizes, accordingly corresponding three VLC tables are used for intra mode coding. 3 counters are used for the adaptation of each VLC table (2 for table entry counter and 1 for sum counter). Therefore 9 counters are used totally.

Initial VLC tables are as shown in Table 1 and these tables are initialized at each slice. If SDIP partition is selected from the VLC table, additional flag is transmitted so that the partition type is determined between 2NxhN and hNx2N.

Table.1 VLC table for intra mode coding

| index | mode | codeword |  | index | mode | codeword |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | SPLIT | 1 |  | 0 | 2Nx2N | 1 |
| 1 | 2Nx2N | 01 |  | 1 | NxN | 01 |
| 2 | 2NxhN | 00 |  | 2 | 2NxhN | 00 |
|  | hNx2N |  |  |  | hNx2N |  |

(a) CU size > min                        (b) CU size = min

# Experimental Results

The proposed method was implemented on the HM4 SDIP software and the performance was measured under Low Complexity intra only configuration. Common test condition is used for measuring the performance of the proposed method besides the SDIP option.

Table 2 shows the experimental results in terms of BD rate reduction. The average gain for all intra LC configuration is 0.1%. There are no noticeable encoder and decoder complexity changes.

Table.2 BD rate (%) reduction of the proposed method

|  | w/o class F |  |  | w/t class F |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra LC |  |  | All Intra LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class B | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class C | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class D | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class E | -0.2% | -0.4% | -0.3% | -0.2% | -0.4% | -0.3% |
| Class F |  |  |  | 0.0% | 0.0% | 0.0% |
| Overall | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 3 also shows the experimental results of the combination test including the proposed method and the Huawei’s SDIP chroma extension scheme [1].

Table.3 BD rate (%) reduction of the combination test for proposed method and JCTVC-G267

|  | w/o class F |  |  | w/t class F |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra LC |  |  | All Intra LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -2.1% | -1.6% | -0.1% | -2.1% | -1.6% |
| Class B | -0.1% | -2.5% | -2.5% | -0.1% | -2.5% | -2.5% |
| Class C | -0.1% | -1.0% | -1.1% | -0.1% | -1.0% | -1.1% |
| Class D | 0.0% | -1.2% | -1.0% | 0.0% | -1.2% | -1.0% |
| Class E | -0.2% | -2.6% | -2.3% | -0.2% | -2.6% | -2.3% |
| Class F |  |  |  | -0.3% | -1.8% | -1.8% |
| Overall | -0.1% | -1.9% | -1.7% | -0.2% | -1.9% | -1.7% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |

# Conclusion

In this document, VLC improvement for intra partitioning on SDIP was proposed. The proposed scheme shows 0.1% bit-rate reduction without causing any encoder or decoder complexity changes. It is recommended to adopt the proposed method into HM.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

- JCT-VC, “CE6.c Report on SDIP chroma extension scheme”, JCTVC-G267, JCT-VC Meeting, Geneva, CH, November, 2011.