| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G145 |
| --- | --- |

| Title: | Non-CE6: Reducing Line Buffers for intra mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jaehyun Lim Yongjoon Jeon Seungwook Park Byeongmoon Jeon 221, Yangjae-dong, Seoucho-gu, Seoul 137-130, Korea | Tel: Email: | jaehyun.lim@lge.com yongjoon.jeon@lge.com seungwook.park@lge.com bm.jeon@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This document presents intra mode data compression scheme for reducing intra mode line buffer size for MPM decision. Furthermore, the effect of intra mode line buffer removal is also investigated in this contribution. The simulation result reportedly shows that the line buffer compression scheme brings negligible BD rate changes for both of all intra HE and LC configurations. And the line buffer removal scheme brings 0.1% and 0.0% BD rate increase for all intra HE and LC configurations respectively. No encoder and decoder time changes are observed for both of schemes.

# Introduction

In the current HM, MPM can be determined from the intra mode of neighboring PUs as shown in Figure 1. Since LCUs are processed in a raster scan order, the intra modes of bottom 4x4 blocks of an LCU row are required to be stored in line buffers for the coding of a next LCU row and the size of the line buffers is proportional to the picture width.

Figure.1 MPM candidates

In this contribution, we propose two methods to reduce the line buffer memory size. One is an intra mode 2:1 compression method for reducing line buffer size. The other is removing whole line buffer memory for intra mode and assigning planar mode for MPM decision when the upper block belongs to the upper LCU.

# Proposed Methods

## Line buffer 2:1 compression

In order to reduce the line buffer size of intra mode data, it is not necessary to store the intra mode of every block. For each PU that needs to access the intra mode of the upper LCU row, only the compressed intra mode in the line buffer can be accessed. Figure 2 shows a proposed 2:1 intra mode compression of one line.

Figure.2 The proposed 2:1 intra mode compression of one line

For every two 4x4 blocks, the intra mode of the right 4x4 blocks are stored in the line buffer. For example, the motion data of block 1 represents the motion data of block 0 and 1. In this way, only half memory is required.

## Line buffer removal

In this part, the whole line buffer for intra mode is removed. When the current PU needs to access the intra mode of the upper LCU for MPM decision, the intra mode of above neighbor is assigned to the planar mode. Therefore any storing of intra mode in line buffer memory is not required.

In example, if above and left neighbor block has intra mode of 10 and 5 respectively, left MPM candidate is determined to 10 but above MPM candidate is determined to 0 instead of 5 when the upper block belongs to the upper LCU as shown in Figure 3.

Figure.3 An example of intra mode line buffer removal

# Experimental Results

The proposed method was implemented on the HM4 software and the performance was measured under all intra, random access and low delay B configurations (both of High Efficiency and Low Complexity). Common test condition is used for measuring the performance of the proposed method.

Table 1 shows the experimental results of intra mode 2:1 compression scheme in terms of BD rate reduction. Almost no BD rate increase is observed without any encoding and decoding time changes

Table.1 BD rate (%) reduction of the proposed intra mode 2:1 compression

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% | 0.1% |
| Class C | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | -0.1% | 0.0% | -0.4% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.1% | -0.2% | 0.0% | -0.1% | 0.1% |
| Class C | 0.0% | 0.0% | -0.1% | 0.0% | 0.3% | -0.1% |
| Class D | 0.1% | 0.1% | 0.4% | 0.0% | -0.2% | -0.5% |
| Class E | 0.1% | 0.6% | 1.3% | 0.0% | 0.3% | 0.6% |
| Overall | 0.0% | 0.1% | 0.3% | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 2 shows the experimental results of intra mode line buffer removal scheme in terms of BD rate reduction. Almost no BD rate increase is observed without any encoding and decoding time changes

Table.2 BD rate (%) reduction of the proposed intra mode line buffer removal

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class C | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class E | 0.1% | 0.1% | 0.2% | 0.1% | 0.0% | 0.1% |
| Overall | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.2% | 0.0% | 0.0% | 0.1% | -0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class D | 0.0% | 0.1% | -0.3% | 0.0% | 0.0% | 0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | -0.3% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | -0.1% | -0.1% | 0.0% | 0.2% | -0.1% |
| Class D | 0.0% | 0.2% | -0.3% | 0.0% | 0.1% | 0.2% |
| Class E | 0.0% | 0.0% | 0.6% | 0.0% | -1.1% | 0.1% |
| Overall | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Conclusion

In this document, intra mode line buffer 2:1 compression and line buffer removal scheme was proposed. Almost no bit rate increase is observed without any encoding and decoding time changes for the proposed line buffer removal scheme as well as the proposed 2:1 compression scheme. Therefore it is recommended to adopt the proposed line buffer removal for intra mode into HM to save the memory of storing intra mode for MPM decision.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Proposed change to WD text

8.3.1 Derivation process for luma intra prediction mode

Inputs to this process are:

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture,

–	a luma location ( xLC, yLC ) specifying the top-left luma sample of the current largest coding unit relative to the top-left luma sample of the current picture,

–	a variable log2PUSize specifying the size of the current prediction unit,

–	variable arrays IntraPredMode (If available) that are previously (in decoding order) derived for adjacent coding units.

Output of this process is the variable IntraPredMode[ xB ][ yB ].

…

IntraPredMode[ xB ][ yB ] is derived as the following ordered steps.

- The derivation process for neighbouring treeblocks specified in subclause XXX with ( xB,  yB ) given as input and the output is assigned to tbAddrA and tbAddrB specifying the treeblock addresses of treeblocks covering ( xBA,  yBA ) and ( xBB, yBB ) respectively where ( xBA,  yBA ) is set equal to ( xB-1,  yB ) and ( xBB,  yBB ) is set equal to ( xB,  yB-1 ).

- For N being either replaced A or B, the variables intraPredModeN are derived as follows.

- If the treeblock with address tbAddrN is not available, intraPredModeN is set equal to -1

- Otherwise, if the coding unit covering ( xBN,  yBN ) is not coded as intra mode, intraPredModeN is set equal to Intra_DC,

(Line buffer 2:1 compression case)

- Otherwise, if yB-1 is smaller than yLC and (xB%8) is equal to 0, intraPredModeA is set equal to IntraPredMode[ xBA ][ yBA ] and intraPredModeB is set equal to IntraPredMode[ xB+4 ][ yBB ]

(Line buffer removal case)

- Otherwise, if yB-1 is smaller than yLC, intraPredModeA is set equal to IntraPredMode[ xBA ][ yBA ] and intraPredModeB is set equal to Intra_Planar

- Otherwise, intraPredModeN is set equal to IntraPredMode[ xBN ][ yBN ], where IntraPredMode is the variable array assigned to the coding unit covering the luma location ( xBN, yBN ).

- For N being either replaced A or B, the variables candIntraPredModeN are derived as follows.

- If intraPredModeN is greater than or equal to intraPredModeNum

- If intraPredModeNum is equal to 4 then candIntraPredModeN is set equal to mapIntraPredMode4[ intraPredModeN ]

- Otherwise candIntraPredModeN is set equal to mapIntraPredMode10[ intraPredModeN ].

- Otherwise, candIntraPredModeN is set equal to intraPredModeN

- The candModeList[x] is derived as follows:

- If both candIntraPredModeN are not available, then the value 3 is assigned to candModeList[ 0 ] and the value 0 is assigned to candModeList[ 1 ]

- Otherwise, if only one candIntraPredModeN is available or if both candIntraPredModeN are the same, candModeList[ x ] is derived by applying the following procedure:

  - candIntraPredModeN is assigned to candModeList[ 0 ]

  - If candIntraPredModeN is not equal to 0, the value 0 is assigned to candModeList[ 1 ]

  - Otherwise, the value 3 is assigned to candModeList[ 1 ]

- Otherwise, both candIntraPredModeN are assigned to the candidate modes list with the smaller of the two candidates at candModeList[ 0 ] and the larger at candModeList[ 1 ].

- IntraPredMode[ xB ][ yB ] is derived by applying the following procedure:

- If prev_intra_pred_flag[ xB ][ yB ] is true, the IntraPredMode[ xB ][ yB ] is set equal to  candModeList[mpm_idx][ xB ][ yB ]]

- Otherwise IntraPredMode[ xB ][ yB ] is derived by applying the following ordered steps:

  - IntraPredMode[ xB ][ yB ] = rem_intra_pred_mode

  - If candModeList[ 0 ] is greater than candModeList[ 1 ], then two values are swapped. [Ed. (WJ): may be better expression needed]

  - When IntraPredMode[ xB ][ yB ] is equal or greater than candModeList[ 0 ], the value of IntraPredMode[ xB ][ yB ] is increased by one

When IntraPredMode[ xB ][ yB ] is equal or greater than candModeList[ 1 ], the value of IntraPredMode[ xB ][ yB ] is increased by one