| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G146 |
| --- | --- |

| Title: | Non-CE8: Information on ALF coefficient re-designing process |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jaehyun Lim Seungwook Park Byeongmoon Jeon 221, Yangjae-dong, Seoucho-gu, Seoul 137-130, Korea | Tel: Email: | jaehyun.lim@lge.com seungwook.park@lge.com bm.jeon@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This document presents an investigation report on complexity reduction for ALF. The purpose of the investigation is to reduce the ALF complexity by reducing the loop number of coefficient refine process. The simulation result reportedly shows that the proposed method achieves 17%/4%/3% encoding time decrease with 0.0%/0.1%/0.2% bit rate increase for all intra HE, random access HE and low delay B HE configurations respectively.

# Introduction

From the profile analysis for ALF encoding process, coefficient re-design part is determined to the most dominant part for ALF complexity. In the current HM 4, two more re-designing processes are performed after the first ALF coefficient decision. In this document, the changes in performance and encoding time are investigated by reducing this number of re-design.

# Proposed Methods

In this report, 2 cases are tested. One is reducing the number of re-design process to one, and the other is reducing the number of re-design process to zero.

# Experimental Results

The proposed method was implemented on the HM4 software and the performance was measured under three High Efficiency configurations (all intra, random access and low delay B).

Table 1 shows the experimental results of reducing the number of re-design process to one in terms of BD rate reduction. No significant performance changes are observed and also 8%/2%/1% encoding time reductions are observed in all intra HE, random access HE and low delay B HE cases respectively.

Table.1 Results in case of reducing the number of re-design to 1

|  | All Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.3% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | -0.3% | -0.4% |
| Class E | 0.0% | 0.0% | 0.0% |  |  |  | 0.1% | 0.2% | 0.6% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 92% |  |  | 98% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |

Table 2 shows the experimental results of reducing the number of re-design process to zero in terms of BD rate reduction. 0.0%/0.1%/0.2% bit rate increases are observed and 17%/4%/3% encoding time reductions are observed in all intra HE, random access HE and low delay B HE cases respectively.

Table.2 Results in case of reducing the number of re-design to 0

|  | All Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.1% | -0.1% | -0.1% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | -0.2% |
| Class C | 0.0% | 0.0% | 0.0% | 0.1% | -0.1% | 0.0% | 0.2% | -0.1% | -0.1% |
| Class D | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% | -0.2% | -0.2% |
| Class E | 0.0% | 0.0% | 0.0% |  |  |  | 0.3% | 0.3% | 0.6% |
| Overall | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% |
| Enc Time[%] | 83% |  |  | 96% |  |  | 97% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |

# Conclusion

In this document, the changes by reducing the number of ALF coefficient re-design are investigated in performance and encoding time. By reducing this number of re-design, significant encoding time decrease with the marginal loss in coding performance is observed. Therefore it is recommended to adopt the proposed reduction of numbers on ALF coefficient re-design process.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).