| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G149 |
| --- | --- |

| Title: | Options for High-Level Syntax for Multistandard Scalability |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Stephan Wenger 433 Hackensack Ave., 7th Floor Hackensack, NJ 07601 USA | Tel: Email: | +1-415-713-5473 stewe@stewe.org |
| Source: | Vidyo, Inc. |  |  |

_____________________________

# Abstract

Discussed are options for high level syntax for multistandard scalability, with a focus on the case of an H.264/AVC or SVC base layer, and one or more spatial/SNR HEVC enhancement layers, and with RTP transport/multiplexing.  A few remarks are included with respect to other standards and other transport/multiplexing schemes. It is the author’s believe that multistandard-scalability is do-able with only moderate increases in high level syntax complexity.  The document is presented for information.

# Introduction, Constraints and Limitations

Multistandard Scalability—that is, at least one layer in layer hierarchy is in compliance with a standard other than at least one other layer—has recently been subject of discussions in JCT-VC as well as assorted requirement groups.  “Scalability”, herein, is to be interpreted broadly, for example to include multiview technologies akin MVC.

The majority of this document covers multistandard scalability/multiview coding using H.264 AVC or SVC for base layer, and HEVC as an enhancement layer.  The key motivation for this choice has been the reflector discussions, which, in their vast majority, considered such a layer hierarchy.  Note that the “base layer” could actually be a hierarchy of temporal layers, and they could conform both to the non-scalable profiles of H.264/AVC as well as to SVC.

For this document, the aforementioned restriction is motivated by at least two factors.  First, H.264/AVC and HEVC share the NAL unit concept, that (as shown below) can make the integration of NAL units of the two standards into a single bitstream somewhat more straightforward than the integration of older, start code based high level syntax mechanisms.  Second, HEVC is currently “open” in the sense that, at this point, we do not need to worry about changes to non backward-compatible versions.  Further, both ISO/IEC and ITU allow for normatively referencing their own standards without trouble, facilitating the specification of multistandard scalability if so desired.

That said, let us also note this early that, while we focus on a certain layering structure, we do not want to advocate a solution that may not be future proof towards other video standards (i.e. the one after HEVC, or existing stuff such as VP8, or MPEG-2).

We further focus this document on the use of RTP as the transport/multiplexing scheme.  This choice is mostly motivated by qualification: while the authors believe to have a solid understanding what can be done using RTP (and related IETF technologies) for transport, multiplexing, and signaling, their knowledge of other relevant transport, multiplexing, and signaling standards is limited.

Section xxx contains a—perhaps overly brief—analysis of multistandard scalability outside the constraints mentioned.

# Recap: The NAL unit concept

Both H.264/AVC and HEVC use NAL units as their highest syntactical structure.  A NAL unit is a segment of coded video – often a slice – and starts with a header.  The basic H.264/AVC NAL unit header is depicted below.

+---------------+

|0|1|2|3|4|5|6|7|

+-+-+-+-+-+-+-+-+

|F|NRI|  Type   |

+---------------+

The forbidden_zero bit (F) is apparently used to prevent start code emulation in certain MPEG-2 environments.  Nal_reference_idc (NRI) was intended for the signaling of a transport priority.  From a video coding compliance viewpoint, the value 00 indicates that the NAL unit is not required for the reconstruction of reference pictures, and, therefore, could be discarded without loosing the integrity of the reference picture chain.

The NAL_unit_type indicates the type of the NAL unit.  Of the 32 possible values of this five bit field, 17 have been assigned by the 2009 version of H.264 (and its ISO/IEC counterpart 14496/10); 6 are reserved for future use by the ITU and ISO/IEC; and the remaining 9 are left “Unspecified” (which should be viewed as a codeword for “can be used by other SDOs for their purposes”).  With respect to the latter, all 8 type values are assigned by RFC 6190s and RFC6184 (which are the latest of the series of RFCs defining RTP payload formats for the H.264 “family”).

We note a number of architectural features of the NAL unit concept.

First, the NAL unit header serves as a payload header in many environments including the H.264 and SVC transports for RTP (RFC 3984/RFC, and RFC6190).  Especially for RTP environments, fixed length fields and octet aligned headers are very greatly favored.

Second, NAL unit types are semantically “above” the profiles and levels, because, in a self-contained bitstream, it is not possible to identify the profile/level without identifying a sequence parameter set through its NAL unit type.  It would be architecturally a very bad idea to assign NAL unit types (even when restricted to NAL unit types other than the sequence parameter set) through a profile/level based mechanism.

# Options for Multiplexing

There are several options for multiplexing NAL units towards a multistandard-scalable decoder, some of which are described below.

## Option 1: Use of previously unused H.264/AVC NAL unit type values for HEVC

This option has been described in JCTVC-E375.  As we are still free in defining the NAL unit types for HEVC, we could conceivably use only those six values unassigned by any version of H.264/AVC for HEVC.

This solution is hardly future-proof.  Specifically, NAL unit types are a limited resource due to the fixed length nature of the fields in H.264’s NAL unit header.  It also seriously limits the number of NAL unit types HEVC (and, potentially, its payload formats or other system layer specs relying on “unspecified” NAL unit values could conceivably use.

## Option 2: Use of ONE previously unused H.264/AVC NAL unit type, w/ indirection

This option allows us to pick only a single NAL unit type, and include something like an extension header (similar to SVC extended NAL unit types) that signals the HECV NAL unit type, as shown below.

+---------------+---------------+

|0|1|2|3|4|5|6|7|0|1|2|3|4|5|6|7|

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|F|NRI| Type=22 |open |  Type   |

+---------------+---------------+

The NAL unit header of the first octet has a fixed NAL unit type (here: 22), which would need to be specified in new revisions of H.264 and 14496-10.  Such a NAL unit header would signal the presence of another octet that could, for example, take the format as shown above.  Bits 0..2 are left open for future extensions.  The 5 bit Type field are available to signal HEVC NAL unit types in their own, separate, numbering space.

When implementing this in the most straightforward way in the HEVC spec, we incur an overhead of 1 octet for NAL unit (even for HEVC-only video) just for supporting multistandard scalability.  That does not strike us as an agreeable tradeoff.

Of course it may be possible to restrict the use of indirection only to multistandard scalability scenarios, and use a different form of NAL unit header for HEVC.  However, a bitstream containing those indirection NAL units may well not be self-contained, as we cannot use a profile/level mechanism to indicate their legality, as described above.  Finally, this option is (to a lesser extent as option 1) not very future-proof, either.  Once more the NAL unit types for HEVC would be statically assigned to a fixed length codeword.

On the upside, all standardization effort necessary to support the concept of multistandard scalability, including (hopefully) the conformance aspect, could stay within ITU and ISO/IEC.

## Option 3: External multiplexing through RTP

With “external multiplexing” we refer to technologies that involve two video bitstreams that are multiplexed and semantically linked together through a multiplexing mechanism.  Many such multiplexing mechanisms exist and support different application scenarios.  Examples include MPEG-2 systems, MPEG-4 systems, RTP, various file formats, and so on.

While relying on external multiplexing may seem awkward for people involved predominantly in the video coding world, it’s the reality in systems design.

The focus here lies on the use of RTP (RFC 3550) and its payload formats, along with signaling support using SDP syntax.  This protocol stack is commonly used in interactive video, and also in some IPTV scenarios.

There are a number of issues when using RTP to semantically tie together two RTP streams into a single layered coding media session.  The two most critical ones are discussed below.  First, the existence of such a semantic relationship needs to be signaled, and, second, NAL units in the two session need to be associated with each other.

### Signaling

In the SIP world, signaling of layered media in the sense of this document requires RFC 5583 (decoding dependency) over RFC 3388 (Media Line Grouping) over RFC 4566 (Session Description Protocol).  All three RFCs, and the payload format specifications have to be taken in combination to understand the signaling.

The Session Description Protocol (RFC 4566) is not so much as a protocol but rather a language specification designed to describe (comparatively simple) multimedia sessions. Media Line Grouping (RFC 3388) allows to semantically bind different media lines together (without clarifying the relationship of those media lines). Decoding Dependency (RFC 5583) allows to signal layered (or multiple descriptive) relationships of media lines.  The payload specific media type definitions in the RTP payload formats (for example RFC 6184 for non-scalable H.264 and RFC 6190 for SVC) include media specific definitions in SDP syntax.  The complete description can be used, for example for signaling in the context of SIP Offer Answer (RFC 3264) for video conferencing, or directly for session announcement as used in IPTV contexts.

The following is an oversimplified description how those standards fit together.

For each layer, or set of layers that are transported in a single RTP session, one SDP “media line” (m=) is required.  This m= line describes the properties of the media carried inside the RTP session.  It includes, for example, the codec type, codec specific parameters (i.e. profile, level), and a mapping to the Payload Type (PT) field in the RTP header.

Most SIP systems use an “Offer-Answer” mechanism (according to RFC 3264) to negotiate media.  An “offerer” sends potentially many m= lines with its choices of available media.  The “answerer” replies with the m= lines it can/wishes to support.  There is no limitation on how many m= lines are being sent or received.  Accordingly, in a layered system, an offerer can send one m= line for a base layer RTP session and one m= line for an enhancement layer RTP session, and an answerer can accept both, thereby indicating that it supports decoding both base and enhancement layer.

Media Line Grouping and Decoding dependency work jointly together to “bind” those (per RFC 3264 not necessarily related) m= lines together.  Grouping is used to signal that the m= lines should be negotiated as a package, and Decoding Dependency can be used to signal base and enhancement layer properties (among other things, i.e. multiple description coding is also supported).

Each m= line contains its own information about the codec being used, i.e. the standard it conforms to.  There is no restriction in the aforementioned standards on the codecs signaled by the m= lines that are being bound together.  (See recent discussions on the IETF mmusic mailing list clarifying this issue.)  Accordingly, even today it is possible to signal a multistandard layered bitstream using SDP.  Below is an example.  Two layers (L1 and L2, green), carry video according to MIME media type H264 and HEVC (blue).  (Note that the HEVC media type obviously is not yet registered, so the syntax below is currently not legal).  The final line, shown in yellow, ties together the media lines as follows:

- to decode payload type 98, you have a layered decoding relationship with layer 1, and you need either data from payload type 96 and payload type 97 (which could, for example, be two temporal layers)

- to decode payload type 99, you have a layered decoding relationship with layer 1, and you need data from payload type 97.

v=0

o=svcsrv 289083124 289083124 IN IP4 host.example.com

s=LAYERED VIDEO SIGNALING Seminar

t=0 0

c=IN IP4 192.0.2.1/127

a=group:DDP L1 L2

m=video 40000 RTP/AVP 96 97

b=AS:90

a=framerate:30

a=rtpmap:96 H264/90000

a=rtpmap:97 H264/90000

a=mid:L1

m=video 40002 RTP/AVP 98 99

b=AS:64

a=framerate:30

a=rtpmap:98 HEVC/90000

a=rtpmap:99 HEVC/90000

a=mid:L2

a=depend:98 lay L1:96,97; 99 lay L1:97

The example provided assumes the transmission of base and enhancement layer over independent transport addresses (that is: IP address and port # combinations).  This is the preferred use of RTP in the IETF.  Work is underway that allows multiplexing the different RTP sessions on the same transport address (likely through a technique known as SSRC multiplexing), but those work has not yet matured into RFCs, and is, therefore, not further described.

### Association of NAL units across RTP sessions

Inter-media synchronization between RTP sessions, in general, uses the RTP timestamp.  The timestamp is a 32 bit field in the RTP header; its clock rate is payload specific, and for video, in most cases, a 90 khz timestamp base is used.  With one (historic) exception, RTP and its payload specifications mandate the RTP timestamp to be the “capture” time.  It is commonly viewed to be a time-shifted version of what is called the “presentation timestamp” in the MPEG world.

RTP packets carrying H.264 AVC or SVC (according to RFC 6184 and RFC 6190, respectively), and presumably also according to a future HEVC payload format standard, normally contain NAL units of one coded picture identified by a single presentation timestamp.  (There is also a packetization mechanism known as MTAP that allows carrying information pertaining to more than one coded picture in a single RTP packet.  We ignore this mode, because a) the MTAP format contains information that allows reconstructing the equivalent of the RTP timestamp of all NAL units it is carrying, and b) MTAPs are not in wide use.)  Association of NAL units in RTP packets to Access Units is performed through matching RTP timestamps.

The RTP timestamp has so far been sufficient to associate NAL units (depacketized from RTP packets) with NAL units of other RTP sessions in the single standard scalable scenario.  There is no reason to believe that an identical mechanism wouldn’t work in the multistandard scalability case, as long as both base layer and enhancement layers have been coded by the same encoder.

# Other Options

We should note that what is listed here is not mutually exclusive with the options above.

## MPEG-2 transport

We don’t know enough about the scalability support of MPEG-2 transport to have an informed opinion how multistandard scalability would fit into this model.

## ISO File Format based formats

We believe that the ISO file format family is sufficiently flexible to signal a layering dependency using multiple standards with minimal additions.

## Jingle

Jingle does currently not support any layered codecs.  However, as they use RTP, the transport and mux part seems straightforward.  The video codec related signaling seems currently to be underspecified, and given the preference of the XMPP for “patent free” technologies, it is unlikely that multistandard scalability using two ITU or ISO/IEC standards will be on the top of their to-do list.

## HTTP streaming (AHS, DASH, similar proprietary protocols)

Most of these solutions stream either ISO file format chunks of data, or MPEG-2 transport streams (or both).  Above remarks apply.  In addition, the “session description” (called sometimes “manifest”) needs to be able to signal multistandard scalability.  In most of the http streaming solutions, the manifest is coded in XML, a highly flexible language.  Accordingly, while multistandard support probably needs to be specified, we are not aware of anything that would architecturally prohibit such an extension.

# Patent rights declaration(s)

Vidyo, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).