| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G150 |
| --- | --- |

| Title: | Proposed Error Pattern Files for JCT-VC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Stephan Wenger Vidyo, Inc. 433 Hackensack Ave., 7th Floor Hackensack, NJ 07601, USA | Tel: Email: | +1-415-713-5473 stewe@stewe.org |
| Source: | Vidyo, Inc. |  |  |

_____________________________

# Abstract

Proposed are error pattern files generated using the ITU-T G.1050 simulation software, with loss rates of 1%, 3%, 5%, and 10%.  Described are the setup of the simulator, the file format, and some characteristics of the patterns.

# Introduction

AHG14 identified the need for error patterns that reflect today’s IP networks (especially the Internet).  We created such patterns using the ITU-T Rec. G.1050 network simulation tool with a setup that, in our opinion, closely enough reflects the behavior of a real-world Internet “connection” with snapshots at loss rates of 1%, 3%, 5%, and 10%.  Instructions are provided that should allow a person somewhat familiar with a Linux (or Cygwin) environment to generate the patterns using the configuration files included in this document.  The author will provide the patterns on request by memory stick throughout the meeting and, if this proposal is accepted, will work with an interested ftp hoster to provide the pattern on their side.  The only reason for not including the patterns in the ZIP file is their size (several MB each).

# Simulation Parameters and Simulator Setup

G.1050 is a network simulator akin perhaps more well known tools such as ns or gns.  It operates by simulating the active and passive elements along the path of a network.  Each of these elements needs to be set up with parameters.

Interconnections between active network elements, called “wires”, include parameters such as delay, bit error rate, and capacity.  As in the real world, wires introduce (only) bit errors and delay.  In the simulations for JCT-VC, the bit error rate is set to 10E-9 and the delay to virtually zero.  Accordingly, practically no packets get lost through activities “on the wire”.

Active network elements can be the source of packet loss because of queue built-up (with pro-active flush in case the queue gets too long in the time domain) and queue overrun (due to bursty input traffic). Active network element can have many (simulated) inputs to a given queue from which it receives traffic, and only so much bandwidth on its outgoing “wire”, leading to simulated congestion and losses.  The simulation parameter handling pro-active queue flushing (in case of stale packets) is the key parameter used for adjusting the simulated network’s behavior to the target packet loss rates.  Further, there is hard queue tail-drop flushing in all active network elements at approximately 30ms built-up, which results in the simulated network in a maximum network latency of roughly 240 ms—an operation point (barely) suitable for interactive video.

In order to make up a realistic scenario, the author selected one of the 40+ pre-configured network models to roughly match the traceroute below, which happens to be the traceroute between the author’s home office and a video conference switch used by the author to telecommute to his employer.  The link bandwidths between the nodes of the traceroute were “guestimated”, except for the access link, which was set to 10 Mbit/s down and 2 Mbit/s up to reflect a common DSL/cable modem operation point.  The network model “p4u4” configured for the simulation assumes eight active network elements, which roughly match the distinct network elements located at the end of some form of long-distance pipe (or otherwise loaded) in the traceroute, as identified by the measured delay.  In the traceroute, elements marked in yellow reflect borders of administrative domains (stewe.org -> Comcast -> GlobalCrossing -> he-net), and elements marked in red reflect active elements with higher latency (presumed due to queue built-up due to limited outlink capacity).

traceroute to main.vidyo.com (64.71.164.194), 64 hops max, 52 byte packets

1  192.168.1.1 (192.168.1.1)  1.570 ms  1.172 ms  1.000 ms

2  c-24-5-176-1.hsd1.ca.comcast.net (24.5.176.1)  28.554 ms  23.308 ms  30.061 ms

3  te-7-4-ur02.sanmateo.ca.sfba.comcast.net (68.87.197.133)  9.385 ms  11.336 ms  9.589 ms

4  68.87.226.250 (68.87.226.250)  12.461 ms  13.808 ms  13.104 ms

5  pos-2-1-0-0-cr01.sacramento.ca.ibone.comcast.net (68.86.90.141)  15.012 ms  17.825 ms  14.482 ms

6  pos-0-8-0-0-cr01.sanjose.ca.ibone.comcast.net (68.86.85.78)  17.060 ms  16.729 ms  15.762 ms

7  pos-0-4-0-0-pe01.11greatoaks.ca.ibone.comcast.net (68.86.87.150)  20.735 ms  66.658 ms  19.990 ms

8  208.178.58.5 (208.178.58.5)  29.291 ms  19.298 ms  23.771 ms

9  te6-2-10g.ar4.sjc2.gblx.net (67.16.142.201)  17.607 ms

po4-20g.ar2.sjc2.gblx.net (67.16.142.214)  34.364 ms

po4-20g.ar4.sjc2.gblx.net (67.16.142.209)  19.522 ms

10  po6-20g.ar3.sjc2.gblx.net (67.17.70.253)  16.989 ms  17.258 ms  19.932 ms

11  hurrican-electric-llc.port-channel100.ar3.sjc2.gblx.net (64.214.174.246)  21.622 ms  27.396 ms  24.593 ms

12  10gigabitethernet1-2.core1.fmt2.he.net (72.52.81.177)  20.085 ms  19.153 ms  21.037 ms

13  advanced-colocation.gigabitethernet5-5.core1.fmt2.he.net (72.52.101.50)  20.556 ms  20.396 ms  18.528 ms

14  64.71.164.194 (64.71.164.194)  18.987 ms  26.511 ms  28.092 ms

Figure 1: Traceroute between Author's home network and main.vidyo.com

G.1050 includes pre-configured traffic generators.  For the video stream under test, the pre-configured generated “HD_vbr” was chosen.  This generator creates traffic at (nominally) 1.5 Mbit/s, with 1-second peaks up to 2 Mbit/s, mostly “full” (1380 bytes—close to MTU) packets and packet creation bursts that ought to reflect a fairly standard IbbbPbbbI GOP structure.   Other traffic generators used in the network model include those for VoIP-fax, http (web-surfing), Over-the-top (OTT) video in SD resolutions (TCP traffic), and Peer to peer (multiple TCP) traffic.  Traffic according to (at least) those characteristics is injected or consumed both in the sending endpoint and in active network elements of the core.

All those parameter settings primarily reflect an operation point the author is interested in: a virtual connection between a video conference endpoint receiving something like 720p60 (or perhaps 1080p30) at 1.5 mbit/s, VBR, over a limited access link to a video conferencing server located in a co-lo location (that is: not subject to any access link restrictions).  The operation point also seems suitable for a downlink of a digital TV signal, though (in this case) 1.5 Mbit/s is probably closer to 720p30.

# Error pattern generation and file format

Generated by the configuration files of this submission are four files, named 1.out, 3.out, 5.out, and 10.out, with average packet loss rates of approximately 1%, 3%, 5%, and 10%, respectively.  Further included are four parameter files 1.param, 3.param, 5.param, and 10.param, which were used (in addition to the standard installation of the G.1050 software) to generate the .out files.  In order to reproduce the results, copy the param files into the soft/tc directory, and run “runone.ss x”, where x is any of 1, 3, 5, and 10.  Doing so will result in subdirectories in the “out” directory called 1, 3, 5, and 10, which include each a file HD_vbr.out, which are the patterns for the respective error rate.

The file format of the .out file is described in G.1050, and can be summarized as follows.  Depicted below are the first 15 lines of the 1 percent pattern.

Source:,"TIA TR30.3: TIA-921B"

Creation Date:,10/22/2011

Description:, out/1/HD_vbr

Content-Encoding :, ASCII

Delay Unit:,ms

Time, Delay, Drop

0.000000000,81.918020000,0

0.030424000,79.411300000,0

0.061512000,79.411300000,0

0.077098000,79.411300000,0

0.092701000,79.411300000,0

0.123900000,79.411300000,0

0.139608000,79.411300000,0

0.155107000,79.411300000,0

0.170722000,79.411300000,0

0.186294000,79.411300000,0

0.201897000,79.411300000,0

0.217533000,79.411300000,0

Figure 2: Start of 1.out

Seven lines of headers are followed by n lines of packet information, each reflecting a single packet.

Each line contains <time> <delay> <drop>.

<time> is a float, reflecting the absolute reception (?) sending (?)  time of the packet in milliseconds.

<delay> is a float, reflecting the transmission time in milliseconds.

<drop> is a flag, when 0 the packet has arrived, when 1 the packet has been dropped.

The error rate of the error patterns are based only on the number of “dropped” packets (drop==0) and the number of lines in the file.

The number of lines in the .out files is between 160000 and 274000.  It’s currently unclear to the author why otherwise unchanged simulation parameters yield to such big differences in packets.  Future versions of this document will hopefully address this issue.

# Future Work

- Sanity-check results

- Determine suitability of results for JCT-VC, especially wrt. long error bursts

- Understand why patterns are of different lengths

# Statistics

Depicted below are the statistics of the error patterns as generated by the simulator.  The drawings should be self-explanatory.  The “pumping” effect in both bursty on non-bursty packet losses is a sign of an insufficiently well balanced network, but can quite commonly observed on the Internet as well (especially when using multi-TCP traffic such as peer-to-peer downloads), so they are of no major concern to the author.

## 1%

## 3%

## 5%

## 10%

# Patent rights declaration(s)

Vidyo, Inc.  does not have any current or pending patent rights relating to the technology described in this contribution.