| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G155 WG11 number: m21708 |
| --- | --- |

| Title: | Non-CE1: On CABAC context initialization |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chuohao Yeo Yih Han Tan Zhengguo Li 1 Fusionopolis Way #21-01 Connexis (South Tower) Singapore 138632 | Tel: Email: | +65 6408 2000 chyeo@i2r.a-star.edu.sg |
| Source: | Institute for Infocomm Research |  |  |

_____________________________

# Abstract

This contribution presents an implementation of the 8-bit CABAC context initialization proposed by HHI in JCTVC-F268. In this proposal, the initial CABAC context variable is directly computed using an 8 bit initialization value instead of first computing a PIPE index and its internal sub-state before converting that to a CABAC state. Coding results on HM-4.0 reportedly show an average Luma BD-Rate of 0.0% for each of the AI-HE, RA-HE and LB-HE configurations.

# Introduction

In HM4, when CABAC is used as the entropy coder, CABAC context variables are initialized based on the slice QP values [1]. From WD4, the following process is used to initialize the context variable to determine the CABAC state index (pStateIdx) and the MPS value (valMPS):

preCtxState = Clip3( 1, 126, ( ( m  Clip3( 0, 51, SliceQPY ) ) >> 4 ) + n )
if( preCtxState  <=  63 )  {
	pStateIdx = 63 − preCtxState
	valMPS = 0
} else {
	pStateIdx = preCtxState − 64
	valMPS = 1
}

m and n are two integer table entries determining the slope and y-intercept to be used respectively for each context variable to be initialized. There is no specification on the bitwidth needed to store m and n, but in the current HM4.0 reference software, m ranges from -60 to 75, and n ranges from -73 to 169, and 2 bytes are used for each of m and n, for a total of 4 bytes per initialization entry.

In JCTVC-F268, context initialization using 8 bit initialization values [2] was proposed. In that proposal, the slope to use is specified by a 4 bit slope index, and the state corresponding to QP=26 is specified by a 4 bit value that indicates both the PIPE index and MPS value. One of the tests in CE6 Subtest C, Test 6, checks this tool when used with CABAC [3].

However, in the implementation provided by the proponents of JCTVC-F268, when this initialization is used, the PIPE index and its internal sub-state are first computed before they are converted to an initial CABAC state. In this contribution, we present a method that uses the same concept to perform context initialization except that initial CABAC state is directly computed, and report results for an implementation based on this method.

# Proposed approach

In the proposed approach, similar to JCTVC-F268, we also use a 4 bit value to specify the slope to use via a look-up table, and a 4 bit value to specify the state corresponding to QP=26. The context initialization follows the process below:

m = tabCtxSlopes[ m_idx ] 
n  = n_idx << 3
preCtxState = Clip3( 1, 126, ( ( m  ( Clip3( 0, 51, SliceQPY ) – 26 ) + 8 ) >> 4 ) + n )
if( preCtxState  <=  63 )  {
	pStateIdx = 63 − preCtxState
	valMPS = 0
} else {
	pStateIdx = preCtxState − 64
	valMPS = 1
}

The variables m_idx ( 0 ≤ m_idx ≤ 15 ) and n_idx ( 0 ≤ n_idx ≤ 15 ) are obtained from the 8 bit initialization value as follows:

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Value | m_idx |  |  |  | n_idx |  |  |  |

The entries of tabCtxSlopes are as follows:

| m_idx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Value | -96 | -64 | -32 | -24 | -16 | -8 | -4 | 0 |
| m_idx | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| Value | 4 | 8 | 16 | 24 | 32 | 64 | 96 | 128 |

Therefore, the initial CABAC state and MPS value to use is directly computed without needing to first compute the PIPE index and its internal sub-state.

In the current implementation, the existing context initialization values are simply converted to the 8 bit values in the proposed method by quantizing the slope and quantizing the value of CABAC state at QP=26.

# Experimental results

The implementation is on HM 4.0. Tests were conducted on the High-Efficiency settings of the Intra, Random Access and Low Delay configurations as specified in the common test conditions [4]. The results are shown in the table below.

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.1% | 0.0% |
| Class C | 0.0% | 0.1% | 0.0% |
| Class D | 0.0% | 0.2% | 0.0% |
| Class E | 0.0% | -0.1% | 0.0% |
| Overall | 0.0% | 0.1% | 0.0% |
|  | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 98% |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | 0.0% | -0.2% | -0.2% |
| Class B | 0.0% | 0.0% | 0.2% |
| Class C | 0.0% | -0.1% | 0.1% |
| Class D | 0.0% | 0.2% | 0.0% |
| Class E |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 98% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | 0.0% | -0.1% |
| Class C | 0.0% | 0.2% | 0.1% |
| Class D | 0.0% | 0.5% | 0.0% |
| Class E | 0.0% | -0.2% | 0.6% |
| Overall | 0.0% | 0.1% | 0.1% |
|  | 0.0% | 0.2% | 0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 98% |  |  |

From the results, no significant coding loss is observed.

# Conclusions

In this contribution, we have presented a method for initializing CABAC context variables using 8 bit initialization values as proposed in JCTVC-F268. In our implementation, the initial CABAC context variables are directly computed. Coding results on HM4.0 shows no significant coding loss. We recommend adopting this proposal into the next HEVC test model.

# Reference

[1] B. Bross et al., “WD4: Working Draft 4 of High-Efficiency Video Coding,” JCTVC-F803, Torino, Italy, Jul. 2011.

[2] D. Marpe et al., “Unified PIPE-Based Entropy Coding for HEVC,” in JCTVC-F268, Torino, Italy, Jul. 2011.

[3] R. Joshi et al., “Description of Core Experiment 1: Entropy Coding,” JCTVC-F901, Torino, Italy, Jul. 2011.

[4] Frank Bossen, “Common test conditions and software reference configurations,” JCTVC-F900, Torino, Italy, Jul. 2011.

# Patent rights declaration(s)

Institute for Infocomm Research may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix – Proposed text

Section 9.3.1.1 is modified as follows:

#### Initialisation process for context variables

Outputs of this process are the initialised CABAC context variables indexed by ctxIdx.

Table 9-1 to Table 9-25 contain the values of the variables n_idx ( 0 ≤ n_idx ≤ 15 ) and m_idx ( -7 ≤ m_idx ≤ 8 ) used in the initialisation of context variables that are assigned to all syntax elements in subclauses Error: Reference source not found to Error: Reference source not found except for the end-of-slice flag and those related to CAVLC.

For each context variable, the two variables pStateIdx and valMPS are initialised.

NOTE 1 – The variable pStateIdx corresponds to a probability state index and the variable valMPS corresponds to the value of the most probable symbol as further described in subclause Error: Reference source not found.

Table 9-xx contains the values of the slope, ctxSlopesValTable, for various slope index m_idx.

Table 9-xx – Specification of ctxSlopesValTable[m_idx]

| m_idx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ctxSlopesVal | -96 | -64 | -32 | -24 | -16 | -8 | -4 | 0 |
| m_idx | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| ctxSlopesVal | 4 | 8 | 16 | 24 | 32 | 64 | 96 | 128 |

The two values assigned to pStateIdx and valMPS for the initialisation are derived from SliceQPY, which is derived in Equation Error: Reference source not found. Given the two table entries ( m_idx, n_idx ), the initialisation is specified by the following pseudo-code process:

m = ctxSlopesValTable[ m_idx + 7 ] 
n  = n_idx << 3
preCtxState = Clip3( 1, 126, ( ( m  ( Clip3( 0, 51, SliceQPY ) – 26 ) ) >> 4 ) + n )
if( preCtxState  <=  63 )  {
	pStateIdx = 63 − preCtxState
	valMPS = 0	(9-1)
} else {
	pStateIdx = preCtxState − 64
	valMPS = 1
}

Tables 9-21 to 9-45 are updated as follows. (note that m_idx, n_idx and values are not highlighted).

Table 9-1 – Values of variables m_idx and n_idx for alf_cu_flag ctxIdx

| Initialisation variables | alf_cu_flag ctxIdx |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| m_idx | 0 | 0 | 0 |
| n_idx | 8 | 8 | 8 |

Table 9-2 – Values of variables m_idx and n_idx for split_coding_unit_flag ctxIdx

| Initialisation variables | split_coding_unit_flag ctxIdx |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| m_idx | -2 | -2 | -2 | -3 | -2 | -2 | -3 | -2 | -2 |
| n_idx | 7 | 9 | 11 | 6 | 8 | 10 | 6 | 8 | 9 |

Table 9-3 – Values of variables m_idx and n_idx for skip_flag ctxIdx

| Initialisation variables | skip_flag ctxIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| m_idx | 0 | 0 | 0 | 0 | 0 | 0 |
| n_idx | 8 | 8 | 8 | 8 | 8 | 8 |

Table 9-4 – Values of variables m_idx and n_idx for cu_qp_delta ctxIdx

| Initialisation variables | cu_qp_delta ctxIdx |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| m_idx | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| n_idx | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 |

Table 9-5 – Values of variables m_idx and n_idx for pred_type

| Initialisation variables | pred_type ctxIdx |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| m_idx | 0 | NA | 0 | -1 | 1 | 1 | 0 | 3 | -2 | -2 |
| n_idx | 9 | NA | 8 | 7 | 11 | 7 | 7 | 9 | 7 | 7 |

Table 9-6 – Values of variable m_idx and n_idx for prev_intra_luma_pred_flag ctxIdx

| Initialisation variables | prev_intra_luma_pred_flag ctxIdx |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| m_idx | 0 | 0 | 0 |
| n_idx | 7 | 6 | 6 |

Table 9-7 – Values of variable m_idx and n_idx for rem_intra_luma_pred_mode ctxIdx

| Initialisation variables | rem_intra_luma_pred_mode ctxIdx |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| m_idx | -1 | -1 | 0 |
| n_idx | 8 | 7 | 7 |

Table 9-8 – Values of variable m_idx and n_idx for intra_chroma_pred_mode ctxIdx

| Initialisation variables | intra_chroma_pred_mode ctxIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| m_idx | 0 | 0 | 0 | 0 | 0 | 0 |
| n_idx | 8 | 8 | 8 | 8 | 8 | 8 |

Table 9-9 – Values of variable m_idx and n_idx for merge_flag ctxIdx

| Initialisation variables | merge_flag ctxIdx |  |
| --- | --- | --- |
|  | 0 | 1 |
| m_idx | 0 | 0 |
| n_idx | 8 | 8 |

Table 9-10 – Values of variable m_idx and n_idx for merge_idx ctxIdx

| Initialisation variables | merge_idx ctxIdx |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| m_idx | 0 | 0 | 0 | 0 | 0 | 1 | -2 | -1 |
| n_idx | 8 | 8 | 8 | 8 | 8 | 6 | 8 | 8 |

Table 9-11 – Values of variable m_idx and n_idx for inter_pred_flag ctxIdx

| Initialisation variables | inter_pred_flag ctxIdx |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| m_idx | -1 | -1 | -2 | 0 |
| n_idx | 7 | 8 | 9 | 8 |

Table 9-12 – Values of variable m_idx and n_idx for ref_idx_lc, ref_idx_l0, ref_idx_l1 ctxIdx

| Initialisation variables | ref_idx_lc, ref_idx_l0, ref_idx_l1 ctxIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| m_idx | -2 | -3 | 0 | -2 | -3 | -3 |
| n_idx | 6 | 9 | 8 | 5 | 8 | 3 |

Table 9-13 – Values of variable m_idx and n_idx for abs_mvd_greater0_flag and abs_mvd_greater1_flag ctxIdx

| Initialisation variables | abs_mvd_greater0_flag, abs_mvd_greater1_flag ctxIdx |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| m_idx | 0 | 0 | 0 | 0 |
| n_idx | 8 | 8 | 8 | 8 |

Table 9-14 – Values of variable m_idx and n_idx for mvp_idx_lc, mvp_idx_l0, mvp_idx_l1 ctxIdx

| Initialisation variables | mvp_idx_lc, mvp_idx_l0, mvp_idx_l1 ctxIdx |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| m_idx | 0 | 0 | 0 | 0 |
| n_idx | 8 | 8 | 8 | 8 |

Table 9-15 – Values of variable m_idx and n_idx for no_residual_data_flag ctxIdx

| Initialisation variables | no_residual_data_flag ctxIdx |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| m_idx | -4 | -3 | -3 | -4 | -5 | -4 | -4 | -4 |
| n_idx | 6 | 8 | 8 | 10 | 6 | 8 | 8 | 9 |

Table 9-16 – Values of variable m_idx and n_idx for split_transform_flag ctxIdx

| Initialisation variables | split_transform_flag ctxIdx |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| m_idx | 0 | 2 | 4 | -1 | 0 | -5 | -5 | -5 | -2 | -5 | -5 | -5 |
| n_idx | 0 | 4 | 5 | 6 | 2 | 5 | 6 | 6 | 3 | 5 | 6 | 7 |

Table 9-17 – Values of variable m_idx and n_idx for cbf_luma ctxIdx

| Initialisation variables | cbf_luma ctxIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| m_idx | -4 | -1 | -3 | -5 | -2 | -5 |
| n_idx | 10 | 8 | 9 | 7 | 8 | 4 |

Table 9-18 – Values of variable m_idx and n_idx for cbf_cb ctxIdx

| Initialisation variables | cbf_cb ctxIdx |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| m_idx | -5 | -3 | -2 | -2 | -5 | -5 | -2 | -3 | -4 | -6 | -2 | -5 |
| n_idx | 7 | 5 | 7 | 7 | 5 | 6 | 7 | 7 | 2 | 6 | 7 | 8 |

Table 9-19 – Values of variable m_idx and n_idx for cbf_cr ctxIdx

| Initialisation variables | cbf_cr ctxIdx |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| m_idx | -5 | -3 | -1 | -2 | -5 | -5 | -3 | -4 | -3 | -6 | -4 | -2 |
| n_idx | 7 | 5 | 7 | 7 | 5 | 6 | 7 | 8 | 2 | 6 | 7 | 7 |

Table 9-20 – Values of variable m_idx and n_idx for last_significant_coefficient_x and last_significant_coefficient_y ctxIdx

| Initialisation variables | last_significant_coefficient_x and last_significant_coeff_y ctxIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| m_idx | 3 | 2 | 2 | 4 | 2 | 2 | 3 | 3 | 3 | 3 | 3 | 3 | 4 | 3 | 4 |
| n_idx | 6 | 7 | 8 | 8 | 7 | 7 | 6 | 6 | 6 | 5 | 5 | 6 | 4 | 4 | 4 |
|  | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 |
| m_idx | 4 | 3 | 2 | 4 | 5 | 5 | 6 | 5 | 3 | 4 | 5 | 4 | 3 | 2 | 3 |
| n_idx | 4 | 5 | 5 | 4 | 9 | 9 | 10 | 9 | 8 | 8 | 7 | 7 | 7 | 7 | 7 |
|  | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 |
| m_idx | 2 | 2 | 2 | 3 | 4 | 4 | 4 | 3 | 4 | 5 | 5 | 4 | 3 | 4 | 5 |
| n_idx | 7 | 7 | 7 | 9 | 6 | 6 | 7 | 7 | 6 | 7 | 5 | 5 | 5 | 7 | 5 |
|  | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 |
| m_idx | 5 | 5 | 5 | 5 | 5 | 4 | 4 | 5 | 2 | 4 | 4 | -1 | 3 | 4 | 4 |
| n_idx | 5 | 4 | 3 | 3 | 3 | 9 | 9 | 10 | 9 | 8 | 8 | 10 | 7 | 7 | 6 |
|  | 60 | 61 | 62 | 63 | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 | 72 | 73 | 74 |
| m_idx | 5 | 3 | 2 | 2 | 3 | 4 | 4 | 4 | 3 | 4 | 5 | 5 | 4 | 3 | 4 |
| n_idx | 5 | 5 | 7 | 7 | 9 | 6 | 6 | 7 | 7 | 6 | 7 | 5 | 5 | 5 | 7 |
|  | 75 | 76 | 77 | 78 | 79 | 80 | 81 | 82 | 83 | 84 | 85 | 86 | 87 | 88 | 89 |
| m_idx | 5 | 5 | 5 | 5 | 5 | 5 | 4 | 4 | 5 | 2 | 4 | 4 | -1 | 3 | 4 |
| n_idx | 5 | 5 | 4 | 3 | 3 | 3 | 9 | 9 | 10 | 9 | 8 | 8 | 10 | 7 | 7 |
|  | 90 | 91 | 92 | 93 | 94 | 95 | 96 | 97 | 98 | 99 | 100 | 101 | 102 | 103 | 104 |
| m_idx | 4 | 5 | 3 |  |  |  |  |  |  |  |  |  |  |  |  |
| n_idx | 4 | 5 | 5 |  |  |  |  |  |  |  |  |  |  |  |  |

Table 9-21 – Values of variable m_idx and n_idx for significant_coeff_flag ctxIdx (A)

| Initialisation variables | significant_coeff_flag ctxIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| m_idx | -2 | -2 | -2 | -3 | -2 | -2 | -2 | -3 | -3 | -3 | -2 | -3 | -3 | -3 | -3 | -2 |
| n_idx | 11 | 10 | 9 | 8 | 9 | 9 | 9 | 9 | 9 | 9 | 9 | 9 | 8 | 8 | 9 | 10 |
|  | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| m_idx | -1 | 0 | -2 | -2 | -1 | 0 | -1 | -2 | -1 | 1 | -1 | -3 | -1 | 0 | 0 | -3 |
| n_idx | 8 | 7 | 6 | 8 | 8 | 7 | 6 | 7 | 7 | 7 | 6 | 6 | 6 | 7 | 6 | 12 |
|  | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 |
| m_idx | -3 | -3 | -1 | 0 | -1 | -2 | -3 | -1 | 0 | -1 | -2 | -2 | 5 | 3 | -2 | -6 |
| n_idx | 10 | 10 | 5 | 8 | 9 | 10 | 11 | 3 | 7 | 8 | 9 | 10 | 10 | 9 | 8 | 6 |
|  | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 |
| m_idx | 1 | 2 | 0 | -6 | -3 | -3 | 6 | 0 | -6 | -6 | 2 | 3 | 1 | -5 | -5 | -2 |
| n_idx | 8 | 8 | 8 | 7 | 8 | 8 | 8 | 7 | 5 | 6 | 7 | 9 | 7 | 7 | 2 | 7 |
|  | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 | 72 | 73 | 74 | 75 | 76 | 77 | 78 | 79 |
| m_idx | 3 | 2 | -4 | -5 | -4 | 2 | 2 | -5 | 0 | -5 | 0 | 3 | 2 | 1 | 3 | 2 |
| n_idx | 7 | 7 | 3 | 5 | 6 | 7 | 4 | 2 | 3 | 6 | 7 | 10 | 8 | 8 | 4 | 8 |
|  | 80 | 81 | 82 | 83 | 84 | 85 | 86 | 87 |  |  |  |  |  |  |  |  |
| m_idx | 2 | 1 | 2 | -2 | 1 | 2 | 2 | 3 |  |  |  |  |  |  |  |  |
| n_idx | 8 | 9 | 10 | 3 | 7 | 8 | 8 | 9 |  |  |  |  |  |  |  |  |

Table 9-22 – Values of variable m_idx and n_idx for significant_coeff_flag ctxIdx (B)

| Initialisation variables | significant_coeff_flag ctxIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| m_idx | -1 | -1 | -2 | -3 | -2 | -2 | -2 | -3 | -2 | -2 | -1 | -4 | -4 | -3 | -2 | 0 |
| n_idx | 9 | 8 | 8 | 7 | 8 | 8 | 8 | 8 | 7 | 7 | 8 | 8 | 7 | 7 | 8 | 8 |
|  | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| m_idx | 0 | -1 | -3 | -2 | -1 | 0 | -2 | -3 | -2 | 0 | -1 | -4 | -4 | -2 | -1 | 0 |
| n_idx | 7 | 6 | 5 | 6 | 6 | 6 | 5 | 6 | 6 | 6 | 6 | 6 | 6 | 7 | 7 | 10 |
|  | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 |
| m_idx | 0 | -1 | 0 | 1 | 0 | -1 | -2 | -1 | 1 | 0 | 0 | 0 | 5 | 3 | 3 | -4 |
| n_idx | 8 | 8 | 4 | 7 | 8 | 9 | 10 | 2 | 6 | 7 | 8 | 9 | 9 | 8 | 7 | 6 |
|  | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 |
| m_idx | 0 | 3 | 6 | 1 | -4 | -2 | 1 | 3 | -5 | -6 | 2 | 3 | 4 | 3 | -5 | 1 |
| n_idx | 7 | 7 | 6 | 6 | 6 | 6 | 8 | 9 | 4 | 5 | 7 | 8 | 6 | 4 | 1 | 5 |
|  | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 | 72 | 73 | 74 | 75 | 76 | 77 | 78 | 79 |
| m_idx | 4 | 4 | 5 | -5 | 5 | 4 | 2 | -6 | 3 | 3 | 3 | 4 | 3 | 2 | 4 | 2 |
| n_idx | 4 | 4 | 3 | 4 | 3 | 5 | 7 | 7 | 4 | 6 | 10 | 9 | 8 | 7 | 3 | 8 |
|  | 80 | 81 | 82 | 83 | 84 | 85 | 86 | 87 |  |  |  |  |  |  |  |  |
| m_idx | 1 | -1 | -2 | 0 | 1 | 1 | 2 | 3 |  |  |  |  |  |  |  |  |
| n_idx | 8 | 8 | 10 | 0 | 7 | 7 | 7 | 8 |  |  |  |  |  |  |  |  |

Table 9-23 – Values of variable m_idx and n_idx for significant_coeff_flag ctxIdx (C)

| Initialisation variables | significant_coeff_flag ctxIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| m_idx | -1 | -1 | -2 | -3 | -2 | -2 | -2 | -3 | -2 | -2 | -1 | -4 | -4 | -3 | -2 | 0 |
| n_idx | 9 | 8 | 8 | 7 | 8 | 8 | 8 | 8 | 7 | 7 | 8 | 8 | 7 | 7 | 8 | 8 |
|  | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| m_idx | 0 | -1 | -3 | -2 | -1 | 0 | -2 | -3 | -2 | 0 | -1 | -4 | -4 | -2 | -1 | 0 |
| n_idx | 7 | 6 | 5 | 6 | 6 | 6 | 5 | 6 | 6 | 6 | 6 | 6 | 6 | 7 | 7 | 10 |
|  | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 |
| m_idx | 0 | -1 | 0 | 1 | 0 | -1 | -2 | -1 | 1 | 0 | 0 | 0 | 5 | 3 | 3 | -4 |
| n_idx | 8 | 8 | 4 | 7 | 8 | 9 | 10 | 2 | 6 | 7 | 8 | 9 | 9 | 8 | 7 | 6 |
|  | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 |
| m_idx | 0 | 3 | 6 | 1 | -4 | -2 | 1 | 3 | -5 | -6 | 2 | 3 | 4 | 3 | -5 | 1 |
| n_idx | 7 | 7 | 6 | 6 | 6 | 6 | 8 | 9 | 4 | 5 | 7 | 8 | 6 | 4 | 1 | 5 |
|  | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 | 72 | 73 | 74 | 75 | 76 | 77 | 78 | 79 |
| m_idx | 4 | 4 | 5 | -5 | 5 | 4 | 2 | -6 | 3 | 3 | 3 | 4 | 3 | 2 | 4 | 2 |
| n_idx | 4 | 4 | 3 | 4 | 3 | 5 | 7 | 7 | 4 | 6 | 10 | 9 | 8 | 7 | 3 | 8 |
|  | 80 | 81 | 82 | 83 | 84 | 85 | 86 | 87 |  |  |  |  |  |  |  |  |
| m_idx | 1 | -1 | -2 | 0 | 1 | 1 | 2 | 3 |  |  |  |  |  |  |  |  |
| n_idx | 8 | 8 | 10 | 0 | 7 | 7 | 7 | 8 |  |  |  |  |  |  |  |  |

Table 9-24 – Values of variable m_idx and n_idx for coeff_abs_level_greater1_flag ctxIdx

| Initialisation variables | coeff_abs_level_greater1_flag ctxIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| m_idx | -2 | -3 | -2 | -2 | -2 | -1 | 0 | -1 | -2 | -1 | -2 | -2 | -1 | -1 | -1 |
| n_idx | 9 | 4 | 5 | 6 | 7 | 9 | 6 | 7 | 7 | 7 | 10 | 8 | 8 | 9 | 8 |
|  | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 |
| m_idx | -2 | -3 | -3 | -3 | -2 | -1 | -1 | -2 | -2 | -1 | -1 | -3 | -2 | -2 | -1 |
| n_idx | 7 | 2 | 4 | 4 | 5 | 7 | 5 | 6 | 6 | 6 | 9 | 8 | 8 | 8 | 7 |
|  | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 |
| m_idx | 1 | 3 | 3 | 3 | 2 | 1 | 0 | 0 | 1 | 0 | 2 | -1 | 0 | 2 | 3 |
| n_idx | 8 | 3 | 4 | 6 | 7 | 9 | 6 | 7 | 7 | 7 | 10 | 7 | 8 | 8 | 8 |
|  | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 |
| m_idx | 0 | 3 | 3 | 2 | 1 | 1 | 2 | 2 | 0 | 2 | -1 | -1 | 2 | -6 | -2 |
| n_idx | 6 | 1 | 2 | 3 | 4 | 7 | 5 | 5 | 5 | 6 | 8 | 6 | 6 | 7 | 7 |
|  | 60 | 61 | 62 | 63 | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 | 72 | 73 | 74 |
| m_idx | 0 | 2 | -1 | -1 | 0 | 0 | 1 | 0 | 1 | 1 | 0 | -1 | -1 | 0 | 2 |
| n_idx | 8 | 4 | 5 | 5 | 6 | 8 | 5 | 6 | 6 | 6 | 9 | 7 | 8 | 8 | 7 |
|  | 75 | 76 | 77 | 78 | 79 | 80 | 81 | 82 | 83 | 84 | 85 | 86 | 87 | 88 | 89 |
| m_idx | 0 | -2 | -1 | -1 | 0 | 0 | 2 | 2 | 2 | 1 | -1 | 1 | 1 | 1 | 1 |
| n_idx | 6 | 2 | 2 | 3 | 3 | 7 | 5 | 5 | 5 | 5 | 8 | 7 | 7 | 7 | 7 |
|  | 90 | 91 | 92 | 93 | 94 | 95 | 96 | 97 | 98 | 99 | 100 | 101 | 102 | 103 | 104 |
| m_idx | 2 | 4 | 3 | 3 | 3 | -5 | -5 | 3 | 2 | -1 | -3 | 2 | 2 | 3 | -3 |
| n_idx | 8 | 3 | 4 | 5 | 6 | 9 | 5 | 5 | 5 | 6 | 10 | 7 | 6 | 5 | 10 |
|  | 105 | 106 | 107 | 108 | 109 | 110 | 111 | 112 | 113 | 114 | 115 | 116 | 117 | 118 | 119 |
| m_idx | -2 | 2 | -3 | 4 | 4 | 2 | -5 | 3 | 2 | 2 | 0 | 4 | 6 | 6 | 4 |
| n_idx | 6 | 3 | 4 | 2 | 2 | 7 | 5 | 6 | 6 | 4 | 8 | 6 | 3 | 3 | 6 |
|  | 120 | 129 | 130 | 131 | 132 | 133 | 134 | 135 | 136 | 137 | 138 | 139 | 140 | 141 | 142 |
| m_idx | 0 | 2 | -1 | -1 | 0 | 0 | 1 | 0 | 1 | 1 | 0 | -1 | -1 | 0 | 2 |
| n_idx | 8 | 4 | 5 | 5 | 6 | 8 | 5 | 6 | 6 | 6 | 9 | 7 | 8 | 8 | 7 |
|  | 135 | 136 | 137 | 138 | 139 | 140 | 141 | 142 | 143 | 144 | 145 | 146 | 147 | 148 | 149 |
| m_idx | 0 | -2 | -1 | -1 | 0 | 0 | 2 | 2 | 2 | 1 | -1 | 1 | 1 | 1 | 1 |
| n_idx | 6 | 2 | 2 | 3 | 3 | 7 | 5 | 5 | 5 | 5 | 8 | 7 | 7 | 7 | 7 |
|  | 150 | 151 | 152 | 153 | 154 | 155 | 156 | 157 | 158 | 159 | 160 | 161 | 162 | 163 | 164 |
| m_idx | 2 | 4 | 3 | 3 | 3 | -5 | -5 | 3 | 2 | -1 | -3 | 2 | 2 | 3 | -3 |
| n_idx | 8 | 3 | 4 | 5 | 6 | 9 | 5 | 5 | 5 | 6 | 10 | 7 | 6 | 5 | 10 |
|  | 165 | 166 | 167 | 168 | 169 | 170 | 171 | 172 | 173 | 174 | 175 | 176 | 177 | 178 | 179 |
| m_idx | -2 | 2 | -3 | 4 | 4 | 2 | -5 | 3 | 2 | 2 | 0 | 4 | 6 | 6 | 4 |
| n_idx | 6 | 3 | 4 | 2 | 2 | 7 | 5 | 6 | 6 | 4 | 8 | 6 | 3 | 3 | 6 |

Table 9-25 – Values of variable m_idx and n_idx for coeff_abs_level_greater2_flag ctxIdx

| Initialisation variables | coeff_abs_level_greater2_flag ctxIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| m_idx | -2 | -1 | -1 | -1 | -1 | -2 | -1 | -1 | -1 | -1 | -2 | -2 | -2 | -2 | -2 |
| n_idx | 6 | 8 | 8 | 9 | 10 | 7 | 8 | 8 | 9 | 10 | 8 | 9 | 9 | 9 | 11 |
|  | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 |
| m_idx | -2 | -1 | -1 | -1 | -2 | -1 | 0 | 0 | 0 | -1 | -2 | -1 | 0 | 0 | -1 |
| n_idx | 4 | 6 | 7 | 7 | 8 | 6 | 7 | 7 | 7 | 8 | 8 | 8 | 8 | 8 | 9 |
|  | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 |
| m_idx | -1 | 0 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 0 | 1 | 1 | 0 | 0 | 1 |
| n_idx | 6 | 7 | 8 | 9 | 10 | 7 | 8 | 8 | 9 | 10 | 8 | 9 | 9 | 9 | 11 |
|  | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 |
| m_idx | -2 | 0 | 0 | 0 | -1 | 1 | 1 | 1 | 2 | 2 | 1 | -6 | 0 | -1 | -6 |
| n_idx | 4 | 5 | 6 | 7 | 7 | 5 | 7 | 7 | 7 | 7 | 7 | 8 | 7 | 8 | 9 |
|  | 60 | 61 | 62 | 63 | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 | 72 | 73 | 74 |
| m_idx | -3 | -2 | -1 | 0 | 0 | -2 | -1 | -1 | 0 | 0 | -1 | -1 | 0 | 1 | 1 |
| n_idx | 5 | 7 | 8 | 8 | 9 | 6 | 7 | 8 | 8 | 10 | 7 | 8 | 8 | 8 | 10 |
|  | 75 | 76 | 77 | 78 | 79 | 80 | 81 | 82 | 83 | 84 | 85 | 86 | 87 | 88 | 89 |
| m_idx | -2 | 0 | 0 | 0 | 0 | -1 | 0 | -1 | -2 | 0 | -1 | 1 | 0 | 0 | 0 |
| n_idx | 4 | 6 | 6 | 7 | 7 | 6 | 7 | 7 | 7 | 7 | 7 | 7 | 8 | 8 | 8 |
|  | 90 | 91 | 92 | 93 | 94 | 95 | 96 | 97 | 98 | 99 | 100 | 101 | 102 | 103 | 104 |
| m_idx | 1 | 0 | 1 | 0 | -1 | -6 | 1 | -1 | -2 | -4 | 1 | 0 | 1 | -2 | -4 |
| n_idx | 5 | 7 | 9 | 9 | 11 | 6 | 7 | 8 | 9 | 10 | 8 | 8 | 8 | 8 | 10 |
|  | 105 | 106 | 107 | 108 | 109 | 110 | 111 | 112 | 113 | 114 | 115 | 116 | 117 | 118 | 119 |
| m_idx | -6 | -5 | 3 | 5 | -5 | 3 | 1 | 5 | -3 | 2 | 6 | 0 | 0 | 6 | 0 |
| n_idx | 5 | 6 | 3 | 4 | 11 | 6 | 7 | 8 | 10 | 8 | 3 | 8 | 8 | 3 | 8 |
|  | 120 | 129 | 130 | 131 | 132 | 133 | 134 | 135 | 136 | 137 | 138 | 139 | 140 | 141 | 142 |
| m_idx | -3 | -2 | -1 | 0 | 0 | -2 | -1 | -1 | 0 | 0 | -1 | -1 | 0 | 1 | 1 |
| n_idx | 5 | 7 | 8 | 8 | 9 | 6 | 7 | 8 | 8 | 10 | 7 | 8 | 8 | 8 | 10 |
|  | 135 | 136 | 137 | 138 | 139 | 140 | 141 | 142 | 143 | 144 | 145 | 146 | 147 | 148 | 149 |
| m_idx | -2 | 0 | 0 | 0 | 0 | -1 | 0 | -1 | -2 | 0 | -1 | 1 | 0 | 0 | 0 |
| n_idx | 4 | 6 | 6 | 7 | 7 | 6 | 7 | 7 | 7 | 7 | 7 | 7 | 8 | 8 | 8 |
|  | 150 | 151 | 152 | 153 | 154 | 155 | 156 | 157 | 158 | 159 | 160 | 161 | 162 | 163 | 164 |
| m_idx | 1 | 0 | 1 | 0 | -1 | -6 | 1 | -1 | -2 | -4 | 1 | 0 | 1 | -2 | -4 |
| n_idx | 5 | 7 | 9 | 9 | 11 | 6 | 7 | 8 | 9 | 10 | 8 | 8 | 8 | 8 | 10 |
|  | 165 | 166 | 167 | 168 | 169 | 170 | 171 | 172 | 173 | 174 | 175 | 176 | 177 | 178 | 179 |
| m_idx | -6 | -5 | 3 | 5 | -5 | 3 | 1 | 5 | -3 | 2 | 6 | 0 | 0 | 6 | 0 |
| n_idx | 5 | 6 | 3 | 4 | 11 | 6 | 7 | 8 | 10 | 8 | 3 | 8 | 8 | 3 | 8 |

Notes:

- Table 9-24 should have three additional context initialization entries.

- Column 1 of Table 9-25 appears not used in the HM4.0 reference software.

- Table 9-29 is should have 1 additional context initialization entry.

- Table 9-33 have entries that do not match the values used in the HM4.0 reference software.

- Table 9-35 should have 6 additional context initialization entries.

- Tables 9-41 to 9-43 have entries that do not match the values used in the HM4.0 reference software.

- The number of context initialization entries in Tables 9-44 and 9-45 do no match the description in Table 9-20, and the values in those tables are also different from values used in HM4.0 reference software.