| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 19-30 November, 2011 | Document: JCTVC-G163 M21716 |
| --- | --- |

| Title: | Non-CE9: simplification of merge/skip TMVP ref_idx derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongjoon Jeon Seungwook Park Joonyoung Park Byeongmoon Jeon #221 Yangjae-dong, Seoucho-gu, Seoul 137-130, Korea | Tel: Email: | yongjoon.jeon@lge.com seungwook.park@lge.com jy.park@lge.com bm.jeon@lge.com |
| Source: | LG |  |  |

_____________________________

# Abstract

This contribution reports the results when the derivation process of reference indices for temporal merging candidate is simplified. Five simplifications SP01 to SP06 are tested. Simulation results revealed that no coding loss is obtained in all cases except low delay configurations of SP01. Based on test results, it is recommended to adopt one of the five simplifications SP02, SP03, SP04, SP05 and SP06 because the refIdx derivation process can be simplified without any harm on coding efficiency. Among the four recommended simplifications, SP05 and SP06 are the most preferred since they are the simplest ones.

# Introduction

In HM4.0, the reference indices for temporal merging candidate of merge/skip modes are derived as follows.

For the derivation of refIdxLXA, the following applies.

- If the prediction unit covering luma location ( xP-1, yP + nPSH − 1 ) is available and PredMode is not MODE_INTRA, refIdxLXA is assigned to refIdxLX[ xP-1, yP + nPSH − 1 ].

- Otherwise, refIdxLXA is assigned to -1.

For the derivation of refIdxLXB, the following applies.

- If the prediction unit covering luma location ( xP + nPSW − 1, yP-1 ) is available and PredMode is not MODE_INTRA, refIdxLXB is assigned to refIdxLX[ xP + nPSW − 1, yP-1 ].

- Otherwise, refIdxLXB is assigned to -1.

For the derivation of refIdxLXC, the following applies.

- If the prediction unit covering luma location ( xP+nPSW, yP-1 ) is available and PredMode is not MODE_INTRA, refIdxLXC is assigned to refIdxLX[ xP+nPSW, yP-1 ].

- Otherwise if the prediction unit covering luma location ( xP-1, yP+nPSH ) is available and PredMode is not MODE_INTRA, refIdxLXC is assigned to refIdxLX[ xP-1, yP+nPSH ].

- Otherwise if the prediction unit covering luma location ( xP-1, yP-1 ) is available and PredMode is not MODE_INTRA refIdxLXC is assigned to refIdxLX[ xP-1, yP-1 ].

- Otherwise, refIdxLXC is assigned to -1.

The variable refIdxLX is derived as follows.

- If refIdxLXA is equal to refIdxLXB and refIdxLXB is equal to refIdxLXC, the following applies.

  - If  refIdxLXA is equal to -1 then refIdxLX = 0

  - Otherwise refIdxLX = refIdxLXA

- Otherwise if refIdxLXA is equal to refIdxLXB, the following applies.

  - If  refIdxLXA is equal to -1 then refIdxLX = refIdxLXC

  - Otherwise refIdxLX = refIdxLXA

- Otherwise if refIdxLXB is equal to refIdxLXC, the following applies.

  - If  refIdxLXB is equal to -1 then refIdxLX = refIdxLXA

  - Otherwise refIdxLX = refIdxLXB

- Otherwise if refIdxLXA is equal to refIdxLXC, the following applies.

  - If  refIdxLXA is equal to -1 then refIdxLX = refIdxLXB

  - Otherwise refIdxLX = refIdxLXA

- Otherwise if refIdxLXA is equal to -1, the following applies.

  - refIdxLX = min( refIdxLXB, refIdxLXC )

- Otherwise if refIdxLXB is equal to -1, the following applies.

  - refIdxLX = Min( refIdxLXA, refIdxLXC )

- Otherwise if refIdxLXC is equal to -1, the following applies.

  - refIdxLX = Min( refIdxLXA, refIdxLXB )

- Otherwise, the following applies.

  - refIdxLX = Min( Min( refIdxLXA, refIdxLXB ), refIdxLXC )

# Proposed simplifications

The following four simplifications are proposed.

SP01. Fix to 0

- refIdxLX = 0

SP02. Min. of left and above

For the derivation of refIdxLXA, the following applies.

- If the prediction unit covering luma location ( xP-1, yP + nPSH − 1 ) is available and PredMode is not MODE_INTRA, refIdxLXA is assigned to refIdxLX[ xP-1, yP + nPSH − 1 ].

- Otherwise, refIdxLXA is assigned to -1.

For the derivation of refIdxLXB, the following applies.

- If the prediction unit covering luma location ( xP + nPSW − 1, yP-1 ) is available and PredMode is not MODE_INTRA, refIdxLXB is assigned to refIdxLX[ xP + nPSW − 1, yP-1 ].

- Otherwise, refIdxLXB is assigned to -1.

The variable refIdxLX is derived as follows.

- If refIdxLXA is equal to refIdxLXB, the following applies.

  - If  refIdxLXA is equal to -1 then refIdxLX = 0

  - Otherwise refIdxLX = refIdxLXA

- Otherwise if refIdxLXA is equal to -1, the following applies.

  - refIdxLX = refIdxLXB

- Otherwise if refIdxLXB is equal to -1, the following applies.

  - refIdxLX = refIdxLXA

- Otherwise, the following applies.

  - refIdxLX =  Min( refIdxLXA, refIdxLXB )

SP03. If low delay condition, Min. of left and above. Otherwise, fix to 0.

- If low delay condition,

  - SP02 (Min. of left and above)

- Otherwise,

  - refIdxLX = 0

* low delay condition is the same condition used in the TMVP derivation process

- If all PicOrderCnts of pictures in all reference picture lists are smaller than PicOrderCnt( currPic )

SP04. Use left as default. Use above only if left is not available.

- If refIdxLXA is not equal to -1, the following applies.

  - refIdxLX = refIdxLXA

- Otherwise if refIdxLXB is not equal to -1, the following applies.

  - refIdxLX = refIdxLXB

- Otherwise, the following applies.

  - refIdxLX = 0

SP05. Use left as default. Fix to 0 only if left is not available.

- If refIdxLXA is not equal to -1, the following applies.

  - refIdxLX = refIdxLXA

- Otherwise, the following applies.

  - refIdxLX = 0

SP06. Use left as default. Fix to 0 only if left is not available.

- If low delay condition,

  - SP05 (Left as default)

- Otherwise,

  - refIdxLX = 0

# Experimental results

For the simulations, the common test conditions and reference configurations specified in [1] are followed.

Table 1 summarizes the test results for the four simplifications. As shown in Table 1 to Table 4, there is no coding loss in all cases except low-delay configurations of SP01. In case of SP01, the loss for Y component is 0.1% for both LB-HE and LB-LC configurations. Even though the average loss is small, the loss is 0.9% and 0.8% for BQSquare sequence of LB-HE and LB-LC respectively.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class C | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 98% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.3% | 0.6% | 0.1% | 0.7% | 0.8% |
| Class C | 0.0% | 0.1% | 0.2% | 0.0% | 0.4% | 0.3% |
| Class D | 0.3% | 0.8% | 0.4% | 0.2% | 0.8% | 0.3% |
| Class E | 0.0% | 0.1% | 0.5% | 0.1% | 1.0% | 0.6% |
| Overall | 0.1% | 0.3% | 0.4% | 0.1% | 0.7% | 0.5% |
|  | 0.1% | 0.3% | 0.4% | 0.1% | 0.7% | 0.6% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 100% |  |  |

Table 1. Results for SP01 : Fix to 0.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | -0.1% | 0.0% | 0.2% | 0.2% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | -0.1% |
| Class D | 0.0% | 0.3% | 0.3% | 0.0% | 0.3% | 0.0% |
| Class E | 0.1% | 0.3% | 1.0% | 0.0% | 0.0% | -0.1% |
| Overall | 0.0% | 0.2% | 0.2% | 0.0% | 0.2% | 0.0% |
|  | 0.0% | 0.1% | 0.1% | 0.0% | 0.2% | 0.0% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 2. Results for SP02 : Min(left, above).

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.2% | -0.2% | 0.0% | -0.1% | -0.2% |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | -0.1% | 0.0% | 0.2% | 0.2% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | -0.1% |
| Class D | 0.0% | 0.3% | 0.3% | 0.0% | 0.3% | 0.0% |
| Class E | 0.1% | 0.3% | 1.0% | 0.0% | 0.0% | -0.1% |
| Overall | 0.0% | 0.2% | 0.2% | 0.0% | 0.2% | 0.0% |
|  | 0.0% | 0.1% | 0.1% | 0.0% | 0.2% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 3. Results for SP03 : If low delay condition, Min(left, above). Otherwise, fix to 0.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.2% | 0.0% | 0.0% | 0.1% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.2% | -0.1% | 0.0% | 0.1% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | -0.3% |
| Class D | 0.1% | 0.4% | 0.2% | 0.1% | -0.1% | -0.2% |
| Class E | 0.1% | 0.2% | 0.8% | 0.0% | 0.3% | -0.3% |
| Overall | 0.0% | 0.1% | 0.2% | 0.0% | 0.1% | -0.1% |
|  | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

Table 4. Results for SP04 : Use left. If not available, use above.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.1% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class D | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.1% | -0.1% | 0.0% | 0.1% | 0.0% |
| Class C | 0.0% | 0.0% | -0.1% | 0.0% | 0.3% | 0.0% |
| Class D | 0.0% | 0.4% | -0.4% | 0.0% | 0.1% | 0.1% |
| Class E | 0.1% | -0.3% | 0.7% | 0.0% | 0.4% | -0.1% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

Table 5. Results for SP05 : Use left

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U |  | Y | U |  |
| Class A | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class C | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 98% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.1% | -0.1% | 0.0% | 0.1% | 0.0% |
| Class C | 0.0% | 0.0% | -0.1% | 0.0% | 0.3% | 0.0% |
| Class D | 0.0% | 0.4% | -0.4% | 0.0% | 0.1% | 0.1% |
| Class E | 0.1% | -0.3% | 0.7% | 0.0% | 0.4% | -0.1% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

Table 6. Results for SP06 : If low delay condition, use left. Otherwise, fix to 0.

# Conclusion and recommendation

Based on the experimental results we recommend adopting one of the five simplifications (SP02, SP03, SP04, SP05 and SP06) because the process can be simplified without any harm on coding efficiency. Especially SP05 and SP06 are the most preferred since they are the simplest ones.

# References

[1]	F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-F900, Torino, IT, July 2011.

[2]	B. Bross, W.-J. Han,  J.-R. Ohm, G. J. Sullivan, T. Wiegand  “WD4: Working Draft 4 of High-Efficiency Video Coding,” JCT-VC Document, JCTVC-F803, Torino, IT, July 2011

# Patent rights declaration(s)

LG Electronics may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation |ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Required Changes in Working Draft

SP01

The following changes are made in subclause 8.4.2.1.1 Derivation process for luma motion vectors for merge mode.

…

- The derivation process for merging candidates from neighboring prediction unit partitions in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), the width and the height of the prediction unit nPSW and nPSH and the partition index PartIdx as inputs and the output is assigned to the availability flags availableFlagN, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N and the motion vectors mvL0N and mvL1N with N being replaced by A0, A1, B0, B1 or B2.

- The derivation process of reference indices for temporal merging candidate in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), nPSW, nPSH as the inputs and the output is directly assigned to refIdxLX. The reference index for temporal merging candidate refIdxLX is set equal to 0.

- The derivation process for temporal luma motion vector prediction in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), refIdxLX as the inputs and with the output being the availability flag availableFlagLXCol and the temporal motion vector mvLXCol. The variables availableFlagCol and predFlagLXCol (with X being 0 or 1, respectively) are derived as specified below.

availableFlagCol = availableFlagL0Col || availableFlagL1Col

predFlagLXCol = availableFlagLXCol

…

SP02

Subclause 8.4.2.1.3 Derivation process of reference indices for temporal merging candidates is replaced with

8.4.2.1.3 Derivation process of reference indices for temporal merging candidates

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH.

Outputs of this process are (with X being replaced by 0 or 1)

- the reference indices refIdxLX of the current prediction units.

For the derivation of refIdxLXA, the following applies.

- If the prediction unit covering luma location ( xP-1, yP + nPSH − 1 ) is available and PredMode is not MODE_INTRA, refIdxLXA is assigned to refIdxLX[ xP-1, yP + nPSH − 1 ].

- Otherwise, refIdxLXA is assigned to -1.

For the derivation of refIdxLXB, the following applies.

- If the prediction unit covering luma location ( xP + nPSW − 1, yP-1 ) is available and PredMode is not MODE_INTRA, refIdxLXB is assigned to refIdxLX[ xP + nPSW − 1, yP-1 ].

- Otherwise, refIdxLXB is assigned to -1.

The variable refIdxLX is derived as follows.

- If refIdxLXA is equal to refIdxLXB, the following applies.

  - If  refIdxLXA is equal to -1 then refIdxLX = 0

  - Otherwise refIdxLX = refIdxLXA

- Otherwise if refIdxLXA is equal to -1, the following applies.

  - refIdxLX = refIdxLXB

- Otherwise if refIdxLXB is equal to -1, the following applies.

  - refIdxLX = refIdxLXA

- Otherwise, the following applies.

  - refIdxLX =  Min( refIdxLXA, refIdxLXB )

SP03

Subclause 8.4.2.1.3 Derivation process of reference indices for temporal merging candidates is replaced with

8.4.2.1.3 Derivation process of reference indices for temporal merging candidates

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH.

Outputs of this process are (with X being replaced by 0 or 1)

- the reference indices refIdxLX of the current prediction units.

If one of PicOrderCnt of pictures in all reference picture lists are larger than PicOrderCnt( currPic ), refIdxLX is set equal to 0. Otherwise (all PicOrderCnts of pictures in all reference picture lists are smaller than PicOrderCnt( currPic ) ), the following applies.

For the derivation of refIdxLXA, the following applies.

- If the prediction unit covering luma location ( xP-1, yP + nPSH − 1 ) is available and PredMode is not MODE_INTRA, refIdxLXA is assigned to refIdxLX[ xP-1, yP + nPSH − 1 ].

- Otherwise, refIdxLXA is assigned to -1.

For the derivation of refIdxLXB, the following applies.

- If the prediction unit covering luma location ( xP + nPSW − 1, yP-1 ) is available and PredMode is not MODE_INTRA, refIdxLXB is assigned to refIdxLX[ xP + nPSW − 1, yP-1 ].

- Otherwise, refIdxLXB is assigned to -1.

The variable refIdxLX is derived as follows.

- If refIdxLXA is equal to refIdxLXB, the following applies.

  - If  refIdxLXA is equal to -1 then refIdxLX = 0

  - Otherwise refIdxLX = refIdxLXA

- Otherwise if refIdxLXA is equal to -1, the following applies.

  - refIdxLX = refIdxLXB

- Otherwise if refIdxLXB is equal to -1, the following applies.

  - refIdxLX = refIdxLXA

- Otherwise, the following applies.

  - refIdxLX =  Min( refIdxLXA, refIdxLXB )

SP04

Subclause 8.4.2.1.3 Derivation process of reference indices for temporal merging candidates is replaced with

8.4.2.1.3 Derivation process of reference indices for temporal merging candidates

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH.

Outputs of this process are (with X being replaced by 0 or 1)

- the reference indices refIdxLX of the current prediction units.

For the derivation of refIdxLXA, the following applies.

- If the prediction unit covering luma location ( xP-1, yP + nPSH − 1 ) is available and PredMode is not MODE_INTRA, refIdxLXA is assigned to refIdxLX[ xP-1, yP + nPSH − 1 ].

- Otherwise, refIdxLXA is assigned to -1.

The variable refIdxLX is derived as follows.

- If refIdxLXA is not equal to -1, the following applies.

  - refIdxLX = refIdxLXA

- Otherwise, the following applies.

  - For the derivation of refIdxLXB, the following applies.

    - If the prediction unit covering luma location ( xP + nPSW − 1, yP-1 ) is available and PredMode is not MODE_INTRA, refIdxLXB is assigned to refIdxLX[ xP + nPSW − 1, yP-1 ].

    - Otherwise, refIdxLXB is assigned to -1.

  - if refIdxLXB is not equal to -1, the following applies.

    - refIdxLX = refIdxLXB

  - Otherwise, the following applies.

    - refIdxLX = 0

SP05

- Remove the subclause “8.4.2.1.3 Derivation process of reference indices for temporal merging candidates”.

- The following changes are made in subclause “8.4.2.1.1 Derivation process for luma motion vectors for merge mode”.

…

- The derivation process for merging candidates from neighboring prediction unit partitions in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), the width and the height of the prediction unit nPSW and nPSH and the partition index PartIdx as inputs and the output is assigned to the availability flags availableFlagN, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N and the motion vectors mvL0N and mvL1N with N being replaced by A0, A1, B0, B1 or B2.

- The derivation process of reference indices for temporal merging candidate in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), nPSW, nPSH as the inputs and the output is directly assigned to refIdxLX. The reference index for temporal merging candidate refIdxLX is set equal to refIdxLX[ xP − 1, yP + nPSH − 1 ] if the prediction unit covering luma location ( xP − 1, yP + nPSH − 1 ) is available and PredMode is not MODE_INTRA. Otherwise, refIdxLX is set equal to 0.

- The derivation process for temporal luma motion vector prediction in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), refIdxLX as the inputs and with the output being the availability flag availableFlagLXCol and the temporal motion vector mvLXCol. The variables availableFlagCol and predFlagLXCol (with X being 0 or 1, respectively) are derived as specified below.

availableFlagCol = availableFlagL0Col || availableFlagL1Col

predFlagLXCol = availableFlagLXCol

…

SP06

Subclause 8.4.2.1.3 Derivation process of reference indices for temporal merging candidates is replaced with

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH.

Outputs of this process are (with X being replaced by 0 or 1)

- the reference indices refIdxLX of the current prediction units.

If one of PicOrderCnt of pictures in all reference picture lists are larger than PicOrderCnt( currPic ), refIdxLX is set equal to 0. Otherwise (all PicOrderCnts of pictures in all reference picture lists are smaller than PicOrderCnt( currPic ) ), the following applies.

For the derivation of refIdxLXA, the following applies.

- If the prediction unit covering luma location ( xP-1, yP + nPSH − 1 ) is available and PredMode is not MODE_INTRA, refIdxLXA is assigned to refIdxLX[ xP-1, yP + nPSH − 1 ].

- Otherwise, refIdxLXA is assigned to -1.

The variable refIdxLX is derived as follows.

- If refIdxLXA is not equal to -1, the following applies.

  - refIdxLX = refIdxLXA

- Otherwise, the following applies.

  - refIdxLX = 0