| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G170 |
| --- | --- |

| Title: | On issues for interlaced format support in HEVC standard |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information and proposal |  |  |
| Author(s) or Contact(s): | Keiichi Chono Hirofumi Aoki 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN | Tel: Email: | +81 50 3757 5439 chono@ct.jp.nec.com |
| Source: | [Information and Media Processing Labs, NEC Corporation] |  |  |

_____________________________

# Abstract

This contribution presents some items on interlaced video coding to be studied toward the CD of HEVC standard. It is recommended that the following are studied in an AHG on interlaced video:

- Support field_pic_flag and bottom_field_flag syntaxes;

- Keep frame_num syntax in order to identify complementary field pair;

- Study simple interlaced video coding tools including chroma MV alignment, modified down-sample scheme for intra_chromaFromLuma mode, alternate scanning, etc.

- Background

In the previous meeting, JCTVC-F194 proposed support of interlaced format in HEVC standard. The main point of the proposal was to introduce modifications mainly to high-level syntax in order to accommodate interlaced video. Through its review session it was recognized that the current requirements document does not prohibit consideration of interlaced video. This contribution presents some items on interlaced video coding to be studied toward the CD of HEVC standard.

- Issues of interlaced video coding in HEVC standard

2.1	High-level syntax for identifying complementary field pair

The H.264/MPEG-4 AVC standard supports frame_num, field_pic_flag and bottom_field_flag syntaxes that play a role of identifying complementary field pair. The current HEVC WD text does not specify the field_pic_flag and bottom_field_flag syntaxes; HM decoder does not have a means for identifying complementary field pair. Therefore it is recommended that field_pic_flag and bottom_field_flag syntaxes are introduced into high-level syntax. If JCTVC members consider modifications of coding tools for interlaced video, field_pic_flag and bottom_field_flag syntaxes can be signaled in the slice header. Otherwise, they can be signaled in a new SEI message.

2.2	Reference picture management

The H.264/MPEG-4 AVC standard specifies reference picture management for interlaced video. However, provided that a slice-level explicit signaling method for reference picture list is supported, the HEVC standard does not need to specify such a reference picture management for interlaced video. For example, consider an encoding of Pic. 4 in Figure 1. With a cost of signaling overhead bits, the slice-level explicit signaling method allows an encoder to assign reference indices 0, 1, 2, and 3 for Pic. 2, Pic. 3, Pic. 0, and Pic. 1, respectively, so that given a frame_num value, the identical reference field is prioritized than the different reference field.

Figure 1: Coding example of interlaced video.

Note that AHG21 studies a reference picture management scheme based on POC and proposes to remove frame_num syntaxes. If JCTVC members decided to adopt the AHG21’s proposal, it is recommended that frame_num is kept for identifying complementary field pair. POC is not used as the alternative of frame_num for identifying complementary field pair since POC values of fields of a complementary field pair are different.

2.3	Coding tool modifications for interlaced video

2.3.1	Prediction of interlaced video

Sample positions of chroma components relative to those of luma component are different between 4:2:0 progressive and interlaced scans. The H.264/MPEG-4 AVC standard specifies a chroma MV alignment scheme that derives chroma motion vector from luma motion vector by using the parities of the current field picture and its reference field picture. Without the chroma MV alignment, a wrong chroma motion vector is used for inter-frame prediction, and it results in visually unpleasant video quality. The chroma MV alignment should be considered for interlaced video coding in the HEVC standard.

We also need a further consideration of luma down-sampling process for the intra_chromaFromLuma mode that predicts chroma components from down-sampled version of luma component. For 4:2:0 progressive formant video, the current down-sampling process of the intra_chromaFromLuma mode produces down-sampled luma samples on the same positions as chroma samples’s. See Figure 2 (a). However, for 4:2:0 interlaced formant video, it produces down-sampled luma samples on different positions as illustrated in Figure 2 (b) and (c). Modified down-sample schemes, which generate down-sampled luma samples on appropriate positions according to the relationship between luma and chroma sample positions, should be considered for interlaced video coding in the HEVC standard.

Figure 2: Down-sampled luma positions by down-sampling process of HM.

2.3.2	Scanning of transformed coefficients

Alternate scanning of transform coefficients has been a promising technique for interlaced video. The alternate scanning is adopted in the existing standards. Therefore the alternate scanning should be considered for interlaced video coding in the HEVC standard.

# Conclusions and recommendations

This contribution has described some items on interlaced video coding. It is recommended that the following items are studied in an AHG on interlaced video toward the CD of HEVC standard:

- Support field_pic_flag and bottom_field_flag syntaxes;

- Keep frame_num syntax in order to identify complementary field pair;

- Study modifications of coding tools including chroma MV alignment, down-sampling for intra_chromaFromLuma mode, alternate scanning, etc.

# Patent rights declaration(s)

NEC Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- S. Sekiguchi et al., "Proposal on the support of interlace format in HEVC," JCTVC-F194, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, 14-22 July, 2011.