| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G172 WG11 Number: m21725 |
| --- | --- |

| Title: | CE6a: Modified down-sampling filter for LM mode of intra chroma prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Jen Chiu 2200 Mission College Blvd. Santa Clara CA USA 95054 | Tel: Email: | 408-765-9494 yi-jen.chiu@intel.com |
|  | Yu Han 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171388 yu.han@intel.com |
|  | Lidong Xu 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171425 lidong.xu@intel.com |
|  | Wenhao Zhang 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171657 wenhao.zhang@intel.com |
|  | Hong Jiang 2200 Mission College Blvd. Santa Clara CA USA 95054 | Tel: Email: | 916-356-0457 hong.h.jiang@intel.com |
| Source: | Intel Corp. |  |  |

_____________________________

# Abstract

This contribution reports the test result of the 2D modified down-sampling filters, described in CE6a of Torino meeting, for LM mode of intra chroma prediction. Compared to the default 2-tap vertical average filter of LM mode of intra chroma prediction in HM4.0 reference software, a modified 2D 3x2 {{1 2 1},{1 2 1}} down-sample filter demonstrates an average BD bitrate reductions of -0.23% on Y, -0.55% on U, -0.33% on V for AI_HE case and an average -0.20% on Y, -0.84% on U, -0.62% on V for AI_LC case, without significant impact on encoding & decoding time (0% on both Enc/Dec time for AI_HE cases, 0% on Enc time for AI_LC cases, and 1% on Dec time for AI_LC cases), on all of common tests from class A to class F. (On test classes A to E, the BD bitrate improvement of -0.15% on Y, -0.39% on U, -0.17% on V is observed for AI_HE case with up to -0.67% on Y, -1.57% on U, -1.12% on V improvement for the test clip BasketballDrill, and -0.13% on Y, -0.63% on U, -0.39% on V is observed for AI_LC case with up to -0.54% on Y, -2.95% on U, -2.22% on V improvement for the test clip BasketballDrill.) Additonal data of the combined G172 & G173 is reported to demonstrate an average BD bitrate reductions of -0.57% on Y, -1.62% on U, -2.98% on V for AI_HE case and an average -0.48% on Y, -1.58% on U, -3.87% on V for AI_LC case.

# Introduction

The LM mode of intra chroma prediction was adopted into HM reference software at Geneva meeting on March 2011[1]. In LM mode of HM 4.0, the correlation parameters α and β of LM mode are derived based on the 2-tap vertical average down-sampling filtering of the reconstructed luma pixels and on the reconstructed chroma pixels. In Torino meeting, JCTVC committee approved to pursue the core experiment CE6 on two proposals [2, 3] which demonstrated the coding efficiency improvement on 2D modified down-sampling filters, in place of the default 2-tap vertical average filter, to apply on the reconstructed luma pixels of left neighbors and of top neighbors to current PU, and to apply on the internal reconstructed luma pixels of current PU [4].

There are currently two down-sample filters of LM mode of intra chroma prediction in HM4.0 reference software as follows.

- A 3-tap horizontal filter {1 2 1} for the reconstructed luma pixels of top neighbor line to current PU.

- A 2-tap vertical filter {1 1} for the reconstructed luma pixels of left neighbor line to current PU and of the internal pixels of current PU.

This contribution reports the test result of 18 test combinations by applying different filters on three parts: the reconstructed luma pixels of top neighbor line, the reconstructed luma pixels of left neighbor line, and internal reconstructed luma pixels of current PU. Consider the modified down-sampling filters defined in CE6 document [4], the candidate test filters for the three parts are listed in Table 1. There are total 2x3x3=18 test cases. Each test case is denoted as “TiLjIk” (i=0 or 1, j=0, 1, or 2, and k=0, 1, or 2) where the test case “T0L0I0” is HM4.0 anchor.

Table 1 Candidate down-sample filters for each location

| Top neighbors | Filter T0: 121 Filter T1: 101 |
| --- | --- |
| Left neighbors | Filter L0: 11 Filter L1: 101101 Filter L2: 121121 |
| Internal pixels (Current PU) | Filter I0: 11 Filter I1: 101101 Filter I2: 121121 |

# Performances

This test code is integrated in HM4.0 and is complied in Linux OS and the tests are simulated in a compute farm consisting of hundred of CPU cores. The decoder is tested on a single workstation with the following configurations: Windows Server 2003 Enterprise x64 Edition, Intel Xeon CPU E5335 @ 2.00GHz, 8GB RAM, and Visual Studio 2008.

## Down-sample filter combination tests

Table 2 shows the summary the average BD bitrate performance of the 17 tests versus the HM4.0 anchors on the common test condition of classes A to E and of classes A to F.

Table 2 - The average BD bitrate performance of 17 tests versus HM 4.0 anchor

| Filter Comb. | Class A-E |  |  |  |  |  | Class A-F |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI_HE |  |  | AI_LC |  |  | AI_HE |  |  | AI_LC |  |  |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| T0L0I1 | -0.05% | 0.94% | 1.17% | -0.05% | 0.57% | 0.89% | 0.00% | 1.16% | 1.39% | 0.00% | 0.70% | 1.01% |
| T0L0I2 | -0.12% | -0.17% | 0.05% | -0.10% | -0.43% | -0.17% | -0.17% | -0.21% | 0.01% | -0.14% | -0.52% | -0.28% |
| T0L1I0 | 0.02% | -0.01% | -0.06% | 0.01% | -0.06% | -0.08% | 0.05% | 0.04% | 0.00% | 0.03% | 0.01% | -0.01% |
| T0L1I1 | -0.10% | 0.58% | 0.85% | -0.09% | 0.26% | 0.60% | -0.08% | 0.76% | 0.97% | -0.07% | 0.37% | 0.64% |
| T0L1I2 | -0.14% | -0.45% | -0.25% | -0.13% | -0.68% | -0.44% | -0.21% | -0.54% | -0.35% | -0.18% | -0.82% | -0.63% |
| T0L2I0 | 0.01% | -0.03% | -0.04% | 0.00% | -0.04% | -0.06% | 0.03% | -0.04% | -0.02% | 0.01% | -0.03% | -0.04% |
| T0L2I1 | -0.08% | 0.72% | 0.96% | -0.08% | 0.37% | 0.71% | -0.06% | 0.93% | 1.15% | -0.05% | 0.48% | 0.79% |
| T0L2I2 | -0.15% | -0.39% | -0.17% | -0.13% | -0.63% | -0.39% | -0.23% | -0.55% | -0.33% | -0.20% | -0.84% | -0.62% |
| T1L0I0 | 0.02% | 0.03% | 0.01% | 0.02% | 0.01% | -0.02% | 0.04% | 0.07% | 0.05% | 0.03% | 0.04% | 0.01% |
| T1L0I1 | -0.07% | 0.67% | 0.93% | -0.07% | 0.32% | 0.68% | -0.03% | 0.91% | 1.13% | -0.02% | 0.47% | 0.78% |
| T1L0I2 | -0.11% | -0.30% | -0.10% | -0.10% | -0.55% | -0.30% | -0.14% | -0.28% | -0.14% | -0.12% | -0.61% | -0.43% |
| T1L1I0 | 0.04% | 0.03% | -0.04% | 0.03% | -0.02% | -0.06% | 0.09% | 0.11% | 0.04% | 0.07% | 0.08% | 0.02% |
| T1L1I1 | -0.11% | 0.26% | 0.59% | -0.11% | -0.05% | 0.36% | -0.11% | 0.45% | 0.67% | -0.09% | 0.05% | 0.35% |
| T1L1I2 | -0.12% | -0.55% | -0.37% | -0.12% | -0.79% | -0.57% | -0.16% | -0.60% | -0.50% | -0.15% | -0.88% | -0.74% |
| T1L2I0 | 0.03% | 0.00% | -0.06% | 0.02% | -0.03% | -0.07% | 0.06% | 0.03% | -0.02% | 0.05% | 0.03% | -0.01% |
| T1L2I1 | -0.10% | 0.41% | 0.72% | -0.10% | 0.07% | 0.49% | -0.09% | 0.63% | 0.87% | -0.08% | 0.19% | 0.53% |
| T1L2I2 | -0.13% | -0.52% | -0.30% | -0.12% | -0.75% | -0.51% | -0.20% | -0.62% | -0.48% | -0.17% | -0.89% | -0.74% |

The test data of Table 2 indicates that the combination “T0L2I2” (a 2D 3x2 {{1 2 1}, {1 2 1}} down-sample filter) achieves the best coding gain. Table 3 provides more detailed data for “T0L2I2” which it achieves the BD bitrate reductions of -0.23% on Y, -0.55% on U, -0.33% on V for AI_HE case and -0.20% on Y, -0.84% on U, -0.62% on V for AI_LC case on the test classes of A to F (-0.15% on Y, -0.39% on U, -0.17% on V for AI_HE case and -0.13% on Y, -0.63% on U, -0.39% on V for AI_LC case on the test classes of A to E), without significant impact on encoding & decoding time (0% on both Enc/Dec time for AI_HE cases, 0% on Enc time for AI_LC cases, and 1% on Dec time for AI_LC cases). The detailed data of each test case can be found in the accompanied spreadsheets.

With the observation of the above 18 tests, this contribution proposes to apply the 2-D 3x2 {{1 2 1}, {1 2 1}} down-sample filter, instead of the default two-tap vertical average filter, for the reconstructed luma pixels of left neighbor line and for the internal reconstructed luma pixels of current PU. We mark that in addition to the coding gain observed in the common test conditions, this 2-D 3x2 {{1 2 1}, {1 2 1}} down-sample filter provides the identical filter shape as the 1-D horizontal down-sample filter {1 2 1} currently applying on the reconstructed luma pixels of top neighbor line.

Table 3 - The coding efficiency performance of the 2D 3x2 {{1 2 1},{1 2 1}} down-sample filter versus HM 4.0 anchor

| “T0L2I2” | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.03% | 0.58% | 1.56% | -0.06% | 0.55% | 1.50% |
| Class B | -0.07% | -0.25% | -0.21% | -0.07% | -0.47% | -0.40% |
| Class C | -0.36% | -1.21% | -1.02% | -0.28% | -1.68% | -1.53% |
| Class D | -0.25% | -0.89% | -0.76% | -0.20% | -1.25% | -1.19% |
| Class E | 0.00% | -0.17% | -0.50% | -0.02% | -0.25% | -0.32% |
| Class F | -0.65% | -1.34% | -1.13% | -0.54% | -1.91% | -1.78% |
| Overall A-E | -0.15% | -0.39% | -0.17% | -0.13% | -0.63% | -0.39% |
| Overall A-F | -0.23% | -0.55% | -0.33% | -0.20% | -0.84% | -0.62% |
| Nebuta | 0.00% | 0.69% | 2.17% | -0.04% | 1.09% | 2.54% |
| SteamLoco | 0.04% | 2.25% | 4.19% | 0.01% | 1.99% | 3.83% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

Notes: The loss of coding efficiency of class A is from the two 10bit clips “Nebuta” and “SteamLocoMotive”.

## Additional test data of the 2D 3x2 {{1 2 1}, {1 2 1}} down-sample filter on CE6a 4.1.2

Two tests of LM mode simplification techniques were defined in section 4.1.2 of CE6 document [4] as followed.

- SimplA: Sub-sampling the reference luma and chroma samples required for the derivation process of parameters αandβ.

- SimplB: Reduce the additional buffers for LM mode from 10bit to 8bit

This additional test was pursued to collect the test data of the proposed 2-D 3x2 {{1 2 1}, {1 2 1}} down-sample filter, instead of the default 2-tap vertical average filter, for the reconstructed luma pixels of left neighbor line and the internal reconstructed luma pixels of current PU with the condition of SimpA and/or SimpB techniques on CE6a 4.1.2. Tables 4-6 show the average BD bitrate performance of the tests on classes A-F. Examined with Table 2, the test indicates the similar coding gain improvement is observed with the condition of SimpA in Table 4, with the condition of SimpB in Table 5, and with the condition of SimpA and SimpB in Table 6. The detailed data of each test case can be found in the accompanied spreadsheets.

Table 4 - The performance of “T0L2I2” with the condition of “Anchor+SimpA”

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.04% | 0.75% | 1.57% | -0.05% | 0.69% | 1.62% |
| Class B | -0.07% | -0.27% | -0.23% | -0.07% | -0.50% | -0.35% |
| Class C | -0.36% | -1.21% | -1.06% | -0.29% | -1.67% | -1.52% |
| Class D | -0.25% | -0.91% | -0.85% | -0.20% | -1.23% | -1.20% |
| Class E | 0.01% | -0.25% | -0.53% | -0.04% | -0.26% | -0.38% |
| Class F | -0.66% | -1.05% | -1.12% | -0.52% | -1.92% | -1.79% |
| Overall A-E | -0.15% | -0.38% | -0.21% | -0.13% | -0.60% | -0.37% |
| Overall A-F | -0.23% | -0.49% | -0.36% | -0.19% | -0.82% | -0.60% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

Table 5 - The performance of “T0L2I2” with the condition of “Anchor+SimpB”

| "Anchor + LM T0L2I2 filter" vs. "Anchor + LM Sample bit reduction (CE6a 4.1.2.2)" |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
|  | Y |  | U | V | Y |  | U | V |
| Class A | -0.03% |  | 0.56% | 1.48% | -0.05% |  | 0.61% | 1.58% |
| Class B | -0.07% |  | -0.26% | -0.23% | -0.07% |  | -0.45% | -0.35% |
| Class C | -0.35% |  | -1.22% | -1.04% | -0.28% |  | -1.62% | -1.49% |
| Class D | -0.25% |  | -0.89% | -0.83% | -0.21% |  | -1.20% | -1.23% |
| Class E | 0.00% |  | -0.23% | -0.57% | -0.03% |  | -0.28% | -0.32% |
| Class F | -0.63% |  | -1.29% | -1.09% | -0.56% |  | -1.85% | -1.75% |
| Overall A-E | -0.14% |  | -0.41% | -0.22% | -0.13% |  | -0.60% | -0.36% |
| Overall A-F | -0.23% |  | -0.56% | -0.36% | -0.20% |  | -0.80% | -0.59% |
| Enc Time[%] |  | 100% |  |  |  | 101% |  |  |
| Dec Time[%] |  | 101% |  |  |  | 101% |  |  |

Table 6 - The performance of “T0L2I2” with the condition of “Anchor+SimpA+SimpB”

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.03% | 0.72% | 1.69% | -0.05% | 0.76% | 1.58% |
| Class B | -0.06% | -0.28% | -0.22% | -0.06% | -0.43% | -0.36% |
| Class C | -0.36% | -1.17% | -1.06% | -0.28% | -1.60% | -1.48% |
| Class D | -0.25% | -0.87% | -0.80% | -0.20% | -1.18% | -1.19% |
| Class E | 0.00% | -0.19% | -0.50% | -0.03% | -0.27% | -0.29% |
| Class F | -0.65% | -1.23% | -1.12% | -0.52% | -1.80% | -1.65% |
| Overall A-E | -0.14% | -0.36% | -0.16% | -0.13% | -0.55% | -0.35% |
| Overall A-F | -0.23% | -0.51% | -0.32% | -0.19% | -0.76% | -0.57% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

# Text modifications

To support the proposed 2-D 3x2 down-sampling filter, the following text modifications are provided for the section 8.3.3.1.8 titled “Specification of Intra_FromLuma prediction mode”:

Inputs to this process are:

……

Output of this process is:

……

This intra prediction mode is invoked when intraPredMode is equal to 35.

The values of the prediction samples predSamples[ x, y ], with x, y = 0..nS-1, are derived as the following ordered steps:

- Variable k3 and the sample array pY’ are derived as:

k3 = Max( 0, BitDepthC + log2( nS ) – 14 )

pY’[ x, y ] = ( recSamplesL[ 2x-1,2y+1 ] + 2*recSamplesL[ 2x,2y+1 ] + recSamplesL[ 2x+1,2y+1] + 2) >> 2, with x=0, ..nS-1, y = -1

pY’[ x, y ] = ( recSamplesL[ 2x-1, 2y ] + 2*recSamplesL[ 2x, 2y ] + recSamplesL[ 2x+1,2y ] + recSampleL[ 2x-1, 2y+1 ] + 2*recSamlesL[ 2x,2y+1 ] + recSamplesL[ 2x+1,2y+1 ]+4 ) >> 3,    with x=-1..nS-1,  y = 0..nS-1

# Conclusions

This contribution reports the test result of the modified 2D down-sampling filters, described in CE6a of Torino meeting, for LM mode of intra chroma prediction. Compared to the default 2-tap vertical average filter of LM mode of intra chroma prediction in HM4.0, the proposed 2D 3x2 {{1 2 1}, {1 2 1}} down-sample filter demonstrates an average BD bitrate reductions of -0.23% on Y, -0.55% on U, -0.33% on V for AI_HE case and -0.20% on Y, -0.84% on U, -0.62% on V for AI_LC case, without significant impact on encoding & decoding time (0% on both Enc/Dec time for AI_HE cases, 0% on Enc time for AI_LC cases, and 1% on Dec time for AI_LC cases), on the common test classes of A to F. This contribution proposes that this 2-D 3x2 down-sample filter {{1 2 1}, {1 2 1}}, in place of the default 2-tap vertical average filter, to be adopted in the HM reference software for further study by broader community of the video expert.

# References

- J. Chen, V. Seregin, W. Han, J. Kim, B.M. Jeon, “CE6.a.4: Chroma intra prediction by reconstructed luma samples”, JCTVC-E266, Geneva, March, 2011.

- Y.-J. Chiu, Y. Han, L. Xu, W. Zhang, H. Jiang, “Cross-channel techniques to improve intra chroma prediction”, JCTVC-F502, Torino, Italy, July 2011

- Minezawa, K. Sugimoto, S. Sekiguchi, “An improvement to chroma intra prediction from luma”, JCTVC-F173, Torino, Italy, July 2011

- A. Tabatabai, E. Francois, “CE6: Intra coding improvements”, JCTVC-F906, Torino, July, 2011

# Additional data

The below tables report the summary data of the combined G172 & G173 which demonstrates an average BD bitrate reductions of -0.57% on Y, -1.62% on U, -2.98% on V for AI_HE case and an average -0.48% on Y, -1.58% on U, -3.87% on V for AI_LC case.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| JCTVC-G172 | -0.23% | -0.55% | -0.33% | -0.20% | -0.84% | -0.62% |
| JCTVC-G173 | -0.35% | -1.11% | -2.79% | -0.29% | -0.78% | -3.35% |
| combined | -0.57% | -1.62% | -2.98% | -0.48% | -1.58% | -3.87% |

Test results of combination of JCTVC-G172 ( T0L2I2 ) and G173

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.32% | 0.52% | -0.90% | -0.25% | 0.78% | -1.41% |
| Class B | -0.16% | -0.55% | -3.34% | -0.12% | -0.07% | -3.72% |
| Class C | -0.83% | -2.96% | -3.86% | -0.71% | -3.08% | -5.30% |
| Class D | -0.73% | -2.78% | -3.58% | -0.64% | -2.77% | -5.03% |
| Class E | -0.27% | -1.21% | -3.26% | -0.19% | -1.29% | -3.66% |
| Class F | -1.12% | -2.87% | -2.92% | -0.97% | -3.37% | -4.08% |
| Overall A-E | -0.46% | -1.36% | -2.99% | -0.38% | -1.23% | -3.83% |
| Overall A-F | -0.57% | -1.62% | -2.98% | -0.48% | -1.58% | -3.87% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

# Patent rights declaration(s)

Intel Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).