| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G174 WG11 Number: m21727 |
| --- | --- |

| Title: | CE12: Deblocking filter parameter adjustment in slice level |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomoo Yamakage1 Saori Asaka1 Takeshi Chujoh1 Marta Karczewicz2 In Suk Chong2 | Email: | tomoo.yamakage@toshiba.co.jp |
| Source: | Toshiba Corporation1 and Qualcomm2 |  |  |

_____________________________

# Abstract

Appropriate parameters for deblocking filter to improve coding efficiency for CE12 are presented.  Offsets to Qp to derive beta and tc in slice level syntax are introduced in order to adjust subjective and/or objective picture quality.  The purpose of this contribution is to provide placeholder to adjust the picture quality.

When Qp offsets to derive beta and tc offsets are -2 and -5, BD-rate reduction is 0.0% (HE) and 0.4% loss (LC) on average under the common test conditions, with maximum BD-rate reduction of 0.7% (HE) and 0.3% (LC).  When higher Qp (32, 37, 42 and 47) is used, BD-rate reduction is 0.4% loss (HE) and 0.6% loss (LC) on average, with maximum BD-rate reduction of 0.6% (HE) and 0.3% (LC).

# Introduction

At the Geneva meeting in Mar. 2011, a contribution (JCTVC-E193) reported that there is a possibility to optimize deblocking filter strength.  At the Torino meeting in Jul. 2011, contributions (JCTVC-F118 and JCTVC-F143) to improve deblocking filter are introduced to WD4/HM4.

In this contribution, slice level syntax to adjust beta and tc by offsetting Qp is proposed in order to adjust subjective and/or objective picture quality.  Since the deblocking filter in WD4/HM4 is well optimized for the current design, the purpose of this contribution is to provide capability to adjust the picture quality manually.  When Qp offsets to derive beta and tc offsets are -2 and -5, BD-rate reduction is 0.0% (HE) and 0.4% loss (LC) on average under the common test conditions, with maximum BD-rate reduction of 0.7% (HE) and 0.3% (LC).  When higher Qp (32, 37, 42 and 47) is used, BD-rate reduction is 0.4% loss (HE) and 0.6% loss (LC) on average, with maximum BD-rate reduction of 0.6% (HE) and 0.3% (LC).

Results were cross-checked by Mitsubishi, and confirmed the same coding results except one test point for anchor.  This might be affected by a bug in HM4.0 (ticket #203).

# Modification points of WD4

Since WD4/HM4 do not support interface to offset beta and tc, new syntax and decoding process to support offsetting are introduced in slice level.  In addition, alpha offset should be removed since it is not described in decoding process.

There is no additional modification to WD4/HM4 other than this offsetting.  The followings are the modification points of WD4 (highlighted in yellow).

7.3.3	Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| lightweight_slice_flag | u(1) |
| ………… |  |
| if( deblocking_filter_control_present_flag ) { |  |
| disable_deblocking_filter_idc |  |
| if( disable_deblocking_filter_idc != 1 ) { |  |
| slice_alpha_c0_offset_div2 |  |
| slice_beta_offset_div2 |  |
| slice_tc_offset |  |
| } |  |
| } |  |
| } |  |

    - Slice header semantics

(Between “slice_qp_delta” and “collocated_from_l0_flag”)

slice_beta_offset specifies an offset to qPL when obtaining a beta value.

slice_tc_offset specifies an offset to qPL when obtaining a tc value.

        - Decision process for luma block edge

Replace the corresponding sentences as follows:

(WD4)

A variables β is specified as Error: Reference source not found with luma quantization parameter qPL as input.

A variable tC is specified as follows:

–	If bS is greater than 2, the variable tC is specified specified as Error: Reference source not found with luma quantization parameter Clip3(0, 55, qPL + 2 ) as input,

–	Otherwise (bS is equal or less than 2), the variable tC is specified specified as Error: Reference source not found with luma quantization parameter qPL as input.

(Proposed)

A variables β is specified as Error: Reference source not found with luma quantization parameter (qPL+ slice_beta_offset) as input.

A variable tC is specified as follows:

–	If bS is greater than 2, the variable tC is specified specified as Error: Reference source not found with luma quantization parameter Clip3(0, 55, qPL + 2 + slice_tc_offset) as input,

–	Otherwise (bS is equal or less than 2), the variable tC is specified specified as Error: Reference source not found with luma quantization parameter (qPL + slice_tc_offset) as input.

8.6.1.4.2	Filtering process for luma block edge

Replace the corresponding sentences as follows:

(WD4)

Depending on pcm_flag, a variable β is specified as follows:

–	If pcm_flag is equal to 1, the variables β is specified as Error: Reference source not found with luma quantization parameter 0 as input.

–	Otherwise, the variables β is specified as Error: Reference source not found with luma quantization parameter qPL as input.

A variable tC is specified as follows:

–	If bS is greater than 2, the variable tC is specified as Error: Reference source not found with luma quantization parameter Clip3(0, 55, qPL + 2 ) as input,

–	Otherwise (bS is equal or less than 2), the variable tC is specified as Error: Reference source not found with luma quantization parameter qPL as input.

(Proposed)

Depending on pcm_flag, a variable β is specified as follows:

–	If pcm_flag is equal to 1, the variables β is specified as Error: Reference source not found with luma quantization parameter 0 as input.

–	Otherwise, the variables β is specified as Error: Reference source not found with luma quantization parameter (qPL+ slice_beta_offset)as input.

A variable tC is specified as follows:

–	If bS is greater than 2, the variable tC is specified as Error: Reference source not found with luma quantization parameter Clip3(0, 55, qPL + 2 + slice_tc_offset) as input,

–	Otherwise (bS is equal or less than 2), the variable tC is specified as Error: Reference source not found with luma quantization parameter (qPL + slice_tc_offset)as input.

8.6.1.4.3	Filtering process for chroma block edge

Replace the corresponding sentences as follows:

(WD4)

A variable tC is specified as follows:

–	If bS is greater than 2, the variable tC is specified as Error: Reference source not found with luma quantization parameter Clip3(0, 55, qPL + 2 ) as input,

–	Otherwise (bS is equal or less than 2), the variable tC is specified as Error: Reference source not found with luma quantization parameter qPL as input.

(Proposed)

A variable tC is specified as follows:

–	If bS is greater than 2, the variable tC is specified as Error: Reference source not found with luma quantization parameter Clip3(0, 55, qPL + 2 + slice_tc_offset) as input,

- Otherwise (bS is equal or less than 2), the variable tC is specified as Error: Reference source not found with luma quantization parameter (qPL + slice_tc_offset) as input.

# Experimental results

Experiments are conducted based on the test condition JCTVC-F900 and JCTVC-F912.  Beta and tc offsets of -2 and -5 are tested in order to show the capability of adjustment.  Tables 1 and 2 show experimental results for High Efficiency and Low Complexity cases, respectively.  Results were cross-checked by Mitsubishi, and confirmed the same coding results except one test point for anchor.  This might be affected by a bug in HM4.0 (ticket #203).

BD-rate reduction is 0.0% (HE) and 0.4% loss (LC) on average under the common test conditions, with maximum BD-rate reduction of 0.7% (HE) and 0.3% (LC).

Table 1  Results for High Efficiency configuration (common test conditions)

| High Efficiency |  |  | AI | RA | LDB | LDP |
| --- | --- | --- | --- | --- | --- | --- |
| A | S01 | Traffic | -0.5% | -0.4% |  |  |
|  | S02 | PeopleOnStreet | -0.5% | 0.3% |  |  |
|  | Sxx | Nebuta | -0.3% | -0.2% |  |  |
|  | Syy | SteamLocomotive | -0.6% | -0.1% |  |  |
| B | S03 | Kimono | -0.4% | 0.2% | 0.4% | 0.4% |
|  | S04 | ParkScene | -0.7% | -0.3% | -0.3% | -0.1% |
|  | S05 | Cactus | -0.4% | 0.0% | 0.1% | 0.2% |
|  | S06 | BasketballDrive | -0.4% | 0.3% | 0.3% | 0.3% |
|  | S07 | BQTerrace | -0.3% | -0.4% | -0.4% | -0.2% |
| C | S08 | BasketballDrill | -0.4% | 0.1% | 0.2% | -0.2% |
|  | S09 | BQMall | -0.2% | 0.2% | 0.1% | 0.4% |
|  | S10 | PartyScene | -0.1% | 0.0% | -0.1% | 0.0% |
|  | S11 | RaceHorses | -0.2% | 0.3% | 0.4% | 0.5% |
| D | S12 | BasketballPass | -0.2% | 0.6% | 0.5% | 0.5% |
|  | S13 | BQSquare | -0.1% | 0.0% | -0.2% | -0.2% |
|  | S14 | BlowingBubbles | -0.2% | 0.0% | -0.1% | -0.1% |
|  | S15 | RaceHorses | -0.3% | 0.4% | 0.5% | 0.5% |
| E | S16 | Vidyo1 | -0.3% |  | 0.1% | 0.1% |
|  | S17 | Vidyo3 | -0.3% |  | -0.1% | 0.1% |
|  | S18 | Vidyo4 | -0.4% |  | -0.1% | 0.1% |
| Class A |  |  | -0.5% | -0.1% |  |  |
| Class B |  |  | -0.4% | 0.0% | 0.0% | 0.1% |
| Class C |  |  | -0.2% | 0.1% | 0.1% | 0.1% |
| Class D |  |  | -0.2% | 0.3% | 0.2% | 0.2% |
| Class E |  |  | -0.3% |  | 0.0% | 0.1% |
| Total |  |  | -0.3% | 0.1% | 0.1% | 0.1% |
| Encoding time |  |  | 100% | 100% | 100% | 100% |
| Decoding time |  |  | 100% | 100% | 100% | 100% |

Table 2  Results for Low Complexity configuration (common test conditions)

| Low Complexity |  |  | AI | RA | LDB | LDP |
| --- | --- | --- | --- | --- | --- | --- |
| A | S01 | Traffic | 0.2% | 0.1% |  |  |
|  | S02 | PeopleOnStreet | 0.2% | 1.2% |  |  |
|  | Sxx | Nebuta | 0.3% | 0.5% |  |  |
|  | Syy | SteamLocomotive | 0.0% | 0.7% |  |  |
| B | S03 | Kimono | 0.5% | 1.0% | 1.3% | 2.3% |
|  | S04 | ParkScene | -0.1% | 0.0% | 0.0% | 0.4% |
|  | S05 | Cactus | -0.1% | 0.4% | 0.4% | 0.8% |
|  | S06 | BasketballDrive | -0.1% | 0.6% | 0.5% | 1.1% |
|  | S07 | BQTerrace | -0.2% | -0.1% | -0.2% | 0.5% |
| C | S08 | BasketballDrill | -0.3% | 0.0% | -0.1% | 0.2% |
|  | S09 | BQMall | 0.0% | 0.3% | 0.3% | 0.7% |
|  | S10 | PartyScene | -0.1% | 0.0% | -0.1% | 0.0% |
|  | S11 | RaceHorses | 0.0% | 0.6% | 0.6% | 0.8% |
| D | S12 | BasketballPass | -0.1% | 0.4% | 0.6% | 0.7% |
|  | S13 | BQSquare | -0.1% | 0.0% | -0.3% | -0.1% |
|  | S14 | BlowingBubbles | -0.1% | 0.1% | 0.0% | 0.1% |
|  | S15 | RaceHorses | -0.1% | 0.5% | 0.6% | 0.7% |
| E | S16 | Vidyo1 | 0.2% |  | 0.3% | 1.3% |
|  | S17 | Vidyo3 | 0.3% |  | 0.5% | 1.3% |
|  | S18 | Vidyo4 | 0.1% |  | 0.3% | 0.7% |
| Class A |  |  | 0.2% | 0.6% |  |  |
| Class B |  |  | 0.0% | 0.4% | 0.4% | 1.0% |
| Class C |  |  | -0.1% | 0.2% | 0.2% | 0.4% |
| Class D |  |  | -0.1% | 0.2% | 0.3% | 0.3% |
| Class E |  |  | 0.2% |  | 0.4% | 1.1% |
| Total |  |  | 0.0% | -0.5% | 0.3% | 0.7% |
| Encoding time |  |  | 100% | 100% | 100% | 100% |
| Decoding time |  |  | 100% | 101% | 101% | 101% |

When higher Qp (32, 37, 42 and 47) is used, BD-rate reduction is 0.4% loss (HE) and 0.6% loss (LC) on average, with maximum BD-rate reduction of 0.6% (HE) and 0.3% (LC).

Table 3  Results for High Efficiency configuration (Qp = 32, 37, 42 and 47)

| High Efficiency |  |  | AI | RA | LDB | LDP |
| --- | --- | --- | --- | --- | --- | --- |
| A | S01 | Traffic | -0.4% | -0.2% |  |  |
|  | S02 | PeopleOnStreet | -0.3% | 0.5% |  |  |
|  | Sxx | Nebuta | -0.5% | -0.2% |  |  |
|  | Syy | SteamLocomotive | -0.4% | 0.3% |  |  |
| B | S03 | Kimono | 0.0% | 0.2% | 0.6% | 0.4% |
|  | S04 | ParkScene | -0.6% | -0.2% | 0.3% | 0.3% |
|  | S05 | Cactus | -0.3% | 0.2% | 0.4% | 0.5% |
|  | S06 | BasketballDrive | 0.0% | 0.5% | 0.8% | 0.6% |
|  | S07 | BQTerrace | -0.2% | -0.2% | 0.0% | -0.2% |
| C | S08 | BasketballDrill | -0.1% | 0.7% | 1.2% | 0.7% |
|  | S09 | BQMall | -0.2% | 0.5% | 0.5% | 0.6% |
|  | S10 | PartyScene | -0.4% | 0.0% | 0.0% | 0.2% |
|  | S11 | RaceHorses | -0.2% | 0.7% | 0.9% | 0.9% |
| D | S12 | BasketballPass | 0.1% | 1.3% | 1.3% | 1.3% |
|  | S13 | BQSquare | 0.0% | 1.0% | 0.8% | 0.4% |
|  | S14 | BlowingBubbles | -0.3% | 0.7% | 0.8% | 0.3% |
|  | S15 | RaceHorses | -0.2% | 0.9% | 1.4% | 1.1% |
| E | S16 | Vidyo1 | -0.1% |  | 1.0% | 0.8% |
|  | S17 | Vidyo3 | -0.3% |  | 0.8% | 0.7% |
|  | S18 | Vidyo4 | -0.2% |  | 0.9% | 0.8% |
| Class A |  |  | -0.4% | 0.1% |  |  |
| Class B |  |  | -0.2% | 0.1% | 0.4% | 0.3% |
| Class C |  |  | -0.2% | 0.5% | 0.6% | 0.6% |
| Class D |  |  | -0.1% | 1.0% | 1.1% | 0.8% |
| Class E |  |  | -0.2% |  | 0.9% | 0.8% |
| Total |  |  | -0.2% | 0.4% | 0.7% | 0.6% |
| Encoding time |  |  | 101% | 100% | 100% | 100% |
| Decoding time |  |  | 100% | 100% | 101% | 100% |

Table 4  Results for Low Complexity configuration (Qp = 32, 37, 42 and 47)

| Low Complexity |  |  | AI | RA | LDB | LDP |
| --- | --- | --- | --- | --- | --- | --- |
| A | S01 | Traffic | 0.4% | 0.2% |  |  |
|  | S02 | PeopleOnStreet | 0.7% | 1.4% |  |  |
|  | Sxx | Nebuta | 0.2% | 0.8% |  |  |
|  | Syy | SteamLocomotive | 0.3% | 0.4% |  |  |
| B | S03 | Kimono | 0.7% | 0.7% | 1.0% | 1.7% |
|  | S04 | ParkScene | 0.1% | 0.1% | 0.2% | 0.8% |
|  | S05 | Cactus | 0.2% | 0.4% | 0.4% | 0.8% |
|  | S06 | BasketballDrive | 0.2% | 0.6% | 0.7% | 1.1% |
|  | S07 | BQTerrace | 0.1% | 0.0% | 0.0% | 0.5% |
| C | S08 | BasketballDrill | 0.1% | 0.6% | 0.9% | 1.1% |
|  | S09 | BQMall | 0.2% | 0.5% | 0.7% | 1.0% |
|  | S10 | PartyScene | -0.3% | 0.0% | 0.0% | 0.1% |
|  | S11 | RaceHorses | 0.0% | 0.7% | 0.8% | 1.1% |
| D | S12 | BasketballPass | 0.2% | 1.2% | 1.0% | 1.5% |
|  | S13 | BQSquare | 0.0% | 0.5% | 0.1% | 0.6% |
|  | S14 | BlowingBubbles | -0.1% | 0.5% | 0.7% | 0.9% |
|  | S15 | RaceHorses | 0.1% | 1.0% | 0.9% | 1.2% |
| E | S16 | Vidyo1 | 0.3% |  | 0.5% | 1.4% |
|  | S17 | Vidyo3 | 0.3% |  | 0.8% | 1.1% |
|  | S18 | Vidyo4 | 0.3% |  | 0.6% | 1.0% |
| Class A |  |  | 0.4% | 0.7% |  |  |
| Class B |  |  | 0.3% | 0.4% | 0.5% | 1.0% |
| Class C |  |  | 0.0% | 0.5% | 0.6% | 0.8% |
| Class D |  |  | 0.0% | 0.8% | 0.7% | 1.0% |
| Class E |  |  | 0.3% |  | 0.6% | 1.2% |
| Total |  |  | 0.2% | 0.6% | 0.6% | 1.0% |
| Encoding time |  |  | 100% | 100% | 100% | 100% |
| Decoding time |  |  | 101% | 102% | 102% | 101% |

# Conclusion

In this contribution, slice level syntax to adjust beta and tc is proposed in order to adjust subjective and/or objective picture quality.  Since the deblocking filter in WD4/HM4 is well optimized for the current design, the purpose of this contribution is to provide placeholder to adjust the picture quality.

When Qp offsets to derive beta and tc offsets are -2 and -5, BD-rate reduction is 0.0% (HE) and 0.4% loss (LC) on average under the common test conditions, with maximum BD-rate reduction of 0.7% (HE) and 0.3% (LC).  When higher Qp (32, 37, 42 and 47) is used, BD-rate reduction is 0.4% loss (HE) and 0.6% loss (LC) on average, with maximum BD-rate reduction of 0.6% (HE) and 0.3% (LC).

# Patent rights declaration(s)

Toshiba Corporation and Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).