| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G181 |
| --- | --- |

| Title: | Non-CE9: Merging candidate reordering |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hideki Takehara Shigeru Fukushima JVC KENWOOD Corporation 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan | Tel: Email: | +81-46-836-5729 takehara.hideki@ jvckenwood.com fukushima.shigeru@ jvckenwood.com |
| Source: | JVC KENWOOD Corporation |  |  |

_____________________________

# Abstract

In the HM4.0, motion information of the merging candidate that is located inside of the CU is not to be used

In this proposal, instead of avoiding such merging candidate, the merging candidate is moved to the last position in the merge list after the derivation process for merging candidates.

The simulation results report that the proposed technique provides 0.1% BD-rate gain for random access and 0.1%/0.2% gain for low delay B setting.

# Introduction

Merge is one of the techniques to reduce motion information. In the HM4.0 [1], motion information of the merging candidate that is located inside of the CU is not to be used. This is for reducing motion information by avoiding useless candidate.

# Algorithm Description

## Current algorithm (HM4.0)

Figure 1 illustrates the avoided candidate position in the HM4.0. In the merge list construction, merging candidate with imitating 2Nx2N is avoided. To avoid such merging candidate, motion information of all spatial merging candidates need to be compared with motion information to be avoided.

Figure 1 Avoid merging candidate position in the HM4.0

## Proposed algorithm

On the availability checking for merging candidates, a merging candidate with imitating 2Nx2N is not avoided in this proposal. All merging candidates can be compared with the candidate that has motion information with imitating 2Nx2N since the pruning process is invoked with the candidate.

Instead of avoiding such merging candidate, the merging candidate is moved to the last position in the merge list after the derivation process for merging candidates.

Figure 2 illustrates the reordering of the merging candidate.

Figure 2 Merging candidate reordering

## Implementation

### Software

This proposed technique is implemented into HM4.0 software.

### Syntax structure

The syntax of this proposed technique is not revised from HM4.0.

# Experiments

## Test conditions

The common test conditions [2] are used.

## Experimental environment

The experimental environment used is presented in Table1.

Table 1 Experimental environment

| CPU | Intel Core i7-2600K 3.40GHz |
| --- | --- |
| Memory | 16GB (DDR3) |
| OS | Windows 7 Professional 64bit |
| Compiler | Microsoft Visual C++ 2008 Express edition SP1 |
| Executable | x64 |

Notes: The decoding time is measured with the writing of yuv file.

## Simulation results

Error: Reference source not found and Error: Reference source not found present the simulation results for random access, and low delay, respectively.

Table 2 Simulation results for random access

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.3% | -0.3% | -0.1% | 0.0% | -0.1% |
| Class B | -0.1% | 0.0% | -0.1% | -0.1% | -0.1% | 0.0% |
| Class C | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% |
| Class D | 0.0% | 0.0% | -0.1% | -0.1% | -0.2% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
|  | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

Table 3 Simulation results for low delay B

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -0.3% | -0.4% | -0.2% | -0.1% | -0.1% |
| Class C | -0.2% | -0.3% | 0.0% | -0.1% | -0.1% | -0.3% |
| Class D | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| Class E | 0.0% | 0.1% | 0.1% | -0.2% | 0.4% | -0.3% |
| Overall | -0.1% | -0.1% | -0.1% | -0.2% | 0.0% | -0.1% |
|  | -0.1% | -0.1% | -0.1% | -0.2% | 0.0% | -0.1% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

The simulation results report that the proposed technique provides 0.1% BD-rate [3] gain for random access and 0.1%/0.2% gain for low delay B setting. (See the attached excel sheet)

# Conclusions

This contribution reports the proposed technique can bring coding gain with reducing decoding complexity.

Therefore, we suggest that the proposed technique be adopted to the next WD and HM.

# References

[1] “WD4: Working Draft 4 of High-Efficiency Video Coding”,

JCTVC-F803, Torino, July 2011

[2] “Common test conditions and software reference configurations”,

JCTVC-F900, Torino, July 2011

[3] “Improvements of the BD-PSNR model,” ITU-T SG16 Q.6 Document,

VCEG-AI11, Berlin, July 2008.

# Patent rights declaration(s)

JVC KENWOOD Holdings, Inc. or JVC KENWOOD Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# WD changes

8.4.2.1.1  Derivation process for luma motion vectors for merge mode

This process is only invoked when PredMode is equal to MODE_SKIP or PredMode is equal to MODE_ INTER and merge_flag [ xP ][ yP ] is equal to 1, where ( xP, yP ) specify the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture.

Inputs of this process are

a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

–	the luma motion vectors mvL0 and mvL1,

–	the reference indices refIdxL0 and refIdxL1,

–	the prediction list utilization flags predFlagL0 and predFlagL1.

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1 are derived as specified by the following ordered steps:

The derivation process for merging candidates from neighboring prediction unit partitions in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), the width and the height of the prediction unit nPSW and nPSH and the partition index PartIdx as inputs and the output is assigned to the availability flags availableFlagN, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N and the motion vectors mvL0N and mvL1N with N being replaced by A0, A1, B0, B1 or B2.

The derivation process of reference indices for temporal merging candidate in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), nPSW, nPSH as the inputs and the output is directly assigned to refIdxLX.

The derivation process for temporal luma motion vector prediction in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), refIdxLX as the inputs and with the output being the availability flag availableFlagLXCol and the temporal motion vector mvLXCol. The variables availableFlagCol and predFlagLXCol (with X being 0 or 1, respectively) are derived as specified below.

availableFlagCol = availableFlagL0Col || availableFlagL1Col	(8-1)

predFlagLXCol = availableFlagLXCol	(8-2)

The merging candidate list, mergeCandList, is constructed as follows.

A1, if availableFlagA1 is equal to 1

B1, if availableFlagB1 is equal to 1

B0, if availableFlagB0 is equal to 1

A0, if availableFlagA0 is equal to 1

B2, if availableFlagB2 is equal to 1

Col, if availableFlagCol is equal to 1

When merging candidates have the same motion vectors and the same reference indices, the merging candidates are removed from the list except the merging candidate which has the smallest order in the mergeCandList.

The variable numMergeCand and numOrigMergeCand are set to the number of merging candidates in the mergeCandList and the variable maxNumMergeCand is set to 5.

When slice_type is equal to B, the following applies.

The derivation process for combined bi-predictive merging candidates specified in subclause Error: Reference source not found is invoked with mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList, numMergeCand, numOrigMergeCand and maxNumMergeCand given as input and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0combCandk and refIdxL1combCandk, the prediction list utilization flags predFlagL0combCandk and predFlagL1combCandk and the motion vectors mvL0combCandk and mvL1combCandk of every new candidate combCandk being added in mergeCandList. The number of candidates being added numCombMergeCand is set equal to ( numMergeCand – numOrigMergeCand ). When numCombMergeCand is greater than 0, k ranges from 0 to numCombMergeCand − 1, inclusive.

The derivation process for non-scaled bi-predictive merging candidates specified in subclause  is invoked with mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList, numMergeCand, numOrigMergeCand and maxNumMergeCand given as input and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0nscaleCandl and refIdxL1nscaleCandl, the prediction list utilization flags predFlagL0nscaleCandl and predFlagL1nscaleCandl, the motion vectors mvL0nscaleCandl and mvL1nscaleCandl of every new candidate nscaleCandl being added in mergeCandList. The number of candidates being added numNscaleMergeCand is set equal to ( numMergeCand – numOrigMergeCand – numCombMergeCand ). When numNscaleMergeCand is greater than 0, l ranges from 0 to numNscaleMergeCand – 1, inclusive.

The derivation process for zero motion vector merging candidates specified in subclause  is invoked with the mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList, the NumMergeCand and the MaxNumMergeCand as the inputs and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0zeroCandm and refIdxL1zeroCandm, the prediction list utilization flags predFlagL0zeroCandm and predFlagL1zeroCandm, the motion vectors mvL0zeroCandm and mvL1zeroCandm of every new candidate zeroCandm being added in mergeCandList. The number of candidates being added numZeroMergeCand is set equal to ( numMergeCand – numOrigMergeCand – numCombMergeCand – numNscaleMergeCand ). When numZeroMergeCand is greater than 0, m ranges from 0 to numZeroMergeCand – 1, inclusive.

The following assignments are made with N being the candidate at position merge_idx[ xP][ yP ] in the merging candidate list mergeCandList ( N = mergeCandList[ merge_idx[ xP][ yP ] ] ) and X being replaced by 0 or 1:

mvLX[ 0 ] = mvLXN[ 0 ]	(8-3)

mvLX[ 1 ] = mvLXN[ 1 ]	(8-4)

refIdxLX = refIdxLXN	(8-5)

predFlagLX = predFlagLXN	(8-6)

The merging candidates reordering process specified in subclause 8.4.2.1.7 is invoked with the mergeCandList and the NumMergeCand as the inputs and the output is assigned to mergeCandList.

8.4.2.1.2  Derivation process for spatial merging candidates

Inputs to this process are

a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are (with N being replaced by A0, A1, B0, B1 or B2 and with X being replaced by 0 or 1)

the availability flags availableFlagN of the neighbouring prediction units,

the reference indices refIdxLXN of the neighbouring prediction units,

the prediction list utilization flags predFlagLXN of the neighbouring prediction units,

the motion vectors mvLXN of the neighbouring prediction units.

For the derivation of availableFlagN, with N being A0, A1, B0, B1 or B2 and ( xN, yN ) being ( xP – 1,  yP + nPSH ), ( xP − 1,  yP + nPSH − 1 ), ( xP + nPSW,  yP – 1 ), ( xP + nPSW − 1,  yP – 1 ) or ( xP – 1,  yP – 1 ), the following applies.

–	If one of the following conditions is true, the availableFlagN is set equal to 0, both components mvLXN are set equal to 0, refIdxLXN and predFlagLX[ xN, yN ] of the prediction unit covering luma location ( xN, yN ) are assigned respectively to mvLXN, refIdxLXN and predFlagLXN.

N is equal to B2 and availableFlagA0 + availableFlagA1 + availableFlagB0 + availableFlagB1 is equal to 4.

The prediction unit covering luma location ( xN, yN ) is not available or PredMode is MODE_INTRA.

PartMode of the current prediction unit is PART_2NxN or PART_2NxnU or PART_2NxnD and PartIdx is equal to 1 and the prediction units covering luma location ( xP,  yP – 1 ) ( PartIdx = 0 ) and luma location ( xN, yN ) (Cand. N) have identical motion parameters:

mvLX[ xP,  yP – 1 ]  = =  mvLX[ xN, yN ]

refIdxLX[ xP,  yP – 1 ]  = =  refIdxLX[ xN, yN ]

predFlagLX[ xP,  yP – 1 ]  = =  predFlagLX[ xN, yN ]

PartMode of the current prediction unit is PART_Nx2N or PART_nLx2N or PART_nRx2N and PartIdx is equal to 1 and the prediction units covering luma location ( xP – 1,  yP ) ( PartIdx = 0 ) and luma location ( xN, yN ) (Cand. N) have identical motion parameters:

mvLX[ xP – 1,  yP ]  = =  mvLX[ xN, yN ]

refIdxLX[ xP – 1,  yP ]  = =  refIdxLX[ xN, yN ]

predFlagLX[ xP – 1,  yP ]  = =  predFlagLX[ xN, yN ]

PartMode of the current prediction unit is PART_NxN and PartIdx is equal to 3 and the prediction units covering luma location ( xP – 1,  yP ) ( PartIdx = 2 ) and luma location ( xP – 1,  yP – 1 ) ( PartIdx = 0 )  have identical motion parameters:

mvLX[ xP – 1,  yP ]  = =  mvLX[ xP – 1,  yP – 1 ]

refIdxLX[ xP – 1,  yP ]  = =  refIdxLX[ xP – 1,  yP – 1 ]

predFlagLX[ xP – 1,  yP ]  = =  predFlagLX[ xP – 1,  yP – 1 ]

and the prediction units covering luma location ( xP,  yP – 1 ) ( PartIdx = 1 ) and luma location ( xN, yN ) (Cand. N) have identical motion parameters:

mvLX[ xP,  yP – 1 ]  = =  mvLX[ xN, yN ]

refIdxLX[ xP,  yP – 1 ]  = =  refIdxLX[ xN, yN ]

predFlagLX[ xP,  yP – 1 ]  = =  predFlagLX[ xN, yN ]

PartMode of the current prediction unit is PART_NxN and PartIdx is equal to 3 and the prediction units covering luma location ( xP,  yP – 1 ) ( PartIdx = 1 ) and luma location ( xP – 1,  yP – 1 ) ( PartIdx = 0 ) have identical motion parameters:

mvLX[ xP,  yP – 1 ]  = =  mvLX[ xP – 1,  yP – 1 ]

refIdxLX[ xP,  yP – 1 ]  = =  refIdxLX[ xP – 1,  yP – 1 ]

predFlagLX[ xP,  yP – 1 ]  = =  predFlagLX[ xP – 1,  yP – 1 ]

and the prediction units covering luma location ( xP – 1,  yP ) ( PartIdx = 2 ) and luma location ( xN, yN ) (Cand. N) have identical motion parameters:

mvLX[ xP – 1,  yP ]  = =  mvLX[ xN, yN ]

refIdxLX[ xP – 1,  yP ]  = =  refIdxLX[ xN, yN ]

predFlagLX[ xP – 1,  yP ]  = =  predFlagLX[ xN, yN ]

–	Otherwise, availableFlagN is set equal to 1 and the variables mvLX[ xN, yN ], refIdxLX[ xN, yN ] and predFlagLX[ xN, yN ] of the prediction unit covering luma location ( xN, yN ) are assigned respectively to mvLXN, refIdxLXN and predFlagLXN.

8.4.2.1.3  Derivation process of reference indices for temporal merging candidate

Inputs to this process are

a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH.

Outputs of this process are (with X being replaced by 0 or 1)

the reference indices refIdxLX of the current prediction units.

For the derivation of refIdxLXA, the following applies.

If the prediction unit covering luma location ( xP-1, yP + nPSH − 1 ) is available and PredMode is not MODE_INTRA, refIdxLXA is assigned to refIdxLX[ xP-1, yP + nPSH − 1 ].

Otherwise, refIdxLXA is assigned to -1.

For the derivation of refIdxLXB, the following applies.

If the prediction unit covering luma location ( xP + nPSW − 1, yP-1 ) is available and PredMode is not MODE_INTRA, refIdxLXB is assigned to refIdxLX[ xP + nPSW − 1, yP-1 ].

Otherwise, refIdxLXB is assigned to -1.

For the derivation of refIdxLXC, the following applies.

If the prediction unit covering luma location ( xP+nPSW, yP-1 ) is available and PredMode is not MODE_INTRA, refIdxLXC is assigned to refIdxLX[ xP+nPSW, yP-1 ].

Otherwise if the prediction unit covering luma location ( xP-1, yP+nPSH ) is available and PredMode is not MODE_INTRA, refIdxLXC is assigned to refIdxLX[ xP-1, yP+nPSH ].

Otherwise if the prediction unit covering luma location ( xP-1, yP-1 ) is available and PredMode is not MODE_INTRA refIdxLXC is assigned to refIdxLX[ xP-1, yP-1 ].

Otherwise, refIdxLXC is assigned to -1.

The variable refIdxLX is derived as follows.

If refIdxLXA is equal to refIdxLXB and refIdxLXB is equal to refIdxLXC, the following applies.

If  refIdxLXA is equal to -1 then refIdxLX = 0

Otherwise refIdxLX = refIdxLXA

Otherwise if refIdxLXA is equal to refIdxLXB, the following applies.

If  refIdxLXA is equal to -1 then refIdxLX = refIdxLXC

Otherwise refIdxLX = refIdxLXA

Otherwise if refIdxLXB is equal to refIdxLXC, the following applies.

If  refIdxLXB is equal to -1 then refIdxLX = refIdxLXA

Otherwise refIdxLX = refIdxLXB

Otherwise if refIdxLXA is equal to refIdxLXC, the following applies.

If  refIdxLXA is equal to -1 then refIdxLX = refIdxLXB

Otherwise refIdxLX = refIdxLXA

Otherwise if refIdxLXA is equal to -1, the following applies.

refIdxLX = min( refIdxLXB, refIdxLXC )

Otherwise if refIdxLXB is equal to -1, the following applies.

refIdxLX = Min( refIdxLXA, refIdxLXC )

Otherwise if refIdxLXC is equal to -1, the following applies.

refIdxLX = Min( refIdxLXA, refIdxLXB )

Otherwise, the following applies.

refIdxLX = Min( Min( refIdxLXA, refIdxLXB ), refIdxLXC )

8.4.2.1.4  Derivation process for combined bi-predictive merging candidates

Inputs of this process are

a merging candidate list mergeCandList,

reference indices refIdxL0N and refIdxL1N of every candidate N being in mergeCandList,

prediction list utilization flags predFlagL0N and predFlagL1N of every candidate N being in mergeCandList,

motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList,

the number of elements numMergeCand within mergeCandList,

the number of elements numOrigMergeCand within the mergeCandList after the spatial and temporal merge candidate derivation process,

the maximum number of elements maxNumMergeCand within the mergeCandList.

Outputs of this process are

the merging candidate list mergeCandList,

the number of elements numMergeCand within mergeCandList.

reference indices refIdxL0combCandk and refIdxL1combCandk of every new candidate combCandk being added in mergeCandList during the invokation of this process,

prediction list utilization flags predFlagL0combCandk and predFlagL1combCandk of every new candidate combCandk being added in mergeCandList during the invokation of this process,

motion vectors mvL0combCandk and mvL1combCandk of every new candidate combCandk being added in mergeCandList during the invokation of this process,

The function RefPicOrderCnt( refidx, LX ) specifies the value of PicOrderCnt of the picture that is the reference picture RefPicListX[ refidx ] of the current picture with X being 0 or 1. PicOrderCnt of the reference picture shall be maintained until the picture is marked as “non-existing”. The variable PicOrderCnt specifies the PicOrderCnt of the current picture.

[Ed. (BB): PicOrderCnt(.) needs to be defined. Currently the Decoding process for picture order count is not described]

When numOrigMergeCand is greater than 1 and less than maxNumMergeCand, the variable numInputMergeCand is set to numMergeCand, the variables combIdx and combCnt are set to 0, the variable combStop is set to false and the following steps are repeated until combStop is equal to true.

The variables l0CandIdx and l1CandIdx are derived using combIdx as specified in Table 8-2.

The following assignments are made with l0Cand being the candidate at position l0CandIdx and l1Cand being the candidate at position l1CandIdx in the merging candidate list mergeCandList ( l0Cand = mergeCandList[ l0CandIdx ] , l1Cand = mergeCandList[ l1CandIdx ] ).

When all of the following conditions are true,

predFlagL0l0Cand = = 1

predFlagL1l1Cand = = 1

RefPicOrderCnt( refIdxL0l0Cand, L0 ) != RefPicOrderCnt( refIdxL1l1Cand, L1 ) | | mvL0l0Cand != mvL1l1Cand

the following applies.

When no candidate in mergeCandList has the same list0 motion vector and reference index as l0Cand and the same list1 motion vector and reference index as l1Cand, the candidate combCandk with k equal to ( numMergeCand − numInputMergeCand ) is added at the end of mergeCandList ( mergeCandList[ numMergeCand ] = combCandk ) and the reference indices, the prediction list utilization flags and the motion vectors of combCandk are dervied as follows and numMergeCand is incremented by 1.

refIdxL0combCandk = refIdxL0l0Cand		(8-7)

refIdxL1combCandk = refIdxL1l1Cand		(8-8)

predFlagL0combCandk = 1		(8-9)

predFlagL1combCandk = 1		(8-10)

mvL0combCandk[ 0 ] = mvL0l0Cand[ 0 ] 		(8-11)

mvL0combCandk[ 1 ] = mvL0l0Cand[ 1 ] 		(8-12)

mvL1combCandk[ 0 ] = mvL1l1Cand[ 0 ] 		(8-13)

mvL1combCandk[ 1 ] = mvL1l1Cand[ 1 ] 		(8-14)

numMergeCand = numMergeCand + 1	(8-15)

The variable combCnt is incremented by 1.

The variable combIdx is incremented by 1.

When combIdx is equal to ( numOrigMergeCand * ( numOrigMergeCand − 1 ) ) or numMergeCand is equal to maxNumMergeCand or combCnt is equal to 5, combStop is set to true.

Table 8-2 – Specification of l0CandIdx and l1CandIdx

| combIdx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| l0CandIdx | 0 | 1 | 0 | 2 | 1 | 2 | 0 | 3 | 1 | 3 | 2 | 3 |
| l1CandIdx | 1 | 0 | 2 | 0 | 2 | 1 | 3 | 0 | 3 | 1 | 3 | 2 |

8.4.2.1.5  Derivation process for non-scaled bi-predictive merging candidates

Inputs of this process are

a merging candidate list mergeCandList,

reference indices refIdxL0N and refIdxL1N of every candidate N being in mergeCandList,

prediction list utilization flags predFlagL0N and predFlagL1N of every candidate N being in mergeCandList,

motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList,

the number of elements numMergeCand within mergeCandList,

the number of elements numOrigMergeCand within the mergeCandList after the spatial and temporal merge candidate derivation process,

the maximum number of elements maxNumMergeCand within the mergeCandList.

Outputs of this process are

the merging candidate list mergeCandList,

the number of elements numMergeCand within mergeCandList.

reference indices refIdxL0nscaleCandl and refIdxL10nscaleCandl of every new candidate nscaleCandl being added in mergeCandList during the invokation of this process,

prediction list utilization flags predFlagL0nscaleCandl and predFlagL10nscaleCandl of every new candidate nscaleCandl being added in mergeCandList during the invokation of this process,

motion vectors mvL0nscaleCandl and mvL10nscaleCandl of every new candidate nscaleCandl being added in mergeCandList during the invokation of this process,

The function RefPicOrderCnt( refidx, LX ) specifies the value of PicOrderCnt of the picture that is the reference picture RefPicListX[ refidx ] of the current picture with X being 0 or 1. PicOrderCnt of the reference picture shall be maintained until the picture is marked as “non-existing”. The variable PicOrderCnt specifies the PicOrderCnt of the current picture.

[Ed. (BB): PicOrderCnt(.) needs to be defined. Currently the Decoding process for picture order count is not described]

When numOrigMergeCand is greater than 0 and numMergeCand is less than maxNumMergeCand, the variable numInputMergeCand is set to numMergeCand, the variables origIdx and nscaleCnt are set to 0, the variable nscaleStop is set to false and the following steps are repeated until nscaleStop is equal to true.

The following assignments are made with origCand being the candidate at position origIdx in the merging candidate list mergeCandList ( origCand = mergeCandList[ origIdx ] ).

The following is applied one time with X being 0 and Y being 1. When nscaleCnt is less than 1 and numMergeCand is less than maxNumMergeCand, it is applied a second time with X being 1 and Y being 0.

When all of the following conditions are true,

refIdxLXorigCand >= 0

RefPicOrderCnt( refIdxLXorigCand, LX ) != RefPicOrderCnt( refIdxorigCandLX, LY )

abs( PicOrderCnt − RefPicOrderCnt( refIdxLXorigCand, LX ) ) = = 
abs( PicOrderCnt − RefPicOrderCnt( refIdxLXorigCand, LY ) )

refIdxLXorigCand < num_ref_idx_lY_active_minus1 + 1

the following applies.

When no candidate in mergeCandList has a list0 motion vector equal to listX motion vector of origCand, a list1 motion vector equal to the listX motion vector of origCand multiplied by 1 and list0 and list1 reference indices equal to the listX reference index of origCand, the candidate nscaleCandl with l equal to ( numMergeCand − numInputMergeCand )is added at the end of mergeCandList ( mergeCandList[ numMergeCand ] = nscaleCandl ) and the reference indices, the prediction list utilization flags and the motion vectors of nscaleCandl are dervied as follows and numMergeCand is incremented by 1.

refIdxL0nscaleCandl = refIdxLXorigCand		(8-16)

refIdxL1nscaleCandl = refIdxLXorigCand		(8-17)

predFlagL0nscaleCandl = 1		(8-18)

predFlagL1nscaleCandl = 1		(8-19)

mvL0nscaleCandl[ 0 ] = mvLXorigCand[ 0 ] 		(8-20)

mvL0nscaleCandl[ 1 ] = mvLXorigCand[ 1 ] 		(8-21)

mvL1nscaleCandl[ 0 ] = − mvLXorigCand[ 0 ] 		(8-22)

mvL1nscaleCandl[ 1 ] = − mvLXorigCand[ 1 ] 		(8-23)

numMergeCand = numMergeCand + 1	(8-24)

The variable nscaleCnt is incremented by 1.

The variable origIdx is incremented by 1.

When origIdx is equal to numOrigMergeCand or numMergeCand is equal to maxNumMergeCand or nscaleCnt is equal to 1, nscaleStop is set to true.

8.4.2.1.6  Derivation process for zero motion vector merging candidates

Inputs of this process are

a merging candidate list mergeCandList,

reference indices refIdxL0N and refIdxL1N of every candidate N being in mergeCandList,

prediction list utilization flags predFlagL0N and predFlagL1N of every candidate N being in mergeCandList,

motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList,

the number of elements numMergeCand within mergeCandList,

the maximum number of elements maxNumMergeCand within the mergeCandList.

Outputs of this process are

the merging candidate list mergeCandList,

the number of elements numMergeCand within mergeCandList.

reference indices refIdxL0zeroCandm and refIdxL10zeroCandm of every new candidate zeroCandm being added in mergeCandList during the invokation of this process,

prediction list utilization flags predFlagL0zeroCandm and predFlagL10zeroCandm of every new candidate zeroCandm being added in mergeCandList during the invokation of this process,

motion vectors mvL0zeroCandm and mvL10zeroCandm of every new candidate zeroCandm being added in mergeCandList during the invokation of this process,

The variable numRefIdx is derived as follows.

If slice_type is equal to P, numRefIdx is set to num_ref_idx_l0_active_minus1 + 1

Otherwise (slice_type is equal to B), numRefIdx is set to min(num_ref_idx_l0_active_minus1 + 1, num_ref_idx_l1_active_minus1 + 1)

When numMergeCand is less than maxNumMergeCand, the variable numInputMergeCand is set to numMergeCand, the variables zeroIdx and zeroCnt are set to 0, the variable zeroStop is set to false and the following steps are repeated until zeroStop is equal to true.

For the derivation of the reference indices, the prediction list utilization flags and the motion vectors of the zero motion vector merging candidate, the following applies.

If all of the following conditions are true,

slice_type is equal to P

no candidate in mergeCandList has a list0 motion vector equal to (0,0) and reference index equal to zeroIdx or (zeroCnt is equal to 2 and zeroIdx is equal to 0),

the candidate zeroCandm with m equal to ( numMergeCand − numInputMergeCand ) is added at the end of mergeCandList ( mergeCandList[ numMergeCand ] = zeroCandm ) and the reference indices, the prediction list utilization flags and the motion vectors of zeroCandm are dervied as follows and numMergeCand is incremented by 1.

refIdxL0zeroCandm = zeroIdx	(8-25)

refIdxL1zeroCandm = −1	(8-26)

predFlagL0zeroCandm = 1	(8-27)

predFlagL1zeroCandm = 0	(8-28)

mvL0zeroCandm[ 0 ] = 0	(8-29)

mvL0zeroCandm[ 1 ] = 0	(8-30)

mvL1zeroCandm[ 0 ] = 0	(8-31)

mvL1zeroCandm[ 1 ] = 0	(8-32)

numMergeCand = numMergeCand + 1	(8-33)

Otherwise if all of the following conditions are true,

slice_type is equal to B

no candidate in mergeCandList has a list0 motion vector equal to (0,0), a list0 reference index equal to zeroIdx, a list1 motion vector equal to (0,0) and a list1 reference index equal to zeroIdx or (zeroCnt is equal to 2 and zeroIdx is equal to 0)

the candidate zeroCandm with m equal to ( numMergeCand − numInputMergeCand ) is added at the end of mergeCandList ( mergeCandList[ numMergeCand ] = zeroCandm ) and the reference indices, the prediction list utilization flags and the motion vectors of zeroCandm are dervied as follows and numMergeCand is incremented by 1.

refIdxL0zeroCandm = zeroIdx	(8-34)

refIdxL1zeroCandm = zeroIdx	(8-35)

predFlagL0zeroCandm = 1	(8-36)

predFlagL1zeroCandm = 1	(8-37)

mvL0zeroCandm[ 0 ] = 0	(8-38)

mvL0zeroCandm[ 1 ] = 0	(8-39)

mvL1zeroCandm[ 0 ] = 0	(8-40)

mvL1zeroCandm[ 1 ] = 0	(8-41)

numMergeCand = numMergeCand + 1	(8-42)

The variables zeroCnt and zeroIdx are modifed as follows.

If zeroIdx is equal to numRefIdx or zeroCnt is equal to 2, zeroIdx is set to 0.

Otherwise (zeroIdx is less than and zeroCnt is less than 2), zeroIdx and zeroCnt are incremented by 1.

When numMergeCand is equal to maxNumMergeCand, zeroStop is set to true.

8.4.2.1.7  Merging candidates reordering process

Inputs of this process are

a merging candidate list mergeCandList,

a variable PartMode specifying prediction partition mode of current coding unit,

a variable PartIdx specifying the index of the current prediction unit within the current coding unit,

the availability flags availableFlag A1 and availableFlagB1 of the neighbouring prediction units,

the number of elements numMergeCand within mergeCandList.

Outputs of this process is

the merging candidate list mergeCandList.

If PartMode of the current prediction unit is PART_2NxN or PART_2NxnU or PART_2NxnD and PartIdx is equal to 1 and availableFlagB1 is equal to 1, merging candidate derived from B1 position is moved to the last order in the mergeCandList.

If PartMode of the current prediction unit is PART_Nx2N or PART_nLx2N or PART_nRx2N and PartIdx is equal to 1 and availableFlagA1 is equal to 1, merging candidate derived from A1 position is moved to the last order in the mergeCandList.