| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G183 |
| --- | --- |

| Title: | Test result of low delay tile |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kimihiko Kazui, Satoshi Shimada, Junpei Koyama and Akira Nakagawa 4-1-1 Kamikodanaka, Nakahara-ku 211-8588 Kawasaki JAPAN | Tel: Email: | +81-44-754-2639 kazui.kimihiko@jp.fujitsu.com sshimada@jp.fujitsu.com koyama.junpei@jp.fujitsu.com anaka@jp.fujitsu.com |
| Source: | FUJITSU LABORATORIES LTD. |  |  |

_____________________________

# Abstract

This contribution shows the evaluation result of the low delay tile scheme proposed at the 6th JCT-VC meeting.

The proposed scheme introduces an additional mode in which the order of entropy coding is the raster scan order within a picture for the purpose of extending the application area of the tile scheme to low delay video transmission.

BD-Rate reduction of the proposed scheme over the current tile scheme in WD4 is -5.8% for "low delay high efficiency" configuration and -4.6% for "low delay low complexity" configuration.

BD-Rate reduction of the proposed scheme over the scheme without tile is -5.3% for "low delay high efficiency" configuration and -5.5% for "low delay low complexity" configuration.

The software and the simulation results are cross-checked by Sharp (JCTVC-G968).

# Introduction

This contribution shows the evaluation result of the low delay tile scheme proposed in JCTVC-F140[1] at the 6th JCT-VC meeting.

The proposed scheme introduces an additional mode in which the order of entropy coding is the raster scan order within a picture for the purpose of extending the application area of the tile scheme[2] to low delay video transmission. Industries such as broadcasting require very low delay coding with high picture quality[4].

# Evaluation method

## Software

The proposed scheme and the schemes for comparison are implemented onto HM software. Since HM4.1 (the formal version with the tile scheme) is not released yet, the branch of HM4 (HM-4.0-dev-tiles-wpp, revision 1418) is used.

Codes related to Wavelet parallel processing are deactivated for this evaluation test.

## Scenario

We evaluate BD-Rate reduction of the proposed scheme over existing schemes when very low delay coding based on the vertical intra line refresh scheme[1] is performed.

A vertical intra line circulates from left to right inside a picture in order to refresh a whole picture within one refresh cycle. The refresh cycle is set equal to Ceil( picture_width / LCU_width ) in this evaluation. For example, the refresh cycle for class-B sequence is 30 frames.

In order to guarantee correct reconstruction of whole pixels at the last picture of each refresh cycle, proper control is necessary for disabling the reference from the refreshed area to the non-refreshed area during the decoding process.

Figure 1  Vertical intra line refresh scheme

The following three modes are compared.

| Mode # | Description |
| --- | --- |
| Mode 1 | Proposed low delay tile scheme |
| Mode 2 | Existing scheme #1 The scheme without using tile |
| Mode 3 | Existing scheme #2 Scheme based on WD4 tile |

Details of three modes are described in the following.

### Mode 1

Up to two vertical tiles are inserted in each picture (Figure 2).

Tiles are used to restrict the reference from a coding unit (or a prediction unit) in the refreshed area to a coding unit (or a prediction unit) in the non-refreshed area. The boundary position of two tiles coincides with the area boundary in each picture.

The order of entropy coding is the raster scan order within a picture. This is the proposed additional operation mode to the WD4 tile scheme. The additional mode enables reduction of the codec delay down to 1 treeblock line time (e.g 0.9 ms for 1080p60 sequence). This is because entropy coding process for a picture can start as soon the first treeblock line of the picture is captured. Unlike the WD4 tile, entropy coder is not reset at a tile boundary and is allowed to refer a coding unit in other tile during the parsing process.

In order to guarantee correct reconstruction of a pixel, the reference from a coding unit (or a prediction unit) in the refreshed area to a coding unit (or a prediction unit) in the non-refreshed area is prohibited during the decoding process.

The width of the vertical intra line is equal to LCU_width.

See Annex for detail.

Figure 2  Encoding scheme of mode 1

### Mode 2

This mode is one the two possible schemes just using existing schemes. Mode 2 guarantees correct reconstruction of a pixel in the refreshed area without using tiles.

In Mode 2, the width of the vertical intra line extends from LCU_width to ( LCU_width + 16 ) (see Figure 3). The right part of the vertical intra line, whose width is 16 pixels, is regarded as the part of the non-refreshed area. That is, the boundary of the right part and the left part (whose width is LCU_width) is the (ideal) area boundary. The area of vertical intra line is ( 1 + ( 16 / LCU_width ) ) times larger than that in Mode1.

The right part of the vertical intra line is contaminated through in-loop filtering. However, as the range of contamination is restricted within the right side of the right part, the left part of the vertical intra line remains intact. To be specific, the range of contamination is the summation of the range of deblock filtering (i.e. 3 pixels width), SAO (i.e. 1 pixel width), and ALF (i.e. 5 pixels width). Note that the minimum width of the left part of the vertical intra line is 16 pixels because the maximum size of ALF filter for chroma is 5x11.

Additional control at the encoder side is applied to intra prediction in Mode 2 only. This control restricts the direction of intra prediction so that all prediction unit inside the vertical intra line do not refer to pixels located in the right side of the vertical intra line. Thus all reconstructed pixels in the vertical intra line before in-loop filtering at the decoder side are always identical to those at the encoder side.

Figure 3  Encoding scheme of mode 2

### Mode 3

Mode 3 applies the tile scheme in WD4.

Similar to Mode 1, the vertical tile boundary is coincided with the area boundary.

The tile scheme in WD4 defines the order of entropy coding be the raster scan within a tile. If the height of a tile is equal to that of a picture, coding delay becomes at least 1 picture interval. This is due to the causality between the capturing order and the bitstream order. Taking two vertical tiles for example, the output of the coded right tile shall be after completing the encoding process of the left tile in order to output coded tiles sequentially. As the completion of encoding process of the left tile cannot be before capturing the last line of the picture, the output of coded tiles delays at least 1 picture period.

In order to achieve the same low delay operation as in Mode 1, each treeblock line is separated by a horizontal tile boundary (see Figure 4). The number of tiles to be inserted is equal to ( 2*N ), where N is equal to Ceil( picture_height / LCU_height ). The order of entropy coding becomes identical to that in Mode 1 (i.e. the raster scan order within a picture).

Figure 4  Encoding scheme of mode 3

### Common behavior in 3 modes

The range of motion vector search is restricted at the encoder side so that any prediction unit in a refresh area of a processing picture does not refer to pixels in a non-refreshed area of its reference pictures.

## Comparison

Reduction of BD-Rate of Mode1 over Mode 2, and reduction of BD-Rate of Mode 1 over Mode 3 are evaluated.

## Configuration

"Low delay" configurations as specified in the common test conditions[5] are applied for the evaluation.

# Result

Table 1 and Table 2 summarize reduction of BD-Rate of the proposed scheme.

Table 1: Summary results of BD-Rate comparing Mode 1 (Proposal) to Mode 2 (Without tile)

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -3.2% | -4.9% | -3.9% | -3.3% | -4.5% | -3.5% |
| Class C | -4.6% | -5.4% | -5.5% | -4.7% | -5.2% | -5.1% |
| Class D | -6.5% | -7.5% | -7.4% | -6.6% | -7.4% | -7.2% |
| Class E | -8.0% | -9.9% | -9.2% | -8.6% | -9.3% | -8.9% |
| Overall | -5.3% | -6.6% | -6.2% | -5.5% | -6.3% | -5.8% |
|  | -5.3% | -6.6% | -6.2% | -5.5% | -6.3% | -5.8% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 2: Summary results of BD-Rate comparing Mode 1 (Proposal) to Mode 3 (WD4 Tile)

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -5.9% | -9.3% | -7.8% | -4.5% | -5.7% | -5.1% |
| Class C | -4.7% | -8.3% | -8.9% | -4.0% | -5.5% | -5.6% |
| Class D | -3.3% | -6.6% | -6.8% | -2.9% | -3.7% | -3.8% |
| Class E | -10.4% | -16.1% | -15.6% | -7.5% | -11.3% | -10.1% |
| Overall | -5.8% | -9.6% | -9.3% | -4.6% | -6.2% | -5.8% |
|  | -5.8% | -9.7% | -9.3% | -4.6% | -6.2% | -5.8% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 95% |  |  | 92% |  |  |

The proposed scheme improves coding efficiency more than 4.6%.

The result is confirmed by Sharp[3].

# Conclusion

The low delay tile scheme proposed in JCTVC-F140 improves at least 4.6% in BD-Rate when very low delay operation is applied. The result shows that the proposed scheme fulfills the requirement of high-quality, low delay video transmission[4].

We recommend to adopt the proposed scheme in WD.

# References

[1] K.Kazui, S.Shimada, J.Koyama and A.Nakagawa, “Support of very low delay coding in Tile”, JCTVC-F140, July 2011, Torino IT

[2] A.Fuldseth, M.Horowitz, S.Xu, A.Segall and M.Zhou, "Tiles", JCTVC-F335, July 2011, Torino IT

[3] K.Misra and A.Segall, "A Cross-check report for JCTVC-G183 on low delay tile", JCTVC-G968, November 2011, Geneva CH

[4] K.Kazui, J.Koyama, and A.Nakagawa, "Industry needs of very low delay coding", JCTVC-F147, July 2011, Torino IT

[5] "Common test conditions and software reference configurations", JCTVC-F900, July 2011, Torino IT

# Patent rights declaration(s)

FUJITSU LIMITED may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex: Proposed syntax, semantics, parsing process, and decoding process

## Syntax

Newly added parts and modified parts of the syntax are highlighted with yellow marker.

### Sequence Parameter Set

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| .... |  |
| tile_boundary_independence_idr | ue(v) |
| uniform_spacing_idr | u(1) |
| if (uniform_spacing_idr == 1) { |  |
| column_width_minus1 | ue(v) |
| row_height_minus1 | ue(v) |
| } |  |
| else { |  |
| num_columns_minus1 | ue(v) |
| num_rows_minus1 | ue(v) |
| for (i=0; i < num_columns_minus1; i++) |  |
| column_width[i] | ue(v) |
| for (i=0; i < num_rows_minus1; i++) |  |
| row_height[i] | ue(v) |
| } |  |
| if (tile_boundary_independence_idr == 2) { |  |
| for ( i = 0; i < ( num_columns_minus1 + 1 ) * ( num_rows_minus1 + 1 ); i++ ) { |  |
| tile_idx_diff | se(v) |
| } |  |
| } |  |
| processing_order_idx | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

### Picure Parameter Set

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| .... |  |
| column_row_info_present | u(1) |
| if (column_row_info_present == 1) { |  |
| tile_boundary_independence_idr | ue(v) |
| uniform_spacing_idr | u(1) |
| if (uniform_spacing_idr == 1) { |  |
| column_width_minus1 | ue(v) |
| row_height_minus1 | ue(v) |
| } |  |
| else { |  |
| num_columns_minus1 | ue(v) |
| num_rows_minus1 | ue(v) |
| for (i=0; i < num_columns_minus1; i++) |  |
| column_width[i] | ue(v) |
| for (i=0; i < num_rows_minus1; i++) |  |
| row_height[i] | ue(v) |
| } |  |
| if (tile_boundary_independence_idr == 2) { |  |
| for ( i = 0; i < ( num_columns_minus1 + 1 ) * ( num_rows_minus1 + 1 ); i++ ) { |  |
| tile_idx_diff | se(v) |
| } |  |
| } |  |
| processing_order_idx | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

## Semantics

Newly added parts and modified parts of the semantics are highlighted with yellow marker.

### Sequence Parameter Set and Picture Parameter Set

tile_boundary_independence_idr when tile_boundary_independence_idr equals 1, all column and row boundaries break prediction mechanisms (e.g., intra prediction and motion vector prediction). Otherwise, when tile_boundary_independence_idr equals 0, row and column boundaries in pictures associated with the picture parameter set do not break prediction mechanisms. Otherwise, when tile_boundary_independence_idr equals to 2, prediction mechanism of each column and row boundaries is determined according to the value of TileIdx.

tile_idx_diff specifies the variable TileIdx of each tile.

TileIdx of N-th tile in the raster-scan order within a picture is calculated as follows:

TileIdx = TileIdx' + tile_idx_diff

, where TileIdx' equal to TileIdx of (N-1)-th tile in the raster-scan order within a picture. When N is equal to 0, TileIdx' is inferred as 0.

processing_order_idx specifies the decoding order of each treeblock within a picture. When processing_order_idx equals to 0, the decoding order of each treeblock is the raster-scan order within a tile. Otherwise, when processing_order_idx equals to 1, the decoding order of each treeblock is the raster-scan order within a picture.

When processing_order_idx equals to 1, the initialization process of the parsing process in subclause 9.2.2 (when entropy_coding_mode_flag is equal to 0) or the initialization process of the parsing process in subclause 9.3.2 (when entropy_coding_mode_flag is equal to 1) is not invoked at a tile boundary.

## Parsing process

When processing_order_idx is equal to 1, the following definition on availability are applied.

### Parsing process (Subclause 9)

The prediction unit is marked as available, unless any of the following conditions is true in which case the prediction unit is marked as not available:

- tbAddr < 0, where tbAddr is the address of the treeblock to which the prediction unit belongs

- tbAddr > CurrTbAddr, where CurrTbAddr is the address of the treeblock to which the current syntax element belongs

- the prediction unit belongs to a different slice than the current syntax element

[Note: The prediction unit is available even when the prediction unit belongs to a different tile than the current syntax element.]

[Note: Current specification only checks availability related to split_coding_unit_flag and skip_flag. Since split_coding_unit_flag is assigned to a coding tree and skip_flag is assigned to a coding unit, rather than to a prediction unit, the words "prediction unit" should be replaced with proper words such as "coding unit". Above definition is based on the text in WD4.]

## Decoding process

When tile_boundary_independence_idr is equal to 2, the following definition on availability are applied.

### Derivation process for luma intra prediction mode (Subclause 8.3.1)

For N being either replaced A or B, the variables intraPredModeN are derived as follows.

- If the coding unit covering ( xBN,  yBN ) is not available, intraPredModeN is set equal to -1

- Otherwise, if the coding unit covering ( xBN,  yBN ) is not coded as intra mode, intraPredModeN is set equal to Intra_DC,

- Otherwise, intraPredModeN is set equal to IntraPredMode[ xBN ][ yBN ], where IntraPredMode is the variable array assigned to the coding unit covering the luma location ( xBN, yBN ).

The coding unit is marked as available, unless any of the following conditions is true in which case the coding unit is marked as not available:

- tbAddr < 0, where tbAddr is the address of the treeblock to which the prediction unit belongs

- tbAddr > CurrTbAddr, where CurrTbAddr is the address of the treeblock to which the current coding unit belongs

- the prediction unit belongs to a different slice than the current coding unit

[Note: The coding unit is available even when the coding unit belongs to a different tile than the current coding unit.]

[Note: The current specification described in WD4 (JCTVC-F803_d5) does not consider the case when a slice boundary exists inside a treeblock.]

[Note: Since intra prediction mode is assigned to each prediction unit, the words "coding unit" should be replaced with "prediction unit". Above definition is based on the text in WD4.]

### Intra sample prediction (Subclause 8.3.3.1)

The coding unit is marked as available, unless any of the following conditions is true in which case the coding unit is marked as not available:

- tbAddr < 0, where tbAddr is the address of the treeblock to which the prediction unit belongs

- tbAddr > CurrTbAddr, where CurrTbAddr is the address of the treeblock to which the current coding unit belongs

- the prediction unit belongs to a different slice than the current coding unit

- tileIdxNeighbor > tileIdxCurr, where tileIdxCurr is TileIdx of the tile to which the current coding unit belongs and tileIdxNeighbor is TileIdx of the tile to which the coding unit belongs

[Note: The last condition is corresponding to the asymmetric restriction on reference across a tile boundary.]

### Decoding process for coding units coded in inter prediction mode (Subclause 8.4)

The prediction unit in the same picture of the current coding unit is marked as available, unless any of the following conditions is true in which case the prediction unit is marked as not available:

- tbAddr < 0, where tbAddr is the address of the treeblock to which the prediction unit belongs

- tbAddr > CurrTbAddr, where CurrTbAddr is the address of the treeblock to which the current coding unit belongs

- the prediction unit belongs to a different slice than the current coding unit

- tileIdxNeighbor > tileIdxCurr, where tileIdxCurr is TileIdx of the tile to which the current prediction unit belongs and tileIdxNeighbor is TileIdx of the tile to which the prediction unit belongs

[Note: The last condition is corresponding to the asymmetric restriction on reference across a tile boundary.]

[Note: This restriction is not applied to a prediction unit in a reference picture.]

### Deblocking filter process (Subclause 8.6.1)

The deblocking filter process shall be applied to all prediction unit edges and transform unit edges of a picture, except edges at the boundary of the picture, any edges for which the deblocking filter process is disabled by disable_deblocking_filter_idc and any edges coincide with slice boundaries when loop_filter_across_slice_flag is equal to 0. For the transform units and prediction units with edges smaller than 8 samples in either vertical or horizontal direction, only the edges lying on the 8x8 sample grid are filtered.

If any edges coincide with a tile boundary, the following additional rule is applied.

Let tileIdxCurr be TileIdx of the current coding unit and tileIdxNeighbor be TileIdx of the adjacent coding unit. The tile boundary exists between the current coding unit and the adjacent coding unit.

If ( tileIdxNeighbor > tileIdxCurr ), the replacing part of the filtering process is not applied to all pixels in the current coding unit.

If ( tileIdxNeighbor < tileIdxCurr ), the replacing part of the filtering process is not applied to all pixels in the adjacent coding unit.

[Note: Above rule guarantees that a pixel in a tile with higher priority is not affected by a pixel in a tile with lower priority through deblocking filtering.]

### Modification process for luma and chroma samples (Subclause 6.3.6.1.1)

[Note: Sample adaptive offset process]

Let CurrTileIdx be TileIdx of the tile to which the current coding unit belongs.

Let RefTileIdx[ i ][ j ][ k ] (i = 0..nS-1 , j = 0..nS-1, k = 0..1) be TileIdx of the tile to which the pixel with position ( xC + i + hPos[ k ], yC + j + vPos[ k ] ) belongs.

Let variable NonRefFlag[ i ][ j ][ k ] (i = 0..nS-1 , j = 0..nS-1, k = 0..1) be equal to ( ( RefTileIdx[ i ][ j ][ k ] > CurrTileIdx ) ? ( 1 ) : ( 0 ) ).

If saoTypeIdx is equal to one of the values of 1, 2, 3 or 4, the following ordered steps apply:

- Arrays hPos[2] and vPos[2] are specified in Error: Reference source not found.

- A variable edgeIdx is specified as

edgeIdx = 2 + ∑k( Sign( recPicture[ xC + i, yC + j ] –

recPicture[ xC + i + hPos[ k ], yC + j + vPos[ k ] ] ) ) with k = 0..1

If any of NonRefFlag[ i ][ j ][ k ] with k = 0..1 is equal to 1, edgeIdx = -1

- The reconstructed picture buffer is modified as

- If edgeIdx is not equal to -1,

recSaoPicture[ xC + i, yC + j ] = recPicture[ xC + i, yC + j ] + saoValueArray[ edgeTable[ edgeIdx ] ]

- else (edgeIdx is equal to -1),

recSaoPicture[ xC + i, yC + j ] = recPicture[ xC + i, yC + j ]

with i = 0..nS-1 and j = 0..nS-1 where 	edgeTable[5] = { 1, 2, 0, 3, 4}.

### Boundary padding process (Subclause 8.6.3.1)

[Note: Boundary padding process in the current WD does not support the case when a part of a boundary coincide with a slice boundary. Derivation process of s'' is rewritten in order to also cover such case.]

Derivation process of s'' is specified in the following.

- s’’[ xC + x ][ yC + y ] is set to s’[ xC + x ][ yC + y ] for x = -nExtPixelsX..( nS + nExtPixelsX - 1 ) and y = -nExtPixelsY..( nS + nExtPixelsY - 1 ) [Note: Copying whole pixels at the beginning]

- Following steps are applied to derive variable n[ x ][ y ] for x = -nExtPixelsX..( nS + nExtPixelsX - 1 ) and y = -nExtPixelsY..( nS + nExtPixelsY - 1 ).

- If loop_filter_across_slice_flag is equal to 0 and the pixel with position ( xC + x, yC + y ) is in the different slice than the current coding unit, then n[ x ][ y ] is set to 1.

- Else if RefTileIdx, TileIdx of the tile to which the pixel with position ( xC + x, yC + y ) belongs, is larger than CurrTileIdx, TileIdx of the tile to which the current coding unit belongs, then n[ x ][ y ] is set to 1.

- Otherwise, n[ x ][ y ] is set to 0.

- For x = -nExtPixelsX..-1 and y = 0..( nS - 1 ), s’’[ xC + x ][ yC + y ] is set to s’’[ xC ][ yC + y ] if n[ x ][ y ] is equal to 1. [Note: Left side]

- For x = nS..( nS + nExtPixelsX - 1) and y = 0..( nS - 1 ), s’’[ xC + x ][ yC + y ] is set to s’’[ xC + nS - 1 ][ yC + y ] if n[ x ][ y ] is equal to 1. [Note: Right side]

- For x = 0..( nS - 1 ) and y = -nExtPixelsY..-1, s’’[ xC + x ][ yC + y ] is set to s’’[ xC + x ][ yC ] if if n[ x ][ y ] is equal to 1. [Note: Top side]

- For x = 0..( nS - 1 ) and y = nS..( nS + nExtPixelsY - 1), s’’[ xC + x ][ yC + y ] is set to s’’[ xC + x ][ yC + nS - 1 ] if n[ x ][ y ] is equal to 1. [Note: Bottom side]

- For x = -nExtPixelsX..-1 and -nExtPixelsY..-1, if n[ x ][ y ] is equal to 1, [Note: Above left]

  - If n[ 0 ][ -1 ] is equal to 0 and n[ -1 ][ 0 ] is equal to 1, then s’’[ xC + x ][ yC + y ] is set to s’’[ xC ][ yC + y ].

  - Otherwise s’’[ xC + x ][ yC + y ] is set to s’’[ xC + x ][ yC ].

- For x = nS..( nS + nExtPixelsX - 1) and -nExtPixelsY..-1, if n[ x ][ y ] is equal to 1, [Note: Above right]

  - If n[ nS - 1 ][ -1 ] is equal to 0 and n[ nS ][ 0 ] is equal to 1, then s’’[ xC + x ][ yC + y ] is set to s’’[ xC + nS - 1 ][ yC + y ].

  - Otherwise s’’[ xC + x ][ yC + y ] is set to s’’[ xC + x ][ yC ].

- For x = -nExtPixelsX..-1 and nS..( nS + nExtPixelsY - 1), if n[ x ][ y ] is equal to 1, [Note: Bottom left]

  - If n[ 0 ][ nS ] is equal to 0 and n[ -1 ][ nS - 1 ] is equal to 1, then s’’[ xC + x ][ yC + y ] is set to s’’[ xC ][ yC + y ].

  - Otherwise s’’[ xC + x ][ yC + y ] is set to s’’[ xC + x ][ yC + nS - 1 ].

- For x = nS..( nS + nExtPixelsX - 1) and nS..( nS + nExtPixelsY - 1), if n[ x ][ y ] is equal to 1, [Note: Bottom right]

  - If n[ nS - 1 ][ nS ] is equal to 0 and n[ nS ][ nS - 1 ] is equal to 1, then s’’[ xC + x ][ yC + y ] is set to s’’[ xC + nS - 1][ yC + y ].

  - Otherwise s’’[ xC + x ][ yC + y ] is set to s’’[ xC + x ][ yC + nS - 1 ].