| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G184 |
| --- | --- |

| Title: | Non-CE6: Unified neighboring positions for intra mode coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shigeru Fukushima Hiroya Nakamura JVC KENWOOD Corporation 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan | Tel: Email: | +81-46-836-6597 fukushima.shigeru@ jvckenwood.com nakamura.hiroya@ jvckenwood.com |
| Source: | JVC KENWOOD Corporation |  |  |

_____________________________

# Abstract

Intra neighboring positions are changed to be matched with inter neighboring positions.

The simulation results report that the proposed technique provides 0.03% BD-rate [4] gain for intra only settings.

# Introduction

In the 6th JCT-VC meeting, unification of merge/AMVP spatial neighboring positions [1] is adopted. However neighboring positions for intra mode coding are still not unified in the HM4.0 [2].

# Algorithm Description

## Current algorithm (HM4.0)

Figure 1 illustrates spatial neighboring positions in the HM4.0. Different positions are used as reference for intra/inter each other.

Figure 1 Neighboring positions in the HM4.0

## Proposed algorithm

Intra neighboring positions are changed to be matched with inter neighboring positions. Figure 2 illustrates the intra neighboring positions in this proposal.

By this change, following effects can be obtained.

- In encoder, memory access to obtain neighboring information can be reduced.

- In decoder, neighboring information can be obtained without depending on intra/inter mode.

Figure 2 Intra neighboring positions in this proposal

## Implementation

### Software

This proposed technique is implemented into HM4.0 software.

### Syntax structure

The syntax of this proposed technique is not revised from HM4.0.

# Experiments

## Test conditions

The common test conditions [3] are used.

## Experimental environment

The experimental environment used is presented in Table1.

Table 1 Experimental environment

| CPU | Intel Core i7-2600K 3.40GHz |
| --- | --- |
| Memory | 16GB (DDR3) |
| OS | Windows 7 Professional 64bit |
| Compiler | Microsoft Visual C++ 2008 Express edition SP1 |
| Executable | x64 |

Notes: The decoding time is measured with the writing of yuv file.

## Simulation results

Table 2 presents the simulation results for intra only.

Table 2 Simulation results for intra only

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.03% | -0.07% | -0.04% | -0.02% | 0.02% | -0.05% |
| Class B | -0.04% | -0.06% | -0.07% | -0.03% | -0.03% | -0.02% |
| Class C | -0.04% | -0.02% | -0.04% | -0.03% | -0.06% | -0.06% |
| Class D | -0.01% | -0.08% | -0.04% | -0.02% | 0.03% | -0.02% |
| Class E | -0.04% | -0.12% | -0.06% | -0.04% | -0.07% | -0.03% |
| Overall | -0.03% | -0.07% | -0.05% | -0.03% | -0.02% | -0.03% |
|  | -0.03% | -0.07% | -0.05% | -0.03% | -0.02% | -0.04% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

The simulation results report that the proposed technique provides 0.03% BD-rate [4] gain for intra only settings. (See the attached excel sheet)

# Conclusions

This contribution reports the proposed technique can reduce complexity of both encoding and decoding. Furthermore, the proposed technique can bring coding gain.

Therefore, we suggest that the proposed technique be adopted to the next WD and HM.

# References

[1] “Unification of derivation process for merge mode and MVP”,

JCTVC-F419, Torino, July 2011

[2] “WD4: Working Draft 4 of High-Efficiency Video Coding”,

JCTVC-F803, Torino, July 2011

[3] “Common test conditions and software reference configurations”,

JCTVC-F900, Torino, July 2011

[4] “Improvements of the BD-PSNR model,” ITU-T SG16 Q.6 Document,

VCEG-AI11, Berlin, July 2008.

# Patent rights declaration(s)

JVC KENWOOD Holdings, Inc. or JVC KENWOOD Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# WD changes

8.3.1 Derivation process for luma intra prediction mode

Inputs to this process are:

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture,

–	a variable log2PUSize specifying the size of the current prediction unit,

–	a variable nPS specifying the size of the current prediction unit,

–	variable arrays IntraPredMode (If available) that are previously (in decoding order) derived for adjacent coding units.

Output of this process is the variable IntraPredMode[ xB ][ yB ].

Table 8-3 specifies the value for the intra prediction mode and the associated names.

Table 8-3 – Specification of intra prediction mode and associated names

| Intra prediction mode | Associated names |
| --- | --- |
| 0 | Intra_Planar |
| 1 | Intra_Vertical |
| 2 | Intra_Horizontal |
| 3 | Intra_DC |
| Otherwise (4..34) | Intra_Angular |
| 35 | Intra_FromLuma (used only for chroma) |

Table 8-4 specifies the number of luma intra prediction modes intraPredModeNum depending on log2PUSize.

Table 8-4 – Specification of intraPredModeNum

| log2PUSize | intraPredModeNum |
| --- | --- |
| 2 | 18 |
| 3 | 35 |
| 4 | 35 |
| 5 | 35 |
| 6 | 4 |

Table 8-5 specifies the mapping table used for converting the number of intra prediction modes.

Table 8-5 – Specification of mapIntraPredMode4 and mapIntraPredMode10

| value | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| mapIntraPredMode4[ value ] | 0 | 1 | 2 | 3 | 3 | 3 | 1 | 3 | 2 | 2 | 3 | 3 | 1 | 1 | 3 | 3 | 2 |  |
| mapIntraPredMode10[ value ] | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |  |
| value | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| mapIntraPredMode4[ value ] | 2 | 3 | 3 | 3 | 3 | 1 | 1 | 1 | 3 | 3 | 3 | 3 | 2 | 2 | 2 | 2 | 3 | 3 |
| mapIntraPredMode10[ value ] | 3 | 3 | 4 | 5 | 5 | 1 | 1 | 6 | 6 | 7 | 4 | 8 | 8 | 2 | 2 | 9 | 9 | 3 |

IntraPredMode[ xB ][ yB ] labelled 0, 1, 2, .., 34 represents directions of predictions as illustrated in Figure 8-3.

Figure 8-3 – Intra prediction mode directions (informative)

IntraPredMode[ xB ][ yB ] is derived as the following ordered steps. [Ed. (WJ): proponent suggests to move this part to the syntax since the other syntax elements utilize IntraPredMode. But it seems too complex to move all the following process to the syntax table. Maybe it’s better to move this part to the semantics section or simply avoid the use of IntraPredMode to parse the syntax item]

- The derivation process for neighbouring treeblocks specified in subclause XXX with ( xB,  yB ) given as input and the output is assigned to tbAddrA and tbAddrB specifying the treeblock addresses of treeblocks covering ( xBA,  yBA ) and ( xBB, yBB ) respectively where ( xBA,  yBA ) is set equal to ( xB-1,  yB ) and ( xBB,  yBB ) is set equal to ( xB,  yB-1 ).

( xBA,  yBA ) is set equal to ( xB-1,  yB+nPS ) and ( xBB,  yBB ) is set equal to ( xB+nPS,  yB-1 ).

- For N being either replaced A or B, the variables intraPredModeN are derived as follows.

- If the treeblock with address tbAddrN is not available, intraPredModeN is set equal to -1

- Otherwise, if the coding unit covering ( xBN,  yBN ) is not coded as intra mode, intraPredModeN is set equal to Intra_DC,

- Otherwise, intraPredModeN is set equal to IntraPredMode[ xBN ][ yBN ], where IntraPredMode is the variable array assigned to the coding unit covering the luma location ( xBN, yBN ).

- For N being either replaced A or B, the variables candIntraPredModeN are derived as follows.

- If intraPredModeN is greater than or equal to intraPredModeNum

- If intraPredModeNum is equal to 4 then candIntraPredModeN is set equal to mapIntraPredMode4[ intraPredModeN ]

- Otherwise candIntraPredModeN is set equal to mapIntraPredMode10[ intraPredModeN ].

- Otherwise, candIntraPredModeN is set equal to intraPredModeN

- The candModeList[x] is derived as follows:

- If both candIntraPredModeN are not available, then the value 3 is assigned to candModeList[ 0 ] and the value 0 is assigned to candModeList[ 1 ]

- Otherwise, if only one candIntraPredModeN is available or if both candIntraPredModeN are the same, candModeList[ x ] is derived by applying the following procedure:

  - candIntraPredModeN is assigned to candModeList[ 0 ]

  - If candIntraPredModeN is not equal to 0, the value 0 is assigned to candModeList[ 1 ]

  - Otherwise, the value 3 is assigned to candModeList[ 1 ]

- Otherwise, both candIntraPredModeN are assigned to the candidate modes list with the smaller of the two candidates at candModeList[ 0 ] and the larger at candModeList[ 1 ].

- IntraPredMode[ xB ][ yB ] is derived by applying the following procedure:

- If prev_intra_pred_flag[ xB ][ yB ] is true, the IntraPredMode[ xB ][ yB ] is set equal to  candModeList[mpm_idx][ xB ][ yB ]]

- Otherwise IntraPredMode[ xB ][ yB ] is derived by applying the following ordered steps:

  - IntraPredMode[ xB ][ yB ] = rem_intra_pred_mode

  - If candModeList[ 0 ] is greater than candModeList[ 1 ], then two values are swapped. [Ed. (WJ): may be better expression needed]

  - When IntraPredMode[ xB ][ yB ] is equal or greater than candModeList[ 0 ], the value of IntraPredMode[ xB ][ yB ] is increased by one

  - When IntraPredMode[ xB ][ yB ] is equal or greater than candModeList[ 1 ], the value of IntraPredMode[ xB ][ yB ] is increased by one